3GPP TSG-RAN5 Meeting #107	R5-252246

# Malta, Malta, 19th - 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0966 | rev | - | Current version: | 18.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Reference for Isolation Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Reference [4] in H.2 is pointing to wrong reference |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add 37.977 as new reference and reference to it in H.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Can lead to confusion |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, H.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Reference [4] was properly defined in initial discussion paper in R5-197506 but was not corrected in RAN5 CR R5-200322 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< Skip Unchanged Sections >>>

<<< START OF CHANGES >>>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[3]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone"

[4]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

[5]	3GPP TS 38.101-4: "NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements".

[6]	3GPP TS 38.508-1: "5GS; User Equipment (UE) conformance specification; Part 1: Common test environment"

[7]	3GPP TS 38.521-1: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone"

[8]	3GPP TS 38.521-2: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone"

[9]	3GPP TS 38.211: "NR; Physical channels and modulation".

[10]	3GPP TS 38.212: "NR; Multiplexing and channel coding".

[11]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[12]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[13]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity", Stage 2.

[14]	3GPP TS 38.306: "NR; User Equipment (UE) radio access capabilities".

[15]	3GPP TR 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz".

[16]	3GPP TS 36.521-1: "E-UTRA; User Equipment (UE) conformance specification; Radio transmission and reception; Part1: conformance testing"

[17]	3GPP TS 36.211: "Physical Channels and Modulation".

[18]	Recommendation ITU-R M.1545: "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000".

[19]	3GPP TS 36.508: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); Common test environments for User Equipment (UE) conformance testing".

[20]	3GPP TS 38.331: “Radio Resource Control (RRC) protocol specification”.

[21]	3GPP TS 38.521-3: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

[22]	3GPP TS 38.509: "5GS; Special conformance testing functions for User Equipment (UE)".

[23]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[24]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[25]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[26]	3GPP TR 38.903: "NR; Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases".

[27]	3GPP TR 37.977: “Verification of radiated multi-antenna reception performance of User Equipment (UE)”.

<<< Skip Unchanged Sections >>>

## H.2	Wireless cable mode isolation procedure

The following procedure shall be used to verify the wireless cable mode has been established and that the minimum isolation has been achieved

1.	Select any of the three Alignment Options (1, 2, or 3) to mount the DUT inside the QZ.

2.	If the re-positioning concept is applied to demodulation test cases, position the DUT in DUT Orientation 1 if the RX beam peak is within 0o≤≤90o. Otherwise, position the DUT in DUT Orientation 2 (Option 1 or 2). If the re-positioning concept is not applied to demodulation test cases, position the DUT in DUT Orientation 1

3.	Connect the SS (System Simulator) using static propagation conditions with the DUT through the measurement antenna with PolLink= polarization to form the RX beam towards the desired test direction. Allow at least BEAM_SELECT_WAIT_TIME for the UE RX beam selection to complete.

4.	Adjust the DL power of the SS to obtain PDL defined in Table C.0.2-1 at the centre of QZ

5.	Perform the isolation of the branches to achieve the wireless cable mode. The inverse channel matrix approach in [27] is one suitable approach. Alternate approaches are not precluded.

6.	To verify the wireless cable mode and thus the min. isolation between branches

a)	Query SS-RSRPB(PolMeas=PolLink=) from the DUT for the -polarization and convert the two measurements in dBm, i.e., SS-RSRPBB1 and SS-RSRPBB2

b)	Calculate the isolation from -polarization into Branch 1, i.e., ISO,B1 = SS-RSRPBB1 - SS-RSRPBB2 and the isolation into Branch 2, i.e., ISO,B2 = SS-RSRPBB2 - SS-RSRPBB1

c)	Connect the SS (System Simulator) using static propagation conditions with the DUT through the measurement antenna with PolLink= polarization to form the RX beam towards desired test direction. Allow at least BEAM_SELECT_WAIT_TIME for the UE RX beam selection to complete.

d)	Adjust the DL power of the SS to obtain PDL defined in Table C.0.2-1 at the centre of QZ

e)	Query SS-RSRPB(PolMeas=PolLink=) from the DUT for -polarization and convert the two measurements in dBm, i.e., SS-RSRPBB1 and SS-RSRPBB2

f)	Calculate the isolation from -polarization into Branch 2, i.e., ISO,B2 = SS-RSRPBB2 - SS-RSRPBB1 and the isolation into Branch 1, i.e., ISO,B1 = SS-RSRPBB1 - SS-RSRPBB2

If either of the isolations pairs, ISO,B1 and ISO,B2 or ISO,B2 and ISO,B1 exceed 12dB, the wireless cable mode has been achieved.

<<< END OF CHANGES >>>