3GPP TSG-RAN5 Meeting #107 	R5-253593

# St Julian's, Malta, 19th May 2025 – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | 3092 | rev | 1 | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Applicability updates to L2W and L2G reselection cases. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Innovation Center Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - 1. It is optional for UE to support protection against improper reselection to GERAN/UTRAN feature. 2. Applicability needs to be updated for cases impacted by this feature. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - 1. Added PICS for protection against improper reselection to GERAN/UTRAN feature. - 2. Updated applicability for cases impacted by protection against improper reselection to GERAN/UTRAN feature. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - A conformant UE may unfairly fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1, 4.3.1.2, 4.3.1.3,4.3.3,4.4.1,4.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Associated discussion paper R5-253041 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

#### 4.3.1.1	E-UTRA FDD - UTRAN FDD cell reselection: UTRA FDD is of higher priority

##### 4.3.1.1.1	Test purpose

To verify that the UE is able to search and measure neighbouring UTRA FDD cells and compare to the E-UTRA serving cell to meet the inter-RAT cell re-selection requirements when the UTRA is of higher priority.

##### 4.3.1.1.2	Test applicability

This test applies to all types of E-UTRA FDD UE release 8 and forward that support UTRA FDD and not supporting protection against improper reselection to GERAN/UTRAN.

##### 4.3.1.1.3	Minimum conformance requirements

When the measurement rules indicate that UTRA FDD cells are to be measured, the UE shall measure CPICH Ec/Io and CPICH RSCP of detected UTRA FDD cells in the neighbour frequency list at the minimum measurement rate. The parameter NUTRA_carrier is the number of carriers used in the neighbour frequency list. The UE shall filter CPICH Ec/Io and CPICH RSCP measurements of each measured UTRA FDD cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least half the minimum specified measurement period.

If the SServingCell of the E-UTRA serving cell is greater than Snonintrasearch then the UE shall search for inter-RAT layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in 3GPP TS 36.133 [4] clause 4.2.2 as Thigher_priority_search = (60 * Nlayers) seconds, where the parameter Nlayers is the total number of configured higher priority carrier frequencies.

If the SServingCell of the E-UTRA serving cell is less than or equal to Snonintrasearch then the UE shall search for and measure inter-RAT layers of higher or lower priority in preparation for possible re-selection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority inter-RAT layers shall be the same as that defined below for lower priority RATs.

The UE shall evaluate whether newly detectable UTRA FDD cells have met the re-selection criteria in 3GPP TS 36.304 [6] within time NUTRA_carrier * TdetectUTRA_FDD (as defined in table 4.2.2.5.1-1 of 3GPP TS 36.133 [4] clause 4.2.2.5.1) when the SServingCell of the E-UTRA serving cell is less than Snonintrasearch when TreselectionRAT = 0 provided that the re-selection criteria is met by a margin of at least 6 dB.

Cells which have been detected shall be measured at least every NUTRA_carrier * TmeasureUTRA_FDD when the SServingCell of the E-UTRA serving cell is less than Snonintrasearch.

When higher priority UTRA FDD cells are found by the higher priority search, they shall be measured at least every TmeasureUTRA_FDD. If re-selection to any higher priority cell is not triggered within (TevaluateUTRA_FDD + TreselectionRAT) after it is found in a higher priority search, the UE is not required to continue making measurements of the cell to evaluate the ongoing possibility of re-selection.

For a cell that has been already detected, but that has not been re-selected to, the filtering shall be such that the UE shall be capable of evaluating that an already identified UTRA FDD cell has met re-selection criterion defined in 3GPP TS 36.304 [6] within NUTRA_carrier * TevaluateUTRA_FDD when TreselectionRAT = 0 as specified in table 4.2.2.5.1-1 of 3GPP TS 36.133 [4] clause 4.2.2.5.1 provided that the reselection criteria is met by a margin of at least 6dB.

The normative reference for this requirement is TS 36.133 [4] clause 4.2.2.5.1 and A.4.3.1.1.

##### 4.3.1.1.4	Test description

4.3.1.1.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 4.1.

Frequencies to be tested: According to Annex E table E-1 and 3GPP TS 36.508 [7] clauses 4.4.2 and 4.3.1.

Channel Bandwidth to be tested: 10MHz as defined in 3GPP TS 36.508 [7] clause 4.3.1.

1.	Connect the SS (node B emulator) and AWGN noise source to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A Figure A.22 for UE with 2Rx RF band and Annex A, Figure TBD for 4Rx capable UE without any 2Rx RF bands.

2.	The general test parameter settings are set up according to Table 4.3.1.1.4.1-1.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	Message contents are as defined in clause 4.3.1.1.4.3.

5.	There is one E-UTRA FDD cell and one UTRA FDD cell specified in the test. Cell 1 is the cell used for registration according to 3GPP TS 36.508 [7] clause 7.2A.2 with the power level set according to Annex C.0 and C.1.

Table 4.3.1.1.4.1-1: General test parameters for E-UTRA FDD- higher priority UTRA FDD inter RAT cell re-selection test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell 1 | UE is on cell 1 in the initialisation phase, so that reselection to cell 2 occurs during T2 |
| T2 end condition | Active cell |  | Cell 2 | UE shall perform reselection to cell 2 during T2 |
|  | Neighbour cell |  | Cell 1 |  |
| T3 end condition | Active cell |  | Cell 1 | UE shall perform reselection to cell 1 during T3 |
|  | Neighbour cell |  | Cell 2 |  |
| E-UTRA PRACH configuration |  |  | 4 | As specified in table 5.7.1-2 in 3GPP TS 36.211 [9] |
| E_UTRA Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| T1 |  | s | >20 | During T1, cell 2 shall be powered off, and during the off time the primary scrambling code shall be changed, The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T2. |
| T2 |  | s | 85 | T2 needs to be defined so that cell re-selection reaction time is taken into account |
| T3 |  | s | 25 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

4.3.1.1.4.2	Test procedure

The test consists of one active cell and one neighbour cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. E-UTRA cell 1 is already identified by the UE prior to the start of the test. Before T1 the UE is camped on to cell 1. During T1, cell 2 shall be powered off, and during the off time the scrambling code shall be changed. At the start of T2 cell 2 becomes stronger than Threshx_high, the UE is expected to detect cell 2, send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Routing Area Update procedure on cell 2. At the start of T3 cell 2 becomes weaker than Threshserving_low, and the UE reselects to Cell 1.

In the following test procedure "UE responds" means "UE starts transmitting preamble on PRACH for sending the RRC CONNECTION REQUEST message to perform a Routing Area Update procedure".

1.	Ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2.

2.	Set the parameters according to duration T1 in Table 4.3.1.1.5-1 and 4.3.1.1.5-2. Propagation conditions are set according to Annex B clause B.1.1. T1 starts.

3.	During T1, Cell 2 shall be powered off and the SS shall set Cell 2 primary scrambling code = ((current cell 2 primary scrambling code - 50) mod 200 + 100).

4.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 4.3.1.1.5-1 and 4.3.1.1.5-2. The SS waits for random access requests information from the UE to perform cell re-selection procedure on the higher priority cell, Cell 2.

5.	If the UE responds on Cell 2 within 81s from the beginning of time period T2 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

6.	If the UE has re-selected Cell 2 within T2, after the re-selection or when T2 expires, continue with step 7.
Otherwise, if T2 expires and the UE has not yet re-selected Cell 2, skip to step 10.

7.	The SS shall switch the power setting from T2 to T3 as specified in Table 4.3.1.1.5-1 and 4.3.1.1.5-2.

8.	The SS waits for random access requests information from the UE to perform cell re-selection procedure on the lower priority cell, Cell 1.

9.	If the UE has re-selected Cell 1 within T3, after the re-selection or when T3 expires, skip to step 11.
Otherwise, if T3 expires and the UE has not yet re-selected Cell 1, continue with step 10.

10.	Switch off and on the UE and ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2 in Cell 1.

11.	Repeat step 2-10 until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved.

4.3.1.1.4.3	Message contents

Message contents are according to 3GPP TS 36.508 [7] clause 4.6 and 3GPP TS 34.108 [24] clause 6.1.0b with the following exceptions:

Table 4.3.1.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.3-1 Table H.2.3-2 Table H.2.3-13 |
| Default RRC messages and information elements contents exceptions | Table H.3.2-1 |

Table 4.3.1.1.4.3-2: System Information Block type 19:
Inter-RAT E-UTRAN FDD - UTRA FDD is of higher priority cell re-selection

| Derivation Path: 36.508 [7] clause 4.4.4.1 Table 4.4.4.1-1: System Information Block type 19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| priority | 5 |  |  |
| s-PrioritySearch1 | 31 (62dB) |  |  |
| s-PrioritySearch2 | 0 | default value is 0 |  |
| threshServingLow | 18 (36dB) |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE |  | n denotes the index of the entry |  |
| earfcn |  | Downlink EARFCN of Cell 1 |  |
| measurementBandwidth | 50 | Enumerated(6, 15, 25, 50, 75, 100) |  |
| priority | 4 |  |  |
| qRxLevMinEUTRA | -70 (-140 dBm) |  |  |
| threshXhigh | 2 (4 dB) |  |  |
| threshXlow | 25 (50 dB) |  |  |
| eutra-excludeListedCellList | Not present |  |  |
| eutraDetection | TRUE |  |  |
| } |  |  |  |
| nonCriticalExtensions SEQUENCE {} | Not present |  |  |
| } |  |  |  |

##### 4.3.1.1.5	Test requirement

Tables 4.3.1.1.4-1, 4.3.1.1.5-1 and 4.3.1.1.5-2 define the primary level settings including test tolerances for inter-RAT cell re-selection E-UTRA FDD to UTRA FDD test case (UTRA is of higher priority).

Table 4.3.1.1.5-1: Cell specific Test Parameters for Cell 1(E-UTRA FDD)

| Parameter | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| OCNG Patterns defined in D.1.2 (OP.2 FDD) |  | OP.2 FDD |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PCFICH_RB | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDCCH_RA | dB |  |  |  |
| PDCCH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
| Qqualmin for UTRA neighbour cell | dB | -20 |  |  |
| Qrxlevmin for UTRA neighbour cell | dBm | -115 |  |  |
| Qrxlevmin | dBm | -140 |  |  |
|  | dBm/15 kHz | -98 |  |  |
| RSRP | dBm/15 KHz | -82.95 | -82.95 | -82.954 + TT1.3, is to be specified per individual test case |
|  | dB | 15.05 | 15.05 | 15.05 |
|  | dB | 15.05 | 15.05 | 15.05 |
| TreselectionEUTRAN | s | 0 |  |  |
| Snonintrasearch | dB | 50 |  |  |
| Threshx, high (Note 2) | dB | 40 |  |  |
| Propagation Condition |  | AWGN |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: This refers to the value of Threshx, high which is included in E-UTRA system information, and is a threshold for the UTRA target cell. |  |  |  |  |

Table 4.3.1.1.5-2: Cell specific test parameters for Cell 2 (higher priority UTRA FDD)

| Parameter | Unit | Cell 2 (UTRA) |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| UTRA RF Channel Number |  | Channel 2 |  |  |
| CPICH_Ec/Ior | dB | -10 |  |  |
| PCCPCH_Ec/Ior | dB | -12 |  |  |
| SCH_Ec/Ior | dB | -12 |  |  |
| PICH_Ec/Ior | dB | -15 |  |  |
| OCNS_Ec/Ior | dB | -0.941 |  |  |
|  | dB | -∞ | 11.90 | -5.70 |
|  | dBm/3,84 MHz | -70.10 |  |  |
| CPICH_Ec/Io | dB | -∞ | -10.27 | -16.74 |
| CPICH_RSCP | dBm | -∞ | -68.20 | -85.80 |
| Propagation Condition |  | AWGN |  |  |
| Qqualmin | dB | -20 |  |  |
| Qrxlevmin | dBm | -115 |  |  |
| QrxlevminEUTRA | dBm | -140 |  |  |
| UE_TXPWR_MAX_RACH | dBm | 21 |  |  |
| Treselection | s | 0 |  |  |
| Sprioritysearch1 | dB | 62 |  |  |
| Sprioritysearch2 | dB | 0 |  |  |
| Threshserving, low | dB | 36 |  |  |
| Threshx, low (Note) | dB | 50 |  |  |
| NOTE: This refers to the value of Threshx, low which is included in UTRA system information, and is a threshold for the E-UTRA target cell. |  |  |  |  |

The cell reselection delay to higher priority is defined as the time from the beginning of time period T2, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message on cell 2.

The cell re-selection delay to higher priority shall be less than 81 s.

NOTE:	The cell re-selection delay to higher priority cell can be expressed as: Thigher_priority_search +TevaluateUTRA_FDD + TSI-UTRA

Where:

Thigher_priority_search 	See section 4.2.2 of 3GPP TS 36.133 [4]; 60s is assumed in this test case

TevaluateUTRA-FDD	See Table 4.2.2.5.1-1 of 3GPP TS 36.133 [4] clause 4.2.2.5.1

TSI-UTRA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 80.48 s for higher priority cell search, allow 81 s for higher priority cell reselection in the test case.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 4.3.1.2	E-UTRAN FDD - UTRAN FDD cell re-selection: UTRA FDD is of lower priority

##### 4.3.1.2.1	Test purpose

To verify that the UE is able to search and measure neighbouring UTRA FDD cells and compare to the E-UTRA serving cell to meet the inter-RAT cell re-selection requirements when the UTRA is of lower priority.

##### 4.3.1.2.2	Test applicability

This test applies to all types of E-UTRA FDD UE release 8 and forward that support UTRA and not supporting protection against improper reselection to GERAN/UTRAN.

##### 4.3.1.2.3	Minimum conformance requirements

When the measurement rules indicate that UTRA FDD cells are to be measured, the UE shall measure CPICH Ec/Io and CPICH RSCP of detected UTRA FDD cells in the neighbour frequency list at the minimum measurement rate. The parameter NUTRA_carrier_FDD is the number of carriers used in the neighbour frequency list. The UE shall filter CPICH Ec/Io and CPICH RSCP measurements of each measured UTRA FDD cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least half the minimum specified measurement period.

If the SServingCell of the E-UTRA serving cell is greater than Snonintrasearchthen the UE shall search for inter-RAT layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in 3GPP TS 36.133 [4] clause 4.2.2 as Thigher_priority_search = (60 * Nlayers) seconds, where the parameter Nlayers is the total number of configured higher priority carrier frequencies.

If the SServingCell of the E-UTRA serving cell is less than or equal to Snonintrasearch then the UE shall search for and measure inter-RAT layers of higher or lower priority in preparation for possible re-selection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority inter-RAT layers shall be the same as that defined below for lower priority RATs.

The UE shall evaluate whether newly detectable UTRA FDD cells have met the re-selection criteria in 3GPP TS 36.304 [6] within time NUTRA_carrier_FDD * TdetectUTRA_FDD (as defined in table 4.2.2.5.1-1 of 3GPP TS 36.133 [4] clause 4.2.2.5.1) when the SServingCell of the E-UTRA serving cell is less than Snonintrasearch when TreselectionRAT = 0 provided that the re-selection criteria is met by a margin of at least 6 dB.

Cells which have been detected shall be measured at least every NUTRA_carrier * TmeasureUTRA_FDD when the SServingCell of the E-UTRA serving cell is less than Snonintrasearch.

When higher priority UTRA FDD cells are found by the higher priority search, they shall be measured at least every TmeasureUTRA_FDD. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of re-selection. However, the minimum measurement filtering requirements specified later in this section shall still be met by the UE before it makes any determination that it may stop measuring the cell.

For a cell that has been already detected, but that has not been re-selected to, the filtering shall be such that the UE shall be capable of evaluating that an already identified UTRA FDD cell has met re-selection criterion defined in 3GPP TS 36.304 [6] within NUTRA_carrier * TevaluateUTRA_FDD when TreselectionRAT = 0 as specified in table 4.2.2.5.1-1 of 3GPP TS 36.133 [4] clause 4.2.2.5.1 provided that the reselection criteria is met by a margin of at least 6dB.

The normative reference for this requirement is TS 36.133 [4] clause 4.2.2.5.1 and A.4.3.1.2.

##### 4.3.1.2.4	Test description

4.3.1.2.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 4.1.

Frequencies to be tested: According to Annex E table E-1 and 3GPP TS 36.508 [7] clauses 4.4.2 and 4.3.1.

Channel Bandwidth to be tested: 10MHz as defined in 3GPP TS 36.508 [7] clause 4.3.1.

1.	Connect the SS (node B emulator) and AWGN noise source to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A Figure A.22 for UE with 2Rx RF band and Annex A, Figure TBD for 4Rx capable UE without any 2Rx RF bands..

2.	The general test parameter settings are set up according to Table 4.3.1.2.4.1-1.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	Message contents are as defined in clause 4.3.1.2.4.3.

5.	There is one E-UTRA FDD cell and one UTRA FDD cell specified in the test. Cell 1 is the cell used for registration according to 3GPP TS 36.508 [7] clause 7.2A.2 with the power level set according to Annex C.0 and C.1.

Table 4.3.1.2.4.1-1: General test parameters for EUTRA FDD -
lower priority UTRA FDD inter RAT cell re-selection test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell1 | E-UTRAN cell |
| T1 end condition | Active cells |  | Cell1 | UE shall perform reselection to cell 1 during T1 for subsequent iterations of the test |
|  | Neighbour cell |  | Cell2 |  |
| T2 end condition | Active cell |  | Cell2 | UE shall perform reselection to cell 2 during T2 |
|  | Neighbour cell |  | Cell1 |  |
| E-UTRA PRACH configuration |  |  | 4 | As specified in table 5.7.1-2 in 3GPP TS 36.211 [9] |
| E_UTRA Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| T1 |  | s | 85 | T1 need to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 25 | T2 need to be defined so that cell re-selection reaction time is taken into account. |

4.3.1.2.4.2	Test procedure

The test consists of one active cell and one neighbour cell. The UE is requested to monitor the neighbouring cell on one UTRA FDD carrier. In the test there are two successive time periods, with time duration of T1 and T2 respectively. Both Cell 1 and Cell 2 are already identified by the UE prior to the start of the test. At T1 the UE is camped on to Cell 1. Cell 2 is of lower priority than Cell 1. Cell 1 and Cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing Cell 2.

In the following test procedure "UE responds" means "UE starts transmitting preamble on PRACH for sending the RRC CONNECTION REQUEST message to perform a Routing Area Update procedure".

1.	Ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2.

2.	Set the parameters according to T1 in Table 4.3.1.2.5-1 and 4.3.1.2.5-2. Propagation conditions are set according to Annex B clause B.1.1. T1 starts. If the UE is already camped in Cell 1, wait until T1expires and skip to step 5.

3.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 1.

4.	If the UE has re-selected Cell 1 within T1, after the re-selection or when T1 expires, continue with step 5.
Otherwise, if T1 expires and the UE has not yet re-selected Cell 1, skip to step 9.

5.	The SS shall switch the power setting from T1 to T2 as specified in Table 4.3.1.2.5-1 and 4.3.1.2.5-2.

6.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 2.

7.	If the UE responds on Cell 2 during time duration T2 within 21 seconds from the beginning of time period T2 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

8.	If the UE has re-selected Cell 2 within T2, after the re-selection or when T2 expires, skip to step 10.
Otherwise, if T2 expires and the UE has not yet re-selected Cell 2, continue with step 9.

9.	Switch off and on the UE and ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2 in Cell 1.

10.	Repeat step 2-9 until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved.

4.3.1.2.4.3	Message contents

Message contents are according to 3GPP TS 36.508 [7] clause 4.6 and 3GPP TS 34.108 [24] clause 6.1.0b with the following exceptions:

Table 4.3.1.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.3-5 Table H.2.3-6 Table H.2.3-13 |
| Default RRC messages and information elements contents exceptions | Table H.3.2-1 |

Table 4.3.1.2.4.3-2: System Information Block type 19: Inter-RAT E-UTRAN FDD - UTRA FDD is of lower priority cell re-selection

| Derivation Path: 36.508 [7] clause 4.4.4.1 Table 4.4.4.1-1: System Information Block type 19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| priority | 3 |  |  |
| s-PrioritySearch1 | 21 (42dB) |  |  |
| s-PrioritySearch2 | 0 | default value is 0 |  |
| threshServingLow | 0 (0dB) |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE |  | n denotes the index of the entry |  |
| earfcn |  | Downlink EARFCN of Cell 1 |  |
| measurementBandwidth | 50 | Enumerated(6, 15, 25, 50, 75, 100) |  |
| priority | 4 |  |  |
| qRxLevMinEUTRA | -70 (-140 dBm) |  |  |
| threshXhigh | 24 (48 dB) |  |  |
| threshXlow | 1 (2 dB) | Default value |  |
| eutra-excludeListedCellList | Not present |  |  |
| eutraDetection | TRUE |  |  |
| } |  |  |  |
| nonCriticalExtensions SEQUENCE {} | Not present |  |  |
| } |  |  |  |

##### 4.3.1.2.5	Test requirement

Tables 4.3.1.2.4.1-1, 4.3.1.2.5-1 and 4.3.1.2.5-2 define the primary level settings including test tolerances for E-UTRAN FDD- UTRAN FDD inter-RAT cell re-selection test case which UTRA is of lower priority.

Table 4.3.1.2.5-1: Cell specific test parameters for cell 1 (E-UTRA)

| Parameter | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 |  | T2 |
| E-UTRA RF Channel number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| OCNG Patterns defined in D.1.2 (OP.2 FDD) |  | OP.2 FDD |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PCFICH_RB | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDCCH_RA | dB |  |  |  |
| PDCCH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
| Qqualmin for UTRA neighbour cell | dB | -20 |  |  |
| Qrxlevmin for UTRA neighbour cell | dBm | -115 |  |  |
| Qrxlevmin | dBm | -140 |  |  |
|  | dBm/15 kHz | -99.35 |  |  |
| RSRP | dBm/15 KHz | -84.95 | -103.05 |  |
|  | dB | 14.40 | -3.70 |  |
|  | dB | 14.40 | -3.70 |  |
| TreselectionEUTRAN | s | 0 |  |  |
| Snonintrasearch | dB | Not sent |  |  |
| Threshserving, low | dB | 44 |  |  |
| Threshx, low (Note 2) | dB | 42 |  |  |
| Propagation Condition |  | AWGN |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: This refers to the value of Threshx, low which is included in E-UTRA system information, and is a threshold for the UTRA target cell. |  |  |  |  |

Table 4.3.1.2.5-2: Cell specific test parameters for Cell 2(Lower priority UTRA FDD)

| Parameter | Unit | Cell 2 (UTRA) |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| UTRA RF Channel Number |  | Channel 2 |  |
| CPICH_Ec/Ior | dB | -10 |  |
| PCCPCH_Ec/Ior | dB | -12 |  |
| SCH_Ec/Ior | dB | -12 |  |
| PICH_Ec/Ior | dB | -15 |  |
| OCNS_Ec/Ior | dB | -0.941 |  |
|  | dB | 13.80 | 13.80 |
|  | dBm/3,84 MHz | -70 |  |
| CPICH_Ec/Io | dB | -10.18 | -10.18 |
| CPICH_RSCP | dBm | -66.20 | -66.20 |
| Propagation Condition |  | AWGN |  |
| Qqualmin | dB | -20 |  |
| Qrxlevmin | dBm | -115 |  |
| QrxlevminEUTRA | dBm | -140 |  |
| UE_TXPWR_MAX_RACH | dBm | 21 |  |
| Treselection | s | 0 |  |
| Sprioritysearch1 | dB | 42 |  |
| Sprioritysearch2 | dB | 0 |  |
| Threshx, high (Note) | dB | 48 |  |
| NOTE: This refers to the value of Threshx, high which is included in UTRA system information, and is a threshold for the E-UTRA target cell. |  |  |  |

The cell reselection delay to lower priority is defined as the time from the beginning of time period T2, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message on cell 2.

The cell re-selection delay to lower priority shall be less than 21 s.

NOTE:	The cell re-selection delay to lower priority cell can be expressed as: TevaluateUTRA_FDD + TSI-UTRA

Where:

TevaluateUTRA-FDD	See Table 4.2.2.5.1-1 of 3GPP TS 36.133 [4] clause 4.2.2.5.1

TSI-UTRA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 20.48 s for lower priority cell reselection, allow 21 s.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 4.3.1.3	E-UTRAN FDD - UTRAN FDD cell re-selection in fading propagation conditions: UTRA FDD is of lower priority

##### 4.3.1.3.1	Test purpose

To verify that the UE is able to search and measure neighbouring UTRA FDD cells and compare to the E-UTRA serving cell to meet the inter-RAT cell re-selection requirements when the UTRA is of lower priority. The E-UTRA cell is in fading propagation conditions and the UTRA cell is in AWGN propagation conditions.

##### 4.3.1.3.2	Test applicability

This test applies to all types of E-UTRA FDD UE release 8 and forward that support UTRA FDD and not supporting protection against improper reselection to GERAN/UTRAN.

##### 4.3.1.3.3	Minimum conformance requirements

When the measurement rules indicate that UTRA FDD cells are to be measured, the UE shall measure CPICH Ec/Io and CPICH RSCP of detected UTRA FDD cells in the neighbour frequency list at the minimum measurement rate. The parameter NUTRA_carrier_FDD is the number of carriers used in the neighbour frequency list. The UE shall filter CPICH Ec/Io and CPICH RSCP measurements of each measured UTRA FDD cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least half the minimum specified measurement period.

If the SServingCell of the E-UTRA serving cell is greater than Snonintrasearch then the UE shall search for inter-RAT layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in 3GPP TS 36.133 [4] clause 4.2.2 as Thigher_priority_search = (60 * Nlayers) seconds, where the parameters Nlayers is the total number of configured higher priority carrier frequencies.

If the SServingCell of the E-UTRA serving cell is less than or equal to Snonintrasearch then the UE shall search for and measure inter-RAT layers of higher or lower priority in preparation for possible re-selection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority inter-RAT layers shall be the same as that defined below for lower priority RATs.

The UE shall evaluate whether newly detectable UTRA FDD cells have met the re-selection criteria in 3GPP TS 36.304 [6] within time NUTRA_carrier_FDD * TdetectUTRA_FDD (as defined in table 4.2.2.5.1-1 of 3GPP TS 36.133 [4] clause 4.2.2.5.1) when the SServingCell of the E-UTRA serving cell is greater than Snonintrasearch when TreselectionRAT = 0 provided that the re-selection criteria is met by a margin of at least 6 dB.

Cells which have been detected shall be measured at least every NUTRA_carrier * TmeasureUTRA_FDD when the SServingCell of the E-UTRA serving cell is less than Snonintrasearch.

When higher priority UTRA FDD cells are found by the higher priority search, they shall be measured at least every TmeasureUTRA_FDD. If, after it detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of re-selection. However, the minimum measurement filtering requirements specified later in this section shall still be met by the UE before it makes any determination that it may stop measuring the cell.

For a cell that has been already detected, but that has not been re-selected to, the filtering shall be such that the UE shall be capable of evaluating that an already identified UTRA FDD cell has met re-selection criterion defined in 3GPP TS 36.304 [6] within NUTRA_carrier * TevaluateUTRA_FDD when TreselectionRAT = 0 as specified in table 4.2.2.5.1-1 of 3GPP TS 36.133 [4] clause 4.2.2.5.1 provided that the reselection criteria is met by a margin of at least 6dB.

The normative reference for this requirement is TS 36.133 [4] clause 4.2.2.5.1 and A.4.3.1.3.

##### 4.3.1.3.4	Test description

4.3.1.3.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 4.1.

Frequencies to be tested: According to Annex E table E-1 and 3GPP TS 36.508 [7] clauses 4.4.2 and 4.3.1.

Channel Bandwidth to be tested: 10MHz as defined in 3GPP TS 36.508 [7] clause 4.3.1.

1.	Connect the SS (node B emulator) and faders and AWGN noise sources to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A figure A.26 for UE with 2Rx RF band and Annex A, Figure TBD for 4Rx capable UE without any 2Rx RF bands.

2.	The general test parameter settings are set up according to Table 4.3.1.3.4.1-1.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	Message contents are as defined in clause 4.3.1.3.4.3.

5.	There is one E-UTRA FDD cell and one UTRA FDD cell specified in the test. Cell 1 is the cell used for registration according to 3GPP TS 36.508 [7] clause 7.2A.2 with the power level set according to Annex C.0 and C.1.

Table 4.3.1.3.4.1-1: General test parameters for EUTRA FDD- 
lower priority UTRA FDD inter RAT cell re-selection test case in fading conditions

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell1 | E-UTRAN Cell. |
| T1 end condition | Active cells |  | Cell1 | UE shall perform reselection to cell 1 during T1 for subsequent iterations of the test. |
|  | Neighbour cell |  | Cell2 |  |
| T3 end condition | Active cell |  | Cell2 | UE shall perform reselection to cell 2 during T3 |
|  | Neighbour cell |  | Cell1 |  |
| E-UTRA PRACH configuration |  |  | 4 | As specified in table 5.7.1-2 in 3GPP TS 36.211 [9] |
| E_UTRA Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| T1 |  | s | <85 | T1 need to be defined so that cell re-selection reaction time is taken into account. T1 is terminated when the UE starts to send preambles to cell 1 |
| T2 |  | s | 64 | The start of T2 is defined as the time when the UE starts to send PRACH preambles to cell 1 |
| T3 |  | s | <25 | T3 need to be defined so that cell re-selection reaction time is taken into account. T3 is terminated when the UE starts to send preambles to cell 2 |
| T4 |  | s | 64 | The start of T4 is defined as the time when the UE starts to send PRACH preambles to cell 2 |

4.3.1.3.4.2	Test procedure

The test consists of one active cell and one neighbour cell. The UE is requested to monitor the neighbouring cell on one UTRA FDD carrier. In the test there are four successive time periods, with time duration of T1, T2, T3 and T4 respectively. Both Cell 1 and Cell 2 are already identified by the UE prior to the start of the test. At T1 the UE is camped on to Cell 1. Cell 2 is of lower priority than Cell 1. Cell 1 and Cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing Cell 2. Time duration T2 and T4 are not used for cell re-selection in the test.

In the following test procedure, "UE responds on Cell 1" means "UE starts transmitting preamble on PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure according to 3GPP TS 36.508 [7] clause 4.5A.2" and "UE responds on Cell 2" means "UE starts transmitting preamble on PRACH for sending the RRC CONNECTION REQUEST message to perform a Routing Area Update procedure".

1.	Ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2.

2.	Set the parameters according to duration T0 in Table 4.3.1.3.5-1 and 4.3.1.3.5-2.

3.	Set the parameters according to T2 in Table 4.3.1.3.5-3 and 4.3.1.3.5-4. Propagation conditions are set according to Annex B clauses B.1.1 and B.2.2. T2 starts.

4.	The SS monitors for possible random access request information from the UE to perform cell re-selection on Cell 2.

5.	If the UE responds on Cell 2 within T2, then count a fail for the event "Hold out on Cell 1" and skip to step 10.
Otherwise, count a success for the event "Hold out on Cell 1" and after T2 expires continue with step 6.

6.	The SS shall switch the power setting from T2 to T3 as specified in Table 4.3.1.3.5-3 and 4.3.1.3.5-4.

7.	The SS waits for random access request information from the UE to perform cell re-selection on Cell 2.

8.	If the UE responds on Cell 2 during time duration T3 within 21 seconds from the beginning of time period T3, then count a success for the event "Re-select lower priority Cell 2".
Otherwise, count a fail for the event "Re-select lower priority Cell 2".

9.	If the UE responds on Cell 2 within T3, at the moment of the request-reception continue with step 10.
Otherwise, after T3 expires skip to step 17.

10.	The SS shall switch the power setting from T3 to T4 as specified in Table 4.3.1.3.5-3 and 4.3.1.3.5-4.

11.	The SS monitors for possible random access request information from the UE to perform cell re-selection on Cell 1.

12.	If the UE responds on Cell 1 within T4, then count a fail for the event "Hold out on Cell 2" and skip to step 17.
Otherwise, count a success for the event "Hold out on Cell 2" and after T4 expires continue with step 13.

13.	The SS shall switch the power setting from T4 to T1 as specified in Table 4.3.1.3.5-3 and 4.3.1.3.5-4.

14.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 1.

15.	If the UE responds on Cell 1 within T1, at the moment of the request-reception skip to step 17.
Otherwise, after T1 expires continue with step 16.

16.	Switch off and on the UE and ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2 in Cell 1. Set the parameters according to duration T0 in Table 4.3.1.3.5-1 and 4.3.1.3.5-2.

17.	Repeat step 3-16 until a test verdict has been achieved.
Each of the events "Hold out on Cell 1", "Re-select lower priority Cell 2" and "Hold out on Cell 2" is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict.
If both events pass, the test passes. If one event fails, the test fails.

4.3.1.3.4.3	Message contents

Message contents are according to 3GPP TS 36.508 [7] clause 4.6 and 3GPP TS 34.108 [24] clause 6.1.0b with the following exceptions:

Table 4.3.1.3.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.3-5 Table H.2.3-6 Table H.2.3-13 |
| Default RRC messages and information elements contents exceptions | Table H.3.2-1 |

Table 4.3.1.3.4.3-2: System Information Block type 19:
Inter-RAT E-UTRAN FDD - UTRA FDD is of lower priority cell re-selection in fading conditions

| Derivation Path: 36.508 [7] clause 4.4.4.1 Table 4.4.4.1-1: System Information Block type 19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| priority | 3 |  |  |
| s-PrioritySearch1 | 21 (42dB) |  |  |
| s-PrioritySearch2 | 0 | default value is 0 |  |
| threshServingLow | 0 (0dB) |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE |  | n denotes the index of the entry |  |
| earfcn |  | Downlink EARFCN of Cell 1 |  |
| measurementBandwidth | 50 | Enumerated(6, 15, 25, 50, 75, 100) |  |
| priority | 4 |  |  |
| qRxLevMinEUTRA | -70 (-140 dBm) |  |  |
| threshXhigh | 22 (44 dB) |  |  |
| threshXlow | 1 (2 dB) | Default value |  |
| eutra-excludeListedCellList | Not present |  |  |
| eutraDetection | TRUE |  |  |
| } |  |  |  |
| nonCriticalExtensions SEQUENCE {} | Not present |  |  |
| } |  |  |  |

##### 4.3.1.3.5	Test requirement

Tables 4.3.1.3.4.1-1, 4.3.1.3.5-1, 4.3.1.3.5-2, 4.3.1.3.5-3 and 4.3.1.3.5-4 define the primary level settings including test tolerances for E-UTRAN FDD- UTRAN FDD inter-RAT cell re-selection test case which UTRA is of lower priority. Note that the time period for T0 is system implementation dependent.

Table 4.3.1.3.5-1: Cell specific test parameters for Cell 1 (Lower priority E-UTRA FDD)

| Parameter | Unit | Cell 1 |
| --- | --- | --- |
|  |  | T0 |
| E-UTRA RF Channel number |  | 1 |
| BWchannel | MHz | 10 |
| Antenna Configuration |  | 1x2 |
| OCNG Patterns defined in D.1.2 (OP.2 FDD) |  | OP.2 FDD |
| PSS_RA | dB | 0 |
| SSS_RA | dB | 0 |
| PCFICH_RB | dB | 0 |
| PHICH_RA | dB | 0 |
| PHICH_RB | dB | 0 |
| PDCCH_RA | dB | 0 |
| PDCCH_RB | dB | 0 |
| PDSCH_RA | dB | 0 |
| PDSCH_RB | dB | 0 |
| OCNG_RANote 1 | dB | 0 |
| OCNG_RBNote 1 | dB | 0 |
| Qqualmin for UTRA neighbour cell | dB | -20 |
| Qrxlevmin for UTRA neighbour cell | dBm | -115 |
| Qrxlevmin | dBm | -140 |
|  | dBm/15 kHz | -104 |
| RSRP | dBm/15 KHz | -82 |
|  | dB | 22 |
|  | dB | 22 |
| TreselectionEUTRAN | s | 0 |
| Snonintrasearch | dB | Not sent |
| Threshserving, low | dB | 44 |
| Threshx, low (Note 2) | dB | 42 |
| Propagation Condition |  | AWGN |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: This refers to the value of Threshx, low which is included in E-UTRA system information, and is a threshold for the UTRA target cell. |  |  |

Table 4.3.1.3.5-2: Cell specific test parameters for Cell 2 (Lower priority UTRA FDD)

| Parameter | Unit | Cell 2 (UTRA) |
| --- | --- | --- |
|  |  | T0 |
| UTRA RF Channel Number |  | Channel 2 |
| CPICH_Ec/Ior | dB | -10 |
| PCCPCH_Ec/Ior | dB | -12 |
| SCH_Ec/Ior | dB | -12 |
| PICH_Ec/Ior | dB | -15 |
| OCNS_Ec/Ior | dB | -0.941 |
|  | dB | 13.80 |
|  | dBm/3,84 MHz | -70 |
| CPICH_Ec/Io | dB | -10.18 |
| CPICH_RSCP | dBm | -66.20 |
| Propagation Condition |  | AWGN |
| Qqualmin | dB | -20 |
| Qrxlevmin | dBm | -115 |
| QrxlevminEUTRA | dBm | -140 |
| UE_TXPWR_MAX_RACH | dBm | 21 |
| Treselection | s | 0 |
| Sprioritysearch1 | dB | 42 |
| Sprioritysearch2 | dB | 0 |
| Threshx, high (Note) | dB | 44 |
| NOTE: This refers to the value of Threshx, high which is included in UTRA system information, and is a threshold for the E-UTRA target cell |  |  |

Table 4.3.1.3.5-3: Cell specific test parameters for cell 1 (E-UTRA)

| Parameter | Unit | Cell 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 |  | T2 |  | T3 |  | T4 |  |  |
| E-UTRA RF Channel number |  | 1 |  |  |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  |  |  |  |  |
| OCNG Patterns defined in D.1.2 (OP.2 FDD) |  | OP.2 FDD |  |  |  |  |  |  |
| PSS_RA | dB | 0 |  |  |  |  |  |  |
| SSS_RA | dB | 0 |  |  |  |  |  |  |
| PCFICH_RB | dB | 0 |  |  |  |  |  |  |
| PHICH_RA | dB | 0 |  |  |  |  |  |  |
| PHICH_RB | dB | 0 |  |  |  |  |  |  |
| PDCCH_RA | dB | 0 |  |  |  |  |  |  |
| PDCCH_RB | dB | 0 |  |  |  |  |  |  |
| PDSCH_RA | dB | 0 |  |  |  |  |  |  |
| PDSCH_RB | dB | 0 |  |  |  |  |  |  |
| OCNG_RANote 1 | dB | 0 |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB | 0 |  |  |  |  |  |  |
| Qqualmin for UTRA neighbour cell | dB | -20 |  |  |  |  |  |  |
| Qrxlevmin for UTRA neighbour cell | dBm | -115 |  |  |  |  |  |  |
| Qrxlevmin | dBm | -140 |  |  |  |  |  |  |
|  | dBm/15 kHz | -104 |  |  |  |  |  |  |
| RSRP | dBm/15 KHz | -82 | -82 |  | -107 |  | -107 |  |
|  | dB | 22 | 22 |  | -3 |  | -3 |  |
|  | dB | 22 | 22 |  | -3 |  | -3 |  |
| TreselectionEUTRAN | s | 0 |  |  |  |  |  |  |
| Snonintrasearch | dB | Not sent |  |  |  |  |  |  |
| Threshserving, low | dB | 44 |  |  |  |  |  |  |
| Threshx, low (Note 2) | dB | 42 |  |  |  |  |  |  |
| Propagation Condition |  | ETU70 |  |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: This refers to the value of Threshx, low which is included in E-UTRA system information, and is a threshold for the UTRA target cell. |  |  |  |  |  |  |  |  |

Table 4.3.1.3.5-4: Cell specific test parameters for Cell 2 (Lower priority UTRA FDD)

| Parameter | Unit | Cell 2 (UTRA) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 |
| UTRA RF Channel Number |  | Channel 2 |  |  |  |
| CPICH_Ec/Ior | dB | -10 |  |  |  |
| PCCPCH_Ec/Ior | dB | -12 |  |  |  |
| SCH_Ec/Ior | dB | -12 |  |  |  |
| PICH_Ec/Ior | dB | -15 |  |  |  |
| OCNS_Ec/Ior | dB | -0.941 |  |  |  |
|  | dB | 13.80 | 13.80 | 13.80 | 13.80 |
|  | dBm/3,84 MHz | -70 |  |  |  |
| CPICH_Ec/Io | dB | -10.18 | -10.18 | -10.18 | -10.18 |
| CPICH_RSCP | dBm | -66.20 | -66.20 | -66.20 | -66.20 |
| Propagation Condition |  | AWGN |  |  |  |
| Qqualmin | dB | -20 |  |  |  |
| Qrxlevmin | dBm | -115 |  |  |  |
| QrxlevminEUTRA | dBm | -140 |  |  |  |
| UE_TXPWR_MAX_RACH | dBm | 21 |  |  |  |
| Treselection | s | 0 |  |  |  |
| Sprioritysearch1 | dB | 42 |  |  |  |
| Sprioritysearch2 | dB | 0 |  |  |  |
| Threshx, high (Note) | dB | 44 |  |  |  |
| NOTE: This refers to the value of Threshx, high which is included in UTRA system information, and is a threshold for the E-UTRA target cell |  |  |  |  |  |

The probability of re-selection from Cell 1 to Cell 2 during T2 observed during testing shall be less than 10%.

The probability of re-selection from Cell 2 to Cell 1 during T4 observed during testing shall be less than 10%.

The cell reselection delay to lower priority is defined as the time from the beginning of time period T3, to the moment when the UE camps on Cell 2 and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message on Cell 2. In order to evaluate re-selection delay, the SS first needs to verify that the UE is camped on Cell 1 at the start of T3.

The cell re-selection delay to lower priority shall be less than 21 s.

NOTE:	The cell re-selection delay to lower priority cell can be expressed as: TevaluateUTRA_FDD + TSI-UTRA

Where:

TevaluateUTRA-FDD	See Table 4.2.2.5.1-1 of 3GPP TS 36.133 [4] clause 4.2.2.5.1

TSI-UTRA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 20.48 s for lower priority cell reselection, allow 21 s.

For the test to pass, all the events above shall pass.

The statistical pass/ fail decisions are done separated for each event. For an event to pass, the total number of successful loops shall be more than 90% of the cases with a confidence level of 95%.

### << Skipped sections >>

### 4.3.3	E-UTRAN TDD - UTRAN FDD cell re-selection

#### 4.3.3.1	Test purpose

To verify that the UE is able to search and measure neighbouring UTRA FDD cells and compare to the E-UTRA serving cell to meet the inter-RAT cell re-selection requirements when the UTRA is of lower priority.

#### 4.3.3.2	Test applicability

This test applies to all types of E-UTRA TDD UE release 8 and forward that support UTRA FDD and not supporting protection against improper reselection to GERAN/UTRAN.

#### 4.3.3.3	Minimum conformance requirements

When the measurement rules indicate that UTRA FDD cells are to be measured, the UE shall measure CPICH Ec/Io and CPICH RSCP of detected UTRA FDD cells in the neighbour frequency list at the minimum measurement rate. The parameter NUTRA_carrier_FDD is the number of carriers in the neighbour frequency list. The UE shall filter CPICH Ec/Io and CPICH RSCP measurements of each measured UTRA FDD cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least half the minimum specified measurement period.

If the SServingCell of the E-UTRA serving cell is greater than Snonintrasearch then the UE shall search for inter-RAT layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in 3GPP TS 36.133 [4] clause 4.2.2 as Thigher_priority_search = (60 * Nlayers) seconds, where the parameter Nlayers is the total number of configured higher priority carrier frequencies.

If the SServingCell of the E-UTRA serving cell is less than or equal to Snonintrasearch then the UE shall search for and measure inter-RAT layers of higher or lower priority in preparation for possible re-selection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority inter-RAT layers shall be the same as that defined below for lower priority RATs.

The UE shall evaluate whether newly detectable UTRA FDD cells have met the re-selection criteria in 3GPP TS 36.304 [6] within time NUTRA_carrier_FDD * TdetectUTRA_FDD (as defined in table 4.2.2.5.1-1 of 3GPP TS 36.133 [4] clause 4.2.2.5.1) when the SServingCell of the E-UTRA serving cell is greater than Snonintrasearch when TreselectionRAT = 0 provided that the re-selection criteria is met by a margin of at least 6 dB.

Cells which have been detected shall be measured at least every NUTRA_carrier * TmeasureUTRA_FDD when the SServingCell of the E-UTRA serving cell is less than Snonintrasearch.

When higher priority UTRA FDD cells are found by the higher priority search, they shall be measured at least every TmeasureUTRA_FDD. If re-selection to any higher priority cell is not triggered within (TevaluateUTRA_FDD + TreselectionRAT) after it is found in a higher priority search, the UE is not required to continue making measurements of the cell to evaluate the ongoing possibility of re-selection.

For a cell that has been already detected, but that has not been re-selected to, the filtering shall be such that the UE shall be capable of evaluating that an already identified UTRA FDD cell has met re-selection criterion defined in 3GPP TS 36.304 [6] within NUTRA_carrier * TevaluateUTRA_FDD as defined in table 4.2.2.5.1-1 of 3GPP TS 36.133 [4] clause 4.2.2.5.1 when TreselectionRAT = 0.

The normative reference for this requirement is TS 36.133 [4] clause 4.2.2.5.1 and A.4.3.3.

#### 4.3.3.4	Test description

##### 4.3.3.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 4.1.

Frequencies to be tested: According to Annex E table E-1 and 3GPP TS 36.508 [7] clauses 4.4.2 and 4.3.1.

Channel Bandwidth to be tested: 10MHz as defined in 3GPP TS 36.508 [7] clause 4.3.1.

1.	Connect the SS (node B emulator) and AWGN noise source to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A Figure A.22 for UE with 2Rx RF band and Annex A, Figure TBD for 4Rx capable UE without any 2Rx RF bands.

2.	The general test parameter settings are set up according to Table 4.3.3.4.1-1.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	Message contents are as defined in clause 4.3.3.4.3.

5.	There is one E-UTRA TDD cell and one UTRA FDD cell specified in the test. Cell 1 is the cell used for registration according to 3GPP TS 36.508 [7] clause 7.2A.2 with the power level set according to Annex C.0 and C.1.

Table 4.3.3.4.1-1: General test parameters for EUTRA TDD- 
lower priority UTRA FDD inter RAT cell re-selection test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell1 | E-UTRAN cell |
| T1 end condition | Active cells |  | Cell1 | UE shall perform reselection to cell 1 during T1 for subsequent iterations of the test |
|  | Neighbour cell |  | Cell2 |  |
| T2 end condition | Active cell |  | Cell2 | UE shall perform reselection to cell 2 during T2 |
|  | Neighbour cell |  | Cell1 |  |
| E-UTRA PRACH configuration |  |  | 53 | As specified in table 5.7.1-2 in 3GPP TS 36.211 |
| Uplink-downlink configuration of cell 1 |  |  | 1 | As specified in table 4.2.2 in 3GPP TS 36.211 |
| Special subframe configuration of cell 1 |  |  | 6 | As specified in table 4.2.1 in 3GPP TS 36.211 |
| E_UTRA Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| T1 |  | s | 85 | T1 need to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 25 | T2 need to be defined so that cell re-selection reaction time is taken into account. |

##### 4.3.3.4.2	Test procedure

The test consists of one active cell and one neighbour cell. The UE is requested to monitor the neighbouring cell on one UTRA FDD carrier. In the test there are two successive time periods, with time duration of T1 and T2 respectively. Both Cell 1 and Cell 2 are already identified by the UE prior to the start of the test. Cell 2 is of lower priority than Cell 1.

In the following test procedure "UE responds" means "UE starts transmitting preamble on PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure".

1.	Ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2.

2.	Set the parameters according to T1 in Table 4.3.3.5-1 and 4.3.3.5-2. Propagation conditions are set according to Annex B clause B.1.1. T1 starts. If the UE is already camped in Cell 1, wait until T1 expires and skip to step 5.

3.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 1.

4.	If the UE has re-selected Cell 1 within T1, after the re-selection or when T1 expires, continue with step 5.
Otherwise, if T1 expires and the UE has not yet re-selected Cell 1, skip to step 9.

5.	The SS shall switch the power setting from T1 to T2 as specified in Table 4.3.3.5-1 and 4.3.3.5-2.

6.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 2.

7.	If the UE responds on Cell 2 during time duration T2 within 21 seconds from the beginning of time period T2 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

8.	If the UE has re-selected Cell 2 within T2, after the re-selection or when T2 expires, skip to step 10.
Otherwise, if T2 expires and the UE has not yet re-selected Cell 2, continue with step 9.

9.	Switch off and on the UE and ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2 in Cell 1.

10.	Repeat step 2-9 until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved.

##### 4.3.3.4.3	Message contents

Message contents are according to 3GPP TS 36.508 [7] clause 4.6 and 3GPP TS 34.108 [24] clause 6.1.0b with the following exceptions:

Table 4.3.3.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.3-5 Table H.2.3-6 Table H.2.3-13 |
| Default RRC messages and information elements contents exceptions | Table H.3.2-2 |

Table 4.3.3.4.3-2: System Information Block type 19: EUTRA TDD- 
lower priority UTRA FDD inter RAT cell re-selection test case

| Derivation Path: 36.508 clause 4.4.4.1 Table 4.4.4.1-1: System Information Block type 19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| priority | 3 |  |  |
| s-PrioritySearch1 | 21 (42dB) |  |  |
| s-PrioritySearch2 | 0 | default value is 0 |  |
| threshServingLow | 0 (0dB) |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE |  | n denotes the index of the entry |  |
| earfcn |  | Downlink EARFCN of Cell 1 |  |
| measurementBandwidth | 50 | Enumerated(6, 15, 25, 50, 75, 100) |  |
| priority | 4 |  |  |
| qRxLevMinEUTRA | -70 (-140 dBm) |  |  |
| threshXhigh | 24 (48 dB) |  |  |
| threshXlow | 1 (2 dB) | Default value |  |
| eutra-excludeListedCellList | Not present |  |  |
| eutraDetection | TRUE |  |  |
| } |  |  |  |
| nonCriticalExtensions SEQUENCE {} | Not present |  |  |
| } |  |  |  |

#### 4.3.3.5	Test requirement

Tables 4.3.3.4.1-1, 4.3.3.5-1 and 4.3.3.5-2 define the primary level settings including test tolerances for E-UTRAN TDD- UTRAN FDD intra frequency cell re-selection test case which UTRA is of lower priority.

Table 4.3.3.5-1: Cell specific test parameters for Cell 1(E-UTRA TDD)

| Parameter | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 |  | T2 |
| E-UTRA RF Channel number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| OCNG Patterns defined in D.2.1 (OP.2 TDD) |  | OP.2 TDD |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PCFICH_RB | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDCCH_RA | dB |  |  |  |
| PDCCH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
| Qqualmin for UTRA neighbour cell | dB | -20 |  |  |
| Qrxlevmin for UTRA neighbour cell | dBm | -115 |  |  |
| Qrxlevmin | dBm | -140 |  |  |
|  | dBm/15 kHz | -99.35 |  |  |
| RSRP | dBm/15 KHz | -84.95 | -103.05 |  |
|  | dB | 14.40 | -3.70 |  |
|  | dB | 14.40 | -3.70 |  |
| TreselectionEUTRAN | s | 0 |  |  |
| Snonintrasearch | dB | Not sent |  |  |
| Threshserving, low | dB | 44 |  |  |
| Threshx, low (Note 2) | dB | 42 |  |  |
| Propagation Condition |  | AWGN |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: This refers to the value of Threshx, low which is included in E-UTRA system information, and is a threshold for the UTRA target cell. |  |  |  |  |

Table 4.3.3.5-2: Cell specific test parameters for Cell 2(Lower priority UTRA FDD)

| Parameter | Unit | Cell 2 (UTRA) |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| UTRA RF Channel Number |  | Channel 2 |  |
| CPICH_Ec/Ior | dB | -10 |  |
| PCCPCH_Ec/Ior | dB | -12 |  |
| SCH_Ec/Ior | dB | -12 |  |
| PICH_Ec/Ior | dB | -15 |  |
| OCNS_Ec/Ior | dB | -0.941 |  |
|  | dB | 13.80 | 13.80 |
|  | dBm/3,84 MHz | -70 |  |
| CPICH_Ec/Io | dB | -10.18 | -10.18 |
| CPICH_RSCP | dBm | -66.20 | -66.20 |
| Propagation Condition |  | AWGN |  |
| Qqualmin | dB | -20 |  |
| Qrxlevmin | dBm | -115 |  |
| QrxlevminEUTRA | dBm | -140 |  |
| UE_TXPWR_MAX_RACH | dBm | 21 |  |
| Treselection | S | 0 |  |
| Sprioritysearch1 | dB | 42 |  |
| Sprioritysearch2 | dB | 0 |  |
| Threshx, high (Note) | dB | 48 |  |
| NOTE: This refers to the value of Threshx, high which is included in UTRA system information, and is a threshold for the E-UTRA target cell. |  |  |  |

The cell reselection delay to lower priority is defined as the time from the beginning of time period T2, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message on cell 2.

The cell re-selection delay to lower priority test requirement in this case is expressed as:

Cell re-selection delay to lower priority = TevaluateUTRA_FDD + TSI-EUTRA

TevaluateUTRA_FDD = 19.2 s; as specified in 3GPP TS 36.133 [4] clause 4.2.2.5

TSI-EUTRA = 1280 ms; as specified in 3GPP TS 36.133 [4] clause 4.2.2.4

The cell re-selection delay to lower priority shall be less than a total of 21 seconds in this test case (note: this gives a total of 20.48 seconds but the test allows 21 seconds).

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

### << Skipped sections >>

### 4.4.1	E-UTRAN FDD - GSM cell re-selection

#### 4.4.1.1	Test purpose

To verify that the UE is able to search and measure neighbouring GSM cells and compare to the E-UTRA serving cell to meet the inter-RAT cell re-selection requirements.

#### 4.4.1.2	Test applicability

This test applies to all types of E-UTRA FDD UE release 8 and forward that support GSM and not supporting protection against improper reselection to GERAN/UTRAN.

#### 4.4.1.3	Minimum conformance requirements

The cell re-selection delay shall be less than 4 * Tmeasure, GSM + TBCCH in RRC_IDLE state.

When the measurement rules defined in 3GPP TS 36.304 [6] indicates that E-UTRAN inter-frequencies or inter-RAT frequency cells are to be measured, the UE shall measure the signal level of the GSM BCCH carriers if the GSM BCCH carriers are indicated in the measurement control system information of the serving cell, GSM BCCH carriers of lower priority than the serving cell shall be measured at least every Tmeasure,GSM as defined in table 4.2.2.5.3-1 of 3GPP TS 36.133 [4] clause 4.2.2.5.3.

When higher priority GSM BCCH carriers are found by the higher priority search, they shall be measured at least every Tmeasure,GSM, and the UE shall decode the BSIC of the GSM BCCH carrier. If, after detecting a cell in a higher priority search, it is determined that re-selection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of re-selection, or to continuously verify the BSIC of the GSM BCCH carrier every 30s.

However, the minimum measurement filtering requirements specified shall still be met by the UE before it makes any determination that it may stop measuring the cell.

The UE shall maintain a running average of 4 measurements for each GSM BCCH carrier. The measurement samples for each cell shall be as far as possible uniformly distributed over the averaging period.

If continuous GSM measurements are required by the measurement rules in 3GPP TS 36.304 [6], the UE shall attempt to verify the BSIC at least every 30 seconds for each of the 4 strongest GSM BCCH carriers. If a change of BSIC is detected for one GSM cell then that GSM BCCH carrier shall be treated as a new GSM neighbour cell. If the UE detects on a BCCH carrier a BSIC which is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform BSIC re-confirmation for that cell.

The UE shall not consider the GSM BCCH carrier in cell re-selection, if the UE cannot demodulate the BSIC of that GSM BCCH carrier. Additionally, the UE shall not consider a GSM neighbour cell in cell re-selection, if it is indicated as not allowed in the measurement control system information of the serving cell.

The UE shall evaluate the inter-RAT cell re-selection criteria as defined in 3GPP TS 36.304 [6] at least every DRX cycle. The DRX cycle length is 1.28 seconds. When a non-zero value of TreselectionEUTRAN is used, the UE shall only perform re-selection on an evaluation which occurs simultaneously to, or later than the expiry of the TreselectionEUTRAN timer.

At inter-RAT cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels for paging reception of the target inter-frequency cell. For E-UTRAN to GSM cell re-selection the interruption time shall not exceed TBCCH + 50 ms. TBCCH is the maximum time allowed to read BCCH data from a GSM cell as defined in 3GPP TS 45.008 [15] clause 6.2.

For idle mode cell re-selection purposes, the UE shall be capable of monitoring at least:

-	Depending on UE capability, 3 FDD E-UTRA inter-frequency carriers, and

-	Depending on UE capability, 32 GSM carriers

In addition to the requirements defined in 3GPP TS 36.133 [4] clause 4.2.2.9 a UE in RRC_IDLE state shall be capable of monitoring a total of at least 8 carrier frequency layers, which includes serving layer, comprising of any defined in 3GPP TS 36.133 [4] clause 4.2.2.9 combination of E-UTRA FDD, E-UTRA TDD, UTRA FDD, UTRA TDD, GSM (one GSM layer corresponds to 32 cells) , cdma2000 1x and HRPD layers.

The normative reference for this requirement is TS 36.133 [4] clause 4.2.2.5.3 and A.4.4.1.

#### 4.4.1.4	Test description

##### 4.4.1.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 4.1.

Frequencies to be tested: According to Annex E table E-1 and 3GPP TS 36.508 [7] clauses 4.4.2 and

Channel Bandwidth to be tested: 10 MHz as defined in 3GPP TS 36.508 [7] clause4.3.1.

1.	Connect the SS (node B emulator) and AWGN noise sources to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A figure A.14 for UE with 2Rx RF band and Annex A, Figure TBD for 4Rx capable UE without any 2Rx RF bands.

2.	The general test parameter settings are set up according to Table 4.4.1.4.1-1.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	Message contents are as defined in clause 4.4.1.4.3.

5.	There is one E-UTRA FDD cell and one GSM cell specified in the test. Cell 1 (E-UTRA FDD cell) is the cell used for registration with the power level set according to Annex C.0 and C.1 for this test.

Table 4.4.1.4.1-1: General Test Parameters for E-UTRAN FDD - GSM cell re-selection test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell1 | UE shall be forced to cell 1 in the initialisation phase and shall be able to detect and monitor the 4 strongest GSM BCCH carriers in T1. Cell 1 is an E-UTRA FDD cell. |
| Final condition | Neighbour cell |  | Cell2 | UE shall perform reselection to cell 2 during T2. Cell 2 is a GSM cell. |
| E-UTRA RF Channel Number |  |  | 1 | 1 E-UTRA FDD carrier frequency |
| GSM ARFCN |  |  | 1 | 12 GSM BCCH carriers are used |
| PRACH configuration |  |  | 4 | As specified in table 5.7.1-2 in 3GPP TS 36.211 [9] |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| CP length of cell 1 |  |  | Normal |  |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| T1 |  | s | 35 | T1 need to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 35 | T2 need to be defined so that the higher layer search periodicity and cell re-selection reaction time are taken into account. |
| Propagation channel |  |  | AWGN |  |

##### 4.4.1.4.2	Test procedure

The test consists of one active cell and one neighbour cell. The UE is requested to monitor the neighbouring cells on one E-UTRA FDD carrier and twelve GSM cells. In the test there are two successive time periods, with time duration of T1 and T2 respectively. Cell 1 (E-UTRA FDD cell) and Cell 2 (GSM cell) shall belong to different Location Areas. During initialization before the start of the test, the UE is camped on Cell 1. By the end of T1, the UE has identified BSIC on the GSM BCCH carrier of Cell 2 but the signal levels do not meet the re-selection criterion during T1. At the start of T2, the signal levels change such that Cell 2 meets the re-selection criterion. The GSM layer is configured at a lower priority than the serving E-UTRA FDD layer.

In the following test procedure "UE responds" means "UE starts transmitting RR Channel Request message for location update procedure".

1.	Ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2.

2.	Set the parameters according to T1 in Table's 4.4.1.5-1 and 4.4.1.5-2. Propagation conditions are set according to Annex B clause B.1.1. T1 starts. If the UE is already camped in Cell 1, wait until T1 expires and skip to step 5.

3.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 1.

4.	If the UE has re-selected Cell 1 within T1, after the re-selection or when T1 expires, continue with step 5.
Otherwise, if T1 expires and the UE has not yet re-selected Cell 1, skip to step 9.

5.	The SS shall switch the power setting from T1 to T2 as specified in Table's 4.4.1.5-1 and 4.4.1.5-2.

6.	The SS waits for location update information from the UE to perform cell re-selection on Cell 2.

7.	If the UE responds on Cell 2 during time duration T2 within 27.9 seconds from the beginning of time period T2 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

8.	If the UE has re-selected Cell 2 within T2, after the re-selection or when T2 expires, skip to step 10.
Otherwise, if T2 expires and the UE has not yet re-selected Cell 2, continue with step 9.

9.	Switch off and on the UE and ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2 in Cell 1.

10.	Repeat step 2-9 until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved.

##### 4.4.1.4.3	Message contents

Message contents are according to 3GPP TS 36.508 [7] clause 4.6 with the following exceptions:

Table 4.4.1.4.3-1: Common Exception messages for E-UTRAN FDD - GSM cell re-selection test case

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.3-9 Table H.2.3-10 |
| Default RRC messages and information elements contents exceptions | Table H.3.2-1 |

All GSM cell messages indicated shall use the same content as described in the default message content in 3GPP TS 45.008 [15] clause 9 for Rel-4 and later releases, with the exceptions above and as specified in Tables 4.4.1.5-1 and 4.4.1.5-2.

#### 4.4.1.5	Test requirement

Tables 4.4.1.4.1-1, 4.4.1.5-1 and 4.4.1.5-2 defines the primary level settings including test tolerances for E-UTRAN FDD to GSM cell re-selection test case.

Table 4.4.1.5-1: Cell Specific Test requirement Parameters for Cell 1 E-UTRAN FDD cell

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Patterns defined in D.1.2 (OP.2 FDD) |  | OP.2 FDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm | -140 |  |
|  | dBm/15 kHz | -99.4 |  |
| RSRP | dBm/15 KHz | -88.9 | -103.1 |
|  | dB | 10.5 | -3.70 |
|  | dB | 10.5 | -3.70 |
| TreselectionEUTRAN | s | 0 |  |
| Snonintrasearch | dB | Not sent |  |
| Threshserving, low | dB | 44 |  |
| Threshx, low Note 2 | dB | 24 |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: This refers to Threshx, low which is included in E-UTRA system information, and is a threshold for GSM target cell. |  |  |  |

Table 4.4.1.5-2: Cell Specific Test requirement Parameters for Cell 2 GSM cell

| Parameter | Unit | Cell 2 (GSM) |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| Absolute RF Channel Number |  | ARFCN 1 |  |
| RXLEV | dBm | -90.00 | -75.00 |
| RXLEV_ACCESS_MIN | dBm | -105 |  |
| MS_TXPWR_MAX_CCH | dBm | 24 |  |

The cell re-selection delay is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send the RR Channel Request message for location update to Cell 2.

The cell re-selection delay test requirement in this case is expressed as:

Cell re-selection delay = 4 * TmeasureGSM + TBCCH

TmeasureGSM = 6.40 s; as specified in 3GPP TS 36.133 [4] clause 4.2.2.3

TBCCH = 1.9 s; maximum time allowed to read the BCCH data from GSM cell, when being synchronized to a BCCH carrier; as specified in 3GPP TS 45.008 [15] clause 6.2

The cell re-selection delay shall be less than a total of 27.9 seconds in this test case (note: this gives a total of 26 seconds for the TmeasureGSM calculation plus 1.9 s for TBCCH but the test allows 27.9 seconds).

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

### 4.4.2	E-UTRAN TDD - GSM cell re-selection

#### 4.4.2.1	Test purpose

To verify that the UE is able to search and measure neighbouring GSM cells and compare to the E-UTRA serving cell to meet the inter-RAT cell re-selection requirements.

#### 4.4.2.2	Test applicability

This test applies to all types of E-UTRA TDD UE release 8 and forward that support GSM and not supporting protection against improper reselection to GERAN/UTRAN.

#### 4.4.2.3	Minimum conformance requirements

If the SServingCell of the E-UTRA serving cell (or other cells on the same frequency layer) is greater than Snonintrasearch then:

- 	The UE may not search for, or measure GSM cells if the priority of GSM is equal to, or lower than the serving cell.

- 	The UE shall search for and measure GSM cells if the priority of GSM is higher than the serving cell. The minimum rate at which the UE is required to search for and measure such layers may be reduced in this scenario to maintain UE battery life.

If the SServingCell of the E-UTRA serving cell is less than or equal to Snonintrasearch then the UE shall measure, according to the measurement rules defined in 3GPP TS 36.304 [6] at least every Tmeasure,GSM as defined in table 4.2.2.5.3-1 of 3GPP TS 36.133 [4] clause 4.2.2.5.3:

- 	If a detailed neighbour cell list is provided, the signal level of the GSM BCCH carrier of each GSM neighbour cell indicated in the measurement control system information of the serving cell; or

- 	If only BCCH carriers are provided, the signal level of the GSM BCCH carriers indicated in the measurement control system information of the serving cell

If the RSRP of the E-UTRA serving cell is greater than Snonintrasearch then the UE shall search for GSM BCCH carrier at least every Thigher_priority_search where Thigher_priority_search is described in 3GPP TS 36.133 [4] clause 4.2.2 as Thigher_priority_search = (60 * Nlayers) seconds, where the parameter Nlayers is the total number of configured higher priority carrier frequencies. When higher priority GSM BCCH carriers are found by the higher priority search, they shall be measured at least every Tmeasure,GSM, and the UE shall decode the BSIC of the GSM BCCH carrier. If re-selection to any higher priority cell is not triggered within (4 * Tmeasure_GSM + TreselectionRAT) after it has been found in a higher priority search, the UE is not required to continue make measurements of the BCCH carrier to evaluate the ongoing possibility of re-selection, or to continuously verify the BSIC of the GSM BCCH carrier every 30s.

The UE shall maintain a running average of 4 measurements for each GSM BCCH carrier. The measurement samples for each cell shall be as far as possible uniformly distributed over the averaging period.

If continuous GSM measurements are required by the measurement rules in 3GPP TS 36.304 [6], the UE shall attempt to verify the BSIC at least every 30 seconds for each of the 4 strongest GSM BCCH carriers. If a change of BSIC is detected for one GSM cell then that GSM BCCH carrier shall be treated as a new GSM neighbour cell. If the UE detects on a BCCH carrier a BSIC which is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform BSIC re-confirmation for that cell.

The UE shall not consider the GSM BCCH carrier in cell re-selection, if the UE cannot demodulate the BSIC of that GSM BCCH carrier. Additionally, the UE shall not consider a GSM neighbour cell in cell re-selection, if it is indicated as not allowed in the measurement control system information of the serving cell.

The normative reference for this requirement is TS 36.133 [4] clause 4.2.2.5.3 and A.4.4.2.

#### 4.4.2.4	Test description

##### 4.4.2.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 4.1.

Frequencies to be tested: According to Annex E table E-1 and 3GPP TS 36.508 [7] clauses 4.4.2 and 4.3.1.

Channel Bandwidth to be tested: 10MHz as defined in 3GPP TS 36.508 [7] clause4.3.1.

1.	Connect the SS (node B emulator) and AWGN noise source to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A figure A.14 for UE with 2Rx RF band and Annex A, Figure TBD for 4Rx capable UE without any 2Rx RF bands.

2.	The general test parameter settings are set up according to Table 4.4.2.4.1-1.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	Message contents are as defined in clause 4.4.2.4.3.

5.	There is one E-UTRA TDD cell and one GSM cell specified in the test. Cell 1 (E-UTRA TDD cell) is the cell used for registration with the power level set according to Annex C.0 and C.1 for this test.

Table 4.4.2.4.1-1: General test parameters for E-UTRA TDD GSM cell re-selection test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell1 | UE shall be forced to cell 1 in the initialisation phase and shall be able to detect and monitor the 4 strongest GSM BCCH carriers in T1. Cell 1 is an E-UTRA TDD cell. |
| Final condition | Neighbour cell |  | Cell2 | UE shall perform reselection to cell 2 during T2. Cell 2 is a GSM cell. |
| E-UTRA RF Channel Number |  |  | 1 | 1 E-UTRA TDD carrier frequency |
| GSM ARFCN |  |  | 1 | 12 GSM BCCH carriers are used |
| Uplink-downlink configuration of cell 1 |  |  | 1 | As specified in table 4.2.2 in 3GPP TS 36.211 |
| Special subframe configuration for cell 1 |  |  | 6 | As specified in table 4.2.1 in 3GPP TS 36.211 |
| PRACH configuration for cell 1 |  |  | 53 | As specified in table 5.7.1-3 in 3GPP TS 36.211 |
| CP length of cell 1 |  |  | Normal |  |
| Access Barring Information |  |  | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| Propagation channel |  |  | AWGN |  |
| T1 |  | s | 35 | T1 need to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 35 | T2 need to be defined so that the higher layer search periodicity and cell re-selection reaction time are taken into account. |

##### 4.4.2.4.2	Test procedure

The test consists of one active cell and one neighbour cell. The UE is requested to monitor the neighbouring cell on one E-UTRA TDD carrier and twelve GSM cells. In the test there are two successive time periods, with time duration of T1 and T2 respectively. Cell 1 (E-UTRA TDD cell) and Cell 2 (GSM cell) belong to different Location Areas. By the end of T1, the UE has identified BSIC on the GSM BCCH carrier of Cell 2 but the signal levels do not meet the re-selection criterion. At the start of T2, the signal levels change such that Cell 2 meets the re-selection criterion. The GSM layer is configured at a lower priority than the serving E-UTRA TDD layer.

In the following test procedure "UE responds" means "UE starts transmitting RR Channel Request message for location update procedure".

1.	Ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2.

2.	Set the parameters according to T1 in Table's 4.4.2.5-1 and 4.4.2.5-2. Propagation conditions are set according to Annex B clause B.1.1. T1 starts. If the UE is already camped in Cell 1, wait until T1 expires and skip to step 5.

3.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 1.

4.	If the UE has re-selected Cell 1 within T1, after the re-selection or when T1 expires, continue with step 5.
Otherwise, if T1 expires and the UE has not yet re-selected Cell 1, skip to step 9.

5.	The SS shall switch the power setting from T1 to T2 as specified in Table's 4.4.2.5-1 and 4.4.2.5-2.

6.	The SS waits for location update information from the UE to perform cell re-selection on Cell 2.

7.	If the UE responds on Cell 2 during time duration T2 within 27.9 seconds from the beginning of time period T2 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

8.	If the UE has re-selected Cell 2 within T2, after the re-selection or when T2 expires, skip to step 10.
Otherwise, if T2 expires and the UE has not yet re-selected Cell 2, continue with step 9.

9.	Switch off and on the UE and ensure the UE is in State 2A-RF according to 3GPP TS 36.508 [7] clause 7.2A.2 in Cell 1.

10.	Repeat step 2-9 until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved.

##### 4.4.2.4.3	Message contents

Message contents are according to 3GPP TS 36.508 [4] clause 4.6 with the following exceptions:

Table 4.4.2.4.3-1: Common Exception messages for E-UTRAN TDD - GSM cell re-selection test case

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.3-9 Table H.2.3-10 |
| Default RRC messages and information elements contents exceptions | Table H.3.2-2 |

All GSM cell messages indicated shall use the same content as described in the default message content in 3GPP TS 45.008 [15] clause 9 for Rel-4 and later releases, with the exceptions above and as specified in Tables 4.4.2.5-1 and 4.4.2.5-2.

#### 4.4.2.5	Test requirement

Tables 4.4.2.4.1-1, 4.4.2.5-1 and 4.4.2.5-2 defines the primary level settings including test tolerances for E-UTRAN TDD to GSM cell re-selection test case.

Table 4.4.2.5-1: Cell Specific Test requirement Parameters for Cell 1 E-UTRAN TDD cell

| Parameter | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 |  |
| E-UTRA RF Channel number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| OCNG Patterns defined in D.2.2 (OP.2 TDD) |  | OP.2 TDD |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PCFICH_RB | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDCCH_RA | dB |  |  |  |
| PDCCH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
| Qrxlevmin | dBm | -140 |  |  |
|  | dBm/15 kHz | -99.4 |  |  |
| RSRP | dBm/15 KHz | -88.9 |  | -103.1 |
|  | dB | 10.5 |  | -3.70 |
|  | dB | 10.5 |  | -3.70 |
| TreselectionEUTRAN | s | 0 |  |  |
| Snonintrasearch | dB | Not sent |  |  |
| Threshserving, low | dB | 44 |  |  |
| Threshx, low (Note 2) | dB | 24 |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: This refers to Threshx, low which is included in E-UTRA system information, and is a threshold for GSM target cell. |  |  |  |  |

Table 4.4.2.5-2: Cell Specific Test requirement Parameters for Cell 2 GSM cell

| Parameter | Unit | Cell 2 (GSM) |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| Absolute RF Channel Number |  | ARFCN 1 |  |
| RXLEV | dBm | -90.00 | -75.00 |
| RXLEV_ACCESS_MIN | dBm | -105 |  |
| MS_TXPWR_MAX_CCH | dBm | 24 |  |

The cell re-selection delay is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send the RR Channel Request message for location update to Cell 2.

The cell re-selection delay test requirement in this case is expressed as:

Cell re-selection delay = 4 * TmeasureGSM + TBCCH

TmeasureGSM = 6.4 s; as specified in 3GPP TS 36.133 [4] Table 4.2.2.5.3-1 in clause 4.2.2.5.3

TBCCH = 1.9 s; the maximum time allowed to read the BCCH data, when being synchronized to a BCCH carrier as specified in 3GPP TS 45.008 [15] clause 6.2

This gives a total of 25.6 s + TBCCH, allow 26 s + TBCCH in the test case.

The cell re-selection delay shall be less than a total of 27.9 seconds in this test case (note: this gives a total of 26 seconds for the TmeasureGSM calculation plus 1.9 s for TBCCH but the test allows 27.9 seconds).

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

<< End of Changes >>