3GPP TSG RAN Meeting #108						RP-251125

Prague, Czech Republic, June 9-13, 2025

Agenda Item:		16.1

Source:		T-Mobile USA, AT&T, Nokia, Ericsson, Telstra, Verizon

Title:				Requirements for Service aware 6G RAN

Document for:		Approval

# 1	Introduction

In this contribution we discuss requirements for service aware 6G radio access network (RAN) to address new and evolving services effectively. We also propose related 6G RAN requirements to be captured to the TR 38.914 on 6G Scenarios and Requirements.

# 2	Discussion

3GPP wireless technology advancement has been the catalyst for the emerging of new verticals. From limited voice and SMS (2G), and early broadband data with optimized voice (3G), to high-speed broadband data, video and unlimited voice/SMS (4G), and unlimited data, IoT, AR and slicing (5G), the industry is at the stage of massive introduction of new use cases. In a conventional network where eMBB services are the predominant traffic carried by the network, maximizing bit/Hz and using it as the main driver for spectrum efficiency makes sense. However, with more new services, such as IoT and XR, being introduced into the wireless network, it is becoming more and more challenging for the network to manage the radio resources which are shared by different services. Fixed Wireless Access (FWA) has also experienced significant growth in recent years, driven by technological advancements. It shouldn't come as a surprise considering that the current 5G specification is designed for eMBB, i.e. smartphone use cases which primarily are voice, SMS and best effort data applications (video streaming, Web browsing, mapping etc) and radio protocol and procedures are heavily designed to ensure robust mobility performance. While eMBB continues to be an important service provided by operators, new applications/services, such as FWA, XR or AI traffic, have significantly different traffic profiles and performance criteria, and needs to be treated as primary services when the specification are defined for 6G.    As the existing and new services will evolve further and new technologies such as AI and sensing will set new requirements for the network, 6G RAN needs to be even more capable of supporting diversified services and applications effectively.

In the current 5G system, 5QI is used to define the specific quality profile for a service by the core network. Different 5QI value correspondent to a set of service level thresholds such as throughput, latency, error rate etc. 5QI index is passed to the RAN in which the 5QI value, along with other RAN domain parameters, is used by the MAC scheduler to calculate the MAC scheduling priority for each data flow. Such mechanism works for a system carrying data applications with less stringent throughput and latency requirement, such as web browsing and non-real time video streaming, however, RAN will struggle to support applications, such as XR, which require both stringent throughput and latency adjustments in real time unless the RAN is over dimensioned to ensure the quality of service for such applications. This makes it very challenge for operators to launch such new services without compromising the overall system spectrum efficiency and legacy eMBB service performance.

3GPP made the efforts to address this issue by introducing RvQoE which allows the application attributes such as video buffer level to be transferred to the AS layer through RRC containers, however, in commercial network, operators ran into challenges from device ecosystem players who normally prevents the OS from sending the application information/APP IDs to the AS layer for various reasons. Operators have very limited visibility and control on the massive and diversified applications that are going through the network and are not able to effectively meet all applications’ performance targets while maintaining healthy network spectrum and operation efficiency. In addition, the idea of using RRC containers to transfer that information between device and RAN introduces additional signalling overhead which could be non-trivial for a loaded network. To solve this problem, fundamental changes are needed to make RAN to be more adaptive to the applications. Service Aware RAN for 6G aims to address the issue from the basic protocol levels.

3GPP needs to understand

- Current 5G system, especially RAN doesn’t have the visibility of real time application level performance attributes and therefore cannot effectively allocate the radio resources to meet the applications minimum requirements while maintaining healthy overall system spectrum efficiency.

- Current 5G QoS framework is too coarse to allow RAN dynamically allocate radio resource efficiently in real time based on the application layer performance.

- Currently, the 5G RAN does not have visibility of application-level characteristics (requirements, performance, etc.) for addressing the type of requirements that are expected for new service types. Thus, the RAN cannot efficiently utilize RAN resources and features to address other objectives (e.g., to provide resilience to users with critical services, etc.).

- While eMBB continues to be an important service provided by operators, new applications/services, such as XR or AI traffic, have significantly different traffic profiles and performance criteria, and needs to be treated as primary services when the specification are defined for 6G.

# 3 	Text proposal

We propose the following text to the 6G Scenarios and Requirements TR 38.914.

## 5.x	Service aware 6G RAN

6G radio access network (6G RAN) shall have means to address evolving and new services like immersive communication, AI traffic effectively and be adaptive to the applications. 6G RAN will need to obtain more visibility to application-level attributes (e.g., performance, requirements, etc.) to effectively allocate radio resources and utilize the different RAN resources to meet the applications minimum requirements while maintaining healthy overall system spectrum efficiency and utilization of RAN resources.

# 4	Conclusion

In this contribution we have discussed requirements for service aware 6G radio access network (RAN) to address new and evolving services effectively. In section 3 we propose 6G RAN requirements for service aware 6G RAN to be captured to the TR 38.914 on 6G Scenarios and Requirements.