3GPP TSG RAN Meeting #108						RP-251128

Prague, Czech Republic, June 9-13, 2025

Agenda Item:		16.1

Source:		T-Mobile USA

Title:				Requirements for 6G Spectrum

Document for:		Approval

# 1	Introduction

In this contribution we discuss requirements for 6G spectrum We also propose related 6G RAN requirements to be captured to the TBD on 6G Scenarios and Requirements.

# 2	Discussion

In 5G NR, the widest FR1 channel bandwidth is 100 MHz. Because of this limit, operators with more than 100 MHz of contiguous spectrum need to use carrier aggregation to cover such allocations. At least eight contributions at the 3GPP 6G workshop in March 2025 mentioned 200 MHz or wider channel bandwidths for 6G. While most focused on the spectrum above 6 GHz, there was also interest in wider channel bandwidths for spectrum below 6 GHz, including the 2.5 and 3.5 GHz bands.

Further, 3GPP has recommended adding 200MHz ch BW to the test environments for IMT-2030 in an LS to WP5D RP-243325.

One benefits of wider channel bandwidths in FR1 is higher spectral efficiency than CA. Another benefit of wider channel bandwidths in FR1 is potentially Lower MPR and A-MPR than UL CA due to contiguous RB allocations.

Due to the lack of new spectrum in most FR1 frequency bands, there is strong interest in multi-radio spectrum sharing (MRSS), to allow operators to use the same spectrum for 5G and 6G. Since some operators have > 100 MHz in FR1 bands that are used for 5G and it is expected more spedctrum will become available in FR1, some of which may expand these bands it is important to maintain compatibility between the new, wider channel bandwidths for 6G, and 5G carrier aggregation of channel bandwidths at least 200MHz, including the use of the same subcarrier spacing.

# 3 	Text proposal

We propose the following text to the 6G Scenarios and Requirements TR 38.914.

## 5.x	6G Spectrum

6G radio access network (6G RAN) shall support existing frequency bands, including FR1 and FR2. multi-radio spectrum sharing (MRSS) in FR1 shall be supported to allow operators to use the same spectrum for 5G and 6G. Since some operators have > 100 MHz in FR1 bands that are used for 5G and it is expected more spedctrum will become available in FR1, some of which may expand these bands it is important to maintain compatibility between the new, wider channel bandwidths for 6G, and 5G carrier aggregation of channel bandwidths at least 200MHz, including the use of the same subcarrier spacing. In addition, new spectrum band FR3 shall be supported. Due to various reasons, unlicensed spectrum were under utilized in 4G and 5G. With new 6G use cases and demands for more network capacity such as FWA, it is important to include unlicensed spectrum study for new use cases.

# 4	Conclusion

In this contribution we have discussed requirements for 6G spectrum support to address new and evolving services effectively. In section 3 we propose 6G RAN requirements for spectrum to be captured to the TR 38.914 on 6G Scenarios and Requirements.