3GPP TSG RAN Meeting #108					RP-251129

Prague, Czech Republic, June 9-13, 2025

Agenda Item:	16.1

Source:	T-Mobile USA, Ericsson, Nokia

Title:	Internal 3GPP requirements on coverage and efficiency

Document for:	Approval

# 1	Introduction

6G is intended to both do what 5G does but in a better way, and to do what 5G cannot do. The ITU IMT-2030 requirements, which apply to 6G, are set to reflect that intention, with sharper legacy requirements and new requirements defined to support new use cases. Although ambitious, the IMT-2030 requirements are not comprehensive, but rather samples of what constitute a good 6G standard. With 5G as a reference, additional requirements can be motivated, e.g. on reuse of site grids or addressing suboptimal solutions.

# 2	Discussion

To successfully support new 6G use cases, improving spectrum efficiency should take the angle of serving different services on the same spectrum resource efficiently, not just serving one type of services, such as eMBB, efficiently.  6G radio protocol/air interface shall focus on addressing new primary use cases (FWA, ISAC, Massive IoT etc) in addition to eMBB from day one.

3GPP needs to understand

- Current 5G system, especially RAN is designed for eMBB as the primary service. This approach needs to change

- Maximizing bit/Hz, while it is important for achieving the absolute peak spectra efficiency for best effort full buffer traffic, should not be the only focus for 6G

- While eMBB continues to be an important service provided by operators, new applications/services, such as XR or AI traffic, have significantly different traffic profiles and performance criteria, and needs to be treated as primary services when the specification are defined for 6G.

Further during RAN#107 the 6G RAN req. SID was updated in [RP-250810]. Particularly, for coverage the following related statements were captured that:

- Enhanced overall coverage, focus on cell-edge performance and UL coverage”

- Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band

Translating this into the actual requirement should be done. In doing so we should address the fact that most sites support multiple bands, this fact is something that should be considered in the 6G design. Another important point is deployments around 7 GHz and being able to reuse the existing site grids.

Another area which should be looked over is delays created by how scheduling and HARQ timelines are constructed in 5G and work should be done to try to minimize these.

For 5G there is a requirement in [TR 38.913] related to coupling loss, this to make sure that 4G and 5G have the same coverage assuming the same deployment frequency. In 5G this requirement is referred to as extreme coverage. For 6G we propose to have a similar requirement but compare 5G and 6G. We suggest having a different name in 6G that is more appropriate instead of extreme coverage, e.g. coupling loss.

# 3 Text proposal

Based on the discussion above we propose the following text proposal.

## 5.x	Enhanced/similar coverage in a multiple band scenario (UL/DL decoupling)

In multi-band configurations, the band on which the downlink performs the best and the band on which the uplink performs the best can be different. In multi-band configurations, the 6G specification should not restrict the allocation of bands for downlink and uplink so that the performance in any direction is worse than the best single-band performance in that direction.

## 5.x	Reduced scheduling latency

For reasons of reducing delay and improving coverage, it is important that when data is available for transmission, that data can be transmitted with a minimum of delay and interruptions. Delays and interruptions due to other flows or users are scheduled, or for reasons of energy efficiency must be accepted, but delays and interruptions, e.g. due to control signaling, should be minimized.

## 5.x	Reuse of existing site grids

It is important that 6G can be introduced and perform well using existing site grids, without the need for densification. This requirement can be divided into two parts:

- When 6G is introduced in existing bands, using existing sites and antennas, the coverage of 6G should be at least as good as that of 5G.

- When 6G is introduced on new frequency bands, higher than those used for 5G on the site grid in question, larger antennas can be used to compensate for the increased loss (between isotropic antennas), 6G should support the use of antenna sizes required to make higher frequencies useful on exiting site grids. Larger here refers to the number of antenna elements. Useful does not necessarily mean full coverage or coverage equal to that of lower bands, but that a significant fraction of users is covered, directly benefiting those users and providing offloading gains when deployed together with lower band frequency layers.

The former requirement was also used when 5G was introduced (with 4G as a reference), with the name ‘extreme coverage’. For 6G the similar requirement is referred to as ‘coupling loss’.

### 5.x	Coupling loss

The coupling loss is defined as the total long-term channel loss over the link between the UE antenna ports and the eNode B antenna ports, and includes in practice antenna gains, path loss, shadowing, body loss, etc. The maximum coupling loss (MaxCL) is the limit value of the coupling loss at which the service can be delivered and therefore defines the coverage of the service. MaxCL is independent of the carrier frequency. It is defined in the UL and DL as:

-	UL MaxCL = UL Max Tx power - eNB Sensitivity

-	DL MaxCL = DL Max Tx power - UE Sensitivity

The MaxCL is evaluated via link budget analysis (supported by link level simulations). The proposed MaxCL calculation template is given in following table 7.10.1-1:

Table 7.10.1-1: MaxCL calculation template

| Item | Value |
| --- | --- |
| Transmitter |  |
| (1) Tx power (dBm) |  |
| Receiver |  |
| (2) Thermal noise density (dBm/Hz) |  |
| (3) Receiver noise figure (dB) |  |
| (4) Interference margin (dB) |  |
| (5) Occupied channel bandwidth (Hz) |  |
| (6) Effective noise power = (2) + (3) + (4) + 10 log(5) (dBm) |  |
| (7) Required SINR (dB) |  |
| (8) Receiver sensitivity = (6) + (7) (dBm) |  |
| (9) MaxCL = (1) - (8) (dB) |  |

The following assumptions are used.

Table 7.10.1-2: MaxCL calculation assumptions

| UE Tx power | [23dBm] |
| --- | --- |
| DL Tx power | 46dBm |
| eNB receiver noise figure | 5dB |
| UE receiver noise figure | 9dB |
| Interference margin | 0dB |

For a basic MBB service characterized by a downlink data rate of 2Mbps and an uplink data rate of 60kbps for stationary users, the target on maximum coupling loss is 140dB. For mobile users a downlink data rate of 384kbps is acceptable.

For a basic MBB service characterized by a downlink data rate of 1Mbps and an uplink data rate of 30kbps for stationary users, the target on maximum coupling loss is 143dB. At this coupling loss relevant downlink and uplink control channels should also perform adequately.

As the evaluation methodology, link budget and/or link level analysis are used for extreme long-distance coverage in low density areas.

# 4	Conclusion

In this contribution we propose to add the text proposal in section 3 to TR 38.914.