3GPP TSG RAN Meeting #108					RP-251131

Prague, Czech Republic, June 9-13, 2025

Agenda Item:	16.1

Source:	T-Mobile USA, Nokia

Title:	Requirements for support of FWA in 6G

Document for:	Approval

# 1	Introduction

Fixed Wireless Access (FWA) has experienced significant growth in recent years, driven by technological advancements. Wireless operators, particularly those in North America and India, have recognized the increased business potential for in-home high-speed internet in areas with limited access to traditional broadband infrastructures such as cable or fiber. The global deployment of 5G has provided unique opportunities for operators to open new revenue streams by offering in-home broadband services to their customers. The rising demand in both residential and commercial sectors for faster, reliable, and high-speed connectivity is propelling the 5G fixed wireless access market.

The U.S. 5G fixed wireless access market size was estimated at USD 8.35 billion in 2023 and is projected to be worth around USD 343.18 billion by 2034, growing at a compound annual growth rate (CAGR) of 40.18% from 2024 to 2034, according to Precedence Research. With FWA becoming the primary traffic in operators’ networks, it is critical that on Day One of 6G, FWA as a primary service offered by operators can meet customer demand with a sustainable and profitable business model.

# 2	Discussion

The existing network, designed primarily for enhanced Mobile Broadband (eMBB) services, may not be able to meet the demand of FWA devices, which have different mobility profiles and usage patterns. With the increased demand and projected growth of FWA in both enterprise and consumer areas, solutions that can further improve spectrum efficiency and economies of scale for FWA are essential to ensure the long-term success of the business model. Mobile operators seek to enhance FWA's functionality by optimizing systems, such as physical layer (PHY) optimizations, to address the unique needs of FWA users. By revisiting key elements like mobility management, control channel overhead, and other PHY layer enhancements, network capacity could be significantly increased to meet the higher bandwidth demands of fixed users.

Observation

- Current 5G specification/protocol/architecture were designed for general eMBB, without special consideration of FWA

- One-pipe-fits-all type approach may not work well for eMBB and FWA

- While eMBB continues to be an important service provided by operators, FWA, with significant more traffic volume than eMBB, needs to be able to be treated as primary service when the specification is developed in 3GPP

- At the same time it is important to acknowledge that part of the success of FWA is due that it can technically leverage eMBB design. On high-level a similar approach should be taken for 6G. That said FWA is such a successful service, it would warrant targeted enhancements.

In order to support FWA in a sufficient manner in 6G it is important that FWA specific enhancements are studied within the working groups. Aspects to study are air interface enhancements, protocol enhancements and mobility specific enhancements. Further it is clear that FWA devices can support specific HP UE classes

In more detai,it is proposed to study potential enhancements in the following areas

- The potential air interface modifications that provide better spectrum efficiency for FWA

- RAN signalling and protocols that are specifically designed to support fixed wireless access deployment

- Support for link adaptation, channel estimation, and RRM schemes that leverage the unique characteristics of FWA user profiles (mobility, usage pattern etc)

- Device capabilities that are unique for FWA, including exploring the regulatory requirements.

- FWA device enhancement (BF, IC, HPUE)

- Support for interference management/coordination schemes that are uniquely beneficial for FWA deployment.

- RAN architecture options, e.g. modular NAS/AS for FWA.

It is important that candidate enhancements are studied and evaluated in relevant scenarios.

In addition to studied multiple different technical areas within the working groups it is important to put focus on FWA by having a suitable evaluation scenario for FWA. This could e.g. be based on the urban, suburban, and rural scenarios used for eMBB, with modifications of e.g. UE positions, mobility, and antenna types to reflect FWA. An example could be outdoor, roof-mounted CPE antennas directed towards serving base stations.

# 3 Text proposal

Based on the discussion above we propose the following text proposal.

## 5.x	Fixed wireless access (FWA)

The 6GR and 6G RAN architecture shall support fixed wireless access (FWA).

## 4.x	Evaluation environment for FWA

FWA enhancements shall be studied in relevant scenarios. These are created based on eMBB scenarios for relevant environments, with modifications on the UE side to reflect FWA.

# 4	Conclusion

In this contribution we propose to add the text proposal in section 3 to TR 38.914.

We further propose that RAN shall task the RAN working groups to study FWA specific enhancements specifically within the areas of air interface enh, protocol enh, mobility specific enhancements and device enhancements (e.g. HPUE).