3GPP TSG-RAN5 Meeting #107	R5-253453

Malta, Malta, May 19th – 23rd 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3366 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating AMPR for UL MIMO test case for band n41 for power class 1.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16,LTE_NR_B41_Bn41_PC29dBm-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The band n41 power class 1.5 in A-MPR for UL MIMO test case is absent in 6.2D.3 and need to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding test Configuration table for NS_47 in 6.2D.3.4.1 - Adding Message contents exceptions for network signalling value "NS_47" in 6.2D.3.4.3 - Adding UE test requirements for NS_47 in 6.2D.3.5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The band n41 test cases for NS_47 is absent in A-MPR for MIMO test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2D.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-252666. |  |  |  |  |  |  |  |  |

<Start of Changes>

6.2D.3	UE additional maximum output power reduction for UL MIMO

Editor’s Note:

- Supporting of ULFPTx is only completed for NS_04, NS_03, NS_03U.

- TP is FFS for NS_47 for Power Class 1.5.

6.2D.3.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated a unique with network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band (the IE freqBandIndicatorNR) and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2D.1.3-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

6.2D.3.2	Test applicability

The requirements of this test apply in test case 6.5D.2.3 Additional Spectrum Emission mask for UL MIMO for network signalling value NS_03, NS_03U, NS_04, NS_06, NS_07 to all types of NR UE release 15 and forward that support UL MIMO.

The requirements of this test apply in test case 6.5D.2.3 Additional Spectrum Emission mask for UL MIMO for network signalling value NS_03, NS_03U, NS_04 to all types of NR UE release 16 and forward that support UL full power transmission (ULFPTx).

The requirements of this test apply in test case 6.5D.2.4.2 Adjacent channel leakage ratio for network signalling values NS_03U, NS_05U, and NS_100 to all types of NR Power Class 1.5, Power Class 2 and 3 UE release 15 and forward that support 2-layer codebook based UL MIMO.

The requirements of this test apply in test case 6.5D.2.4.2 Adjacent channel leakage ratio for network signalling values NS_03U to all types of NR Power Class 2 and 3 UE release 16 and forward that support UL full power transmission (ULFPTx).

The requirements of this test apply in test case 6.5D.3.3 Additional Spurious Emissions for network signalling value NS_04, NS_07, NS_12, NS_13, NS_14_NS_15, NS_47, NS_21, NS_43, NS_43U, NS_56, NS_50 to all types of NR UE release 15 and forward that support UL-MIMO.

The requirements of this test apply in test case 6.5D.3_1.3 Additional Spurious Emissions for network signalling value NS_04 to all types of NR UE release 16 and forward that support UL full power transmission (ULFPTx).

NOTE:	Test execution is not necessary if 6.5D.2.3, 6.5D.2.4.2 and 6.5D.3.3 are executed.

<Unchanged Skipped>

Table 6.2D.3.4.1-22: Test Configuration table for NS_47 power class 1.5

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies |  |  |  |  |  |  |  | As specified in Table 6.2.3.4.1-18a and 6.2.3.4.1-18b |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 30 MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest (Note 3) |  |  |
| A-MPR test parameters for NS_47 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 30 | Default |  | CP-OFDM | QPSK | Edge_1RB_Left (A1) |  |  |  |
| 2 | Default | 30 | Default |  |  | QPSK | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 3 | Default | 30 | Deafult |  |  | QPSK | Edge 1RB_Right (A3) |  |  |  |
| 4 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 5 | Default | 30 | Default |  |  | QPSK | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 6 | Default | 30 | Default |  |  | QPSK | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 7 | Default | 30 | Default |  |  | QPSK | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 8 | Default | 30 | Default |  |  | QPSK | 80@35 (A5) |  | 40@18 (A5) | 20@9 (A5) |
| 9 | Default | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A1) |  |  |  |
| 10 | Default | 30 | Default |  |  | 16 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 11 | Default | 30 | Default |  |  | 16 QAM | Edge 1RB_Right (A3) |  |  |  |
| 12 | Default | 30 | Default |  |  | 16 QAM | Outer_Full (A2) |  |  |  |
| 13 | Default | 30 | Default |  |  | 16 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 14 | Default | 30 | Default |  |  | 16 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 15 | Default | 30 | Default |  |  | 16 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 16 | Default | 30 | Default |  |  | 16 QAM | 80@35 (A5) |  | 40@18 (A5) | 20@9 (A5) |
| 17 | Default | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A1) |  |  |  |
| 18 | Default | 30 | Default |  |  | 64 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 19 | Default | 30 | Default |  |  | 64 QAM | Edge 1RB_Right (A3) |  |  |  |
| 20 | Default | 30 | Default |  |  | 64 QAM | Outer_Full (A2) |  |  |  |
| 21 | Default | 30 | Default |  |  | 64 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 22 | Default | 30 | Default |  |  | 64 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 23 | Default | 30 | Default |  |  | 64 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 24 | Default | 30 | Default |  |  | 64 QAM | 80@35 (A5) |  | 40@18 (A5) | 20@9 (A5) |
| 25 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A1) |  |  |  |
| 26 | Default | 30 | Default |  |  | 256 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 27 | Default | 30 | Default |  |  | 256 QAM | Edge 1RB_Right (A3) |  |  |  |
| 28 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |
| 29 | Default | 30 | Default |  |  | 256 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 30 | Default | 30 | Default |  |  | 256 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 31 | Default | 30 | Default |  |  | 256 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 32 | Default | 30 | Default |  |  | 256 QAM | 80@35 (A5) |  | 40@18 (A5) | 20@9 (A5) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: For FR1 bands where highest supported SCS is 60 kHz the highest tested SCS is limited to 30 kHz as carrier with SCS=60 kHz cannot be used as PCell. |  |  |  |  |  |  |  |  |  |  |

Editor’s note: The following lines belong at the end of subclause 6.2D.3.4.1. As new tables are added to this section, these lines should always follow the tables

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2 and uplink signals according Annex G.0, G.1, G.2 and G.3.0.

4.	The UL Reference Measurement channels are set according to the applicable Table in clause 6.2D.3.4.1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2D.3.4.3.

<Unchanged Skipped>

6.2D.3.4.3.24	Message contents exceptions for network signalling value "NS_47"

1.	Information element additionalSpectrumEmission is set to NS_47. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.24-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_47"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_47) |  |  |

6.2D.3.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from table 6.2D.3.5-1 to table 6.2D.3.5-19. The allowed A-MPR values specified in table 6.2.3.3.1-1 are in addition to the allowed MPR requirements specified in clause 6.2.2.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2D.1.3-1 apply.

<Unchanged Skipped>

Table 6.2D.3.5-26: UE Power Class 1.5 test requirements (NS_47) for n41

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 6.5 | 13 | 0 | 16 | 5 | 3 | 31+TT | 11.0-TT |
| 2 | 29 | 0 | 2 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 3 | 29 | 0 | 6.5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16.0-TT |
| 4 | 29 | 0 | 4.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 5 | 29 | 0 | 4.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 6 | 29 | 0 | 4.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 7 | 29 | 0 | 4.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 8 | 29 | 0 | 4.5 | 4 | 0 | 24.5 | 3 | 3 | 31+TT | 21.5-TT |
| 9 | 29 | 0 | 6.5 | 13 | 0 | 16 | 5 | 3 | 31+TT | 11.0-TT |
| 10 | 29 | 0 | 2.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 11 | 29 | 0 | 6.5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16.0-TT |
| 12 | 29 | 0 | 4.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 13 | 29 | 0 | 4.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 14 | 29 | 0 | 4.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 15 | 29 | 0 | 4.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 16 | 29 | 0 | 4.5 | 4 | 0 | 24.5 | 3 | 3 | 31+TT | 21.5-TT |
| 17 | 29 | 0 | 6.5 | 13 | 0 | 16 | 5 | 3 | 31+TT | 11.0-TT |
| 18 | 29 | 0 | 4.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 19 | 29 | 0 | 6.5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16.0-TT |
| 20 | 29 | 0 | 5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 21 | 29 | 0 | 5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 22 | 29 | 0 | 5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 23 | 29 | 0 | 5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 24 | 29 | 0 | 5 | 0 | 0 | 24 | 3 | 3 | 31+TT | 21.0-TT |
| 25 | 29 | 0 | 8.5 | 13 | 0 | 16 | 5 | 3 | 31+TT | 11.0-TT |
| 26 | 29 | 0 | 8.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 27 | 29 | 0 | 8.5 | 8 | 0 | 20.5 | 6 | 3 | 31+TT | 14.5-TT |
| 28 | 29 | 0 | 8.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 29 | 29 | 0 | 8.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 30 | 29 | 0 | 8.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 31 | 29 | 0 | 8.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 32 | 29 | 0 | 8.5 | 0 | 0 | 20.5 | 6 | 3 | 31+TT | 14.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-27: UE Power Class 1.5 test requirements (NS_47) for n41 (large FWA form factor with 20 dB or above antenna isolation)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 6.5 | 13 | 0 | 16 | 5 | 3 | 31+TT | 11.0-TT |
| 2 | 29 | 0 | 1.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 3 | 29 | 0 | 6.5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16.0-TT |
| 4 | 29 | 0 | 4 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 5 | 29 | 0 | 4 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 6 | 29 | 0 | 4 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 7 | 29 | 0 | 4 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 8 | 29 | 0 | 4 | 4 | 0 | 25 | 3 | 3 | 31+TT | 22.0-TT |
| 9 | 29 | 0 | 6.5 | 13 | 0 | 16 | 5 | 3 | 31+TT | 11.0-TT |
| 10 | 29 | 0 | 2 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 11 | 29 | 0 | 6.5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16.0-TT |
| 12 | 29 | 0 | 4 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 13 | 29 | 0 | 4 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 14 | 29 | 0 | 4 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 15 | 29 | 0 | 4 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 16 | 29 | 0 | 4 | 4 | 0 | 25 | 3 | 3 | 31+TT | 22.0-TT |
| 17 | 29 | 0 | 6.5 | 13 | 0 | 16 | 5 | 3 | 31+TT | 11.0-TT |
| 18 | 29 | 0 | 4 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 19 | 29 | 0 | 6.5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16.0-TT |
| 20 | 29 | 0 | 4.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 21 | 29 | 0 | 4.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 22 | 29 | 0 | 4.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 23 | 29 | 0 | 4.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 24 | 29 | 0 | 4.5 | 0 | 0 | 24.5 | 3 | 3 | 31+TT | 21.5-TT |
| 25 | 29 | 0 | 7.5 | 13 | 0 | 16 | 5 | 3 | 31+TT | 11.0-TT |
| 26 | 29 | 0 | 7.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 27 | 29 | 0 | 7.5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16.0-TT |
| 28 | 29 | 0 | 7.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 29 | 29 | 0 | 7.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 30 | 29 | 0 | 7.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 31 | 29 | 0 | 7.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 32 | 29 | 0 | 7.5 | 0 | 0 | 21.5 | 5 | 3 | 31+TT | 16.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

<End of Changes>