3GPP TSG RAN Meeting #108							               RP-251143

Prague, Czech Republic, Jun 9-13, 2025

## Status Report to TSG

Agenda item:			10.3.3

| WI / SI Name | New WID on LTE-based 5G Broadcast Phase 2 |  |  |  |
| --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes | Testing part: No |
| Acronym | LTE_terr_bcast_Ph2 |  |  |  |
| Unique ID | 1060081 |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-250794 |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 09/2025 | Performance part: 12/2025 | Testing part: N/A |
| Overall Completion level | Study Item: | Core part: 95% | Performance Part: 0% | Testing part: N/A |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Assunta De Vita |
|  | Company | EBU |
|  | Email | assunta.devita@rai.it |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements from RAN1#120-bis

### 2.1.1.1 Time-frequency interleaver design for MBMS dedicated cells

Agreement

Support at least the following values of time interleaver depth:

- Values M={4,8,16,32} and N={1,2,4,8,16} are supported.

- FFS: UE behaviour in case values of M and N exceed UE capabilities in terms of soft buffer size and maximum TBS supportable in a TTI

- FFS: larger values, including M=64, N={32,64}.

Agreement

In case of time interleaving, in order to reduce the resource wastage due to the potential mismatch between MSI periodicity and the (MxN) pattern:

- Adopt an extended set of configurable periodicities for MCH scheduling period for R19 PMCHs that includes the intermediate values among the periodicities defined for the legacy configurations.

  - May include the case of scheduling different M/N within the same MSI periodicity

  - FFS the values of the new periodicities.

- RAN1 to specify UE behavior, if needed, in case of residual mismatch even after new periodicities are adopted, taking into account the amount of subframe available at the end of the periodicity

  - Option 1: Drop the time-interleaved TBs which do not have all the N RVs transmitted within the MSI periodicity

  - Option 2: Drop the time-interleaved TBs which could not be successfully decoded.

    - It is up to network to decide whether to schedule or not schedule MTCH transmission in the residual mismatch part of the MSI periodicity. UE knows this from MSI (legacy behaviour)

  - Other Options not precluded

Agreement

The number of columns, C, of the frequency interleaver is C= #CBs Nsymb ×m, where #CBs denotes the number of CBs in the TB (including scaling, if any), Nsymb denotes the number of OFDM symbols of the PMCH,

    - Working assumption:

      - m = Nsymbgcd(Nsymb,#CBs), and gcd(.) denotes the operation to compute the greatest common divisor.

Agreement

The following steps are applied to the rows of the frequency interleaver:

- Perform inter-row permutation with a reorder of the row indexes Ri', which is calculated as:

Ri'=R'i-1+d########################,          R'i-1≤R-d-1 #i×dR#,                 else.

With, 1≤i≤R-1, R'0=0, and d=R/R/RC-C+1.

- Perform a cyclic shift to the elements in the j-th row with an offset S(j) defined as:

Sj=modj+12×-1j,C, j=0,1,2,…,R-1

Agreement

LBRM parameters (e.g. NIR) are consistent between the broadcast network and the (multiple) UEs that are receiving the broadcast transmission.

- These multiple UEs may belong to different UE categories, thus having different values of Nsoft, NIR.

- FFS: how the consistent NIR and NCB is calculated

Agreement

The agreement in the previous meeting is updated as follows:

Regarding configuration for time interleaving transmission, the following options can be considered:

- Option 1: Per PMCH configuration, e.g., via pmch-Config

- Option 2: Per MBMS session, e.g., via MBMS-SessionInfo or via MAC CE

- Alt1: If per MBMS session configuration is done, a T_proc of 3 ms between the end of the MSI and the first MTCH in the MCH scheduling period is needed for UE de-rate matching.

- Alt2: If per MBMS session configuration is done, other options (than Alt1 above) are not precluded, e.g. that allow the UE to know the M, N values of the first MTCH scheduled after the MSI

Agreement

Obtain the initial TBS (𝑇𝐵𝑆′) as per current specifications, and calculate the final TBS as 𝑇𝐵𝑆=𝑟𝑜𝑢𝑛{𝑇𝐵𝑆′×𝑁}

- The operation “round{x}” is mapping x to the closest TBS from all the existing TBS values in section 7.1.7.2 TS 36.213.

#### 2.1.2	Agreements from RAN1#121

### 2.1.1.2 Time-frequency interleaver design for MBMS dedicated cells

Agreement

For each CB of the TB, k0n=2RsubblockTC+rvnEmin, where Emin=NLQmG'/C (as per TS 36.212), and rvn∈{0…N-1} is the redundancy version of the n-th subframe, where NL=1, and, rvn values are mapped sequentially to subframes, i.e., rvn=n.

Agreement

Time interleaving parameters are indicated per PMCH configuration, e.g., via a configuration of M,N in pmch-Config. Specify the following set of new MSI periodicities: {rf7, rf14, rf53, rf56, rf112, rf424}.

- Additionally down-select among {rf27 or rf28}, {rf106 or rf108}, {rf211 or rf212}.

- FFS actions on the residual mismatch after applying the new MSI periodicities in conjunction with the configuration of M,N

  - Option 1: Fill the residual space with a service with different TFI configuration (M,N), e.g., via an additional (addressing the residual space) configuration via pmch-Config

Agreement

The soft buffer size for one time-interleaved TB is down-selected from

- Option 1:  NIR=βNsoftM, where:

  - Option 1a: Nsoft is the soft buffer size pertaining to a reference UE category, M is the number of TBs for time-interleaving and β is a scaling factor. FFS: exact value/values/configuration of β.

  - Option 1b: Nsoft is configured in PMCH-Config from {1827072, 3654144, 5481216, 7308288, 9744384, 12789504, 14616576, 17052672, 19488768, 24360960, 29233152, 34105344, 36541440, 38977536, 42631680, 47431680, 303562752}, 1β is configured in PMCH-Config from: {1, 3/2, 6/5, 8/5, 2, 12/5, 8/3, 3, 5, 32};  and M is the number of TBs for time-interleaving.

- Option 2 : The soft buffer size for the code block is denoted as Ncb=min(TBSLBRMC∙RLBRM,Kw), where C and Kw are same as defined in TS 36.212 clause 5.1. TBSLBRM is calculated based on, e.g., nPRB,LBRM, ITBS, N. RLBRM=2/3.

  - FFS: Details of determining TBSLBRM, nPRB,LBRM, ITBS (e.g., based on a notion of maximum coding rate) may need to be considered, to provide the flexibilities afforded by the LTE specifications, keeping in mind the LTE UE categories.

- FFS: impact to soft buffer size for UE receiving unicast considering the case where UE prioritizes receiving broadcast.

Agreement

Confirm the working assumption in the following agreement:

Agreement

The number of columns, C, of the frequency interleaver is C= #CBs Nsymb ×m, where #CBs denotes the number of CBs in the TB (including scaling, if any), Nsymb denotes the number of OFDM symbols of the PMCH,

- Working assumption:

  - m = Nsymbgcd(Nsymb,#CBs), and gcd(.) denotes the operation to compute the greatest common divisor.

Agreement

For PMCH with time interleaving, the maximum TBS a UE supports for the scaled TB is down-selected from the following:

- Option 1: the largest TBS among the multiple maximum TBS of a DL-SCH as per TS 36.306 set by UE category.

- Option 2: the smallest TBS among the multiple maximum TBS of a DL-SCH as per TS 36.306 set by UE category.

- Option 3: signalled by the UE from the multiple maximum TBS of a DL-SCH as per TS 36.306 set by UE, e.g. using a new UE capability

Agreement

Add the following MSI periodicities to the previous RAN1 agreement {rf28, rf108, rf212}.

Agreement

The residual space created by the mismatch between (M,N) and the MSI periodicity may be filled with the last MTCH service with different TFI configuration (M,N), e.g., via an additional (addressing the residual space) optional configuration via pmch-Config.

- In the space before mismatch, all MTCHs within the MSI period have the same (M,N) obtained via the configuration of M,N in pmch-Config.

Agreement

The soft buffer size for one time-interleaved TB is down-selected from

- Option 1:  NIR=βNsoftM, where:

  - Option 1a: Nsoft is the soft buffer size pertaining to a reference UE category, M is the number of TBs for time-interleaving and β is a scaling factor configured from network among values in the interval [0 1]. FFS: the exact values in the interval.

  - Option 1b: Nsoft is configured in PMCH-Config from {1827072, 3654144, 5481216, 7308288, 9744384, 12789504, 14616576, 17052672, 19488768, 24360960, 29233152, 34105344, 36541440, 38977536, 42631680, 47431680, 303562752}, 1β is configured in PMCH-Config from: {1, 3/2, 6/5, 8/5, 2, 12/5, 8/3, 3, 5, 32};  and M is the number of TBs for time-interleaving.

- FFS: impact to soft buffer size for UE receiving unicast considering the case where UE prioritizes receiving broadcast.

Agreement

The number of subframes used for time-interleaving derived from ‘stop MTCH (x+1)’ is expected to be an integer multiple of MxN.

- FFS: spec impact, if any

Agreement

Cyclically shift the bit sequence b0, b1, …bMbit-1 in Section 6.3.1 of TS 36.211 for the ith subframe of the time-interleaved TB by Xi bits, where

- i∈{0,1,…,N-1},

- N denotes the number of subframes to which the time-interleaved TB is mapped

- Xi  is

  - Alt 1: ∑#CBs-1r=#CBs-SiEr, with Si=(i×α) mod #CBs

    - Er is the number of bits in the rth codeblock within a subframe (as defined in TS 36.212)

    - α=1

  - Alt 2:  ∑#CBs-1r=#CBs-SiEr, with Si=(i ×α) mod #CBs

    - Nsymb denotes the number of OFDM symbols within a subframe

    - #CBs denotes the number of CBs in the time-interleaved (scaled) TB

    - Er is the number of bits in the rth codeblock within a subframe (as defined in TS 36.212)

    - α= #CBsN×Nsymb

A new RRC parameter configures between α=1 and α= #CBsN×Nsymb as above

    - If this new RRC parameter is not configured, the cyclic shift described in this agreement is not enabled.

    - A new optional UE capability is introduced to indicate support of this cyclic shift.

    - FFS: Extension of the bit-field corresponding to this RRC parameter by one additional bit, based on NOTE 1 below

  - NOTE 1: The following alternative for Xi may be considered, exclusively based on BLER-vs-SNR performance comparison (with respect to Alt 1 / Alt 2)

    - Xi=Ai×floor(NSC ×QmN), with Ai∈{0,…N-1}

    - Where NSC  is the number of subcarriers available in 1 OFDM symbol for PMCH, e.g.  NSC =Mbit/(QmNsym)

Agreement

The soft buffer size for one time-interleaved TB is calculated as NIR=βNsoftM, where Nsoft is the soft buffer size configured according to a reference UE category, indicated from among { DL Cat4, DL Cat5, … DL Cat26}, M is the number of TBs for time-interleaving and β is configured from: {1/32, 1/5, 1/3, 3/8, 5/12, 1/2, 5/8, 5/6, 2/3, 1}.

Agreement

Rel-19 IE in MBMS Interest Indication message carries following parameters utilized for receiving Rel-19 LTE-based 5G Broadcast service(s):

- Frequency

- New values of sub-carrier spacing = {2.5 kHz, 1.25 kHz}

- New values for bandwidth = {n30, n35, n40}

- A new UE baseband resource Scaling factor (e.g., utilized soft buffer at the UE) used for receiving the time interleaved broadcast

Agreement

In case the time interleaved PMCH transmission (as configured) exceeds a UE’s capability(/es) in terms of soft buffer size or maximum TBS supported in a TTI, the UE is not expected to receive the corresponding time interleaved PMCH transmission.

Agreement

Update of the FFS of a previous agreement:

Agreement

The number of subframes used for time-interleaving derived from ‘stop MTCH (x+1)’ is expected to be an integer multiple of MxN.

Spec impact is up to RAN2.

Agreement

RAN1 asks RAN2 whether/how to address the HARQ handling in the MAC specification for the Rel-19 time-interleaved PMCH transmission with M TBs and N RVs.

Agreement

For clarification to RAN2:

When time-interleaving is enabled, values of N for configuration/indication do not include N=1. When N is not configured, time-interleaving is disabled i.e. N=1.

#### 2.1.2	Remaining Open issues

None

## 2.2	RAN2

#### 2.2.1	Agreements from RAN2#130

### 2.2.1.1 Signalling impacts to support time-frequency interleavers

Agreements

- RAN2 assumes the following needs to be specified in RRC, but the final parameters/values/structure/terminology needs to be confirmed based on L1 parameters list from RAN1:

  - Follow RAN1 agreement (i.e. per PMCH configuration) on the configuration for time interleaving transmission.

  - Wait for RAN1 for the candidate values of the new MCH scheduling period(s).

  - Introduce a field to indicate TBS scaling factor (i.e. N) with candidate values {2,4,8,16,ffs}.

  - Introduce a field to indicate the separation between two transmissions of the same TB (i.e. M) with candidate values {4,8,16,32,ffs}.

  - Presence of the parent IE containing fields for N and M indicates time interleaving is enabled.

  - FFS (pending RAN1 decision):  Introduce a flag to indicate whether frequency interleaving for MTCH is enabled.

- We aim to support co-existence of legacy (pre-Rel19) and R19 transmission and UEs by defining R19 PMCHs to cater to Time Interleaving based configurations, scheduling and transmissions in addition to legacy PMCHs. FFS exact signalling

- RAN2 to adopt and specify minimal multiplexing enhancements for MCH reception that are applied only for scheduling of R19 PMCHs based MTCHs and may include, e.g.:

  - Not allowing multiplexing of two MTCHs in same subframe

  - Not applying Time interleaving to subframe carrying MSI/eMSI/MCCH

  - Not allowing multiplexing of MTCH with MSI/eMSI/MCCH in a sub-frame due to Time interleaving difference

  - FFS (pending RAN1 discussion):	Inserting and/ interpreting padding to account for remaining portion of the subframe in above scenarios

- FFS cell reselection impact (e.g. need to understand whether there is a practical scenario where some changes would be needed)

#### 2.2.2	Remaining Open issues

Finalize signalling for time-frequency interleaving.

## 2.3	RAN3

#### 2.3.1	Agreements from RAN3#128

### 2.3.1.1   Signalling Support

Agreements

MCE decides the TFI and the configuration of TFI, and indicates the configuration of TFI to the eNBs.

Regarding parameters configuration for TFI, per PMCH configuration is adopted in RAN1.

#### 2.3.2	Remaining Open issues

Finalize signalling for time-frequency interleaving.

## 2.4	RAN4

#### 2.4.1	Agreements

#### 2.4.2	Remaining Open issues

Specify performance requirements for time-frequency interleaving.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1#120

- R1-2500872	Time-frequency interleaver design for MBMS dedicated cells	Samsung

- R1-2501057	Introduction of WI “Time and Frequency Interleaving for LTE-based 5G Terrestrial Broadcast” 	EBU

- R1-2500148	Discussion on time-frequency interleaver design for LTE-based 5G Broadcast	ZTE Corporation, Sanechips

- R1-2500150	On time-frequency interleavers for LTE-based 5G Broadcast	Huawei, HiSilicon

- R1-2500582	Design of Time and Frequency Interleaver for LTE-based 5G Broadcast	Shanghai Jiao Tong University

- R1-2501058	Details of Time and Frequency Interleaving for LTE-based 5G Terrestrial Broadcast 	EBU

- R1-2501179	Time and Frequency Interleaving for 5G Broadcast	Qualcomm Incorporated

RAN1#120-bis

- R1-2502548	Higher layer parameters for Rel-19 LTE-based 5G Broadcast after RAN1#120	EBU

- 9.13.1	Time-frequency interleaver design for MBMS dedicated cells

- R1-2501912	Discussion on time-frequency interleaver design for LTE-based 5G Broadcast	ZTE Corporation, Sanechips

- R1-2502209	On time-frequency interleavers for LTE-based 5G Broadcast	Huawei, HiSilicon

- R1-2502390	Views on LTE-based 5G broadcast	Samsung

- R1-2502546	Further details on open issues regarding TFI for LTE-based 5G Broadcast 	EBU

- R1-2502582	Design of Time-frequency interleaver for 5G Broadcast	Shanghai Jiao Tong University

- R1-2502861	Time and Frequency Interleaving for 5G Broadcast	Qualcomm Incorporated

RAN1#121

- R1-2503249	On time-frequency interleavers for LTE-based 5G Broadcast	Huawei, HiSilicon

- R1-2503332	Discussion on time-frequency interleaver design for LTE-based 5G Broadcast	ZTE Corporation, Sanechips

- R1-2503589	Further details on Time and Frequency Interleaving for 5G Broadcast phase-2	Samsung

- R1-2504241	Further discussion on open issues regarding TFI for LTE-based 5G Broadcast	EBU

- R1-2504417	Time and Frequency Interleaving for 5G Broadcast	Qualcomm Incorporated

- R1-2504654	Time and Frequency Interleaving for 5G Broadcast	Qualcomm Incorporated

- R1-2503609	Further discussion on Time-frequency interleaver for 5G Broadcast Phase 2	Shanghai Jiao Tong University

- R1-2504652	Higher layer parameters for Rel-19 LTE-based 5G Broadcast after RAN1#120	EBU

RAN2#130

- R2-2504260	RAN2 impacts to support time-frequency interleavers for LTE-based 5G Broadcast Phase 2	Qualcomm Incorporated, EBU

- R2-2503506	Configuration and scheduling aspects for LTE-based 5G Broadcast Phase 2  Samsung

- R2-2504075	Discussion on time-frequency interleavers for MBMS	Huawei, HiSilicon

RAN3#128

- R3-253182   Consideration on LTE-based 5G Broadcast Phase 2  Huawei

- R3-253183  Introduction of LTE-based 5G Broadcast Phase 2  Huawei

- R3-253253  Discussion on LTE-based 5G Broadcast  ZTE Corporation

- R3-253254  TPs for Signalling Support of LTE-based 5G Broadcast  ZTE Corporation