3GPP TSG RAN4 Meeting #08	RP-251158

Prague, Czech Republic, June 9-13, 2025

Agenda item:		9.4.3.4

Source:		Union Inter. Chemins de Fer

Title:		HPUE in bands n100 and n101

Release:		19

Work Item:	HPUE_NR_FR1_bands_R19-Core

Document for:		Discussion

# Introduction

This contribution discusses the railway FRMCS needs for using HPUE in combination with CA/DC and UL MIMO in the bands n100 and n101 for FRMCS cab-radios, i.e. user equipment installed on trains with external antenna on top of the train roof at approximately 4m over ground (i.e. FWVM devices) .

The contribution proposes to extend the existing WID RP-241932 HPUE_NR_FR1_bands_R19-Core with support of PC2 in band n100 and PC2 and PC1.5 in band n101 for FWVM devices as a necessary step prior to addressing HPUE with CA/DC and UL MIMO. It also proposes an approach to address in the TS 38.101-1 the EU regulatory requirement for 37dB ACLR.

# Discussion

Railway operators need to have the ability with FRMCS cab-radios, to deploy CA/DC using both n100 and n101, and/or UL MIMO in n100 and n101 to achieve improved data throughput at desired locations. Currently in RAN4 both CA/DC and UL MIMO are being defined with WID NR channel BW less than 5MHz for FR1 Phase 2 [1], respectively WID NR_bands_xFeature_R19-Core [2], for PC3 transmit power in bands n100 and n101.

To extend the coverage area where FRMCS cab-radios can beneficially make use of CA/DC and UL MIMO it is necessary to enable the use of CA/DC and UL MIMO in combination with a HPUE power class in bands n100 and n101.

Observation 1: railway operators have a need for CA/DC and UL MIMO with HPUE support in bands n100 and n101 for FRMCS cab-radios, i.e. user equipment installed on trains with external antenna on top of the train roof at approximately 4m over ground.

For the railway FRMCS frequency bands n100 and n101, single carrier PC1 transmit power has been introduced in R18 with WID LTE_NR_HPUE_FWVM_REL18 [3] for FRMCS cab-radios.

To maximize the coverage area and throughput especially at cell edge, for CA/DC and UL MIMO the highest possible transmit power level should be available as that allows the largest basestation intersite distance. This significantly reduces the implementation cost of the FRMCS network infrastructure.

Observation 2: The use of HPUE at the highest possible transmit power level with CA/DC and UL MIMO is essential to reduce the implementation cost of the FRMCS network infrastructure.

Hence the use of PC1 component carriers is preferable. However, UIC understands that, due to constraints within the current 3GPP specifications, it is not possible to support CA/DC or UL MIMO using PC1 in both bands n100 and n101. However, specifying the use of CA/DC and UL MIMO with PC2 in FDD band n100, and PC2 and PC1.5 in TDD band n101 should be possible. A necessary first step for this is to define support of single carrier PC2 in band n100, and PC2 and PC1.5 in band n101.

Observation 3: Current 3GPP specifications do not allow specifying the use of PC1 for CA/DC or UL MIMO in bands n100 and n101.

Observation 4: To enable specifying support of CA/DC and UL MIMO with PC2 and PC1.5 HPUE, a necessary first step is to define for FRMCS cab-radios support of single carrier PC2 in FDD band n100, and PC2 and PC1.5 in TDD band n101.

Within the current RAN4 work the existing WID RP-241932 HPUE_NR_FR1_bands_R19-Core [4] addresses PC2 and PC1.5 HPUE. Its stated objectives are:

- Objective 1.  For TDD band: PC1.5 and PC2 for handheld UE and FWA, and PC1 for FWVM

- Objective 2.  For FDD band: PC2 for handheld UE and FWA, and PC1 for FWVM

From these objectives it can be seen that PC2 and PC1.5 power classes in bands n100 and n101 could be supported by this WID [4]. However, the FRMCS cabradio is considered to be a FWVM device in Ref. [3], whereas the current WID [4] only covers handheld UE and FWA for PC2 and PC1.5.

Observation 4: The current WID RP-241932 HPUE_NR_FR1_bands_R19-Core [4] could support the railway FRMCS needs for single carrier support of PC2 in FDD band n100 and PC2 and PC1.5 in TDD band n101..

Observation 5: The current WID [4] does not enable PC2 and PC1.5 support for FWVM devices such as FRMCS cabradios.

FRMCS cabradios must comply with the European spectrum regulation in DEC(20)02 [5] that requires a 37dB ACLR. Current TS 38.101-1 [6] defines for PC2 and PC1.5 devices an ACLR of 31dB.

Observation 6: Cabradios are required to meet a 37dB ACLR whereas TS 38.101-1 [6] defines for PC2 and PC1.5 devices an ACLR of 31dB.

FRMCS cabradios currently being developed by industry are constructed as a single radio module incorporating a PA and spectrum filtering that is able to achieve the PC1 transmit power level and 37dB ACLR.

For the introduction of PC1 HPUE in bands n100 and n101, as stated in TR37.829 [7],  there was no need for REFSENS exception, general MPR can be used and no A-MPR is necessary as there are no additional regional requirements which are more stringent than current 3GPP requirements for PC1. Hence only the general MPR table of TS 38.101-1 [6] table 6.2.2 is applicable.

This same PC1 radio module will be used to implement cabradios for PC2 or PC1.5 with 37dB ACLR. The practical use case is where a train uses its PC1 HPUE with single carrier in n100 or n101, and, under certain situations requiring higher data throughput, needs to use CA/DC or UL MIMO. In such case the same HPUE module would use power class PC2 or PC1.5 to achieve CA/DC or UL MIMO.

Observation 7: Cabradio implementation for PC2 and PC1.5 will be the same as for PC1 with PA and spectrum filtering that meets 37dB ACLR.

Observation 8: For cabradios in bands n100 and n101 using PC2 and PC1.5 no REFSENS exceptions are defined and only general MPR table of TS 38.101-1 [6] table 6.2.2 is applicable.

# Conclusion

Proposal 1: Based on the above observations, it is proposed to extend WID HPUE_NR_FR1_bands_R19-Core [4] with support for FWVM of PC2 for FDD bands and PC2 and PC1.5 for TDD bands.

Proposal 2: Based on the above observations, it is proposed to add the support of PC2 in FDD band n100 and PC2 and PC1.5 in TDD band n101 to WID HPUE_NR_FR1_bands_R19-Core [4].

Proposal 3: Based on the above observations, it is proposed to add to Table 6.2.1-1: UE Power Class:

“NOTE 9 : PC2 in band n100 and PC2 and PC1.5 in band n101 is only allowed for FRMCS cab-radios (FWVM) under the assumption of implementation based on PC1 HPUE at PC2 and PC1.5 transmit power levels with 37dB ACLR.”

# References

[1]	RP-240832 NR channel BW less than 5MHz for FR1 Phase 2

[2]	RP-242572 NR_bands_xFeature_R19-Core

[3]	RP-223424 LTE_NR_HPUE_FWVM_REL18

[4] 	RP-241932 HPUE_NR_FR1_bands_R19-Core

[5]	ECC Decision (20)02

[6] 	TS 38.101-1

[7] 	TR 37.829