3GPP TSG RAN Meeting #108					RP-251167

Prague, Czech Republic, June 9-13, 2025

Agenda Item:	16.1

Source:	Ericsson, AT&T

Title:	Requirements for supporting aerial UEs in 6G

Document for:	Approval

# 1	Introduction

In this contribution, we discuss the requirements for the 6G Radio and the 6G Radio architecture for supporting aerial UEs.

# 2	Discussion

3GPP networks have supported connectivity for aerial UEs in 4G and 5G. In 4G, the support of aerial UEs was added several releases after the introduction of the generation. In 5G, some support was included from the first release (e.g., uplink power control parameters introduced in 4G for aerial UEs), with some later additions in Rel-18 for uncrewed aerial vehicles (UAVs) and air-to-ground (ATG). We believe that the 6G should also fulfill at least the corresponding requirements.

The 6G Radio should support aerial use cases with the following characteristics:

- Types of aerial vehicles: both UAVs (including various types of drones with different sizes and weights) and crewed aerial vehicles (e.g., air taxis, helicopters, commercial aircraft).

- Altitude: both low altitudes (up to about 300 m above ground level), medium altitudes (from 300 m to 3000 m above ground level), and high altitudes (3 to 16 km).

- UE speeds: all speed ranges corresponding to the types of aerial vehicles mentioned above, namely a few km/h or few tens of km/h for UAVs; up to hundreds of km/h for air taxis and helicopters; up to above 1000 km/h for commercial aircraft; and beyond for high performance aircrafts.

- Types of data traffic: there will be a need for both high-reliability but low data-rate traffic for use cases such as command-and-control of drones and high data rate MBB-type of traffic for UL-heavy use cases such as 4K-video surveillance using drones or for DL-heavy use cases such as in-flight Internet connectivity for commercial aircraft.

Providing connectivity to aerial UEs poses significant challenges compared to serving terrestrial UEs due to the differences in radio propagation conditions, mobility patterns, etc. Moreover, aerial UEs will be served by public networks that are also used by non-aerial users. Thus, managing the interference generated and received by aerial users is of high importance.

In addition, the airspace use cases are tightly regulated, including specific requirements on the types of services that must be provided (e.g., UAV identification, etc.) and dedicated spectrum regulations. The 6G radio should comply with these regulations.

To allow for monetization of connectivity to aerial users and to ensure that the other requirements mentioned above can be met, it is crucial that the 6G Radio network can detect and identify airborne users.

In terms of deployments, both dedicated deployments (e.g., macro deployment with antennas tilted upwards) and shared deployments (i.e., deployments designed for non-airborne UEs but also used by airborne UEs) should be considered.

The 6G architecture should allow for serving aerial UEs using public networks (e.g., operated by a public CSP) as well as dedicated networks (e.g., for critical communications).

# 3 Text proposal

Based on the discussion above we propose the following text proposal.

## 5.x Support for aerial UEs in 6G The 6G Radio shall provide coverage and high-capacity connectivity for airborne UEs while fulfilling aviation and aerial-related spectrum regulations. The 6G Radio shall allow for serving aerial UEs with both dedicated and non-dedicated deployments, with both public and private networks. The 6G Radio and 6G RAN architecture shall support detection, identification, and management of airborne UEs, including interference (to/from the networks, to/from land-based UEs, among aerial UEs, etc).

# 4	Conclusion

In this contribution we propose to add the text proposal in section 3 to TR 38.914.