3GPP TSG RAN Meeting #108          						RP- 251196

Prague, Czech Republic, June 9-13, 2025

Source:	Tejas Networks Ltd.

Title:	Enhancements for solutions for Ambient IoT (Internet of Things) in NR

Agenda item:	15.1.3

Document for:     Discussion and Decision

# Introduction

The existing IoT technology is not suitable in most of the target use cases as mentioned in TR 22.840 [1]. Hence, 3GPP Rel-18 has conducted a study to find a new IoT technology called Ambient IoT which is suitable for the use cases mentioned in TR 22.840 [1]. The Ambient IoT devices are ultra-low power consuming and ultra-low complex making them suitable for very low-end IoT applications. The complexity and power consumption of Ambient IoT Device 1 and 2a are comparable to the existing RFID technology, whereas the number of connections and/or device density is higher than the Narrow Band-IoT (NB-IoT). As per the deployment scenario mentioned in TR38.848 [2], the Ambient IoT devices can communicate with the NR/LTE base stations (BSs) due to their large coverage range and harvest energy from such nearby BS or user equipment (UE) to execute the operation of the Device.

# Rel-19 Ambient IoT Study and Motivation for Enhancement in Rel-20

In Rel-19, two deployment topologies are studied as shown in Fig.1. Rel-19 Ambient IoT technology is low power consuming and cost effective indoor IoT technology, which supports massive connectivity with better interference management then RFID. However, Rel-19 work item for A-IoT specifies the protocols for Device 1 with topology 1 and the coverage is limited to 15 meters [3].

Fig. 1a: D1T1 - Device indoor, BS indoor                     Fig. 1b: D2T2 - Device outdoor, BS indoor

The Rel-19 Ambient IoT (A-IoT) work item includes the following:

- Topology 1 for Device 1 only

- External carrier wave transmitter (CWT) for backscatter communication

- Physical layer uplink and downlink message signals are defined for handling multiple devices by one reader.

- Coverage is limited to 15 meters only for Device 1 with topology 1.

- Due to limited power budget of Device 1, the clock accuracy is low, which increases the R2D signaling overhead for clock calibration.

However, the study in TR 38.769 shows that a higher coverage of up to 40-50m can be achieved with Device 2b. The distance can be further increased by considering an intermediate node which can act as a reader between the device and base station in D2T2 topology. Moreover, higher clock accuracy of Device 2b reduces the midamble overhead and improves the data rate.

# Proposals for Enhancement of Ambient IoT in Rel-20

[RAN1-led]: Ambient IoT Proposal

# Proposal 1.1 – CW inside topology: CW inside topology is more practical than CW outside topology, as the on-demand availability of external CW transmitter is not always feasible for CW outside topology. Thus, CW inside topology should be studied for both D1T1 and D2T2 topology with Device type 1.

# Proposal 1.2 – Specify Pre/mid/post-ambles for Device 2b:  Higher complexity and power budget of Device 2b provides higher coverage and clock accuracy compared to Deice 1. Thus, the specification requirement of Pre/mid/post-ambles for Device 2b is not same as Device 1, which should be defined for both Topologies 1 and 2.

# Proposal 1.3 – Charging cycle of Device 1: As Device 1 is fully powered by RF energy harvesting, a charging period before data transmission period should be defined for Device 1 for both Topologies 1 and 2.

Proposal 1.4 – Sleep mode for Device 2: Device 2b can be designed with higher device complexity than Device 1 and thus could potentially support timer based functions. This opens up possibility of duty cycle based message reception at device side. RAN1/RAN2 can study sleep mode for Device 2b and duty cycle based operation in Rel-20.

Proposal 1.5 – A-IoT for outdoor applications: Enhancement of Ambient IoT in Rel-20 should study the A-IoT solutions for outdoor applications including sensing, positioning, and tracking use cases.

[RAN2-led]: Ambient IoT Proposal

# Proposal 2.1 – Coexistence of release 19 and Release 20 devices: RAN2 WGs to design message formats to be interoperable with release 19 devices and a deployment case of a release 20 capable reader being able to serve both rel19 and rel20 devices.

# Proposal 2.2 – Handling of Multi-reader scenario:  It was agreed in release 19 to no address the case of device operating within the coverage of multi-reader but in most practical deployment scenarios, this cannot be guaranteed. In order to have a useful deployment, Release 20 of AIoT specification should address the A-IoT devices handling of messages received from multiple readers.

# Proposal 2.3 – Study procedures to support DO-A capable AIoT devices:  SA2 has agreed on the WT#1 [4] to study the support of DO-A (Device Originated Autonomous Access) capable AIoT devices. RAN1 and RAN2 WGs should study the reader discovery and initial access procedure for such devices.

# Conclusions:

[RAN1 led objectives]:

- Specify impact of CW inside topology deployment in both D1T1 and D2T2@ scenarios

- Specify the pre/mid/post amble requirements for Device type 2b

- Study impact of charging cycle on device availability in Device type 1

- Specify the sleep mode for Device type 2

- Study impact to AIoT physical layer processing to support outdoor deployment use case

[RAN2 led objectives]:

- Specify procedures to handle deployments with co-existence of Release 19 and Release 20

- Specify RAN2 procedures to handle devices under multiple reader coverage

- Study procedures to support DO-A capable devices

# References:

[1] TR 22.840, Study on Ambient power-enabled Internet of Things, V0.4.0.

[2] TR 38.848, Study on A-IoT (Internet of Things) in RAN, V18.0.0.

[3] TR 38.769, Study on solutions for Ambient IoT (Internet of Things), V1.1.0.

[4] S2-2506092, NEW SID on Architecture support of Ambient power-enabled Internet of Things-Phase 2, Fukuoka City, Fukuoka, JP.