3GPP TSG RAN Meeting #108                                                       RP-251216

Prague, Czech Republic, June 9-13, 2025

Source:	vivo

Title:	Discussion on IMT-2030 technical performance requirements (TPR)

Agenda Item:	16.1

Document for:	Discussion and Decision

Introduction

The objectives of the study item of Study on 6G Scenarios and requirements has been updated in RAN #107 [1] as follows:

This study item aims to

- investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs.

  - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN

- Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors.

- Develop 3GPP requirements for 6G Radio for improvement of existing services and for new services.

- Determine the applicability of legacy services to 6G Radio, and define radio requirements for these, as appropriate.

- Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least:

  - Ensure appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting

  - Energy efficiency and energy saving: both for network and device

  - Enhanced spectral efficiency

  - Enhanced overall coverage, focus on cell-edge performance and UL coverage

  - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz

  - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band

  - Target scalable and forward compatible design for diverse device types

  - Improved spectrum utilization and operations taking into account diverse spectrum allocations

  - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs

  - Aim at a harmonized 6G Radio design for TN and NTN, including their integration

  - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

- Define a time plan and steer work as appropriate for the RAN WGs during the 6G WG SI to deliver high-level decisions at least on the following areas:

  - Fundamental 6G radio design aspects: waveform, numerology, channel coding, etc…

  - Overall high-level aspects of 5G to 6G migration

  - RAN architecture and interfaces, including RAN-Core interface

  - Coordination of 6G AI/ML framework

This study item aims to

- investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs.

  - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN

- Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors.

- Develop 3GPP requirements for 6G Radio for improvement of existing services and for new services.

- Determine the applicability of legacy services to 6G Radio, and define radio requirements for these, as appropriate.

- Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least:

  - Ensure appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting

  - Energy efficiency and energy saving: both for network and device

  - Enhanced spectral efficiency

  - Enhanced overall coverage, focus on cell-edge performance and UL coverage

  - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz

  - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band

  - Target scalable and forward compatible design for diverse device types

  - Improved spectrum utilization and operations taking into account diverse spectrum allocations

  - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs

  - Aim at a harmonized 6G Radio design for TN and NTN, including their integration

  - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

- Define a time plan and steer work as appropriate for the RAN WGs during the 6G WG SI to deliver high-level decisions at least on the following areas:

  - Fundamental 6G radio design aspects: waveform, numerology, channel coding, etc…

  - Overall high-level aspects of 5G to 6G migration

  - RAN architecture and interfaces, including RAN-Core interface

  - Coordination of 6G AI/ML framework

Furthermore, in RAN #107 [2] and CC#1 [3], 3GPP TSG RAN has achieved some agreements on following proposals.

Proposal 1: Define positioning accuracy as a quantitative TPR

Proposal 2: Define Sensing-related capabilities as a TPR.

Proposal 3: define AI-related capabilities as qualitative TPR evaluated by inspection.

Proposal 4: Define energy efficiency as a TPR.

Proposal 5: For user experienced data rate, further discuss applicable deployment scenarios, and revisit the evaluation methodology before setting values for UL

Proposal 6: For bandwidth TPR, it is proposed to set a target value of 400MHz (same definition as IMT-2020)

- TBD on other TPR values

Proposal 7: Define TPR (e.g. joint/composite requirement) related to data rate, latency, reliability, capacity etc. as a candidate IMT-2030 TPR.

- Applicable usage scenario: immersive communication

The following proposals are agreed in CC#1 for IMT-2030 TPRs:

Proposal 2-3: Do not define security, interoperability as IMT-2030 TPRs, these can be covered by submission template for IMT-2030.

Proposal 2-4: Sustainability is covered by energy efficiency, do not define separate TPR for sustainability.

Proposal 1: Define positioning accuracy as a quantitative TPR

Proposal 2: Define Sensing-related capabilities as a TPR.

Proposal 3: define AI-related capabilities as qualitative TPR evaluated by inspection.

Proposal 4: Define energy efficiency as a TPR.

Proposal 5: For user experienced data rate, further discuss applicable deployment scenarios, and revisit the evaluation methodology before setting values for UL

Proposal 6: For bandwidth TPR, it is proposed to set a target value of 400MHz (same definition as IMT-2020)

- TBD on other TPR values

Proposal 7: Define TPR (e.g. joint/composite requirement) related to data rate, latency, reliability, capacity etc. as a candidate IMT-2030 TPR.

- Applicable usage scenario: immersive communication

The following proposals are agreed in CC#1 for IMT-2030 TPRs:

Proposal 2-3: Do not define security, interoperability as IMT-2030 TPRs, these can be covered by submission template for IMT-2030.

Proposal 2-4: Sustainability is covered by energy efficiency, do not define separate TPR for sustainability.

In this contribution, we express our views on the TPR including the definition of TPRs, applicable usage scenarios and the target values.

Discussion

Summary on TPR definition and target values

In this section, we summarize our views on candidate TPRs, including the definition, the target values and applicable usage scenarios. For some TPRs, the definition can be the same as the ones in IMT-2020, such as peak data rate, spectral efficiency etc. as summarized in Table 1.

Table 1 Proposed TPR values

| Candidate IMT-2030 TPR |  | TPR definition | Usage scenarios | View on target value(s) | Enhancement times to IMT-2020 | Moderator proposal in RAN#107 and Agreements in CC#1 |
| --- | --- | --- | --- | --- | --- | --- |
| Peak data rate (Gbit/s) |  | Same as IMT-2020 | IC | DL: 80 Gbit/s UL: 40 Gbit/s | 4x | Alternative 1: Not to define a requirement, e.g. Leave the value for peak data rate for the technology proponent to provide Alternative 2: Define a requirement with reasonable value |
| User experienced data rate (Mbit/s) |  | See chapter 2.2 | IC | Dense Urban-IC: DL: 400~600 Mbit/s: UL: TBD | DL: 4~6x UL: FFS | DL: TBD For UL: - RAN#107 agreement: For user experienced data rate, further discuss applicable deployment scenarios, and revisit the evaluation methodology before setting values for UL Proposal 1: Use system level simulation for the evaluation of UL user experienced data rate with no bandwidth scaling taking UE maximum output power into account. |
| Spectral efficiency | Peak spectral efficiency (bit/s/Hz) | Same as IMT-2020 | IC | DL: 60 Mbit/s UL: 30 Mbit/s | 2x | DL: [1.5-2.5x] of IMT-2020 UL: [1.5-2.5x] of IMT-2020 |
|  | Average spectral efficiency (bit/s/Hz/TRxP) |  | IC | TE; DL; UL. Indoor Hotspot-IC; 18~27; 13.5~20.25. Dense Urban-IC; 15.6~23.4; 10.8~16.2. Rural-IC; 6.6~9.9; 3.2~4.8. | 2~3x | [3x] of IMT-2020 |
|  | 5th percentile user spectral efficiency (bit/s/Hz) |  | IC | TE; DL; UL. Indoor Hotspot-IC; 0.6~0.9; 0.42~0.63. Dense Urban-IC; 0.45~0.675; 0.30~0.45. Rural-IC; 0.24~0.36; 0.09~0.135. | 2~3x | [3x] of IMT-2020 |
| Area traffic capacity (Mbit/s/m²) |  |  | IC | Indoor Hotspot-IC: 40~60 Mbit/s/m2 | 4~6x | [10] |
| Connection density (devices/km²) |  |  | MC | 107~108 devices per km2 | 10~100x | [10x] |
| Mobility (bit/s/Hz @ km/h) |  | See chapter 2.3 | IC | Test environment; Normalized traffic channel link data rate (bit/s/Hz); Mobility (km/h). Indoor Hotspot-IC; 2.25; 10. Dense Urban-IC; 1.68; 30. Rural-IC; 1.2; 120. 0.675; 500. Urban mobility - IC; TBD; TBD. | 1.5x SE for the velocities defined in IMT-2020. Other velocities and test environments: TBD | Keep the velocities defined for IMT-2020. Should new velocity be added? (up to 1000/1200km/h for airplane) Should 50% spectral efficiency be increased for the velocity defined in IMT-2020? e.g. [1.5x] of IMT-2020 |
| Mobility interruption time (ms) |  | See chapter 2.3 | IC and HRLLC | 0ms | 1x | [0 ms] |
| Latency | User plane latency (ms) | Same as IMT-2020 | IC and HRLLC | HRLLC: 0.5-1ms IC: 1-2ms | HRLLC: 1/2-1x IC: 1/4-1/2x | TBD |
|  | Control plane latency (ms) |  |  | 10~15ms | 1/2-3/4x | TBD |
| Reliability |  |  | IC and HRLLC | 1-10-5 | 1x | [1-10-5] |
| Composite requirement |  | See chapter 2.4 | IC | Data rate: DL: 30Mbps UL: 10Mbps |  | Moderator Proposal 2-1: Define TPR (e.g. joint/composite requirement) related to data rate, latency, reliability, capacity etc. as a candidate IMT-2030 TPR. - Applicable usage scenario: immersive communication |
|  |  |  |  | Latency: DL: 10ms UL: 30ms |  |  |
|  |  |  |  | Reliability: DL: 99% UL: 99% |  |  |
|  |  |  |  | Capacity: number of users per km2 Dense urban-IC: 1000 [Indoor hotspot-IC: 10000] |  |  |
| Coverage |  |  |  |  |  | Proposal 2-5: For coverage, further discuss the following three options: - Option 1: define coverage as a quantitative TPR - Option 2: do not define coverage as a TPR, use link budget evaluation approach (same as IMT-2020) - Option 3: define extra connectivity as a qualitative TPR, which was revised from a part of coverage |
| Positioning | Positioning accuracy | See chapter 2.5 |  | 10cm |  | Moderator Proposal 1-1: Define positioning accuracy as a quantitative TPR |
| Bandwidth |  | Same as IMT-2020 |  | 400MHz | 4x | 400MHz for system bandwidth (same definition as IMT-2020) |
| Sensing-related capability | Detection/False alarm probability | See chapter 2.6 | ISAC | TBD |  | Moderator Proposal 1-2: Define Sensing-related capabilities as a TPR. |
|  | Localization accuracy |  |  | TBD |  |  |
|  | Velocity accuracy |  |  | TBD |  |  |
|  | Sensing resolution |  |  | TBD |  |  |
|  | Motion rate accuracy |  |  | TBD |  |  |
| AI-related accuracy |  | See chapter 2.7 | AIAC | Qualitative |  | Moderator Proposal 1-3: define AI-related capabilities as qualitative TPR evaluated by inspection. |
| Energy efficiency | Network EE | See chapter 2.8 | IC | TBD |  | Moderator Proposal 1-4: Define energy efficiency as a TPR. |
|  | Device EE |  |  | TBD | Both unloaded and loaded case should be considered. |  |
| Security |  |  |  |  |  | Proposal 2-3: Do not define security, interoperability as IMT-2030 TPRs, these can be covered by submission template for IMT-2030. |
| Resilience |  |  |  |  |  |  |
| Interoperability |  |  |  |  |  | Proposal 2-3: Do not define security, interoperability as IMT-2030 TPRs, these can be covered by submission template for IMT-2030. |

Spectral efficiency

Peak spectral efficiency

In IMT-2020, refer to the ITU M.2410 [4], the requirement values were defined assuming an antenna configuration to enable eight spatial layers (streams) in the downlink and four spatial layers (streams) in the uplink. According to TR 37.910, the evaluation results of NR is ~1.5x of DL requirement and ~1.6x of UL requirement.

Regarding the assumption of spatial layers (streams) for the requirement of IMT-2030, if the supported number of single user layer larger than eight, there will be significant impact on the number of antennas at device and the MIMO design target, meanwhile, whether the channel conditions can support greater than eight layers also needs to be comprehensively studied. So, considering the possible and practical antenna configuration for device and channel conditional, we suggest for IMT-2030 peak spectral efficiency requirement, assume an antenna configuration to enable eight spatial layers (streams) in both downlink and uplink.

Furthermore, higher modulation (e.g. 1024QAM, 4096 QAM) for downlink is possible. Higher coding rate is also possible depending on the development of channel coding scheme. In addition, lower overhead (i.e., radio resources that are used for physical layer synchronization, reference signals or pilots, control signals, guard bands and guard times) is expected due to technical advancement in 6G timeframe, such as application of AI/ML in PHY. Hence, we propose the minimum requirements for peak spectral efficiency is 2.0 times of IMT-2020, i.e., 60 bit/s/Hz for downlink and 30 bit/s/Hz for uplink.

Average spectral efficiency and 5th percentile user spectral efficiency

Regarding the requirement value(s), some initial simulation evaluation results with extreme antenna array assumption in BS by increasing the antenna elements by 4x/8x/12x of 128 elements and 4x/8x/16x of 32 TXRUs and typical antenna assumptions in UE with 4RX and 2/4TX are provided in the Figure 1 below. In the preliminary simulations, ideal PMI measurement are assumed for the transmission port larger than 32. Other potential and possible performance enhancement technologies, such as higher modulation, enhanced channel coding, enhanced CSI-RS/DMRS, enhanced MIMO schemes, AI for CSI enhancement, overhead reduced and so on, are not modeled or assumed.

Figure 1 Preliminary simulation results of spectral efficiency of Dense Urban

Table 2 Simulation assumptions for spectral efficiency

|  | 5G | 6G |
| --- | --- | --- |
| Scenario | Dense Urban (ISD=200m) | Dense Urban (ISD=200m) |
| Carrier Frequency | 4GHz | 7GHz |
| Simulation band | 20MHz | 20MHz |
| SCS | 30KHz | 30KHz |
| Duplex | DDDSU | DDDSU |
| gNB (M,N,P,Mg,Ng; Mp,Np) | 32P128E: (8,8,2,1,1;2,8) | 128P512E: (16,16,2,1,1;4,16) 256P1024E:(32,16,2,1,1;8,16) 512P1536E:(24,32,2,1,1;8,32) |
| UE (M,N,P,Mg,Ng; Mp,Np) | 2T(1,1,2,1,1;1,1) 4T/4R (1,2,2,1,1;1,2) | 2T(1,1,2,1,1;1,1) 4T/4R (1,2,2,1,1;1,2) |
| DL CSI | SRS-based precoding, non-codebook | SRS-based precoding, non-codebook |
| UL CSI | Non-codebook | Non-codebook |
| DL MIMO mode | MU-MIMO | MU-MIMO |
| DL DMRS port number | 12 | 24 |
| UL MIMO mode | SU-MIMO | SU-MIMO |
| Traffic model | Full buffer | Full buffer |

From the preliminary simulation results, it can be observed that for downlink, with higher number of BS antenna elements and TXRUs in IMT-2030, spectral efficiency can achieve 3~7 times of IMT-2020 requirement. For uplink with 2 transmission antennas in device, the maximum performance is 1.9 times to IMT-2020 requirement for average spectral efficiency. With the potential enhancement of technologies in IMT-2030, the spectral efficiency may be further enhanced.

Hence, we propose the minimum requirements for average spectral efficiency and 5% percentile user spectral efficiency is 2.0~3.0 times of IMT-2020.

User experienced data rate

The definition of user experienced data rate in ITU M.2410 [4] is as below:

User experienced data rate is the 5% point of the cumulative distribution function (CDF) of the user throughput. User throughput (during active time) is defined as the number of correctly received bits, i.e. the number of bits contained in the service data units (SDUs) delivered to Layer 3, over a certain period of time.

In case of one frequency band and one layer of transmission reception points (TRxP), the user experienced data rate could be derived from the 5th percentile user spectral efficiency through equation (3). Let W denote the channel bandwidth and SEuser denote the 5th percentile user spectral efficiency. Then the user experienced data rate, Ruser is given by:

Ruser = W × SEuser

The first paragraph is the general definition, which is applicable for both DL and UL. The second paragraph explains the calculation method, which is only applicable to the downlink. Hence, we have the following suggestion:

User experienced data rate is the 5% point of the cumulative distribution function (CDF) of the user throughput. User throughput (during active time) is defined as the number of correctly received bits, i.e. the number of bits contained in the service data units (SDUs) delivered to Layer 3, over a certain period of time.

For uplink, the user experienced data rate could be derived directly from the 5% point of the cumulative distribution function (CDF) of the user throughput.

For downlink, in case of one frequency band and one layer of transmission reception points (TRxP), the user experienced data rate could be derived from the 5th percentile user spectral efficiency through equation (3). Let W denote the channel bandwidth and SEuser denote the 5th percentile user spectral efficiency. Then the user experienced data rate, Ruser is given by:

Ruser = W × SEuser 	(3)

In case bandwidth is aggregated across multiple bands (one or more TRxP layers), the user experienced data rate will be summed over the bands.

User experienced data rate is the 5% point of the cumulative distribution function (CDF) of the user throughput. User throughput (during active time) is defined as the number of correctly received bits, i.e. the number of bits contained in the service data units (SDUs) delivered to Layer 3, over a certain period of time.

For uplink, the user experienced data rate could be derived directly from the 5% point of the cumulative distribution function (CDF) of the user throughput.

For downlink, in case of one frequency band and one layer of transmission reception points (TRxP), the user experienced data rate could be derived from the 5th percentile user spectral efficiency through equation (3). Let W denote the channel bandwidth and SEuser denote the 5th percentile user spectral efficiency. Then the user experienced data rate, Ruser is given by:

Ruser = W × SEuser 	(3)

In case bandwidth is aggregated across multiple bands (one or more TRxP layers), the user experienced data rate will be summed over the bands.

Mobility

In IMT-2020, mobility TPR is defined as the supported maximum mobile station speed (in km/h) at which a defined QoS can be achieved, which are related to extreme high-speed scenarios such as high-speed trains. The defined QoS is normalize traffic channel link data rate (bit/s/Hz). However, although the user experience is good in most scenarios, there are still poor user experience in some scenarios where the device moving speed is not high, e.g., when entering or exiting buildings, passing under driveway bridges, being in an underground parking lot, or using an elevator, which are referred to as pain point scenarios. The terrible user experiences in these pain point scenarios cannot be tolerated, especially for some important calls or ongoing online conference.

The traditional mobility requirements and evaluation methodology cannot reflect the practical user experience, hence it’s necessary to identify the practical problem in actual deployment scenarios to pursue technical solutions to fundamentally resolve the mobility problem. The mobility capability should be measured in terms of practical typical deployment scenarios and user movements to evaluate radio access technology’s ability to support mobility with minimum service interruption.

Define mobility requirement considering the deployment scenarios and user movements to evaluate technology’s ability to support mobility with minimum service interruption.

Regarding the deployment scenarios, we propose an Urban mobility deployment scenario, where the actual environment of dense urban with car lanes, pedestrian/bicycle sidewalks and buildings are modeled. The actual blocking or reflection from obstacles can be modeled, which is a typical scenario to evaluate the actual mobility performance.

Referring to the definition in IMT-2020, we suggest to add the practical Urban mobility deployment scenario as a test environment to define the mobility classes and minimum requirement.

Mobility classes

|  | Test environments for eMBB |  |  |  |
| --- | --- | --- | --- | --- |
|  | Indoor Hotspot – IC | Dense Urban – IC | Rural – IC | Urban mobility – IC |
| Mobility classes supported | Stationary, Pedestrian | Stationary, Pedestrian, Vehicular (up to 30 km/h) | Pedestrian, Vehicular, High speed vehicular | Pedestrian, Vehicular |

Regarding the requirements of normalized traffic channel link data rate for the velocities defined in IMT-2020, we propose the minimum requirement as 1.5 times of IMT-2020.

Traffic channel link data rates normalized by bandwidth

| Test environment | Normalized traffic channel link data rate (bit/s/Hz) | Mobility (km/h) |
| --- | --- | --- |
| Indoor Hotspot – IC | 2.25 | 10 |
| Dense Urban – IC | 1.68 | 30 |
| Rural – IC | 1.2 | 120 |
|  | 0.675 | 500 |
| Urban mobility – IC | TBD | TBD |

Mobility interruption time

As discussed in section 3.7, the mobility interruption time capability should also be measured in terms of practical typical deployment scenarios and user movements to evaluate radio access technology’s ability to support mobility with minimum service interruption time.

Based on the mobility interruption time definition in M.2410 [4], in our view, following should be taken into account:

The mobility interruption time should be evaluated and minimized to achieve consistent user experience especially in practical deployment scenarios, especially in some pain point scenarios, e.g. passing street corners, entering or exiting elevator, etc., where wireless signal quality changes rapidly in a short period.

Composite requirement

Immersive XR is important and typical use cases for immersive communication, and there are high requirements on data rate, latency and reliability. Meanwhile, system capacity is an important metric for successful commercial deployments, so the capacity with joint requirement on data rate, latency, and reliability is a suitable KPI to assess the system performance to support XR use cases.

Referring to the initial agreement in ITU-R WP5D #48 meeting [5], we have the following suggestion:

This requirement is defined as the number of users per unit area (per km2) that fulfil the required success probability (reliability) of transmitting packets within a required latency, which is the time it takes to deliver a data packet for a given arrive rate from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum performance requirement is summarized in Table 2.

Table 3 Composite requirement

| Test environment | Data rate (Mbits/s) | Latency (ms) | Reliability | Number of users (/km2) |
| --- | --- | --- | --- | --- |
| Dense Urban - IC | DL: 30 UL: 10 | DL: 10 UL: 30 | 99% | 1000 |
| [Indoor hotspot - IC] | DL: 30 UL: 10 | DL: 10 UL: 30 | 99% | [10000] |

Positioning

Positioning is the ability to calculate the approximate position of connected devices. Positioning accuracy describes the closeness of the measured position of the device to its true position value. The accuracy can describe the accuracy either of an absolute position or of a relative position. It can be further broken into a horizontal position accuracy – referring to the position error in a 2D reference or horizontal plane, and into a vertical position accuracy – referring to the position error on the vertical axis or altitude.

This requirement is defined for the purpose of evaluation in the Immersive Communication and Integrated Sensing and Communication usage scenarios.

The preliminary requirements could be 10 cm. Higher positioning capability is considered for some specific usage scenario, such as XR tracking, the positioning accuracy could be 1cm.

Sensing-related capabilities

Sensing-related capabilities refer to the ability to provide functionalities in the radio interface to support sensing tasks, including passive object presence/absence detection, object classification, movement characterization, object localization, etc.

Referring to the discussion in ITU-R WP5D, the following four performance metrics could be used to evaluate the radio access technology’s ability to support sensing related capabilities:

- Detectability (Detection / False Alarm Probability) – Detection probability, PD, is the probability of correctly detecting the presence of the object, and False alarm probability, PFA, is the associated probability that an object is detected given that the object is not present.

- Localization Accuracy - Localization accuracy refers to the closeness of the measured sensing result of position of the target object to its true position value and is the difference between the calculated location and the actual location of an object.

(Ed Note: Whether this accuracy is to be represented in Cartesian or polar coordinate or Euclidian distance is FFS. Components of localization accuracy may include:)

  - Distance accuracy: the difference between the calculated distance and the actual distance of the sensing target

  - Angle accuracy: the difference between the calculated angles and the actual angles of the sensing target

  - Horizontal/Vertical Localization Accuracy (HSA/VSA): It is defined as the difference between the calculated horizontal/vertical position and the actual horizontal/vertical position of an object

- Velocity Accuracy - Velocity accuracy is defined as the difference between the estimated velocity and the actual velocity of the sensing target.

- Sensing Resolution – Is defined as the minimum difference of the measured value (e.g, range, location) of two target objects, in order to distinguish these two target objects. (Ed note: this may be evaluated by an analytical approach).

However, regarding the movement characterization of targets, it is not only linear motion, but also periodic motion, such as drone blade rotation, bird wing flapping, human arm/leg swing, human chest undulation etc. which are also important movement characterization and need to be identified and measured to support some sensing tasks, so we propose the following sensing-related TPR:

Motion rate accuracy – Motion rate accuracy describes the difference between the measured movement frequency caused by part(s) (e.g. blade, chest) of the target object and the true movement frequency.

Define motion rate accuracy as a sensing-related capability.

AI-related capabilities

Applicable AI-related capabilities refer to the ability to provide certain functionalities throughout IMT-2030 to support AI enabled applications. These capabilities include:

-	Data collection

-	Data/Model exchange

-	AI model training

-	AI model inference

-	AI model monitoring

-	AI computing resource management

Energy efficiency

It is agreed in RAN #107 that energy efficiency is a TPR of IMT-2030 [2].

Energy efficiency is a quantifiable metric of sustainability and is important for the success of IMT-2030 technology. The energy efficiency TPR should be defined to ensure that IMT-2030 is designed in an energy efficient way.

Regarding the load cases for energy efficiency, there are two cases to be considered. One is the unloaded case, where there is no traffic data transmission/reception is happening but there are still some signals, such as paging signal, needed to be transmitted/received, power saving technologies are very much needed to reduce the power consumption meanwhile maintain the paging performance (i.e. paging latency). The other one is the loaded case, where there is traffic data for transmission and/or reception but not happening always at all the time, power saving technologies to reduce the power consumption meanwhile minimizing the impact on the data transmission performance (e.g. user perceived throughput (UPT), which is widely used as performance metric in the 3GPP study) are necessary to enhance the energy efficiency. Therefore, the energy efficiency should be considered for both unloaded case and loaded case.

Define energy efficiency for both unloaded case and loaded case.

Define energy efficiency considering both energy saving and communication performance metrics together.

With the reference to the initial agreement in ITU-R WP5D #48 meeting [5], we have the following suggestion:

Energy efficiency is an important metric of sustainability. Network energy efficiency is the capability of a RIT/SRIT to support radio access network energy saving in relation to the traffic capacity provided. Device energy efficiency is the capability of the RIT/SRIT to support device modem power saving in relation to the traffic characteristics.

This requirement is defined for the purpose of evaluation in the Immersive communication usage scenario.

The requirement is the relative energy savings between the selected load case(s) and a reference case. Proponents should report the power model used for the evaluation, as well as impacts on communication performance (e.g. user perceived throughput (UPT), latency) associated with the evaluated power saving technologies, see [IMT-2030 EVAL] for details.

The minimum requirements for Energy efficiency are summarized in Tables below.

Table: The minimum requirements for Network Energy efficiency

| [Test environment(s)] ** | Selected load case(s) | The reference case | Relative power saving (%) |
| --- | --- | --- | --- |
| TBD | [empty load, low load, light load and/or medium load] * | Fully loaded IMT-2030 case * | [X1%, X2%, ...] |

*empty load: L=0, fully loaded case: L=100

** needed only in case of simulation

Table: The minimum requirements for Device Energy efficiency

| Test environment(s) | Selected load case(s) | The reference case | Relative power saving (%) |
| --- | --- | --- | --- |
| N.A | Empty load | Fully loaded IMT-2030 case | A1% |
| Dense Urban-IC | [>= B2% load] | Fully loaded IMT-2030 case | A2% |
| [Indoor hotspot-IC] | [>= B3% load] | Fully loaded IMT-2030 case | A3% |

Conclusions

In summary, in this contribution we expressed our views on the TPR and deployment scenarios, and have following proposals:

- Define mobility requirement considering the practical deployment scenarios and user movements to evaluate technology’s ability to support mobility with minimum service interruption.

- Define motion rate accuracy as a sensing-related capability.

- Define energy efficiency for both unloaded case and loaded case.

- Define energy efficiency considering both energy saving and communication performance metrics together.

References

- RP-250810, “New SID: Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom.

- RP-250819, “Moderator's summary for IMT-2030 TPRs”, Moderator (CMCC).

- RT-250024, “Moderator's summary for IMT-2030 TPRs in CC#1” , Moderator (CMCC).

- R-REP-M.2410-2017-MSW-E.

- R23-WP5D-250204-TD-0249!R1!MSW-E.