3GPP TSG RAN Meeting #108                                                       RP-251217

Prague, Czech Republic, June 9-13, 2025

Source:	vivo

Title:	Discussion on scenarios and 3GPP requirements for 6G radio

Agenda Item:	16.1

Document for:	Discussion and Decision

Introduction

The objectives of the study item  “Study on 6G Scenarios and requirements” has been updated in RAN #107 [1] as follows:

This study item aims to

- investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs.

  - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN

- Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors.

- Develop 3GPP requirements for 6G Radio for improvement of existing services and for new services.

- Determine the applicability of legacy services to 6G Radio, and define radio requirements for these, as appropriate.

- Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least:

  - Ensure appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting

  - Energy efficiency and energy saving: both for network and device

  - Enhanced spectral efficiency

  - Enhanced overall coverage, focus on cell-edge performance and UL coverage

  - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz

  - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band

  - Target scalable and forward compatible design for diverse device types

  - Improved spectrum utilization and operations taking into account diverse spectrum allocations

  - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs

  - Aim at a harmonized 6G Radio design for TN and NTN, including their integration

  - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

- Define a time plan and steer work as appropriate for the RAN WGs during the 6G WG SI to deliver high-level decisions at least on the following areas:

  - Fundamental 6G radio design aspects: waveform, numerology, channel coding, etc…

  - Overall high-level aspects of 5G to 6G migration

  - RAN architecture and interfaces, including RAN-Core interface

  - Coordination of 6G AI/ML framework

In this contribution, we provide our views on the scenarios and requirements for 6G radio.

Scenarios

For the three enhanced usage scenarios, Immersive Communication, Hyper Reliable & Low-latency Communication and Massive Communication, which are envisaged to expand from IMT-2020 (i.e., eMBB, URLLC, and mMTC), whereas some of the deployment scenarios of IMT-2020 in TR 38.913 [2] can be used as starting point with necessary modifications (such as carrier frequency, system bandwidth, antenna configuration and so on) to match the new spectrum available for IMT-2030 and the system design.

The following deployment scenarios in TR 38.913 can be taken as starting point for 6G scenarios:

- - Indoor hotspot

- - Dense Urban

- - Rural

- - Urban macro

- - High speed

- - Urban coverage for massive connection

Regarding the carrier frequency, we propose additional candidate carrier frequency around 7GHz spectrum for these scenarios. Considering requirements on the potential peak data rate (such as 3~5x IMT-2020 requirement) and the range of bands being discussed in WRC, we suggest the aggregated system bandwidth of 2GHz (DL + UL) for 7GHz spectrum while reusing the same BS sites and the ISD as in mid band operation. For the UE antenna configurations, considering the actual applications, various device types, the size and implementation complexity etc., the number of antenna elements at the UE can be considered up to 8 for Tx and Rx.

The following configuration assumptions could be considered for 6G scenarios:

- - Carrier frequency: additional spectrum of 7 GHz.

- - Aggregated system bandwidth:  up to 2 GHz (DL+UL) in 7GHz spectrum.

- - UE antenna elements: up to 8 Tx and Rx antenna elements.

Considering various studies and evaluations among industries and academia on mobility performance for 6G, we propose an Urban mobility deployment scenario, where the actual environment of dense urban with car lanes, pedestrian/bicycle sidewalks and buildings are modeled. The actual blocking or reflection from obstacles can be modeled to reflect the rapid changes in wireless signal quality to evaluate the actual mobility performance. Some of its attributes are listed in Table 1.

Table 1 Attributes for Urban mobility

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 7GHz or Around 4GHz |
| Aggregated system bandwidth | Up to 2GHz (DL+UL) |
| Layout | Macro only BS is located on the building in sideways and middle of the road BS height is 25m Building size is 413m x 230m x 20m |
| ISD | 250m |
| BS antenna elements | Up to [TBD] Tx and Rx antenna elements |
| UE antenna elements | Up to 8 Tx and Rx antenna elements |
| UE distribution | Car lanes and pedestrian/bicycle sidewalks are placed around a road black. 2 lanes in each direction, 4 lanes in total, 1 sidewalk in each direction, 2 sidewalks in total, one building (size 413m x 230m x 20m ) in the center of block, one block size 433m x 250m. UEs are distributed on the car lanes and/or sidewalks. Vehicle user dropping: on the car lanes Pedestrian/bicycle user dropping: on the sidewalks |
| UE mobility | Move within the lane, and randomly change directions, go straight, turn left, turn right, or make a U - turn at the intersection Vehicle user speed: 60km/h Bicycle user speed: 15km/h Pedestrian speed: 3km/h |

Urban mobility scenario shall be considered as 6G deployment scenario.

For the Integrated Sensing and Communication usage scenarios, the agreed scenarios in the RAN1 study on channel modelling for ISAC for NR can be taken as starting point as shown in table 2.

Table 2  ISAC usage scenarios candidate deployment scenarios

| Sensing Targets | Scenarios |
| --- | --- |
| UAVs | RMa-AV, UMa-AV, UMi-AV (TR 36.777) |
| Humans indoors | InF, Indoor Office, [Indoor Room (TR 38.808)], [UMi, UMa] |
| Humans outdoors | UMi, UMa, [RMa] |
| Automotive vehicles (at least outdoors) | Highway, Urban grid, UMa, UMi, RMa |
| Automated guided vehicles (e.g. in indoor factories) | InF |
| Objects creating hazards on roads/railways (examples defined in TR 22.837) | Highway, Urban grid, HST |

Deployment scenarios agreed in RAN1 study on channel modelling for ISAC for NR can be taken as starting point for IMT-2030 integrated sensing and communication usage scenario.

Requirements

Legacy key performance indicators

For some legacy performance indicators, such as spectral efficiency, data rate, latency, reliability, and etc., we propose the definition and requirement to be the same as the ITU IMT-2030 TPRs. For information, we summarize our views on the requirements for these candidate key performance metrics in Table 3.

Table 3 Proposed legacy performance indicators values

| Candidate performance indicators |  | View on target value(s) | Enhancement times to IMT-2020 |
| --- | --- | --- | --- |
| Peak data rate (Gbit/s) |  | DL: 80 Gbit/s UL: 40 Gbit/s | 4x |
| User experienced data rate (Mbit/s) |  | Dense Urban-IC: DL: 400~600 Mbit/s: UL: TBD | DL: 4~6x UL: FFS |
| Spectral efficiency | Peak spectral efficiency (bit/s/Hz) | DL: 60 Mbit/s UL: 30 Mbit/s | 2x |
|  | Average spectral efficiency (bit/s/Hz/TRxP) | Deployment scenario; DL; UL. Indoor Hotspot-IC; 18~27; 13.5~20.25. Dense Urban-IC; 15.6~23.4; 10.8~16.2. Rural-IC; 6.6~9.9; 3.2~4.8. | 2~3x |
|  | 5th percentile user spectral efficiency (bit/s/Hz) | TE; DL; UL. Indoor Hotspot-IC; 0.6~0.9; 0.42~0.63. Dense Urban-IC; 0.45~0.675; 0.30~0.45. Rural-IC; 0.24~0.36; 0.09~0.135. | 2~3x |
| Area traffic capacity (Mbit/s/m²) |  | Indoor Hotspot-IC: 40~60 Mbit/s/m2 | 4~6x |
| Connection density (devices/km²) |  | 107~108 devices per km2 | 10~100x |
| Mobility interruption time (ms) |  | 0ms | 1x |
| Latency | User plane latency (ms) | HRLLC: 0.5-1ms IC: 1-2ms | HRLLC: 1/2-1x IC: 1/4-1/2x |
|  | Control plane latency (ms) | 10~15ms | 1/2-3/4x |
| Reliability |  | 1-10-5 | 1x |

For the legacy key performance indicators, the definition and requirements can be same as the ITU IMT-2030 TPRs, including

- Peak data rate, peak spectral efficiency

- average spectral efficiency, 5% user spectral efficiency

- User experienced data rate

- Area traffic capacity

- Control plane latency, user plane latency

- Reliability

- Connection density

- Mobility interruption time

Minimize optional features and avoid multiple options for the same purpose

Hard lessons learned from the very first release of NR specifications is that there are too many optional features and, in some cases, there are multiple options for the same purpose. The 6G radio design should take the lessons learnt from 5G NR into account! Excessive optional features and multiple choices for certain feature delayed overall development of the system. Various design options and features in NR, resulting in numerous configurations and excessive UE capabilities led to lower market adoption rate. For example, five different PUCCH formats are defined for physical UL control channels, three different HARQ codebooks (and even with some variations for some of them) are defined for HARQ feedback, both high layer and low layer repetitions are introduced for the purpose of robustness (and even multiple options and types of repetitions in each of the protocol layer), and various procedures for handover/mobility (e.g., L3 handover, LTM, CHO), etc. There are several hundred feature groups defined in NR for MIMO technology alone! Such an overdesign not only increases unnecessary flexibility without performance benefit, it makes the system overcomplicated, increases risks of fragmentation, and tremendous difficulty in maintenance and evolution. Many options are defined without proper and careful consideration of justifications on real deployment benefits. Consequently, making decision on real product development becomes overwhelming, while lots of efforts are spent on the works that are never been implemented nor commercialized.

Therefore, the 6G radio design should avoid overdesign and defining excessive optional features. It is desirable to minimize, if not possible to completely eliminate, adoption of multiple options for the same purpose.

Avoid defining excessive optional features and minimize, if not possible to completely eliminate, adoption of multiple options for the same purpose.

Energy efficiency

Energy efficiency is a quantifiable metric of sustainability and is important for the success of IMT-2030 technology. The energy efficiency TPR should be defined to ensure that IMT-2030 is designed in an energy efficient way.

For evaluation of energy efficiency, two cases of load levels can be considered. One is the unloaded case, where there is no need of traffic data transmission / reception  but there are still some signals, such as paging signal, need to be transmitted / received, power saving technologies are very much needed to reduce the power consumption meanwhile maintain the paging performance (i.e. paging latency). The other is the loaded case, where there are traffic data for transmission or reception but not all the time, power saving technologies to reduce the power consumption meanwhile minimizing the impact on the data transmission performance (e.g. user perceived throughput (UPT)) are necessary to improve the energy efficiency.

Under the unloaded case, even with the same configuration of discontinuous reception (DRX), due to different power consumption in different sleep states and transition power from different sleep states to non-sleep states, the average energy consumption of the device is also different. Therefore, using the sleep ratio and sleep duration cannot fully characterize the energy consumption of the device. In addition, in unloaded case, the latency for paging/control signaling delivery may also be important for some services and device types. In unloaded case, the energy efficiency could be defined taking both the impact on paging/control signaling latency and energy consumption into account. For the definition of energy efficiency for unloaded case, two options can be considered.

Option 1, define EE=VP, where v is the delivery/reception rate of paging, v = sum(1/ti)/N, ti is the time interval between the arrival of paging and its successful delivery to the receiver. Option 2, define EE as the relative power consumption with tolerable/less than [TBD] latency deterioration.

Under the loaded case, due to the traffic characteristics, two sub-cases can be considered, sleep duration and data transmission duration. It is well understood that the power consumptions of device in different power states are different, technologies for reduction in power consumption can be studied meanwhile minimizing the impact on the latency of data transmission. Therefore, in loaded case, the energy efficiency could be defined by taking both the data rate and power consumption into account. For the definition of energy efficiency in loaded case, two options can be considered.

Option 1, define EE=VP  or EE=PV ,  where P is the power consumption, v is the user perceived throughput (UPT), which is widely used in 3GPP evaluations in all SI/WIs and reflects the user's experience data rate, and P=EC/T, where EC is the energy consumption during an evaluation time T, EC= Pon ×Ton + Psleep ×Tsleep+ Ptransition ×Ttransition . Option 2, define EE as the relative power consumption with tolerable/less than [TBD] data rate deterioration.

Although in the above discussion, device side energy efficiency is assumed, the network side energy efficiency is also very important. The definition of network side energy efficiency shall also be carefully studied, common definition for both device and network side energy efficiency is preferable, if possible.

Both unloaded case and loaded case should be considered for energy efficiency.

The definition of energy efficiency should jointly consider both power/energy consumption and communication related performance metric, such as data rate, and latency for control signaling/paging or data.

For unloaded case, two options can be considered:

- - Option 1:  EE=VP  , where P is the power consumption, v is the delivery/reception rate of paging, v = sum(1/ti)/N, ti is the time interval between the arrival of paging and its successful delivery to the receiver.

- -  Option 2:  EE is the relative power consumption with tolerable/less than [TBD] latency deterioration.

For loaded case, two options can be considered:

- - Option 1:  EE=VP  or E=PV , where P is the power consumption, v is the user perceived throughput (UPT).

- - Option 2:  EE is the relative power consumption with tolerable/less than [TBD] data rate deterioration.

Coverage

Regarding 6G coverage requirements, there are two aspects need to be considered. One is to ensure that 6G deployment around 7GHz achieves comparable coverage footprint as 5G mid-band, when re-using existing 5G mid-band (~3.5GHz) site grid for 6G deployments. The other is to improve user experience in case of same spectrum band for 5G and 6G. Table 4 provides achievable data rate under Urban deployment @4GHz by 5G system at different coupling loss. ~ 2x data rate improvement for both coupling loss values are expected for 6G system.

Table 4  Coverage improvement by 6G system

|  | MCL (dB)note 2 | UL Data rate (5G) | Data rate (6G) |
| --- | --- | --- | --- |
| Urban (4GHz TDD) note 1 | 131 | 1Mbps | [2x] |
| Urban (4GHz TDD) | 144 | 50Kbps | [2x] |

Note 1: MCL with corresponding data rate for 5G system is from TR 38.830.

Note 2: MCL = Total transmit power – Receiver sensitivity + gNB antenna gain (component 2) as defined in [TR 38.830], assuming 64 TXRU.

Furthermore, to meet wide area requirement for LPWA service, an additional coverage requirement of ~10dB extension compared to normal coverage bottleneck channel is required, e.g., [154] dB MCL. At this coupling loss, data rate around [2.5kbps] shall be achieved, and all relevant downlink and uplink control channels should also perform adequately.

Support 6G coverage comparable to 5G mid-band, reusing existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz.

Support 6G overall coverage improvement in same spectrum band as 5G, by [2x] data rate boost for normal coverage, and ~ 10dB extension for LPWA with [2.5kbps] cell-edge data rate.

Diverse device types

In the era of 6G, the mobile system will provide its services to a myriad of devices, from low-end LPWA devices to high-end advanced eMBB devices. Despite their diverse form factor and distinct service scenarios, common framework with scalable design for these device types is desirable to minimize deviation in access technology and leverage economy of scale.

Clear blueprint for 6G baseband capability should be decided first to avoid compatibility issue in later release.  Limited number of device types can be considered, for example, three device types can be considered,

Device type A is targeting smartphones, immersive eMBB device, etc.

Device type B is targeting reduced capability eMBB device, e.g. wearable/XR, etc.

Device type C is targeting IoT/LPWA device.

Our views on the three device types on U6G (6425-7125MHz) are provided in the following Table 5.

Table 5 Device types suggestion

|  | Device type A | Device type B | Device type C |
| --- | --- | --- | --- |
| Downlink peak data rate | ~10Gbps | 200Mbps ~ 1Gbps | ~10Mbps |
| Uplink peak data rate | ~2Gbps | 50Mbps ~ 200Mbps | ~5Mbps |
| Supported maximum downlink channel bandwidth | ≥ 200 MHz | 100 MHz | ~5MHz for FDD ~20MHz for TDD |
| Supported maximum uplink channel bandwidth | 200MHz | 100 MHz | ~5MHz for FDD ~20MHz for TDD |
| Downlink MIMO layer | ≥ 4 layer | 2 layer | ≤ 2 layer |
| Uplink MIMO layer | ≥ 2 layer | 1 layer | 1 layer |

Considering lower cost and complexity, and extreme coverage type C device (IoT/LPWA) will support limited bandwidth. According to cost reduction analysis in LTE and NR, bandwidth around 5MHz for low band in FR1 achieves sufficiently low cost and complexity, and further reduction of bandwidth would only bring marginal benefit. For higher band in FR1, if supported for LPWA, minimum bandwidth around 20MHz would be reasonable choice, scaling with the subcarrier spacing, facilitates unified design for different bands in FR1. For type B device (reduced capability eMBB, wearable/ XR), similar bandwidth as NR eMBB can be considered, while for device A device (smart phone / immersive eMBB), larger bandwidth more than 200MHz can be supported. With 5~20MHz bandwidth for device type C, common design for initial access and common channels compatible with all three UE categories discussed above can be easily supported.

Similar as LTE and NR LPWA device as well as Redcap/eRedcap, type C device will typically support single antenna (optionally support two antennas), and may have smaller antenna gain due to complexity and small device form factors restriction, i.e., downlink MIMO layer no larger than 2 is supported by type C device. For type B and type A devices, larger number of antennas with higher antenna gain can be supported, e.g., ≥ 4 layers for type A device and 2 layers for type B device.

To meet various service requirements, e.g., from smart cities, smart metering to immersive multimedia, different device types support different data rates. With limited bandwidth and limited number of Rx/Tx antennas, type C device supports 10 Mbps, while up to10 Gbps can be supported by type A device to deliver lightning-fast data speeds and immersive multimedia experiences. In-between type A and type C device, 200Mbps ~ 1Gbps data rate can be considered for type B device.

Consider three device types for 6G system:

- Type A:  Smart phone, immersive eMBB device

- Type B:  Reduced capability eMBB device, wearable/XR

- Type C:  IoT/LPWA

Flexible spectrum utilization

One key driver of the 6G radio is to exploit more spectrum resources, e.g., FR3. On the other hand, there are more and more existing spectrums of the legacy 2G/3G/4G and even 5G system, especially in low band and narrow bandwidth, are going to be re-farmed for 6G. It is thus desirable to introduce a flexible mechanism to efficiently utilize these diverse and fragmented spectrums.

Despite the traditional Carrier Aggregation feature is still expected to be an important mechanism to utilize the fragmented spectrums, it is recognized to be inefficient in some aspects. For example, the broadcast of synchronization signals, system information, paging, etc., is inevitable for each carrier, resulting in high system overhead, as well as unnecessary network energy consumption. Moreover, the framework of CA is less flexible in dynamic activation/switching of the aggregated carrier resources, making it less adaptive to the burst traffic, rapid channel variation and recovery of beam/link failure. Furthermore, the existing CA framework does not efficiently exploit the UE hardware resources, such as the baseband processers, buffers, RF capabilities, etc., among the aggregated carriers. Although a step towards improving performance has been taken in 5G by supporting UL Tx switching in some predefined scenarios, it is desirable to do better in 6G. Therefore, new modeling of fragmented spectrums can be studied in 6G, e.g., single cell multiple carriers (SCMC).

Nevertheless, even for the not-yet-re-farmed 5G spectrum, it is beneficial for the 6G radio to dynamically share the spectrum resources to further improve the spectrum efficiency. The so-called Dynamic Spectrum Sharing (DSS), or Mutli-radio Spectrum Sharing (MRSS), is one of the key requirements for the 6G radio.

Study mechanisms to efficiently utilize the diverse and fragmented spectrums for 6G radio, with low system overhead and network energy consumption, quick adaptation to burst traffic and channel variation, better utilization of UE hardware resources, etc.

Study MRSS/DSS mechanism for 6G to utilize the spectrum resources of 5G.

Common 6G radio design

Reducing power consumption is a critical for eMBB devices (e.g., smartphone, XR), which ensures optimal user experience. While LPWA devices also need to have very low power consumption to extend battery life, often requiring devices to operate for several years without changing a battery. In this sense, the 6G network shall support technologies to improve energy efficiency that are mutually required by eMBB and LPWA devices. Power saving features e.g., eDRX with very long cycle, LP-WUS, reduced UE capability update (e.g., to single Rx, reduced MIMO layers) that are introduced in 4G/5G may still be leveraged in the 6G network. However, it is expected to introduce common 6G Radio design for power saving features that are commonly applicable to eMBB and LPWA devices with minimal differences.

eMBB devices/services may encounter coverage issues in specific scenarios (e.g., subways, garages, elevators, remote areas), which can result in undesirable service interruption. In addition, also in order to support vertical use cases, it requires the LPWA devices to operate on the wide-coverage areas e.g., smart cities, agricultural IoT, asset tracking, smart metering etc. Therefore, it is crucial for the 6G network to support coverage native design for ensuring seamless service continuity across diverse use cases/scenarios. Extended coverage is yet another common 6G Radio design aspect applicable for both consumer eMBB and LPWA applications. Moreover, the RAN working group studies shall also focus on developing efficient mechanisms to enable resource sharing for common control channels (e.g., for carrying SI and Paging) and supporting extended coverage  for both initial access and RRC connected mode.

In particular deployment scenario where an operator serves both eMBB and LPWA devices in same coverage area, the operator can deploy common features such as power saving and coverage extension technologies to meet service needs of various device types adaptively. A device in deeper coverage requires larger amounts of radio resources to access the 6G network, which on the other hand leads to significant challenge in supporting massive connections to cater increasing number of eMBB and LPWA devices. To address load congestion issue in dense areas, in terms of common 6G Radio design aspects,  unified access control (UAC) feature that are commonly applicable to eMBB and LPWA shall also be considered.

Introduce common features e.g., Power saving, Coverage extension, Unified access control (UAC) across different device types.

TN and NTN

NTN is an important deployment scenario introduced in the later release of 5G NR, and is expected to be an integral part of the system design to provide ubiquitous connectivity in 6G. The NR technology was primarily designed with TN deployment in mind, which does not take into account the characteristic of NTN. Consequently, some additional “patches” are introduced on top of NR to support the NTN operation. For example, an additional Koffset is introduced in NR to cope with the propagation delay in NTN that is much larger than that in TN. Such kind of patches introduced in each release make the NR-NTN system diverging away from the TN in every release, and as a consequence, the “normal” NR UE cannot benefit from the ubiquitous connectivity provided by NTN.

Therefore, the design of 6G radio should consider the characteristics of TN and NTN together from the beginning. A unified radio design for TN and NTN is beneficial to provide the ubiquitous connectivity for every 6G UE, without need for NTN-specific hardware, functionality or requirement. Thus, a 6G UE can directly (re)select and access to the NTN network when the TN network is not available. Moreover, a unified radio design natively supports the interworking between TN and NTN, e.g., mobility. From specification perspective, it is also helpful to minimize the design and maintenance efforts in the future releases.

Strive to a unified 6G radio design for TN and NTN operation and support interworking between TN and NTN networks.

System simplification

In 5G, both standalone and non-standalone architecture options were supported in RAN, including 7 different options. Supporting different architecture options complicates the specification design, significantly increased the complexity of products for both network and UE side. In addition, the non-standalone architecture led to increased UE power consumption which significantly impacted user experience, it also delayed the deployment of standalone 5G for operators.

Based on the above experience in 5G, it is suggested that only standalone architecture is considered for 6G RAN. Aggregating of 5G and 6G spectrum can be achieved by MRSS with carrier aggregation, or data-aggregation at core network level, i.e. dual stack.

Support standalone architecture only for 6G RAN.

Conclusions

In summary, in this contribution we expressed our views on the deployment scenarios and requirements on 6G radio, and have following proposals:

- The following deployment scenarios in TR 38.913 can be taken as starting point for 6G scenarios:

- - Indoor hotspot

- - Dense Urban

- - Rural

- - Urban macro

- - High speed

- - Urban coverage for massive connection

The following configuration assumptions could be considered for 6G scenarios:

- - Carrier frequency: additional spectrum of 7 GHz.

- - Aggregated system bandwidth:  up to 2 GHz (DL+UL) in 7GHz spectrum.

- - UE antenna elements: up to 8 Tx and Rx antenna elements.

Urban mobility scenario shall be considered as 6G deployment scenario.

Deployment scenarios agreed in RAN1 study on channel modelling for ISAC for NR can be taken as starting point for IMT-2030 integrated sensing and communication usage scenario.

For the legacy key performance indicators, the definition and requirements can be same as the ITU IMT-2030 TPRs, including

- Peak data rate, peak spectral efficiency

- average spectral efficiency, 5% user spectral efficiency

- User experienced data rate

- Area traffic capacity

- Control plane latency, user plane latency

- Reliability

- Connection density

- Mobility interruption time

Avoid defining excessive optional features and minimize, if not possible to completely eliminate, adoption of multiple options for the same purpose.

Both unloaded case and loaded case should be considered for energy efficiency.

The definition of energy efficiency should jointly consider both power/energy consumption and communication related performance metric, such as data rate, and latency for control signaling/paging or data.

For unloaded case, two options can be considered:

- - Option 1:  EE=VP  , where P is the power consumption, v is the delivery/reception rate of paging, v = sum(1/ti)/N, ti is the time interval between the arrival of paging and its successful delivery to the receiver.

- -  Option 2:  EE is the relative power consumption with tolerable/less than [TBD] latency deterioration.

For loaded case, two options can be considered:

- - Option 1:  EE=VP  or E=PV , where P is the power consumption, v is the user perceived throughput (UPT).

- - Option 2:  EE is the relative power consumption with tolerable/less than [TBD] data rate deterioration.

Support 6G coverage comparable to 5G mid-band, reusing existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz.

Support 6G overall coverage improvement in same spectrum band as 5G, by [2x] data rate boost for normal coverage, and ~ 10dB extension for LPWA with [2.5kbps] cell-edge data rate.

Consider three device types for 6G system:

- Type A:  Smart phone, immersive eMBB device

- Type B:  Reduced capability eMBB device, wearable/XR

- Type C:  IoT/LPWA

Study mechanisms to efficiently utilize the diverse and fragmented spectrums for 6G radio, with low system overhead and network energy consumption, quick adaptation to burst traffic and channel variation, better utilization of UE hardware resources, etc.

Study MRSS/DSS mechanism for 6G to utilize the spectrum resources of 5G.

Introduce common features e.g., Power saving, Coverage extension, Unified access control (UAC) across different device types.

Strive to a unified 6G radio design for TN and NTN operation and support interworking between TN and NTN networks.

Support standalone architecture only for 6G RAN.

References

- RP-250810, “New SID: Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom.

- TR 38.913, Study on Scenarios and Requirements for Next Generation Access Technologies.