3GPP TSG RAN Meeting #108					RP-251238

Prague, Czech Republic, June 9-13, 2025

Agenda Item:	16.1

Source:	Ericsson, CHTTL, Fujitsu, Intel Corporation, NTT DOCOMO, Semtech, Sony, T-Mobile USA, Verizon

Title:	Minimum requirements for Massive Communication for IoT

Document for:	Approval

# 1	Introduction

One of the IMT-2030 usage scenarios is Massive Communication, which is described as follows in [1]:

Massive Communication This usage scenario extends massive Machine Type Communication (mMTC) of IMT-2020 and involves connection of massive number of devices or sensors for a wide range of use cases and applications. Typical use cases include expanded and new applications in smart cities, transportation, logistics, health, energy, environmental monitoring, agriculture, and many other areas such as those requiring a variety of IoT devices without battery or with long-life batteries. This usage scenario would require support of high connection density, and depending on use cases, different data rates, low power consumption, mobility, extended coverage, and high security and reliability.

In this contribution to the RAN study on 6G scenarios and requirements [2][3], we discuss and provide input on the minimum requirements for Massive Communication.

# 2	Discussion

## 2.1	Continued support for IoT devices from Day 1 of 6G

A wide range of Internet-of-Things (IoT) use cases can be successfully served by the low-end 4G/5G based IoT technologies defined in 3GPP (NB-IoT, eMTC, Cat-1/1bis, Cat-4, RedCap, eRedCap). These technologies are expected to live on for a long time, and 6G must be able to coexist with these technologies.

Eventually, 6G should be able to replace the legacy 4G/5G technologies for all significant use cases including IoT use cases. To facilitate a smooth migration to 6G, it is important that IoT use cases are considered already in the first 6G release. Otherwise, the alternative would be to retrofit a non-backwards-compatible IoT solution into 6G at a later stage, which would come with several drawbacks. SA2 is also expected to study how to support cellular IoT enablers in 6G according to the draft SA2 SID in [4].

## 2.2	Minimum requirements for 6G Massive Communication

Based on the discussion on continued support for diverse device types from Day 1 of 6G (see Section 2.1) and existing 5G requirements (see Annex in the end of this contribution), the following minimum requirements for Massive Communication can be considered.

### 2.2.1	UE complexity and form factor

The legacy 3GPP technologies optimized for IoT use cases employ at least some of the following main techniques to facilitate UE implementations with low complexity and small form factor.

Single antenna: NB-IoT, eMTC, Cat-1/1bis, RedCap, and eRedCap UEs can all be implemented with either one or two antennas. The minimum requirement for Massive Communication should also be a single antenna to be able to achieve low enough complexity and small form factor, at least in FR1 bands. Means to restrict the implementation of a single antenna to low-end UEs and to limit their access to specific cells or carriers (in the same way as, e.g., for RedCap) should be considered.

Modest peak rates: See Section 2.2.2.

Limited bandwidth: Cat-1/1bis, Cat-4, RedCap, and eRedCap UEs support 20 MHz UE RF bandwidth in FR1 bands. NB-IoT and eMTC have 200 kHz and 1.4 MHz UE bandwidths, respectively. To maintain a common air interface (same sync signals, etc.) for 6G IoT UEs and other 6G UEs, it is desired that the minimum UE bandwidth is not too small. For this reason, the minimum requirement should not be smaller than 3~5 MHz and could potentially be different for low/mid/high band. Reducing the UE bandwidth further than this is not expected to provide significant additional complexity reduction. A reasonable tentative value is 5 MHz at least for low FR1 bands.

Half-duplex FDD operation: In FDD bands, NB-IoT, eMTC, RedCap, and eRedCap UEs can be implemented with half-duplex operation, meaning that they are not required to transmit and receive at the same time. Other 4G/5G UEs including Cat-1/1bis and Cat-4 only support full-duplex operation, which requires a duplex filter, or possibly multiple duplex filters if the UE supports multiple bands, which can become complex and increase the device cost. It can be troublesome to introduce support for half-duplex FDD retroactively in the standard and in products. Therefore, it is desired that half-duplex FDD operation is properly supported already in the first 6G release.

### 2.2.2	Peak data rate

Cat-1/1bis and eRedCap UEs support DL/UL peak rates of 10/5 Mbps and 10/10 Mbps, respectively. Very roughly speaking, NB-IoT and eMTC UEs have an order of magnitude lower peak rates, and Cat-4 and RedCap UEs have an order of magnitude higher peak rates.

Too high peak rate should be avoided for complexity and power consumption reasons. Too low peak rate should be avoided for spectral efficiency, latency, and energy consumption reasons. A reasonably high peak rate will be beneficial when somewhat larger amounts of data need to be transferred efficiently, for example during software/firmware upgrade. The minimum required peak rate for Massive Communication may need further discussion, but a reasonable tentative value is 5~10 Mbps.

### 2.2.3	Latency

The 5G latency requirement (described in the Annex in the end of this contribution) is <10 seconds for small infrequent packets transmitted in uplink in a bad coverage scenario (164 dB MaxCL). In better coverage, much better latency can be expected, both in the control plane (CP) and the user plane (UP). The CP and UP latency requirements for 5G eMBB (also described in the Annex) are 10 ms and 4 ms, respectively.

For Massive Communication in 6G, it may be reasonable to require a maximum CP latency corresponding to an end-to-end latency of no more than ~100 ms, allowing an IoT device to set up a connection and send a small packet relatively quickly (at least for a UE in normal coverage, e.g., up to ~144 dB MaxCL).

Furthermore, a UP latency requirement allowing delivery of the equivalent data volume of ~30 kbps in both DL and UL within a packet delay budget of ~50 ms may be reasonable (at least for a UE in normal coverage, e.g., up to ~144 dB MaxCL), corresponding to 30,000*0.050=1500 bits within the ~50 ms. This may enable support of for example a conversational voice service, which can be useful for some IoT services.

### 2.2.4	Coverage

The 5G extreme coverage requirement (described in the Annex) is 164 dB MaxCL for a data rate of 160 bps, which is supported by NB-IoT (and by eMTC with CE mode B). This coverage is achieved through extensive time repetition of the physical signals/channels in both DL and UL.

A coverage target needs to be determined for Massive Communication in 6G. Unless there is a clear market need for extreme coverage through massive repetition, it may be enough with an extended coverage region that provides somewhat higher cell-edge data rates. A MaxCL of ~154 dB may be a reasonable tentative target, which may correspond to ~10 dB extension compared to normal coverage (~144 dB MaxCL). Many UEs will not need the 10-dB coverage extension, but UEs with a single antenna may need at least a few dB for coverage recovery. The coverage enhancement techniques can be supported by 6G IoT UEs as well as ordinary 6G UEs.

### 2.2.5	UE battery life

There will be Massive Communication use cases in 6G that require long battery lives. The complexity reduction techniques listed above can help reduce power consumption, and additional power saving techniques such as extended discontinuous reception (eDRX) and wake-up receivers (WUR) are beneficial for UEs that need a long battery life. These additional power saving techniques can be supported by 6G IoT UEs as well as ordinary 6G UEs.

### 2.2.6	Connection density

Connection density requirements are expected to be addressed in the IMT-2030 TPR discussion [5][6] and are not addressed further in this contribution.

# 3	Text proposal

Based on the discussion above we propose the following text proposal.

## 5.x	Minimum requirements for Massive Communication

The 6GR and 6G RAN architecture shall support the following minimum requirements for Massive Communication:

- The minimum number of UE receive antennas is 1, at least for FR1 bands.

- The minimum PHY peak data rate is [5~10] Mbps in DL and [5~10] Mbps in UL.

- The minimum UE transmit/receive RF bandwidth is [5] MHz, at least for low FR1 bands.

- Half-duplex FDD operation is supported (beside full-duplex FDD operation and TDD operation).

- The maximum CP latency is [FFS] ms (at least for a UE in normal coverage, e.g., up to [144] dB MaxCL) to enable a maximum end-to-end latency of [100] ms.

- The maximum UP latency is [50] ms for simultaneous delivery of [1500] bits in both DL and UL (at least for a UE in normal coverage, e.g., up to [144] dB MaxCL).

- A coverage of up to [154] dB MaxCL is supported (with a cell-edge data rate which is FFS), which is commonly applicable to any UEs.

# 4	Conclusion

In this contribution we propose to add the text proposal in section 3 to TR 38.914 clause 5.

# References

M.2160-0 (11/2023), “Recommendation ITU-R M.2160-0: Framework and overall objectives of the future development of IMT for 2030 and beyond”, ITU-R, November 2023.

RP-250810, “Revised SID: Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom, RAN#107, March 2025.

TR 38.914, “Study on 6G Scenarios and requirements”, RAN.

S2-2504686, “New Study on Architecture for 6G System”, ZTE (Moderator), SA2#169, May 2025.

RP-243325, “Draft Reply to ITUR_WP5D_TEMP_167 = RP-243202 on Minimum requirements related to technical performance for IMT-2030 radio interface(s)”, RAN#106, December 2024.

RP-250819, “Moderator's summary for IMT-2030 TPRs”, Moderator (CMCC), RAN#107, March 2025.

TS 22.261, “Service requirements for the 5G system”, SA1.

TR 38.913, “Study on scenarios and requirements for next generation access technologies”, RAN.

# Annex: Existing 5G requirements

For 5G, TS 22.261 [7] specifies the following service requirements related to low-end devices:

## 6.4 Resource efficiency ### 6.4.1 Description […] For small form factor UEs it will be challenging to have more than 1 antenna due to the inability to get good isolation between multiple antennas. Thus, these UEs need to meet the expected performance in a 5G network with only one antenna. […] ### 6.4.2 Requirements #### 6.4.2.1 General […] The 5G system shall support small form factor UEs with single antenna. NOTE: Small form factor UEs are typically expected to have the diagonal less than 1/5 of the lowest supported frequency wave length. […] The 5G system shall support UEs with different characteristics such as data rate, power consumption, etc.

For 5G, the RAN study item on scenarios and requirements defines several mMTC-related requirements in TR 38.913 [8] including the following ones:

## 7.6 Latency for infrequent small packets For infrequent application layer small packet/message transfer, the time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point at the mobile device to the radio protocol layer 2/3 SDU egress point in the RAN, when the mobile device starts from its most "battery efficient" state. For the definition above, the latency shall be no worse than 10 seconds on the uplink for a 20 byte application packet (with uncompressed IP header corresponding to 105 bytes physical layer) measured at the maximum coupling loss (MaxCL) of 164dB. Analytical evaluation is the baseline evaluation methodology and system level evaluation can be considered if needed. […] ## 7.10 Coverage MaxCL in uplink and downlink between device and Base Station site (antenna connector(s)) for a data rate of 160bps, where the data rate is observed at the egress/ingress point of the radio protocol stack in uplink and downlink. The target for coverage should be 164dB. Link budget and/or link level analysis are used as the evaluation methodology. […] ## 7.11 UE battery life UE battery life can be evaluated by the battery life of the UE without recharge. For mMTC, UE battery life in extreme coverage shall be based on the activity of mobile originated data transfer consisting of 200bytes UL per day followed by 20bytes DL from MaxCL of 164dB, assuming a stored energy capacity of 5Wh. The target for UE battery life for mMTC should be beyond 10 years, 15 years is desirable. Analytical evaluation is used as the evaluation methodology. […] ## 7.17 Connection density Connection density refers to total number of devices fulfilling a target QoS per unit area (per km2), where the target QoS is to ensure a system packet drop rate less than 1% under given packet arrival rate l and packet size S. Packet drop rate = (Number of packet in outage) / (number of generated packets), where a packet is in outage if this packet failed to be successfully received by destination receiver beyond packet dropping timer. The target for connection density should be 1 000 000 device/km2 in urban environment. 3GPP should develop standards with means of high connection efficiency (measured as supported number of devices per TRxP per unit frequency resource) to achieve the desired connection density. Analytical, link level evaluation and system level evaluation are to be performed for Urban coverage for massive connection (Urban environment).

TR 38.913 [8] furthermore includes the following general 5G requirements on latency:

## 7.4 Control plane latency Control plane latency refers to the time to move from a battery efficient state (e.g., IDLE) to start of continuous data transfer (e.g., ACTIVE). The target for control plane latency should be 10ms. Analytical evaluation is used as the evaluation methodology. NOTE1: For satellite communications link, the control plane should be able to support RTT of up to 600ms in the case of GEO and HEO, up to 180ms in the case of MEO, and up to 50ms in the case of LEO satellite systems. ## 7.5 User plane latency The time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point via the radio interface in both uplink and downlink directions, where neither device nor Base Station reception is restricted by DRX. For URLLC, the target for user plane latency should be 0.5ms for UL, and 0.5ms for DL. Furthermore, if possible, the latency should also be low enough to support the use of the next generation access technologies as a wireless transport technology that can be used within the next generation access architecture. NOTE1: The reliability KPI also provides a latency value with an associated reliability requirement. The value above should be considered an average value and does not have an associated high reliability requirement. For eMBB, the target for user plane latency should be 4ms for UL, and 4ms for DL. NOTE2: For eMBB value, the evaluation needs to consider all typical delays associated with the transfer of the data packets in an efficient way (e.g. applicable procedural delay when resources are not preallocated, averaged HARQ retransmission delay, impacts of network architecture). When a satellite link is involved in the communication with a user equipment, the target for user plane RTT can be as high as 600ms for GEO satellite systems, up to 180ms for MEO satellite systems, and up to 50ms for LEO satellite systems. NOTE3: For the satellite case, the evaluation needs to consider the max RTT that is associated with the GEO satellite systems. Analytical evaluation is used as the evaluation methodology.