3GPP TSG RAN Meeting #108	RP-251242

Prague, Czech Republic, June 9 – 13, 2025

Source:	Intel Corporation

Title:	Discussion on Discussion on 5G-A AI/ML-based CSI compression in Rel-20

Agenda item:	15.1.1

Document for:	Discussion

# Introduction

In this contribution, we share our views on the scope of Rel-20 WI for 5G-A on AI/ML for air interface, with focus on AI/ML-based CSI compression.

# Discussion on Rel-20 AI/ML Air-Interface Objectives

## Background: Terminology

The following terminologies from the Rel-18/19 SIs on AI/ML for air interface are relevant to the discussion in the present document.

### AI/ML-based CSI compression sub-use-cases

| Case | Target CSI slot(s) | Whether the UE uses past CSI information | Whether the network uses past CSI information |
| --- | --- | --- | --- |
| 0 | Present slot | No | No |
| 1 | Present slot | Yes | No |
| 2 | Present slot | Yes | Yes |
| 3 | Future slot(s) | Yes | No |
| 4 | Future slot(s) | Yes | Yes |
| 5 | Present slot | No | Yes |

Of the above sub-use-cases, Cases 0 (no temporal domain compression and/or prediction), 2, and 3 were prioritized during Rel-19 studies.

### Solutions to address inter-vendor coordination for two-sided AI/ML models for CSI compression

| Option | Prioritized Sub-Option(s) | Description | Mapping to Directions A/B/C |
| --- | --- | --- | --- |
| 1 | 1 | Fully standardized reference model (structure + parameters) | Direction C |
| 2 | 2 | Standardized dataset | - |
| 3 | 3a-1 | Standardized reference model structure + Parameter exchange from the NW-side to the UE-side - Parameters exchanged from the NW-side to UE-side is CSI generation part | Direction A Parameters received at the UE or UE-side goes through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., potential re-training, re-development of a different model, and/or offline testing. |
| 4 | 4-1 | Standardized data / dataset format + Dataset exchange between NW-side and UE-side - Dataset exchanged from the NW-side to UE-side consists of (target CSI, CSI feedback) | Direction A Parameters received at the UE or UE-side goes through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., potential re-training, re-development of a different model, and/or offline testing. |
| 5 | 5a-1 | Standardized model format + Reference model exchange between NW-side and UE-side - Model exchanged from the NW-side to UE-side is CSI generation part | Direction A Parameters received at the UE or UE-side goes through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., potential re-training, re-development of a different model, and/or offline testing. |

## Status from RAN1 and RAN4 on studies on AI/ML-based CSI compression

During RAN1 #121 meeting, RAN1 concluded the Rel-19 studies on AI/ML-based CSI compression covering the following aspects:

- Performance-complexity trade-off

- Inter-vendor collaboration complexity

- Other specification impacts, including model monitoring and data collection.

On the performance-complexity trade-off, a key observation from the studies during Rel-19 was that most of the gains in terms of SGCS can be realized with encoders/decoders AI/ML models of sizes around 1M parameters and complexity of around 10M FLOPs.

Further, RAN1 identified the most promising candidate solutions to the inter-vendor coordination problem.

These were concluded as being feasible in addressing the inter-vendor coordination needs and include:

- Direction C

- Direction A, sub-option 3a-1

  - With target CSI sharing from NW-side

  - Without target CSI sharing from NW-side

- Direction A, sub-option 4-1

Potential specification impacts that apply commonly to the three prioritized solutions to the inter-vendor coordination problem were identified by RAN1 with the following recommendations for Rel-20 [1]:

- Recommend Case 0

  - Cases 2 and 3 can be considered for further enhancement built upon the specification of case 0.

- Recommend following specification impacts for the two-sided CSI compression use case:

  - Model pairing procedure including ID and applicability reporting

  - Inference aspects including target CSI type, measurement and report configuration, CQI / RI determination, payload determination, quantization configuration / codebook, UCI mapping, CSI processing criteria and timeline, priority rules for CSI reports

  - NW- and UE-side data collection for training

    - Target CSI format

    - Note: The framework defined in BM and CSI prediction use cases could be reused.

  - Specify performance monitoring, if needed

For the three candidate solutions to the inter-vendor coordination problem, RAN1 identified potential specification impacts:

- Reference model / structure specification (for sub-option 3a-1 and Direction C)

  - Note: The so called fully specified reference model discussed in RAN1 for Direction C is assumed to be equivalent to the to-be-specified model, if specified, for performance requirements in RAN4.

  - Note: The model structure(s) for the reference encoder in inter-vendor collaboration sub-option 3a-1 can be same as/equivalent to the structure of the RAN4 defined/specified (if defined/specified for testability) reference encoder.

    - To reduce the workload of the potential normative work scope, the model structure(s) for the reference encoder in inter-vendor collaboration sub-option 3a-1 is(are) same as/equivalent to the structure of the RAN4 defined/specified (if defined/specified for testability) reference encoder.

  - Note: RAN1 has studied and agreed on a scalable model structure for feasibility study during Rel-19 study, and considers that can serve as a starting point for the model structure specification for the potential normative work.

- Format and contents of parameter/dataset/information exchange: dataset (for 4-1 and 3a-1 with target CSI)/ encoder parameter (for 3a-1) / additional information (for 3a-1 and 4-1).

The above notes with commentary on the relationship to studies in RAN4 can be seen as consistent with agreements in RAN4:

Agreements: If RAN1 direction C agreed, RAN4 and RAN1 should co-ordinate on model structure etc. for direction C due to the similarities between option 3/4 and direction C - Duplicated work is to be avoided

However, RAN1 was not able to converge on recommendation(s) regarding whether to pursue down-selection or pursue normative work for all of the identified promising solutions for inter-vendor coordination.

# Discussion on Objectives for Rel-20 AI/ML Air-Interface

In our view, Rel-20 WI on AI/ML for air interface should include normative work on CSI compression with a clearly defined scope.

Regarding down-selection to sub-use-case 0 (no temporal domain compression or prediction), the RAN1 recommendation seems reasonable and should be followed.

On inter-vendor coordination, in the following, we outline few key observations regarding the RAN1-identified feasible solutions for inter-vendor coordination below.

- Direction C (Standardized reference models for encoder and decoder)

  - Ensures a minimum performance requirement

  - Can allow for a certain level of adaptation at the UE- and NW-sides as long as compatibility and required minimum performance relative to the reference model-pair are ensured.

    - This can facilitate localized models and re-training/adapting to field data to a limited extent.

  - Requires specification of one or more standardized model structures that offer a certain level of scalability in terms of the bandwidth (number of subbands), number of Tx antenna ports, CSI payload sizes, etc. for possibly different model tokenization options.

    - Common aspect between Direction C and Direction A, sub-option 3a-1

  - Requires specification of one or more Encoder-Decoder-Quantization codebook-Training dataset tuples consistent with the specified model structure(s)

  - Can be seen as equivalent to the currently expected option pursued by RAN4 to specify encoder-decoder model(s) for defining performance requirements in RAN4

- Direction A, sub-option 3a-1 (Model parameters for a specified model structure for encoder part exchanged from NW- to UE-side)

  - Can potentially offer improved performance compared to Direction C via the ability to adapt/re-train models at NW- and/or UE-sides with incorporation of field data for training beyond what may be possible when conforming to standardized reference models for encoder-decoder

  - Requires signalling support including definition of format for model parameters for the CSI generation part from the NW- to UE-side.

  - Requires specification of one or more standardized model structures that offer a certain level of scalability in terms of the bandwidth (number of subbands), number of Tx antenna ports, CSI payload sizes, etc. for possibly different model tokenization options.

    - Common aspect between Direction C and Direction A, sub-option 3a-1

- Direction A, sub-option 4-1 (Dataset for encoder part for model training exchanged from NW- to UE-side)

  - Can potentially offer improved performance compared to Direction C via the ability to adapt/re-train models at NW- and/or UE-sides with incorporation of field data for training beyond what may be possible when conforming to standardized reference models for encoder-decoder

  - Requires signalling support including definition of format for dataset for the CSI generation part from the NW- to UE-side.

In general, it would not be advisable to pursue normative work for all the identified feasible solutions (effectively, count up to four solutions at the high-level itself) for a single issue.

From the key observations above and the detailed observations from the evaluations and studies reported in TR 38.843 on the issue of inter-vendor coordination, the consideration of Direction C seems rather practical in the context of RAN4 testability defining minimum performance requirements.

However, it would be worth considering another solution that can offer better flexibility in model adaptation which is crucial towards addressing the challenge of realizing well-generalized models based on synthetic data for training. This can ensure that the feature can offer good achievable performance in scenarios with significant mismatch between the corresponding actual field data and the synthetic data used to train/develop the standardized reference models.

Consequently, with a view to leveraging the common requirement of specifying a scalable model structure between Direction C and Direction A, sub-option 3a-1, the latter can also be pursued for normative work. In this case, reference model structure (common to Directions C and A, sub-option 3a-1) and reference model (for Direction C) can be developed in cooperation between RAN4 and RAN1 while utilizing the studies conducted in RAN1 so far on model structure scalability.

Overall, this would offer standardized reference model(s) with specified Encoder-Decoder-Quantization codebook-Training dataset tuple(s) that use a standardized model structure offering a limited degree of scalability for different application scenarios and is/are used to define the minimum performance requirements. In addition, signalling support to be introduced to facilitate exchange of encoder parameters (for the CSI generation part) for the standardized model structure from the NW-side to the UE-side as per Direction A, sub-option 3a-1. Further, both with and without target CSI sharing can be supported for Direction A, sub-option 3a-1, where “with target CSI sharing” implies sharing of training dataset.

Direction A, sub-option 4-1 offers another path to enable more flexible development of encoder-decoder pairs with sharing of only training dataset from the NW-side to UE-side and may require specification of model structure (at least in part) to ensure sufficient alignment between NW- and UE-sides. Introducing yet another solution beyond the above two would not be desirable – not only from the perspective of workload but also from the perspective of offering multiple solutions to the same problem to the industry at large.

Based on the discussion above, the following are proposed for the Rel-20 WI on AI/ML-based CSI compression.

Proposal 1: Consider the following objectives for scoping of the Rel-20 WI on AI/ML for air interface:

- Provide specification support for AI/ML-based CSI compression using two-sided models for CSI compression sub-use-case 0, involving Spatial-Frequency domain compression.

  - Specify model pairing procedure including ID and applicability reporting [RAN1, RAN2]

  - Specification support for inference aspects including target CSI type, measurement and report configuration, CQI / RI determination, payload determination, quantization configuration / codebook, UCI mapping, CSI processing criteria and timeline, priority rules for CSI reports  [RAN1, RAN2]

  - NW- and UE-side data collection for training [RAN1, RAN2]

    - Target CSI format

    - Note: The framework defined in BM and CSI prediction use cases could be reused.

  - Specify solutions for inter-vendor coordination for two-sided models [RAN1, RAN4]

    - (For Direction A-sub-option 3a-1 and Direction C) Specify reference model structure for encoder-decoder pair for CSI compression

      - Note: The model structure(s) for the reference encoder in inter-vendor collaboration sub-option 3a-1 can be same as/equivalent to the structure of the RAN4 defined/specified (if defined/specified for testability) reference encoder.

      - Note: RAN1 has studied and agreed on a scalable model structure for feasibility study during Rel-19 study, and considers that can serve as a starting point for the model structure specification for the potential normative work

    - (For Direction C) Specify reference model for encoder-decoder pair for CSI compression for the specified model structure

      - Note: The so called fully specified reference model discussed in RAN1 for Direction C is assumed to be equivalent to the to-be-specified model, if specified, for performance requirements in RAN4

    - (For Direction A-sub-option 3a-1) Specify format and contents of parameters and any additional information exchange from NW-side to UE-side: dataset (when target CSI is shared)/ encoder parameters/ additional information (e.g., performance target in terms of SGCS and/or NMSE)

    - (For Direction A-sub-option 3a-1) Specify signalling support for delivery of parameters and any additional information from NW-side to UE-side

  - Specify performance monitoring, if needed [RAN1, RAN2]

  - Define RRM core requirements for the above [RAN4]

  - Define performance requirements for the above [RAN4]

# Conclusions

In this contribution, we discussed the outcome of the studies in Rel-19 for CSI compression and the conclusions made by RAN1 and shared our views on the potential scoping of Rel-20 WI on AI/L for air interface. Based on the presented discussion, the following proposal is made.

Proposal 1: Consider the following objectives for scoping of the Rel-20 WI on AI/ML for air interface:

- Provide specification support for AI/ML-based CSI compression using two-sided models for CSI compression sub-use-case 0, involving Spatial-Frequency domain compression.

  - Specify model pairing procedure including ID and applicability reporting [RAN1, RAN2]

  - Specification support for inference aspects including target CSI type, measurement and report configuration, CQI / RI determination, payload determination, quantization configuration / codebook, UCI mapping, CSI processing criteria and timeline, priority rules for CSI reports  [RAN1, RAN2]

  - NW- and UE-side data collection for training [RAN1, RAN2]

    - Target CSI format

    - Note: The framework defined in BM and CSI prediction use cases could be reused.

  - Specify solutions for inter-vendor coordination for two-sided models [RAN1, RAN4]

    - (For Direction A-sub-option 3a-1 and Direction C) Specify reference model structure for encoder-decoder pair for CSI compression

      - Note: The model structure(s) for the reference encoder in inter-vendor collaboration sub-option 3a-1 can be same as/equivalent to the structure of the RAN4 defined/specified (if defined/specified for testability) reference encoder.

      - Note: RAN1 has studied and agreed on a scalable model structure for feasibility study during Rel-19 study, and considers that can serve as a starting point for the model structure specification for the potential normative work

    - (For Direction C) Specify reference model for encoder-decoder pair for CSI compression for the specified model structure

      - Note: The so called fully specified reference model discussed in RAN1 for Direction C is assumed to be equivalent to the to-be-specified model, if specified, for performance requirements in RAN4

    - (For Direction A-sub-option 3a-1) Specify format and contents of parameters and any additional information exchange from NW-side to UE-side: dataset (when target CSI is shared)/ encoder parameters/ additional information (e.g., performance target in terms of SGCS and/or NMSE)

    - (For Direction A-sub-option 3a-1) Specify signalling support for delivery of parameters and any additional information from NW-side to UE-side

  - Specify performance monitoring, if needed [RAN1, RAN2]

  - Define RRM core requirements for the above [RAN4]

  - Define performance requirements for the above [RAN4]

# References

- R1-2505019 - Text proposal to capture the output of Rel-19 study of Agenda Item 9.1.4.1 into TR 38.843, RAN1 #121, May 2025.