3GPP TSG RAN Meeting #108														 RP-251247

Prague, Czech Republic, June 9 – 13, 2025

Agenda item:	16.1

Source:	Telstra Limited

Title:	TR 38.914 text proposal for Extreme Long Distance coverage

Document for:	Agreement

## Background and discussion

At TSG RAN#107, the second phase of the 6G Scenarios and Requirements study was initiated for RAN to study requirements and deployment scenarios for 6G.

This contribution presents a text proposal for the new 3GPP TR 38.914 (“Study on 6G Scenarios and requirements”), to add the deployment scenario for long range cells. The key characteristics of this scenario are Macro cells with very large area coverage supporting basic data speeds and voice services, with low to moderate user throughput and low user density.

This remains an equally important deployment scenario for 6G and should be included in the Technical Report. The fundamental definition of the scenario is re-used from 5G. The traffic model has been updated to reflect more modern usage observed in our mobile network in areas serviced with such extreme coverage.

## Proposal

- We propose the following text for introduction of long distance coverage for approval into TR 38.914.

TEXT PROPOSAL for TR 38.914:

### 4.x.x	Extreme long distance coverage in low density areas

The extreme Long Range deployment scenario is defined to allow for the provision of services for very large areas with low density of users whether they are humans and machines (e.g. Low ARPU regions, wilderness, areas where only highways are located, etc). The key characteristics of this scenario are Macro cells with very large area coverage supporting basic data speeds and voice services, with low to moderate user throughput and low user density.

Table 4.x.x: Attributes for extreme rural

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Below 3 GHz With a priority on bands below 1GHz Around 700 MHz |
| System Bandwidth | 40 MHz (DL+UL) |
| Layout | Single layer: Isolated Macro cells |
| Cell range | 100 km range (Isolated cell) to be evaluated through system level simulations. Feasibility of Higher Range shall be evaluated through Link level evaluation (for example in some scenarios ranges up to 150-300km may be required). |
| User density and UE speed | User density: NOTE1 Speed up to 160 km/h |
| Traffic model | Average data throughput at busy hours/user: 30 kbps User experienced data rate: DL: up to 2 Mbps while stationary and 384 kbps while moving UL: up to 1 Mbps while stationary and 100 kbps while moving |

NOTE1:	Evaluate how many users can be served per cell site when the range edge users are serviced with the target user experience data rate.