3GPP TSG RAN Meeting #108	                                              	                    RP-251252

Prague, Czech Republic, June 9-13, 2025

Agenda item:	15.1.1

Title:	Discussion on AI/ML for NR air interface enhancements work scope in Rel-20

Source:	OPPO

Document for:	Discussion

# Introduction

In Rel-18, the technologies of AI/ML have been introduced into 3GPP RAN1 working group and intensively studied. At the beginning of Rel-19, for the case of beam management and positioning, RAN1 step into normative work. Later on, the use case of CSI prediction has been studied from Q1 2025. As for CSI compression with two-side model, the study will proceed until the end of Rel-19 [1].

With two continuous study phases on CSI compression, we tend to believe RAN working group can complete its normative work in Rel-20 as 5G-Advance.

Provide specification support for the following aspects: - AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]: - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback - Identification related signalling is part of the above objective - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models - Signalling mechanism of applicable functionalities/models - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2 - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024. - CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025 Study objectives: - CSI feedback enhancement [RAN1]: - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing necessary specification impact analysis, as well as, other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843. - Necessity and details of model Identification concept and procedure in the context of LCM for two-sided models [RAN2/RAN1] - CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]: - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods - Model transfer/delivery [RAN2/RAN1]: - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study

In this contribution, we will briefly discuss how to handle the use case of CSI compression with two-side model in Rel-20 5G-A for AI/ML.

# CSI compression

During the 2nd study phase of CSI compression in Rel-19, two major issues have been identified. The first issue is to balance the performance and complexity/overhead. The second issue is to address the training collaboration among vendors.

## Evaluation Observations

During the Rel-18 and Rel-19 study phase, evaluations have been conducted by many companies. Performance gain from the perspective of intermediate KPI (e.g., SGCS) and eventual throughput are achieved over legacy non-AI benchmark. Furthermore, based on the observations in RAN1#121, Rel-19 evaluations show improved performance/complexity trade-off compared to Rel-18 evaluations for temporal domain Case 0 with optimized AI/ML model structures.

Observation 1: Performance gain of CSI compression on UPT can be achieved with trade-off between complexity when compared with legacy CSI feedback.

## Inter-vendor collaboration

In the current Rel-19 study phase of two-side model, the issue of inter-vendor collaboration has been widely discussed and evaluated in RAN1. Both of Option 4-1, Option 3a-1 and Direction C (Option 1) are confirmed feasible.

Those Options can be listed below.

- Option 4-1: Dataset exchange from the NW-side to UE-side consists of (target CSI, CSI feedback).

- Option 3a-1: Parameters exchange from the NW-side to UE-side is CSI generation part with standardized reference model structure (including with target CSI and without target CSI).

- Direction C (Option 1): Fully standardized reference model(s) and parameters with specified CSI generation part and/or CSI reconstruction part

From our understanding, anyone of the above options can address the problem of inter-vendor collaboration.

Observation 2: For two-sided model, the inter-vendor collaboration issue is addressable via any of Option 4-1, Option 3a-1 and Direction C in RAN1.

## Normative work for CSI compression

With the above being said, we believe the issues of performance/complexity tradeoff and inter-vendor collaboration can be addressed in RAN1 and other related working groups. In Rel-20, it is time to deliver the study/evaluation results of two-side model into reality.

Proposal 1: Considering the fact that performance/complexity tradeoff and inter-vendor collaboration are addressable, there is no obstacle to specify CSI compression in Rel-20 5G-advance.

As far as we know, there could be parallel study/working items on AI/ML in Rel-20. Specifically, AI/ML could be a totally new study item for 6G, whereas there could be another AI/ML normative work (e.g. CSI compression with two-side model) for NR. Considering the large effort companies contributed to AI/ML during Rel-18 and Rel-19, it makes sense to avoid duplicated work as much as possible during Rel-20. For instance, if the RAN working group(s) specify the CSI compression with two-side model for NR, then we should not study it again for 6G CSI feedback in parallel. From this sense, the workload can be manageable and reasonable.

Proposal 2: It makes sense to avoid duplicated work between Rel-20 NR WI and 6G SI, e.g. on CSI compression with two-side model.

# Aspects of CSI compression for normative work

If RAN agrees to specify the CSI compression with two-sided model, the following design should be completed.

## Inter-vendor collaboration

In the last RAN1#121 meeting, a few of conclusions and agreements were achieved to address the issue of inter-vendor collaboration. RAN1 has identified the following impacts:

- Reference model / structure specification (for Option 3a-1 and Direction C): to avoid the duplicated work and reduce the workload, RAN1 has agreed the model structure(s) for the reference encoder in Option 3a-1 is(are) same as/equivalent to the structure of the RAN4 defined/specified (if defined/specified) reference encoder. Moreover, to satisfy the requirement on the performance and scalability, it is recommended that RAN1 and RAN4 can cooperate on the specification of reference encoder structure.

- Format and contents of parameter/dataset/information exchange: this part can be specified in RAN1. To reduce the workload, RAN1 should avoid the duplicated work on the common parts among different options. For example, the dataset in Option 4-1 should follow the same format as that of Option 3a-1 with target CSI exchange. Secondly, regarding the different target CSI types, such as eigenvector in spatial-frequency/angular-delay domain and raw channel matrix, we suggest focussing on only one type of target CSI with eigenvector in spatial-frequency domain, and then specify the reference encoder structure based on this type of target CSI as the model input.

The reason why RAN1 consider both Direction A (Option 3a-1 and Option 4-1) and Direction C as feasible lies in the expectation that RAN1 can reuse the reference model (to-be-specified) for performance tests in RAN4. This is to avoid duplicated work in RAN1 given small amount of TU allocated.

However, consider the fact that the interoperability and RAN4 testing feasibility is a separate study in RAN4, there is a chance that the reference encoder will not be specified in RAN4. If that’s the case, we think the only way to address inter-vendor collaboration issue is to exchange dataset, i.e. Option 4-1 under Direction A.

Agreement For addressing inter-vendor collaboration complexity, RAN1 identifies the following specification impacts for supporting sub-option 3a-1 (including with target CSI and without target CSI), sub-option 4-1, and Direction C - Reference model / structure specification (for sub-option 3a-1 and Direction C) - Note: The so called fully specified reference model discussed in RAN1 for Direction C is assumed to be equivalent to the to-be-specified model, if specified, for performance requirements in RAN4. - Note: The model structure(s) for the reference encoder in inter-vendor collaboration sub-option 3a-1 can be same as/equivalent to the structure of the RAN4 defined/specified (if defined/specified for testability) reference encoder. - To reduce the workload of the potential normative work scope, the model structure(s) for the reference encoder in inter-vendor collaboration sub-option 3a-1 is(are) same as/equivalent to the structure of the RAN4 defined/specified (if defined/specified for testability) reference encoder. - Note: RAN1 has studied and agreed on a scalable model structure for feasibility study during Rel-19 study, and considers that can serve as a starting point for the model structure specification for the potential normative work. - Format and contents of parameter/dataset/information exchange: dataset (for 4-1 and 3a-1 with target CSI)/ encoder parameter (for 3a-1) / additional information (for 3a-1 and 4-1). - The workload can be reduced by identifying the common part among different aspects. Agreement For inter-vendor collaboration, RAN1 concludes that both Direction A and Direction C are feasible. For Direction A, RAN1 concludes that the feasible sub-options are sub-option 4-1 and sub-option 3a-1, and in case of sub-option 3a-1, with and without target CSI sharing from NW side. - Performance: Direction C with standardized reference model (structure + parameters) ensures a minimum performance requirement, while Direction A with standardized reference model structure + parameters (sub-option 3a-1), or standardized dataset exchange (sub-option 4-1) from NW-side to UE-side may achieve better performance. - Direction C addresses inter-vendor collaboration complexity: From RAN1 perspective, Direction C (a.k.a., option 1) eliminates the inter-vendor collaboration complexity if it is feasible to standardize the model. The fully standardized reference model (structure + parameters) should be trained using synthetic data. The model structure and parameters are assumed to be equivalent to the to-be-specified model, if specified, for performance requirements in RAN4. RAN1 concludes that the scalable model structure specification and model parameters specification is feasible, but the conclusion on interoperability and RAN4 testing feasibility is a separate study in RAN4. - For Direction A with sub-option 4-1 and 3a-1, RAN1 concludes that standardized signaling, if feasible and specified, can be used for model parameter / dataset exchange to alleviate / resolve the inter-vendor collaboration complexity. The standardized signaling, if needed, may be over-the-air, or other approaches. For OTA and non-OTA based standardized signaling, as well as the feasibility of standardized signaling, related status have been provided in [R2-2503169]. For sub-option 3a-1, the specified encoder (a.k.a., CSI generation part) structure should be determined using synthetic data, and the scalable encoder structure specification is feasible.

Proposal 3: To address the inter-vendor collaboration, we consider the following solutions

- If RAN4 specifies the reference model structure(s) and/or parameters satisfying feasibility and scalability, support to specify the format and content of parameter/dataset/information exchange in RAN1

- Otherwise, only support dataset exchange (i.e. Option 4-1) in RAN1

## Spatial-frequency (Case 0) vs. Temporal-spatial-frequency (Case 2/3)

In Rel-19, the spatial-frequency domain CSI compression (Case 0), temporal domain Case 2 with time-frequency-spatial domain CSI compression and Case 3 with joint CSI prediction and compression are further evaluated. However, considering the workload in limited time budget, it is recommended to treat Case 0 in higher priority in normative phase to make the two-sided model workable. Then, if time permitted, Case 2 and Case 3 can be considered for further enhancement built upon the specification of Case 0 by reusing the common part as much as possible.

Proposal 4: Recommend the spatial-frequency domain CSI compression (Case 0) into Rel-20 5G-A normative work.

## Data collection for training

In terms of data collection for training, it is necessary for NW to collect the ground-truth label. For Option 4-1 to resolve the inter-vendor collaboration issue, NW-side CSI generation part and CSI reconstruction part models are trained firstly based on NW-side dataset. Basically, NW-side data are collected with ground-truth CSI reporting from multiple UEs. Therefore, mechanisms/signaling of the ground-truth reporting should be specified to facilitate the NW-side data collection for model training.

Proposal 5: For model training at NW-side, specify mechanism/signaling for ground-truth reporting by UE in Rel-20 5G-Advance.

As for model training at UE-side, it is necessary to specify the mechanisms/signaling to facilitate the UE-side data collection, so that UE can train its own CSI generation part without NW-side data sharing. Since different UEs/UE-vendors may have different requirements for UE-side data collection, it is better up to UE to trigger the procedure for data collection.

Proposal 6: For model training at UE-side, support UE requested RS configuration/transmission.

## Performance monitoring

After the functionality of CSI compression is activated with two-side model, there should be mechanisms to monitor its performance. Since the encoder and decoder are deployed at UE-side and NW-side respectively, it is straightforward to monitor its performance at both UE-side and NW-side.

Regarding NW-side monitoring, reuse the legacy codebook for target CSI reporting. Considering the reporting overhead and workload, possible enhancement with higher-resolution parameter combination on the codebook is out of AI/ML scope.

Regarding UE-side monitoring, we support of SGCS reporting by UE to assist NW to perform the monitoring. Whether SGCS is calculated through Case 2-1 with proxy model or Case 2-2 with SGCS estimator can be up to UE implementation. To our understanding, the SGCS reporting has been supported in CSI prediction agenda item, which brings very limited additional work to UE-side monitoring for the use case of two-sided CSI compression.

Proposal 6: To monitor the performance of CSI compression, support:

- NW-side monitoring with target CSI reporting by UE via legacy codebook

  - Possible enhancement with higher-resolution parameter combination on the codebook is not supported

- UE-side monitoring with SGCS reporting by reusing the framework defined for Rel-19 CSI prediction

# Objectives for CSI compression

With the above being discussed and the agreement in RAN1#121, we would like to suggest Rel-20 5G-A objectives for two-sided model CSI compression involving RAN1, RAN2 and RAN4 as below.

Specification support for spatial-frequency domain CSI compression (i.e. Case 0)

- CSI feedback enhancement with two-sided model [RAN1]

  - Option 4-1 to address the inter-vendor collaboration

    - In case that the workload is manageable, Option 3a-1 can be supported if RAN4 specifies the reference encoder(s) satisfying feasibility and scalability

      - Note: The known structure(s) of Option 3a-1 reuse the same structure(s) of the above-mentioned refence encoder(s) specified by RAN4

    - Signalling/mechanism to support dataset(s)/parameters exchange from NW-side to UE-side

    - Model pairing procedure including ID and applicability reporting

  - Data collection for training

    - Signalling/mechanisms for ground-truth reporting to facilitate NW-side data collection for training

    - Signalling/mechanisms for UE-side data collection for training

  - Inference aspects

    - Target CSI type, measurement and report configuration, CQI/RI determination, payload determination, quantization configuration/codebook, UCI mapping, CSI processing criteria and timeline, priority rules for CSI reports

  - Model monitoring

    - Specify necessary signalling/mechanisms for NW-side monitoring and UE-side monitoring

- Model identification procedure in the context of LCM for two-sided models [RAN2/RAN1]

- CN/OAM/OTT data collection of UE-sided model training [RAN2/RAN1]

  - Regarding the FS_NR_AIML_Air study, specify the corresponding contents of UE-side data collection

- For CSI compression, specify core requirements for AI/ML LCM procedures and UE features [RAN4]:

  - Specify the necessary RAN4 core requirements for CSI compression.

  - Specify the necessary RAN4 core requirements for LCM procedures including performance monitoring.

- For CSI compression, specify performance requirements and test cases for AI/ML LCM procedures and UE features [RAN4]:

  - Specify necessary performance requirements and tests (including metrics) for CSI compression

  - Specify necessary performance requirements and tests (including metrics) for LCM procedure including performance monitoring.

- Testability and interoperability [RAN4]

  - Specify the testing framework and procedure for CSI compression, specify(or finalize) testing(or reference) model(s) for CSI compression, in collaboration with RAN1, and including at least:

    - Relation to legacy requirements

    - Performance monitoring and LCM aspects

    - Generalization aspects

    - UE processing capability and limitations

  - RAN5 aspects related to testability and interoperability to be addressed on a request basis

NOTE: offline training is assumed for the purpose of this project.

# 3. Conclusion

To draw attention from readers, allow us to repeat observations and proposals in what follows.

Normative work for Rel-20 5G-A

Observation 1: Performance gain of CSI compression on UPT can be achieved when compared with legacy CSI feedback.

Observation 2: For two-sided model, the inter-vendor collaboration issue is addressable via any of Direction A/B/C in RAN1.

Proposal 1: Considering the fact that performance/complexity tradeoff and inter-vendor collaboration are addressable, there is no obstacle to specify CSI compression in Rel-20 5G-advance.

Proposal 2: It makes sense to avoid duplicated work between Rel-20 NR WI and 6G SI, e.g. on CSI compression with two-side model.

Potential scope for CSI compression with two-sided model

Proposal 3: To address the inter-vendor collaboration, we consider the following solutions

- If RAN4 specifies the reference model structure(s) and/or parameters satisfying feasibility and scalability, support to specify the format and content of parameter/dataset/information exchange in RAN1

- Otherwise, only support dataset exchange (i.e. Option 4-1) in RAN1

Proposal 4: Recommend the spatial-frequency domain CSI compression (Case 0) into Rel-20 5G-A normative work.

Proposal 5: For model training at NW-side, specify mechanism/signaling for ground-truth reporting by UE in Rel-20 5G-Advance.

Proposal 5: For model training at UE-side, support UE requested RS configuration/transmission.

Proposal 6: To monitor the performance of CSI compression, support:

- NW-side monitoring with target CSI reporting by UE via legacy codebook

  - Possible enhancement with higher-resolution parameter combination on the codebook is not supported

- UE-side monitoring with SGCS reporting by reusing the framework defined for Rel-19 CSI prediction

Objectives for CSI compression

Specification support for spatial-frequency domain CSI compression (i.e. Case 0)

- CSI feedback enhancement with two-sided model [RAN1]

  - Option 4-1 to address the inter-vendor collaboration

    - In case that the workload is manageable, Option 3a-1 can be supported if RAN4 specifies the reference encoder(s) satisfying feasibility and scalability

      - Note: The known structure(s) of Option 3a-1 reuse the same structure(s) of the above-mentioned refence encoder(s) specified by RAN4

    - Signalling/mechanism to support dataset(s)/parameters exchange from NW-side to UE-side

    - Model pairing procedure including ID and applicability reporting

  - Data collection for training

    - Signalling/mechanisms for ground-truth reporting to facilitate NW-side data collection for training

    - Signalling/mechanisms for UE-side data collection for training

  - Inference aspects

    - Target CSI type, measurement and report configuration, CQI/RI determination, payload determination, quantization configuration/codebook, UCI mapping, CSI processing criteria and timeline, priority rules for CSI reports

  - Model monitoring

    - Specify necessary signalling/mechanisms for NW-side monitoring and UE-side monitoring

- Model identification procedure in the context of LCM for two-sided models [RAN2/RAN1]

- CN/OAM/OTT data collection of UE-sided model training [RAN2/RAN1]

  - Regarding the FS_NR_AIML_Air study, specify the corresponding contents of UE-side data collection

- For CSI compression, specify core requirements for AI/ML LCM procedures and UE features [RAN4]:

  - Specify the necessary RAN4 core requirements for CSI compression.

  - Specify the necessary RAN4 core requirements for LCM procedures including performance monitoring.

- For CSI compression, specify performance requirements and test cases for AI/ML LCM procedures and UE features [RAN4]:

  - Specify necessary performance requirements and tests (including metrics) for CSI compression

  - Specify necessary performance requirements and tests (including metrics) for LCM procedure including performance monitoring.

- Testability and interoperability [RAN4]

  - Specify the testing framework and procedure for CSI compression, specify (or finalize) testing (or reference) model(s) for CSI compression, in collaboration with RAN1, and including at least:

    - Relation to legacy requirements

    - Performance monitoring and LCM aspects

    - Generalization aspects

    - UE processing capability and limitations

  - RAN5 aspects related to testability and interoperability to be addressed on a request basis

NOTE: offline training is assumed for the purpose of this project.

# 4. References

[1] RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface