3GPP TSG RAN Meeting #108								RP-251254

Prague, Czech Republic, June 9-13, 2025

Agenda Item:	15.1.5

Source:	OPPO

Title:	Discussion on NR NTN Phase 4 work scope in Rel-20

Document for:	Discussion & Decision

Introduction

RAN1#107 meeting agrees the following conclusion for GNSS resilient operation [1]. In this paper, we discuss the checkpoint on GNSS resilient operation for Rel-20 as well as some suggestions on the potential WID scope.

Conclusion for GNSS resilient operation - Study Item with checkpoint to determine whether we have normative work and the scope based on results of study item phase. - Study should consider Rel-16 as baseline for discussion. Study should focus on NR NTN and be backward compatible with legacy UEs. Aim to minimize physical layer procedures impact to 5GA and avoid physical layer channel/signal changes. - Objective to be narrowed down for RAN#108 - No additional NR NTN enhancements with RAN1 impact can be considered at this point given limited 5GA TUs

Discussion

GNSS resilience operation

In last RAN#107 meeting, there was a consensus to focus on the GNSS resilient operation for Rel-20, and other enhancement is not further considered for NR NTN. Here the GNSS resilient may refer to two cases. In case 1, the UE is capable of GNSS measurement but in some situation, e.g., in tunnel, there is potential degradation on the GNSS performance.  On the other hand, in case 2, the UE is not capable of GNSS at all. We suggest that the two cases are to be considered in Rel-20.

Proposal 1: For GNSS resilient operation, consider the following two cases:

- Case 1: UE is capable of GNSS measurement but in some situation, e.g., in tunnel, there is potential degradation on the GNSS performance.

- Case 2: UE is not capable of GNSS.

Maintenance for UL synchronization

According to TR38.821, two solutions are identified to maintain UL timing advance and frequency synchronization in NTN deployment. Option 1 is autonomous maintenance at UE with UE known location and satellite ephemeris, and Option 2 is network-assisted maintenance. It should be noted that in Option 2, NW-assisted pre-compensation would target for both timing advance and frequency offset on service link, in addition to TA pre-compensation on feeder link which has already been supported in current specification. For residual TA error and frequency offset error, it can be further compensated by legacy PRACH preamble and RACH procedure.

- [TR38.821]: - 6.3.4 Maintenance for UL timing advance and frequency synchronization For the timing advance (TA) in the initial access and the subsequent TA maintenance, the following solutions are identified with an illustration of the definition of terminology given in Figure 6.3.4-1: ● Option 1: Autonomous acquisition of the TA at UE with UE known location and satellite ephemeris. […] ● Option 2: Timing advanced adjustment based on network indication In this way, the common TA, which refers to the common component of propagation delay shared by all UEs within the coverage of same satellite beam/cell, is broadcasted by the network per satellite beam/cell. The calculation of this common TA is conducted by the network with assumption on at least a single reference point per satellite beam/cell. The indication for UE-specific differential TA from network as the Rel-15 TA mechanism is also needed. For satisfying the larger coverage of NTN, extension of value range for TA indication in RAR, either explicitly or implicitly, is identified. Whether to support negative TA value in corresponding indication will be determined in the normative phase. Moreover, indication of timing drift rate, from the network to UE, is also supported to enable the TA adjustment at UE side. […] For the UL frequency compensation, at least for LEO system, the following solutions are identified with consideration on the beam specific post-compensation of common frequency offset at the network side: […] ● Option-2: The required frequency offset for UL frequency compensation at least in LEO systems is indicated by the network to UE. The acquisition on this value can be done at the network side with detection of UL signals, e.g., preamble.

For GNSS resilient operation, there will be accordingly two potential solutions to maintain UL synchronization: Option 1 with UE known location being acquired by NR positioning instead of GNSS; Option 2 with NW-assisted pre-compensation for UL. For Option 2, it was already studied its feasibility. For Option 1, we investigated if it is possible that idle UE performs DL TDOA positioning using legacy SSB signals from multiple satellites. In our simulation assumptions, four satellite positions are considered with two satellite positions on the orbit 0 and two other satellite positions on the orbit 1, as shown in Fig. 1. The positioning accuracy error is within 561 m at 95% CDF point with the aid of 4 satellite positions as depicted in Fig. 2. This ensures the timing offset is within CP length and the frequency error is within 0.1 ppm at 2GHz carrier frequency. If in Rel-20 we can consider larger bandwidth and other reference signal than SSB, e.g., PRS, the positioning accuracy error is expected to be further reduced.

Proposal 2: For GNSS resilient operation, consider the following two options:

- Option 1: Autonomous maintenance at UE with UE location acquired by NR positioning.

- Option 2: NW-assisted pre-compensation for both timing advance and frequency offset on service link.

Fig. 1: orbit 0 is the serving cell orbit (600km), orbit 1 is modelized at 400km. The TDOA is based on 4 satellite positions, with two on the orbit 0 and two others on the orbit 1

Fig. 2: positioning error CDF

On the other hand, it was concluded that this study should aim to minimize physical layer procedures impact to 5GA and avoid physical layer channel/signal changes. Given the limited TU, we propose not to consider PRACH enhancement for GNSS resilient operation.

Proposal 3: No PRACH enhancement for GNSS resilient operation.

With the above analysis, we suggest some WI objectives for Rel-20 GNSS resilient operation:

Proposal 4: Objectives for Rel-20 NR NTN GNSS resilient operation [RAN1, RAN2, RAN4]:

- Specify necessary enhancement, if any, for network-assisted pre-compensation

- Study and specify, if feasible, UE location estimation based on NR positioning for idle UE

Note: GNSS resilient includes UE capable of GNSS with reduced GNSS performance and UE incapable of GNSS.

Conclusion

In this paper, we discussed the checkpoint on the GNSS resilient operation and possible WID scope for Rel-20 NR NTN. Our proposals are summarized as follows:

Proposal 1: For GNSS resilient operation, consider the following two cases:

- Case 1: UE is capable of GNSS measurement but in some situation, e.g., in tunnel, there is potential degradation on the GNSS performance.

- Case 2: UE is not capable of GNSS.

Proposal 2: For GNSS resilient operation, consider the following two options:

- Option 1: Autonomous maintenance at UE with UE location acquired by NR positioning.

- Option 2: NW-assisted pre-compensation for both timing advance and frequency offset on service link.

Proposal 3: No PRACH enhancement for GNSS resilient operation.

Proposal 4: Objectives for Rel-20 NR NTN GNSS resilient operation [RAN1, RAN2, RAN4]:

- Specify necessary enhancement, if any, for network-assisted pre-compensation

- Study and specify, if feasible, UE location estimation based on NR positioning for idle UE

Note: GNSS resilient includes UE capable of GNSS with reduced GNSS performance and UE incapable of GNSS.

Reference

[1] RP-250812 Summary for RAN Release 20 5G-Adv