3GPP TSG RAN Meeting #108										       RP-251276

Prague, Czech Republic, June 9-13, 2025

Status Report to TSG

Agenda item:		10.4.6

| WI / SI Name | New LTE band for 5G broadcast for region 3 utilizing a geosynchronous satellite |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | LTE_band_5G_bcast_GSO-Core |  |  |  |  |
| Unique ID | 1071064 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-251419 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: Sep/2025 | Performance part: Mar/2026 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 75% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Serwah, Oh |
|  | Company | Astrum Mobile |
|  | Email | serwah.oh@astrum-mobile.com |
|  | Name | Han, Bin |
|  | Company | Qualcomm Incorporated |
|  | Email | binhan@qti.qualcomm.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

2.4	RAN4

2.4.1	Agreements

RAN4#114bis

Band definition and system parameters

Sub-topic 2-1: New Suffix

Issue 2-1-1: Whether to use a new suffix for new SDO band in TS 36.102 and TS 36.108

- Agreement:

  - UE RF: To use suffix in TS 36.102 to specify the corresponding SDO band, system parameters, UE RF requirements  for LTE based 5G broadcast over GSO satellite.

  - SAN RF: To consider suffix in TS 36.108 to specify the corresponding SDO band, system parameters, SAN RF requirements (if needed) for LTE based 5G broadcast over GSO satellite.

Sub-topic 2-2: Operating band, transmission bandwidth, channel arrangement

Issue 2-2-1: Operating band

- Agreement:

  - The new SDO band operating in 1467 to 1492 MHz can be numbered as 246

Issue 2-2-2: Transmission bandwidth configuration(NRB)

- Agreement:

  - To define 50RB transmission bandwidth configuration for SCS=15kHz for 10MHz channel bandwidth in TS 36.102 and TS 36.108

Issue 2-2-3: Channel bandwidth

- Agreement:

  - To define PMCH of 10MHz bandwidth configuration for 15kHz SCS in TS 36.102 and TS 36.108 as following table.

Table 5.3C-2: 5G broadcast PMCH bandwidths per operating band in TS 36.102 and TS 36.108

| Operating Band | PMCH bandwidth |
| --- | --- |
|  | 10MHz |
| 246 | Yes |

Issue 2-2-4: Channel spacing

- Agreement:

  - To reuse the same PMCH channel spacing for LTE-based 5G Broadcast operating for satellite.

    - Nominal channel spacing between adjacent broadcast channels is defined as follows

        - Nominal Channel spacing = PMCH bandwidth

    - where PMCH bandwidth is the broadcast bandwidth indicated by upper layer signaling pmch-Bandwidth in the MBSFN area (see TS 36.331 [7]).

Issue 2-2-5: Channel raster, carrier frequency and EARFCN

- Agreements:

  - The channel numbers for LTE based 5G terrestrial broadcast band are proposed as 228051 -<1>- 227480

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| 246 | 100 | 1467 | 228051 | 228051 -<1>- 228300 | N/A |  |  |

UE RF requirements

Sub-topic 3-1: UE RF requirements

Issue 3-1-1: Reference sensitivity

- Agreement:

  - Add a new chapter 7.3C

  - Specify Reference sensitivity as -97 dBm for 10MHz PMCH.

Issue 3-1-2: Maximum input level

- Agreement:

  - Add a new chapter 7.4C

  - To specify the maximum input level requirements as -40dBm which is in line with NTN UE maximum input level requirement of 36.102.

Issue 3-1-3: ACS

- Agreement:

  - Add a new chapter 7.5C

  - Take the following ACS requirements as the baseline.

Table 7.5C-1: Adjacent channel selectivity

| Rx Parameter | Units | PMCH bandwidth |
| --- | --- | --- |
|  |  | 10 MHz |
| ACS | dB | 33 |

Table 7.5C-2: Test parameters for Adjacent channel selectivity, Case 1

| Rx Parameter | Units | PMCH bandwidth |
| --- | --- | --- |
|  |  | 10 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |
| PInterferer | dBm | REFSENS +45.5dB |
| BWInterferer | MHz | 5 |
| FInterferer (offset) | MHz | 7.5+0.0075 / -7.5-0.0075 |
| NOTE: The interferer consists of the Reference measurement channel specified in Annex A.1.3. |  |  |

Table 7.5C-3: Test parameters for Adjacent channel selectivity, Case 2

| Rx Parameter | Units | PMCH bandwidth |
| --- | --- | --- |
|  |  | 10 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -53.5 |
| PInterferer | dBm | -40 |
| BWInterferer | MHz | 5 |
| FInterferer (offset) | MHz | 7.5+0.0075 / -7.5-0.0075 |
| NOTE: The interferer consists of the Reference measurement channel specified in Annex A.1.3. |  |  |

Issue 3-1-4: In-band blocking

- Agreement:

  - Add a new chapter 7.6C.2 for in-band blocking

  - To specify in-band blocking for 10MHz PMCH for LTE based 5G broadcast utilizing satellite in TS 36.102 as the baseline.

Table 7.6C.2-1: In band blocking parameters

| Rx parameter | Units | PMCH bandwidth |
| --- | --- | --- |
|  |  | 10 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 6 |
| BWInterferer | MHz | 5 |
| FIoffset, case 1 | MHz | 7.5+0.0025 |
| FIoffset, case 2 | MHz | 12.5+0.0125 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.18. |  |  |

Table 7.6C.2-2: In-band blocking

| Operating band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |
| 246 | FInterferer | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer center frequencies NOTE 4: BW refers to 10 MHz |  |  |  |  |

Issue 3-1-5: Out-of-band blocking

- Agreement:

  - Add a new chapter 7.6C.3 for out-of-band blocking

  - To specify out-of-band blocking for 10MHz PMCH for LTE based 5G broadcast utilizing satellite in TS 36.102 as following tables.

Table 7.6C.3-1: Out-of-band blocking parameters

| Rx Parameter | Units | PMCH bandwidth |
| --- | --- | --- |
|  |  | 10 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 6 |
| NOTE: Reference measurement channel is specified in Annex A.1.3. |  |  |

Table 7.6C.3-2: Out of band blocking

| Operating band | 1 | Units | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Range 1 | Range 2 | Range 3 |
|  | FInterferer (CW) | MHz | FDL_low -40 to FDL_low -85 | FDL_low -85 to FDL_low -110 | FDL_low -110 to 1 MHz |
|  |  |  | FDL_high +30 to FDL_high + 75 | FDL_high +75 to FDL_high +100 | FDL_high +100 to +12750 MHz |
| 246 | PInterferer | dBm | -44 | -30 | -15 |

Issue 3-1-6: Narrow band blocking

- Agreement:

  - Narrow-band blocking requirements are not applicable to LTE based 5G terrestrial broadcast.

Issue 3-1-7: Spurious response

- Agreement:

  - No spec impact.

Issue 3-1-8: Intermodulation characteristics

- Agreement:

  - Add a new chapter to capture Intermodulation characteristics for LTE-based 5G Broadcast.

Issue 3-1-9: Spurious emissions

- Agreement:

  - No spec impact.

Issue 3-1-10: Reference measurement channels

- Agreement:

  - To specify the RMC of 15kHz SCS for 10MHz PMCH in TS 36.102 as following tables as the baseline. FFS on other SCSs

Table A.1.3.1-1 Fixed Reference Channel for PMCH Receiver Requirements (15 kHz SCS)

| Parameter | PMCH (15 kHz SCS) |  |  |
| --- | --- | --- | --- |
|  | Unit | Value |  |
| Reference channel |  | R.PMCH.1 | R.PMCH.2 |
| PMCH bandwidth | MHz | 10 | 10 |
| Allocated resource blocks |  | 50 | 50 |
| Allocated subframes per Radio Frame(Note1) |  | 10 | 10 |
| Modulation |  | QPSK | 16QAM |
| Target Coding Rate |  | 1/3 | 3/4 |
| Information Bit Payload (Note 2) |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | TBD | TBD |
| Number of Code Blocks per Sub-Frame (Note 3) |  | 1 | 3 |
| Binary Channel Bits Per Subframe |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | TBD | TBD |
| MBMS UE Category |  | ≥ 1 | ≥ 2 |
| Note 1: 2 OFDM symbols are reserved for PDCCH; and no CRS allocated as per TS 36.211 [4]. Note 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |

SAN RF requirements

Sub-topic 4-0: SAN RF requirements set

- Agreement:

  - For all requiremens listed in sub-topic 4-1, necessity for certain requirements require further verification to consider requirements structure approach in TS36.104 (for TN broadcast), as well as TS38.108 (NTN SAN specification).

  - All SAN RF related agreements are considered as baseline, subject to verification next RAN4 meeting.

  - Proponent are encouraged to provide more details on their current and/or future satellite characteristics.

Sub-topic 4-1: SAN RF requirements

Issue 4-1-1: Satellite Access Node output power

- Agreement:

  - No specification impact.

Issue 4-1-2: RE power control dynamic range

- Agreement:

  - No specification impact.

Issue 4-1-3: Total power dynamic range

- Agreements

  - FFS on whether total power dynamic range requirement applies for LTE based 5G broadcast over Satellite

Issue 4-1-4: Transmit ON/OFF power

- Agreement:

  - No specification impact.

Issue 4-1-5: Transmitted signal quality

- Agreement:

  - No specification impact.

Issue 4-1-6: Occupied bandwidth

- Agreement:

  - No specification impact.

Issue 4-1-7: Adjacent Channel Leakage Power Ratio

- Agreement:

  - Reuse the legacy requirements for GEO

Issue 4-1-8: EVM

- Agreement:

  - To specify the EVM requirements for LTE based 5G broadcast over satellite based on the following table.

Table 6.5.2-1: EVM requirements for E-UTRA carrier

| Modulation scheme | Required EVM (%) |
| --- | --- |
| QPSK | 17.5 % |
| 16QAM | 12.5 % |
| NOTE 2: Only QPSK and 16QAM apply. |  |

Issue 4-1-9: Out-of-band emissions

- Agreement:

  - Reuse the legacy requirements.

Issue 4-1-10: Transmitter spurious emissions

- Agreement:

  - Reuse the legacy requirements.

Issue 4-1-11: Transmitter intermodulation

- Agreement:

  - No specification impact, not applicable to SAN.

Issue 4-1-12: Window length

- Agreement:

  - To reuse EVM window length for 10MHz defined in 36.104.

Sub-topic 4-2: SAN RF conformance requirements

- Agreement:

  - As per WID: RAN4 to further discuss whether SAN RF conformance requirements and tests will be defined.

RAN4#115

Topic #1: UE RF requirements

Issue 1-2-1: 15kHz SCS for 10MHz PMCH

- Agreement:

  - Specify 15kHz SCS for 10MHz PMCH in Rel-19.

Issue 1-2-2: The RMC of 15kHz SCS

- Agreement:

  - The RMC of 15kHz SCS to be specified in TS 36.102 based on the following Table 1:

Table 1 Fixed Reference Channel for PMCH Receiver Requirements (15 kHz SCS)

| Parameter | PMCH (15 kHz SCS) |  |  |
| --- | --- | --- | --- |
|  | Unit | Value |  |
| Reference channel |  | R.PMCH.1 | R.PMCH.2 |
| PMCH bandwidth | MHz | 10 | 10 |
| Allocated resource blocks |  | 50 | 50 |
| Allocated subframes per Radio Frame(Note1) |  | 10 | 10 |
| Modulation |  | QPSK | 16QAM |
| Target Coding Rate |  | 1/3 | 1/2 |
| Information Bit Payload (Note 2) |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 3624 | 9912 |
| Number of Code Blocks per Sub-Frame (Note 3) |  | 1 | 2 |
| Binary Channel Bits Per Subframe |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 10200 | 20400 |
| MBMS UE Category |  | ≥ 1 | ≥ 1 |
| Note 1: 2 OFDM symbols are reserved for PDCCH; and no CRS allocated as per TS 36.211 [4]. Note 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |

Issue 1-2-3: Out of band blocking requirements

- Agreement:

  - Define the Range 1 starting from 15MHz below and above frequency range of band 246

Table 7.6C.3-2: Out of band blocking

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pw | dBm | REFSENS + 6 dB |  |  |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| 246 | Finterferer (CW) | MHz | -95 < f – FDL_low < -15 or 15 < f – FDL_high < 85 | -95 < f – FDL_low ≤ -120 or 85 ≤ f – FDL_high < 110 | 1 ≤ f ≤ FDL_low – 120 or FDL_high + 110 ≤ f ≤ 12750 |

Issue 1-2-4:  Spurious response

- Agreement:

  - To specify the spurious response for LTE based 5G broadcast utilizing geosynchronous satellite as table 7.7C-1 and table 7.7C-2 in TS 36.102.

Table 7.7C-1: Spurious response parameters

| Rx parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 10 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |
|  |  | 6 |
| NOTE: Reference measurement channel is specified in Annex A.1.3. |  |  |

Table 7.7C-2: Spurious response

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

Issue 1-2-5: Release Independent

- Agreement:

  - The new proposed band for LTE based 5G broadcast utilizing satellite is specified in a release independent manner from Rel-17.

Topic #2: SAN RF requirements

Issues 2-2-0: Applicability of SAN RF minimum requirements for LTE based 5G broadcast over GSO

- Agreement:

  - To update applicability rule for SAN RF minimum requirements to accommodate LTE based 5G broadcast over GSO in TS 36.108.

    - For the legacy SAN RF requirements which can be used for LTE based 5G broadcast over GSO, no need to change specifications expecting updating the applicability rule.

    - For the new requirements which are only applicable to LTE based 5G broadcast over GSO, add the new requirements to the corresponding clauses.

Issue 2-2-1: Total power dynamic range

- Agreement:

  - Total power dynamic range requirement is not applied for LTE based 5G broadcast over GSO.

    - Assuming the transmission bandwidth is fixed for LTE based 5G broadcast over GSO

Issue 2-2-2: Transmitted signal quality

- Agreement:

  - frequency error shall be specified.

Issue 2-2-3: Occupied bandwidth

- Agreement:

  - This requirement shall be applicable, but it don’t need change anything in TS section.

Issue 2-2-4: Satellite Access Node output power

- Agreement:

  - This requirement shall be applicable, but it don’t need change anything in TS section.

Issue 2-2-5: Transmitter spurious emissions

- Agreement:

  - Reuse legacy requirements and follow ITU requirements (ITU Resolution 761 in WRC-19).

Issue 2-2-6: SAN RF conformance requirements

- Agreement:

  - To be discussed after Core completion.

2.4.2	Remaining Open issues

The remaining issues need to be solved:

- To complete Big CR to TS36.108

4.	References

RAN4 #114bis

- R4-2504070         Work plan for WI						          Astrum

- R4-2503747	Proposals on Band definition & system parameters               ZTE and Sanechips

- R4-2504053	Proposals on Band definition & system parameters	          Qualcomm and Astrum

- R4-2503549	Proposals on UE RF requirements			          Apple

- R4-2503748	Proposals on UE RF requirements                                          ZTE and Sanechips

- R4-2504054	Proposals on UE RF requirements				Qualcomm

- R4-2503749	Proposals on SAN RF requirements			           ZTE and Sanechips

- R4-2504093	Proposals on SAN RF requirements		                      Astrum

- R4-2504643         Topic summary for [114bis][318]                                             Astrum

5G_Broadcast_GSO_NTN_band

- R4-2504725	Way Forward for [114bis][318]                                                Astrum &Qualcomm

5G_Broadcast_GSO_NTN_band		                                 & ZTE& Huawei

RAN4 #115

- R4-2505515	Draft CR to TS 36.102			                                 Astrum

- R4-2505738	Proposals on UE RF requirements				Qualcomm

- R4-2506232	Proposals on UE RF requirements                                          	ZTE and Sanechips

- R4-2505739	Draft CR to TS 36.102				                     Qualcomm

- R4-2505740	Draft CR to TS 36.102				                     Qualcomm

- R4-2506234          Draft CR to TS 36.102	                                                      	ZTE and Sanechips

- R4-2505516	Draft CR to TS 36.108	                                                      Astrum

- R4-2506233	Proposals on SAN RF requirements			           ZTE and Sanechips

- R4-2507831         Proposals on SAN RF requirements                                         Huawei and HiSilicon

- R4-2506235         Draft CR to TS 36.108                                                              ZTE and Sanechips

- R4-2507832         Draft CR to TS 36.108                                                              Huawei and HiSilicon

- R4-2507850         Topic summary for [115][318]                                                  Astrum

5G_Broadcast_GSO_NTN_band

- R4-2508648         Way Forward for [115][318]                                                    Astrum &Qualcomm

5G_Broadcast_GSO_NTN_band		                                 & ZTE& Huawei

- R4-2505517         Big CR to TS 36.102                                                                Astrum