3GPP TSG RAN Meeting #108																        RP-251281

Prague, Czech Republic, June 9th-13th, 2025

Source:	Xiaomi

Title:	Scope of AI/ML in 5G-A Rel-20

Agenda Item:	15.1.1

Document for:	Discussion and decision

Introduction

During Rel-19 AI/ML, aspects related to two-sided model based CSI compression, UE data collection and etc., were studied according to the SID [1].

Study objectives: - CSI feedback enhancement [RAN1]: - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing necessary specification impact analysis, as well as, other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843. - Necessity and details of model Identification concept and procedure in the context of LCM for two-sided models [RAN2/RAN1] - CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]: - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods - Model transfer/delivery [RAN2/RAN1]: - Determine whether there is a need to consider standardized solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study - Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis NOTE: offline training is assumed for the purpose of this project. NOTE: the outcome of the study objectives should be captured in TR 38.843 for future reference. NOTE: Coordination with SA/SA WGs of the ongoing study/work as it may relate to their required work.

For CSI compression, performance-complexity trade-off, inter-vendor collaboration complexity and interoperability and testability were discussed. Observations and recommendations are captured in [2]. For UE data collection, RAN2 discussed the feasibility of identified options in Rel-18 and the unsolved issues are still under SA discussion. Based on the study outcome in [2] for CSI compression and the discussion status in RAN2 and SA for UE data collection, we provide our consideration on the potential scope for 5G-A Rel-20 AI/ML air interface normative work.

Two-sided AI/ML model based CSI compression

In this section, we provide views on use cases, inter-vendor collaboration training, interoperability and testability for two-sided AI/ML model.

Use cases

In Rel-19, the following Case 1-5 on top of Case 0 were studied to improve system performance [3]. In addition to case 0, Case 2 and Case 3 were agreed to be prioritized as well.

Table 1: Summary of use cases for two-sided AL/ML Model based CSI compression

| Case | Target CSI slot(s) | Whether the UE uses past CSI information | Whether the network uses past CSI information |
| --- | --- | --- | --- |
| 0 | Present slot | No | No |
| 1 | Present slot | Yes | No |
| 2 | Present slot | Yes | Yes |
| 3 | Future slot(s) | Yes | No |
| 4 | Future slot(s) | Yes | Yes |
| 5 | Present slot | No | Yes |

Based on Rel-19 study, the following recommendation is made in [2].

Based on the study, RAN1 recommends the following with potential RAN1 specification impacts common to all inter-vendor collaboration options. - Recommend Case 0 - Case 2 and 3 can be considered for further enhancement built upon the specification of case 0.

According to the recommendation in RAN1, it is very natural to include Case 0 in the normative work scope. The remaining issue is whether to include Case 2 and Case 3 into the normative work scope. In the following paragraph, we will discuss the necessity for Case 2 and Case 3 on top of Case 0.

Case 2：

- Performance & complexity

Regarding Case 2, the observation regarding performance and complexity for Case 2 was made as follows in [2].

- Temporal domain Case 2 can achieve higher gain compared to Case 0 with similar or increased complexity - Some companies achieved gain using Case 2 AI/ML models having same/similar/lower complexity as their Case 0 AI/ML models, while some other companies achieved gain using Case 2 AI/ML models having higher complexity than their Case 0 AI/ML models. - Note: The Case 2 evaluations for this summary were done under no UCI loss.

Based on the above observation, better performance compared with Case 0 can be achieved by Case 2. However, some companies observed that Case 2 achieves the gain with similar complexity as Case 0, while some companies observed that Case 2 achieves the gain with higher complexity. In theory, the complexity of Case 2 should be higher than that of Case 0 since additional adaptation layer (e.g., Conv-LSTM or latent adaptation layer) is needed. However, the model complexity is highly related to model pre/post-processing method. For example, it is observed in [2] that the complexity of encoder could be 0.2M, 16M, 113M or 1760M FLOPs when SGCS gain is around 20%. It should be noticed that 1706M FLOPs is required only if model optimization is not applied. If knowledge distillation and post-processing via recurrent quantizer latent vector are adopted, it could be significantly reduced to a comparable magnitude of complexity as that of case 0, i.e., 113M to 16M FLOPs. The encoder complexity could be further reduced to 0.2M FLOPs with adopting proper input type of encoder, e.g., angle-delay domain channel information. Hence, model complexity is not a critical issue since it can be addressed by adopting advanced pre-/post processing approaches. Typically, the model complexity usually could be up to more than several thousand M FLOPs for CSI prediction and beam prediction. Considering model complexity can be as low as 10M FLOPs assuming optimization, it is affordable for case 2 from implementation perspective.

Last but not least, even when complexity of both Case 0 and Case 2 is set to less than 10M FLOPs, Case 2 can still bring about 5.3% SGCS gain compared with Case 0 when the complexity is restricted to the same level as that of case 0.

- Specification efforts

The following specification impacts on Case 0 were identified in RAN1.

- Model pairing procedure including ID and applicability reporting

- Inference aspects including target CSI type, measurement and report configuration, CQI / RI determination, payload determination, quantization configuration / codebook, UCI mapping, CSI processing criteria and timeline, priority rules for CSI reports

- NW and UE side data collection for training,

  - Target CSI format

  - Note: The framework defined in BM and CSI prediction use cases could be reused.

- Specify performance monitoring, if needed

Since Case 2 is designed on top of Case 0, most design of Case 0 can be reused. Additional specification work may include following two aspects:

- Additional configuration/reporting of temporal aspects for training dataset collection

- Signalling for mitigating the impact of UCI loss

For data collection, the collected sample is CSI related information. The existing CSI framework could be reused to support CSI configuration or reporting. The standardization effort is expected to be small. For the second aspect, RAN1 has identified two options for mitigating the impact of UCI loss in [4], which have been comprehensively studied. Hence, additional specification effort for Case 2 is limited.

Observation 1: Compared with Case 0, temporal domain Case 2 can achieve higher gain with acceptable complexity.

Observation 2: On top of Case 0, additional specification efforts to support Case 2 is limited, including:

- Additional configuration/reporting of temporal aspects for training dataset collection

- Signaling for mitigating the impact of UCI loss

According to the analysis above, we think Case 2 could achieve better performance with acceptable complexity and limited specification effort. Thus, we prefer to include Case 2 into the normative work scope.

Proposal 1: Include Case 2 for Rel-20 5G-A AI/ML normative work of two-sided AI/ML model based CSI compression.

Case 3：

- Performance & complexity

For Case 0, larger performance loss is expected since UE only reports the history measured channel information. In order to further reduce performance loss due to channel aging, Case 3 was studied in Rel-19, and the observation regarding performance and complexity for Case 3 was made as follows in [2].

- Under CSI prediction, temporal domain Case 3 can achieve better performance than Rel-18 Doppler eType II benchmark. The amount of gain of Case 3 over Rel-18 Doppler eType II benchmark is similar to the amount of gain of Case 0 over Rel-16 eType II. - On average, for each inference, FLOPs of Case 3 is higher than that of Case 0.

The benchmark of Case 3 is Rel-18 Doppler eType II codebook since both of them is used to report future channel information. Based on above observation, Case 3 can achieve better performance than legacy Rel-18 eType II Doppler codebook.

In [2], compared with Rel-18 eType II Doppler codebook, it has been observed that about 4% SGCS gain could be achieved by using an encoder with complexity less than 10M FLOPs. Furthermore, if encoder with complexity in terms of 10M~100M FLOPs is assumed, up to 28% SGCS gain could be achieved compared with Rel-18 eType II Doppler codebook. Similar to Case 2, the observed magnitude of complexity is very divergent across companies.  This reason is that different companies develop their model with different pre/post processing or optimization design for encoder and CSI prediction model, such as knowledge distillation and/or pruning. It implies the complexity problem may not be critical which can be solved by utilizing proper implementation methods.

- Specification efforts

On top of Case 0, the following three aspects may be involved in the standardization.

- Additional configuration/reporting of temporal aspects for training dataset collection.

- Training dataset labels and performance monitoring labels definition

- Payload determination, UCI mapping, and CSI processing criteria and timeline

Similar to Case 2, the existing CSI framework can be reused for training dataset collection. For the second aspect, RAN1 already had comprehensive study on the candidate solution options, which can be used as baseline for the normative work. For the third aspect, the mechanism of defining the payload, UCI mapping, and CSI processing criteria and timeline defined for Rel-18 eType II Doppler codebook could be reused as baseline. In short, the standardization effort for Case 3 is acceptable.

Observation 3: Compared with Case 0, temporal domain Case 3 can achieve higher gain with affordable complexity.

Observation 4: On top of Case 0, the following additional specification efforts are required for Case 3.

- Additional configuration/reporting of temporal aspects for training dataset collection

- Training dataset labels and performance monitoring labels definition

- Payload determination, UCI mapping, and CSI processing criteria and timeline

According to the analysis above, we think Case 3 could achieve better performance and address the CSI aging problem. In addition, the complexity issue can be solved by model optimization and specification effort is limited as well. Thus, we prefer to include Case 3 into the normative work.

Proposal 2: Include Case 3 for Rel-20 5G-A AI/ML normative work of two-sided AI/ML model based CSI compression.

Inter-vendor collaboration training

During Rel-19, five options on inter-vendor training collaboration were studied and the details of these directions or options on inter-vendor training collaboration could be found in Table 2.

Table 2: Inter-vendor training collaboration Options

| Options | Directions | Details of different options |  |
| --- | --- | --- | --- |
| Option 1 | C | Fully standardized reference model (structure + parameters) |  |
| Option 2 | C | Standardized dataset |  |
| Option 3a | A | Standardized reference model structure + Parameter exchange | 3a-1: CSI generation part (with/without target CSI) |
|  |  |  | 3a-2: CSI reconstruction part |
|  |  |  | 3a-3: both CSI generation part and reconstruction parts |
| Option 3b (z4) | B |  | Parameters exchanged from the NW-side to UE-side is CSI generation part |
| Option 4 | A | Standardized data / dataset format + Dataset exchange | 4-1: target CSI, CSI feedback |
|  |  |  | 4-2: CSI feedback, reconstructed target CSI |
|  |  |  | 4-3: target CSI, CSI feedback, reconstructed target CSI |
| Option 5a | A | Standardized model format + Reference model (CSI generation part, CSI reconstruction part or both) exchange |  |
| Option 5b (z4) | B | Standardized model format +Model exchanged from the NW-side to UE-side is CSI generation part |  |

According to the output of study item on CSI compression, option 3a-1, Option 4 and direction C are on the table.  In the following paragraph, we discuss our understanding on the maturity of above options and which can be included in R20 normative work scope.

    - Direction A Option 4-1

- Performance

RAN1 has studied the performance of Option 4-1 when dataset distribution between NW-side and UE-side is different. Based on the evaluation result, -2.4% ~ 0% relative to upper-bound performance is observed. This implies Option 4-1 could be used to address inter-vendor collaboration training with trivial performance loss. In addition, as discussed in 2.2.2 and 2.2.3, compared with Option 3a-1 and Direction C, Option 4-1 leads to less performance loss relative to upper-bound performance.

- Feasibility

RAN1 has concluded Option 4-1 is feasible. In Option 4-1, dataset exchange between gNB and UE is involved. During RAN2 #129bis meeting, RAN2 analyzed and discussed the feasibility of OTA and non-OTA-based information exchange for Direction A, including Option 4-1 (dataset transfer) and Option 3a-1 (model parameter transfer). However, RAN2 has no consensus on the feasibility of OTA solution, especially considering challenges for CP to transfer large amount of dataset. On the other hand, RAN2 concluded dataset transfer in Option 4-1 could be transferred from gNB to UE-side OTT server via non-OTA solution. RAN3 further confirmed it is feasible via non-OTA OAM solution in [5]. Though the final confirmation from SA5 on non-OTA OAM solution and SA2 on non-OTA CN solution is needed, RAN2 assumes both solutions can be feasible based on existing architecture with potential signaling enhancements.

- Specification impact

RAN1 identifies the following specification impacts for Option 4-1.

- Format and contents of dataset/additional information exchange

For Case 0, the format and content of dataset have been studied. The contents of dataset could be spatial-frequency domain precoder vector or angle-delay domain channel information. The format of data could be codebook-based Rel-16 eType 2 or float 32. Additionally, the specification effort for higher layer to support non-OTA is also manageable if we consider OAM solution only. The required specification effort to support Option 4-1 is not significant. For Option 3a-1, standardization of the model format and the model parameter delivery is necessary, the involved standardization effort is equivalent to that of data format definition and data set delivery in Option 4-1. While, besides that, model structure definition is required for Option 3a-1, which would incur significant specification effort. For Direction C, not only the model structure, but also the model parameters need standardization. Considering the complicated model structure and large number of parameters (e.g., at least 1M parameters for small AI model), the required standardization effort is much larger than the data format definition and message delivery procedure. Considering these aspects, the required specification effort in Option 4-1 is less than that of Option 3a-1 and Direction C.

Observation 5: Option 4-1 non-OTA OAM solution is confirmed as feasible at least from RAN’s perspective, and it requires less specification efforts with better performance compared with Option 3a-1 and Direction C.

    - Direction A Option 3a-1

- Performance

RAN1 also studied the performance of Option 3-1 and observed minor to moderate performance loss (-0.4% ~ -3.9%) relative to upper-bound performance. This implies that Option 3a-1 could also be used to address inter-vendor collaboration issue. However, compared with Option 4-1, the performance loss of Option 3a-1 is larger.

- Feasibility

From RAN1 perspective, RAN1 concluded Option 4-1 is feasible. Similar conclusion as option 4-1 was made on Option 3a-1 by RAN2/RAN3. However, RAN3 and SA5 have concerns on sharing model parameter. The reason is RAN3 previously concluded that model algorithm is out of 3GPP scope during Rel-17 AI/ML-based NG-RAN study. Therefore, transferring model parameter directly is not preferred considering the key objective for two-sided model is to support interoperability and avoid any proprietary information exposure.

Observation 6: RAN3 concluded AI model algorithm is out of 3GPP scope during Rel-17 study, the required model parameter sharing in option 3a-1 may not be supported by RAN3.

- Specification impact

To support Option 3a-1, model structure should be standardized, which requires huge standardization effort. To reduce the workload, RAN1 considered to reuse the standardized model structure in RAN4 for testing purpose. However, we observe there’s a potential mismatch between study in RAN1 and RAN4 on the model structure. In our understanding, RAN1 agreed to consider transformer as the scalable model structure, while RAN4’s study mainly focuses on CNN structure. If the scalable transformer structure is used to specify the reference encoder, specification collaboration between RAN1 and RAN4 is needed.

As the AI/ML technology is developing and evolving very quickly, the standardized model structure may be outdated. It is important to ensure a future-proof model structure to be specified for forward-compatibility. From this perspective, Option 3a-1 is not a suitable solution to address the inter-vendor collaboration training complexity.

Observation 7: Significant specification efforts on model structure are required for Option 3a-1, and the future-proof model structure is not ensured for forward-compatibility.

    - Direction C option 1

- Performance

RAN1 has studied the performance of Direction C when dataset distribution between NW-side and UE-side is different. Assuming decoder is fixed and standardized, minor to moderate loss (-1.56% ~ -3.81%) relative to upper-bound performance is observed. However, if actual field data is used for training model, moderate to significant loss (-6.8% ~ -12%) is observed compared with upper-bound performance. The performance loss of Direction C is larger than that of Option 4-1 from above evaluation results as well.

- Feasibility

Based on outcome of study item, both RAN1 and RAN4 have concluded that Direction C is feasible. However, RAN2/RAN3/SA were not involved in the evaluation of Direction C, it is unclear whether/how much specification impact will be introduced to other WGs, especially RAN2.

- Specification impact

For Direction C, reference model including model structure and model parameters should be specified. Similar to the situation of Option 3a-1, standardization effort for reference model for Direction C is also significant. Hence, Direction C is not a suitable solution to address the inter-vendor collaboration training complexity as well.

Observation 8: Significant specification efforts are required to specify model for Direction C, and the future-proof model is not ensured.

Based on above discussion for the three inter-vendor training options, the performance, feasibility and specification impact for them are summarized in Table 2.

Table 3: Comparison among 3 inter-vendor training collaboration Options

| Options Factors | Option 4-1 | Option 3a-1 | Direction C |
| --- | --- | --- | --- |
| Performance loss compared with upper bound | Minor | Minor to moderate | Minor to moderate |
| Feasibility | - RAN1: Yes - RAN2: Yes, via non-OTA - RAN3: Yes, via non-OTA OAM - RAN4: not studied | - RAN1: Yes - RAN2: Yes, via non-OTA - RAN3: Yes, via non-OTA OAM - RAN4: not studied | - RAN1: Yes - RAN2/3: not studied - RAN4: Yes |
| Specification impact | Smaller | Larger | Larger |

Considering the performance, feasibility and specification impact of the three options, we suggest Option 4-1 non-OTA based dataset transfer via OAM is prioritized in normative work.

Proposal 3: Prioritize Direction A Option 4-1 non-OTA based dataset transfer via OAM in normative work.

Interoperability and testability for two-sided AI/ML model

During Rel-19 SI, RAN4 has study the convergence between SGCS reported for different company UE encoders when operating with test decoders for test feasibility issue.

From the simulation results of option 3, 4a/4b, the main observations are as below:

For option 3 track 2:

- There is reasonable SGCS convergence when using the mixed dataset for training and test.

- It is possible for UE vendors to train an encoder based on a frozen test decoder that is functional.

- The results considering “own” datasets demonstrate that there is divergence in the datasets, and attention must be paid to alignment of the training datasets.

For option 4a/4b:

- When mixed dataset is used, it is feasible to train a functional “own” decoder based on the frozen encoder, and then a functional “own” encoder based on the “own” decoder, and vice versa.

- Two lower complexity encoders were considered and tested based on the frozen decoder.

  - A transformer with 3.63M FLOPs

  - A CNN encoder with 4.14M FLOPs

- When the lower complexity encoders were examined using the mixed dataset for all training and the mixed test set for testing, SGCS convergence (- 1 to 3.5%) and interoperability with all companies’ “own” decoders was observed for the considered structures.

The above study in RAN4 is aligned with direction C in RAN1, and RAN4 has confirmed the test feasibility.

There are other options in RAN1 which would provide more flexibility for exchanged datasets and model parameter, e.g., direction A. If direction A is adopted by RAN1, there is no test feasibility issue in RAN4 either. Since the main task in RAN4 is to define test case, RAN4 can choose the current specified/captured encoder, decoder and training dataset to test for direction A. Therefore, RAN4 can confirm test feasibility and interoperability for CSI compression.

Observation 9: RAN4 can confirm test feasibility and interoperability for CSI compression.

If RAN1 direction C is adopted, the current agreement in RAN4 #115 is as below:

Agreements: If RAN1 direction C agreed, RAN4 and RAN1 should co-ordinate on model structure etc. for direction C due to the similarities between option 3/4 and direction C - Duplicated work is to be avoided

Observation 10: If RAN1 direction C is adopted, RAN4 and RAN1 should co-ordinate on model structure.

Besides, core and performance part shall also be specified.

Proposal 4: Specify Core requirements and Performance requirement for CSI compression associated UE feature and AI/ML LCM procedures.

- Specify necessary RAN4 core requirements and for CSI compression and LCM procedures including performance monitoring

- Specify necessary performance requirements and test cases for CSI compression (including performance monitoring) and UE features enabled by UE-sided models

UE data collection

Since Rel-18, RAN2 has studied UE data collection over two releases. Following way forward on UE-side data collection was agreed in RAN #106 meeting:

For SA From RAN perspective, transfer of data over UP for Solution 2 in SA2 should be studied and provide feedback on fulfilement of visibility and controllability requirements. Transfer of data over UP for Solution 3 can be studied by SA5 RAN has already concluded that transfer of data over CP for Solution 2 and 3 can meet the visibility and controllability RAN requirements, even though some scalability concerns have been raised for high amount of data transfer. RAN is not expecting SA2 and SA5 to study the transfer of data over CP for now. For RAN2 - During Rel-19 RAN2 continues studies on AIML UE sided data collection (excluding 1b) - Study RAN aspects related to data transfer over UP - Discuss level of NG-RAN involvement in the control and configuration of UE side data collection. - Discuss NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion). - Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain. - Discuss on the scalability aspects of CP - RAN2 can Liaise with other WGs if/when needed General RAN will make a final decision on way forward taking into account SA inputs when available. RAN will have a checkpoint in Q2, 2025

In below section, we further discuss our observations and suggestions on UE-side data collection in R20 based on current status of different WGs.

RAN2 status

During RAN2 #130 meeting, RAN2 discussed feasibility of CP solutions with no conclusion on feasibility.

- Capture in the TR issues with CP solution related to large amount of data (if there is a large amount of data) to be transferred for a UE, need for segmentation and it’s implications.

However, it is observed that, for large amount of data, using CP solution to transfer massive size of dataset has been extensively discussed for NW-side dataset transfer via OTA and RAN2 has not reached consensus on its feasibility:

- For OTA solutions ‘gNB -> NW dataset/model parameters collection entity (if needed) -> gNB -> UE -> UE training entity (a server inside MNO or an OTT server)’, RAN2 identified following candidate solutions but there’s no consensus on feasibility: - OTA solution 1a: gNB -> UE via CP - OTA solution 2: CN -> UE via gNB - OTA solution 3: OAM -> UE via gNB

Though NW dataset/model parameter was discussed under the assumption that CP is used for DL dataset transfer, the same constraint on RRC message size also applicable to UL for UE-side data collection, as UL RRC only support 5 more segments than DL RRC. Therefore, in our understanding, there's also no consensus on whether data transfer over CP for UE-side data transfer with massive size is feasible or not. Therefore, we suggest to deprioritize discussion on Solution 2/3 via CP.

Observation 11: Restriction using CP for massive data size transfer for UE-side data collection in UL is the same as OTA solutions for two-sided model NW dataset/model parameter transfer in DL, which makes CP solution not feasible.

Proposal 5: UE-side data transfer Solution 2 and Solution 3 via CP is not considered within scope of R20.

Furthermore, RAN2 didn't discuss any UP solution after RAN #106 meeting. Whether UP solution is feasible or not is up to SA2/SA5, which is analyzed as below.

SA5 status

RAN/RAN2 has received a reply LS from SA5 in S5-250828. RAN2 further replied that SA5 assumption is not correct, since there's no UP tunnel terminated at gNB in 5GA. Hence, in our understanding, UP solution via OAM is not feasible and out of scope of UE-side data collection.

Observation 12: UE-side data collection via UP towards OAM with gNB-terminated UP tunnel is out of scope of 5GA.

SA2 status

RAN received a reply LS from SA2 in S2-2505713, indicating that SA2 has started a new SI working on UE-side data collection UP solution. However, no conclusion has been reached yet and the approval of outcome of this SI is expected in Dec 2025.

In our understanding, without any SA2 conclusion, it is difficult for RAN/RAN2 to further progress/discuss UP solution. Therefore, we suggest to set a check point at Dec 2025, and decide the scope of UE-side data collection (if any) based on the outcome of SA2 study.

Proposal 6: RAN set a checkpoint in Dec 2025, waiting for SA2 conclusion on UP solution before starting any WI work for UE-side data collection in RAN/RAN2.

Objective on AI/ML air interface in 5GA R20

Based on above discussion, we suggest that the objective on AI/ML air interface in 5GA R20 includes the following aspects:

Proposal 7: The objective on AI/ML air interface in 5GA R20 includes the following items.

- CSI compression with two-sided model, encompassing [RAN1, RAN2, RAN3, RAN4]

- Spatial-Frequency domain Case 0, Time-Spatial-Frequency domain CSI compression case 2 (utilizing past CSI information on both UE side and gNB side to improve the CSI feedback performance for present slot), Time-Spatial-Frequency domain CSI compression case 3 (utilizing past CSI information on UE side to improve the CSI feedback performance for future slot(s))

- Specify necessary data format, data contents or additional information exchange procedure/signaling to facilitate inter-vendor collaboration [RAN1, RAN2, RAN3]

- Specify necessary measurements, signaling/ mechanism(s) to support two-sided model LCM operations

- Specify necessary CSI measurement and/or CSI reporting enhancement for two-sided model based CSI compression

- Further study testability and interoperability for two-sided models in RAN4 for Direction A Option 4-1 if adopted in RAN1.

- Core requirements and performance requirement for CSI compression associated UE feature and AI/ML LCM procedures [RAN4]

- Specify necessary RAN4 core requirements and for CSI compression and LCM procedures including performance monitoring

Conclusions

In this contribution, the observations and proposals on two-side AI/ML model and UE data collection are summarized as follows.

Observations

Observation 1: Compared with Case 0, temporal domain Case 2 can achieve higher gain with acceptable complexity.

Observation 2: On top of Case 0, additional specification efforts to support Case 2 is limited, including:

- Additional configuration/reporting of temporal aspects for training dataset collection

- Signaling for mitigating the impact of UCI loss

Observation 3: Compared with Case 0, temporal domain Case 3 can achieve higher gain with affordable complexity.

Observation 4: On top of Case 0, the following additional specification efforts are required for Case 3.

- Additional configuration/reporting of temporal aspects for training dataset collection

- Training dataset labels and performance monitoring labels definition

- Payload determination, UCI mapping, and CSI processing criteria and timeline

Observation 5: Option 4-1 non-OTA OAM solution is confirmed as feasible at least from RAN’s perspective, and it requires less specification efforts with better performance compared with Option 3a-1 and Direction C.

Observation 6: RAN3 concluded AI model algorithm is out of 3GPP scope during Rel-17 study, the required model parameter sharing in option 3a-1 may not be supported by RAN3.

Observation 7: Significant specification efforts on model structure are required for Option 3a-1, and the future-proof model structure is not ensured for forward-compatibility.

Observation 8: Significant specification efforts are required to specify model for Direction C, and the future-proof model is not ensured.

Observation 9: RAN4 can confirm test feasibility and interoperability for CSI compression.

Observation 10: If RAN1 direction C is adopted, RAN4 and RAN1 should co-ordinate on model structure.

Observation 11: Restriction using CP for massive data size transfer for UE-side data collection in UL is the same as OTA solutions for two-sided model NW dataset/model parameter transfer in DL, which makes CP solution not feasible.

Observation 12: UE-side data collection via UP towards OAM with gNB-terminated UP tunnel is out of scope of 5GA.

Proposals

Proposal 1: Include Case 2 for Rel-20 5G-A AI/ML normative work of two-sided AI/ML model based CSI compression.

Proposal 2: Include Case 3 for Rel-20 5G-A AI/ML normative work of two-sided AI/ML model based CSI compression.

Proposal 3: Prioritize Direction A Option 4-1 non-OTA based dataset transfer via OAM in normative work.

Proposal 4: Specify Core requirements and Performance requirement for CSI compression associated UE feature and AI/ML LCM procedures.

- Specify necessary RAN4 core requirements and for CSI compression and LCM procedures including performance monitoring

- Specify necessary performance requirements and test cases for CSI compression (including performance monitoring) and UE features enabled by UE-sided models

Proposal 5: UE-side data transfer Solution 2 and Solution 3 via CP is not considered within scope of R20.

Proposal 6: RAN set a checkpoint in Dec 2025, waiting for SA2 conclusion on UP solution before starting any WI work for UE-side data collection in RAN/RAN2.

Proposal 7: The objective on AI/ML air interface in 5GA R20 includes the following items.

- CSI compression with two-sided model, encompassing [RAN1, RAN2, RAN3, RAN4]

- Spatial-Frequency domain Case 0, Time-Spatial-Frequency domain CSI compression case 2 (utilizing past CSI information on both UE side and gNB side to improve the CSI feedback performance for present slot), Time-Spatial-Frequency domain CSI compression case 3 (utilizing past CSI information on UE side to improve the CSI feedback performance for future slot(s))

- Specify necessary data format, data contents or additional information exchange procedure/signaling to facilitate inter-vendor collaboration [RAN1, RAN2, RAN3]

- Specify necessary measurements, signaling/ mechanism(s) to support two-sided model LCM operations

- Specify necessary CSI measurement and/or CSI reporting enhancement for two-sided model based CSI compression

- Further study testability and interoperability for two-sided models in RAN4 for Direction A Option 4-1 if adopted in RAN1.

- Core requirements and performance requirement for CSI compression associated UE feature and AI/ML LCM procedures [RAN4]

- Specify necessary RAN4 core requirements and for CSI compression and LCM procedures including performance monitoring

Reference

- RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, Sep, 2024.

- RAN1#121Chairman’s notes, May, 2025.

- RAN1#116 Chairman’s notes, Feb, 2024.

- RAN1#120 Chairman’s notes, Feb, 2025.

- R3-253961, “Reply LS on signalling feasibility of dataset and parameter sharing”, RAN3, May, 2025.