3GPP TSG RAN Meeting #108	RP-251308

Prague, Czech Republic, 9-13 June, 2025

# Source:	MediaTek Inc.

# Title:	Discussion on new WID: NR FR1 DL fragmented carriers in Rel-20

# Document for:	Information

Agenda Item:	15.4.4

- Introduction

The intra-band non-contiguous carrier aggregation (NC CA) has been introduced in the RAN4 specs from Rel-11. The requirements defined in the current specification are based on the partially-shared architecture which requires separate Rx RF chains and Rx LOs for receiving each of the non-contiguous component carriers that limit the CA capabilities as the maximum supported number of component carriers. In the SID[1] for NR DL fragmented carriers, RAN4 has concluded [2] it would be feasible under certain conditions for a NR UE to operate one Rx RF chain mode to receive two DL non-contiguous carriers.

- Discussions

During the study phase, RAN4 has identified to enable the one Rx RF chain architecture, there can be no REFSENS degradation in certain condition or slightly REFSENS degradation depends on e.g., Wgap of the two non-contiguous carriers. The receiver adjacent channel selectivity can be maintained when the channel bandwidth of the two carriers are equal. The concerns of REFSENS degradation on Pcell was raised during the study. To manage the impact of Pcell performance degradation, the following operation scenarios can be considered for intra-band non-contiguous DL CA:

a.) FDD band DL fragmented carriers

i. CBW of both DL fragmented carriers + Wgap are small enough

ii. UE located so near to the gNB that UL Tx power level is not too large

iii. For higher order CA combination, both intra-band DL fragmented carriers are SCC (Pcell UL is in another band)

iv. When UE is capable of interference measurement, self-assessment on the pontetial performance degradation and/or reporting the results to network for enabling/disabling one Rx RF chain operation

v. The frequency allocation of PCC and SCC to avoid worst case configuration on DL performance i.e., PCC/SCC assignment

b.) TDD band DL fragmented carriers

With above justifications, we propose the work item scope in bellow:

Proposal 1: We propose work item scope for Rel-20 NR FR1 DL fragmented carriers:

Objective of Core part WI or Testing part WI

-	Introducing new UE behavior for reducing the number of UE Rx RF chains (e.g. from partially-shared Rx RF chains per CC to one Rx RF chain) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a NR CA combination for the inter-operator co-located scenario, considering:

- Specify allowable RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Up to 6 dB DL received power level imbalance between the two non-contiguous CCs [RAN4];

- Down select approaches among the SI[1] outcomes by RAN#111 to manage the impact of Pcell performance degradation [RAN4];

  - Based on the down-selection results, introduce the corresponding mechanism, if needed [RAN4, RAN2];

- Determine and specify UE behaviour of CA fallback with/without semi-statically architecture switch hardware resources (i.e. Rx RF chains) based on the SI[1] outcome [RAN4, RAN2];

  - Re-use current fallback procedure as much as possible to minimize RAN2 impact

NOTE 1:	 No RAN1 impact is foreseen

Objective of Performance part WI

The objective of the performance part is:

-	Depending on the down-selected approach, specify the corresponding RRM test cases, if needed.

-	Specify UE demod requirements to ensure zero impact to performance degradation under concerned scenarios.

- Conclusion

In this RAN #108 meeting, we provided draft WID objectives on NR FR1 DL fragmented carriers in Rel-20 based on the conclusion of the SID on NR FR1 DL fragmented carriers in Rel-19. To specify the related core/performance requirements for FR1 DL fragmented carriers, we propose the followings:

Proposal 1: We propose work item scope for Rel-20 NR FR1 DL fragmented carriers:

Objective of Core part WI or Testing part WI

-	Introducing new UE behavior for reducing the number of UE Rx RF chains (e.g. from partially-shared Rx RF chains per CC to one Rx RF chain) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a NR CA combination for the inter-operator co-located scenario, considering:

- Specify allowable RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Up to 6 dB DL received power level imbalance between the two non-contiguous CCs [RAN4];

- Down select approaches among the SI[1] outcomes by RAN#111 to manage the impact of Pcell performance degradation [RAN4];

  - Based on the down-selection results, introduce the corresponding mechanism, if needed [RAN4, RAN2];

- Determine and specify UE behaviour of CA fallback with/without semi-statically architecture switch hardware resources (i.e. Rx RF chains) based on the SI[1] outcome [RAN4, RAN2];

  - Re-use current fallback procedure as much as possible to minimize RAN2 impact

NOTE 1:	 No RAN1 impact is foreseen

Objective of Performance part WI

The objective of the performance part is:

-	Depending on the down-selected approach, specify the corresponding RRM test cases, if needed.

-	Specify UE demod requirements to ensure zero impact to performance degradation under concerned scenarios.

References

[1] RP-243002 SID: Study on NR FR1 DL fragmented carriers, RAN#106, MediaTek Inc.

[2] R4-2508123, Draft big TR for 38.755: FS_NR_DL_Frag_Carrier, RAN4 #115, MediaTek Inc.