3GPP TSG RAN Meeting #108				RP-251323

Prague, Czech Republic, June 9-13, 2025

Source:	Skyworks Solutions, Inc.

Title:	On HPUE for 5G-Advanced in Rel-20

Agenda Item:	15.4.1

Document for:	Discussion

# Introduction

A summary of the early Release 20 5G-Advanced RAN4 led topics was provided in RAN#107 in [1]. In this paper, we share our views on properly dimensioning the 5G Advanced HPUE work and propose options to streamline the work.

# HPUE for Release 20 5G-Advanced

## Early RAN4 Release 20 topics relevant to HPUE

The Moderator Summary [1] lists a number of high-level candidate topics for RAN4 led Work Items in Release 20, the relevant HPUE related topics are copied below in italics:

Below we list the candidate objectives for each potential RAN4 Rel-20 project as the starting point for the further discussions of RAN4 Rel-20 package. The candidate objectives below are listed without priority, and those highlighted by blue are the candidate objectives with the common interests shown among companies during RAN#107. The further discussions on the detailed objectives will be needed before making decisions on RAN4 Rel-20 package until September.

UE RF centric evolution

- FR1 HPUE enhancement (PC2/PC1.5/PC1) for single carrier and EN-DC/NR-CA

  - This item receives common interest,but needs discussion on the feasibility of some use cases and also need down-selection.

- Power enhancement with relaxed UE Tx EVM and/or uplink throughput improvement with advanced BS receiver (AI based receiver is not precluded as the starting point)

  - TBD on objectives should include Tx EVM relaxation, uplink throughput improvement, or both.

  - Study phase is needed if it is considered and FFS whether to treat it in UE RF or demod package

  - There are still concern on whether Tx EVM should be part of Rel-20 package for 5G-A.

- Tx requirements enhancement for U6G (n104)

- Power classes framework

- Optimize MPR/A-MPR

- MPR reduction by leveraging asymmetric ACLR

NTN enhancements

- NTN HPUE

  - Study whether the high power than PC1 for IoT-NTN and/or NR-NTN, and what high power classes should be considered.

  - FFS whether the proposed high power class can be applied to handheld + FWA or only FWA.

  - Need differentiate NR-NTN and IoT-NTN in terms of power class to be discussed.

- NTN power domain enhancements

In both NR TN and NTN cases, HPUE related Release-20 UERF topics have received strong support. In this contribution we provide our views on the proposed HPUE topics and propose additional options.

## HPUE options for Release 20 5G Advanced

Within the current NR specification there are already a number of HPUE cases that are covered:

- Single band and single CC power classes:

  - PC5 1Tx (20dBm) for NRU

  - PC3 default power class (23dBm) for FDD/TDD TN and NTN

    - 1Tx and 2Tx for NR with optional power boosting

    - 2Tx for NRU

  - PC2 (26dBm) for FDD/TDD TN (+NTN in Rel-19)

    - 1Tx and 2Tx for NR with optional power boosting

  - PC1.5 (29dBm) for TDD TN

    - 2Tx for NR (+NR-NTN in Rel-19)

    - 4Tx for FWA

  - PC1 (31dBm) for NR (FWA+IoT NTN in Rel-19)

  - Rel-19 is introducing means to decrease Minimum Power Reduction (MPR) in some circumstances

- Intra-band contiguous and non-contiguous CC power classes:

  - PC3 default power class (23dBm) for 1Tx and 2Tx FDD/TDD TN

  - PC2 1Tx and 2Tx (26dBm) for FDD/TDD TN

  - PC1.5 (29dBm) introduced in Rel-19

    - 1Tx and 2Tx for contiguous case

    - 1Tx for non-contiguous case

  - Rel-19 is introducing means to decrease Minimum Power Reduction (MPR) in some circumstances

- Inter-band power classes:

  - PC3 default power class (23dBm) for 1/2/3Tx with PC3 per band (PC5 for NR-U bands)

  - PC2 (26dBm) for 1/2/3Tx with PC3/PC2 per band

  - PC1.5 (29dBm) 1/2/3Tx with PC3/PC2/PC1.5 per band

  - With increased power option when the sum of the band power capability is higher than the inter-band power class.

Release 20 candidate HPUE topics builds on the above and a number of new HPUE cases were proposed by companies. A non-exhaustive list of proposals is copied here:

- PC1 with 4Tx UL for FWA. PC1 with 2Tx and UL-MIMO.

- PC1.5 FDD with 2Tx. PC1.5 with 1Tx. PC1.5 TDD and FDD with 3Tx

- PC1 with inter-band and intra-band UL CA (4Tx and 8Tx enhancements)

  - 4x26 dBm. 8x23 dBm. 2x29 dBm (New request for Rel-20)

  - Combination of 23 and 26 (and 29) dB PC1.5 with 4Tx or 8Tx for UL intra-band and inter-band CA

- Reduce UE MPR with relaxed Tx EVM requirements and advanced BS receiver for 64QAM and 256QAM. Reduce UE MPR with relaxed Tx EVM requirements for QPSK and 16QAM

- Specify ACLR of 26dB and corresponding MPR requirements for U6GHz (n104). Specify RF requirements for 200MHz CBW for U6GHz (n104)

- High power class framework

- Power class handling for CA (allowing maximum output power per band for different CA combos)

- Specify the UE RF requirements of MPR reduction by defining different regions corresponding to asymmetric ACLR.

- PC1.5/PC1 for handheld NR-NTN and IoT-NTN devices

- Higher power than PC1 for handheld NR-NTN and IoT-NTN devices

- PC1/PC2 for RedCap NTN UE

- NTN power domain enhancements

- MPR reduction with emission relaxation under NTN deployment.

The proposals above can be classified in a few categories:

- New power class configuration based on existing PA smartphone implementation

- New power class configuration based on PA smartphone implementation yet to be developed

- Power boosting, MPR improvements.

In our view, RAN4 should prioritize new power classes based on existing PA implementations and enabling as much MPR reduction and/or power boosting options as possible.

Going into some level of details, there are PC1 options that can use existing PA implementations in principle, but it should be within a feasible linearity target. Current PC1 definition calls for 37dB ACLR while PC3 is 30dB ACLR and PC2/PC1.5 are 31dB ACLR. So, although it seems feasible to achieve 32dBm with 8x23dBm or 4x26dBm, it cannot be the case if the ACLR target is 37dB. However, the 1dB margin to the PC1 31dBm level, can be traded to a 3dB improvement in ACLR. Thus, a PC1 ACLR target of less or equal to 34dB is preferred and correspond to a similar absolute power interference than PC1.5.

Also, there are request for 1Tx PC1.5 implementations (including PC1 with two Tx with PC1.5 PAs). If we understand that some scenarios like NTN, indoor CPE and Automotive may see little benefit from multiple Tx without specific work in RAN1, we believe that a new PC1.5 1Tx PA and/or filter implementation may suffer from the same issue than the existing 1Tx PC1 case in terms of both filter and PA low cost component availability. If PA and filter technologies exist to achieve 29dBm power handling, it is unlikely to correspond to the high-volume technology or products developed for smartphones.

In our view RAN4 has not exhausted yet all options for increased power out of a PC2 1Tx PA including:

- Power boosting options

- Reduced MPR options

- Power increase with relaxed in-band and/or out-of-band requirements.

Based on the above observations, we believe it is important to streamline the HPUE options for Release 20 to limit the amount of work (needed to allow time for 6G study and spectrum related work) and insure availability of high volume/low-cost components. Here are a few principles we propose to achieve this.

Proposal 1: For Release 20 HPUE work should focus on combinations of existing PA implementations (23dBm/30dB ACLR and 26dBm/31dB ACLR) and enabling all power boosting options.

Proposal 2: For PC1 based on 8x23dBm or 4x26dBm PAs, study the viability of up to 34dB ACLR to enable the reuse of current PC3 and PC2 PA implementations.

Proposal 3: As an alternative to 1Tx PC1.5, study all the options for PC2 power boosting and apply to its 1/2/3/4/8Tx combinations.

# References

- RP-250817 Moderator's summary for RAN4 led REL-20 topics, RAN4 Chair (Huawei), 3GPP TSG RAN Meeting #107, Incheon, Korea, March 12-14, 2025.