3GPP TSG RAN Meeting #108	RP-251339

Prague, CZ, June 9-13, 2025

Agenda item:	16.2

Source:	Nordic Semiconductor ASA

Title:	Study on 6G IoT

Document for:	Discussion

# 1	Introduction

At the TSG RAN #107 meeting in Incheon, Korea a revised SID was approved to study 6G scenarios and requirements [1]. In this contribution we provide our view on key 6G IoT elements that should be included in the study.

# 2	Discussion

## 2.1	Deployed cellular IoT technologies

Starting from LTE R13, 3GPP introduced different IoT technologies, among which the following are deployed and their KPIs summarized in Table below:

|  | Cat-NB2 R14 | Cat-M1 R14 | Cat1-bis R14 | RedCap R17 | [eRedCap R18] |
| --- | --- | --- | --- | --- | --- |
| Bitrate [Mbs] DL/UL#7 | 0.127/0.159 | 0.6/1 | 10/5 | 54-113/56-120 per layer | 10/10 |
| Mod. DL/UL#7 | QPSK/QPSK | 16QAM/QPSK | 64/16 QAM | 64/16 QAM | 64/16 QAM |
| RF BW | 180kHz | 1.44Mhz | 20Mhz | 20Mhz | 20Mhz |
| SCS | 15kHz | 15kHz | 15kHz | 15/30kHz | 15/30kHz |
| BB PRBs #4 | 1 | 6 | 100 | 106/51 | 106/51 or 25/12 |
| UE Rx ant. | 1 | 1 | 1 | 2 | 1 |
| UE Tx ant. | 1 | 1 | 1 | 1 | 1 |
| UE Tx Pow. [dBm] | 14/20/23 | 20/23 | 23 | 23+ | 23+ |
| Network Tx ant, | 2 | 4 | 4 | >=8 | >=8 |
| DL cov enh | Rep. (20dB) | Rep. (10dB) | -3dB | DL beamforming | DL beamforming |
| UL cov enh | Rep (20dB) | Rep. (10dB) | 0dB | Rep. (10dB) | Rep. (10dB) |
| Duplex | HD-only/TDD* In field HD-FDD | HD/FD in field HD-FDD | HD/FD/TDD in field FD-FDD | FD/TDD* in field FD-FDD | HD/FD/TDD* not in field |
| Duplex switch (HD-FDD or TDD) | 1ms | 1ms | puncturing | 13us | 13us |
| Sched delay DL | n+5ms | n+2ms | same slot | same slot | same slot |
| Ack proc delay | n+13ms | n+4ms | n+4ms | n+1ms/0.357ms | n+1ms/0.357ms |
| Sched delay UL | n+8ms | n+4ms | n+4ms | n+1ms/0.429ms | n+1ms/0.429ms |
| Coding DL/UL/CTRL | Viterbi/TBCC/Viterbi | TBCC/TBCC/ Viterbi/ | TBCC/TBCC/ Viterbi/ | LDPC/LDPC/Polar | LDPC/LDPC/Polar |

*TDD not deployed

Among IoT cellular technologies it can be noticed that TDD has not been deployed, for NB-IoT as well as RedCap at the moment.  While NB-IoT, LTE-M were deployed with HD-FDD and RedCap with FD-FDD.

Observation-1: Low-band FDD bands are the most suitable for IoT due to their good coverage. Both half-duplex and full-duplex FDD are currently deployed in cellular IoT vertical.

It is slightly unclear what would be IoT use-cases which would require bitrates as high as 100-200Mbits. Perhaps some forms of XR wearables, low-end smartphones, video surveillance? Asset trackers, metering, connected cities, smart-watches would suffice with 10-20Mbits, while enjoying small-size and low-complexity instead.

Observation-2: Cellular IoT use-cases are different, however majority of cellular IoT use-cases do not require 100-200Mbit/s and complexity that comes with it.

Proposal-1: For the lowest IoT use-cases in 6G, target for 10-20 Mbps peak rates.

## 2.2	Half-duplex bringing complexity down

A half-duplex implementation is an efficient way to reduce complexity on the module level. The relaxation from simultaneous transmission and reception simplifies BB and mainly RFFE design at the UE. However, in previous generations half-duplex support in first releases (Rel-8 of LTE and R-15 of NR) has been missing.

Contrary, for LPWA, half-duplex support has been specified from its first release (i.e. R13), and field adopted half-duplex LTE-M version instead of full-duplex. For NB-IoT, half-duplex was deployed instead of TDD. Contrary, for LTE Cat1bis half-duplex has not been adopted at all, and for NR half-duplex has not been adopted yet.

In addition to complexity reduction, half-duplex reduces also the form factor of the communication module, this because duplexer technology cannot be implemented in the basic silicon. Special semiconductors and large node sizes are required for duplexers, making full-duplex solution difficult to use in small form wearables.

From network point of view, half-duplex does not necessarily impact spectral efficiency of the cell, if scheduling algorithm is designed flexibly and duplexing time-gap is small (e.g. 13us). While efficient scheduler may not be difficult to develop, it may be non-trivial to introduce it only after a full-duplex scheduler has been finalized. This is because full-duplex scheduling choices can be made so that they are not compatible with half-duplex requirements introduced in later releases. Therefore, it is essential that half-duplex requirements are known to network vendors since the first release of 6G.

Proposal-2: In 6G, all FD-FDD, HD-FDD and TDD are strived to be supported from initial 6G release. RAN1 should study how to support unified scheduling designed for all three duplexing modes.

## 2.3	Narrow UL BW

In addition to half-duplex, the key component of low-complex and small-form module is narrow-enough UL transmission bandwidth. Existing 3GPP requirements can be met with lower complexity, when UL transmission BW is low, avoiding need for additional filtering.  When it comes to DL, from Nordic point of view, there is no need to restrict DL BW to within <5MHz. DL could be 10 or 20MHz to ensure efficient broadcast design. However, UL should be confined within <5MHz transmission BW in 6G IoT standard.

Proposal-3: While balanced <5MHz UL/DL RF BW is a good baseline for 6G IoT, unbalance minimum RF BW between UL (e.g. 5MHz) and DL (e.g. 10MHz) could be studied to simplify unified 6G RAT broadcast/SI design while ensuring low-complex UL RF implementations.

## 2.4	1 Rx

Number of received antennas has been often a subject of disputes in RAN plenary sessions. Small number of antennas is not liked by MNOs, because only single MIMO layer can be used for such devices in DL, reducing spectral efficiency on licensed spectrum. However, in our opinion, 1Rx is a crucial feature to be able to reduce complexity of cellular IoT solution and being able to compete with non-cellular technologies like Wi-Sun, LoRA, DECT NR+, and others. We believe that single Rx antenna should be allowed with low-enough throughput devices, such as devices with <10-20Mbits peak rate. This will ensure that impact to MNO license band spectral efficiencies is minimized.

Proposal-4: To minimize impact to license band spectral efficiency, 1Rx support shall be allowed in 6G unified RAT at least for the lowest-bitrate IoT use-cases, i.e. <=10-20Mbits.

## 2.5	sub-PRB allocations and UL repetitions

The sub-PRB scheduling is in our opinion an important building block for ensuring coverage. And this being true particularly in NTN scenarios. This is why we believe sub-PRB scheduling should be part of 6G standard, and if not used in TN, used at least in NTN deployments. Additionally, small number of repetitions should be supported to improve UL coverage of 6G IoT solution.

Proposal-5: Study feasibility of sub-PRB allocations in 6G unified RAT, which would be beneficial at least in NTN scenarios.

Proposal-6: Support small number of UL repetitions (max 10dB) for 6G unified RAT.

## 2.5 Protocol overhead and configuration flexibility

One learning from NR is that protocol configuration flexibility is making it an extremely complex system, and the overhead of excessive configurability and capabilities is eating capacity which otherwise could be used for customer data. While overhead may be negligible when operating on 100MHz carrier, when 3/5MHz carrier is being deployed, overhead is significant part of available bitrate.

Generally speaking, the design of system should target heavily reducing the need for excess DL broadcast, minimize the use and size of SI messages, and aim for minimalistic DL control channel design. There should be high threshold when introducing new SI broadcast or extend their content (essential or on demand).

Finally, feature discussions carried in WGs should be completely redesigned. Instead of having one baseline capability and 20 optional on top. There should be only e.g. three capability profiles 1) minimum set of features 2) selected enhanced set of features 3) all features specified. Once profile is indicated, all features in the profile are mandatory not only for UE, but also for network. This will simplify not only ecosystem discussions and IoDT but also reduces overhead in capability exchange.

Proposal-7: Study potential techniques on how to reduce configurability complexity and system overhead (broadcast, capabilities) especially for the RF BW limited UEs in 6G unified RAT

- at least for 6G IoT consider introducing capability profiles (e.g. 3). Features within indicated profile are mandatory for both UE and network.

# 3	Conclusions

In this contribution, we discussed the 6G IoT core design elements.  The following is observed and proposed:

Observation-1: Low-band FDD bands are the most suitable for IoT due to their good coverage. Both half-duplex and full-duplex FDD are currently deployed in cellular IoT vertical.

Observation-2: Cellular IoT use-cases are different, however majority of cellular IoT use-cases do not require 100-200Mbit/s and complexity that comes with it.

Proposal-1: For the lowest IoT use-cases in 6G, target for 10-20 Mbps peak rates.

Proposal-2: In 6G, all FD-FDD, HD-FDD and TDD are strived to be supported from initial 6G release. RAN1 should study how to support unified scheduling designed for all three duplexing modes.

Proposal-3: While balanced <5MHz UL/DL RF BW is a good baseline for 6G IoT, unbalance minimum RF BW between UL (e.g. 5MHz) and DL (e.g. 10MHz) could be studied to simplify unified 6G RAT broadcast/SI design while ensuring low-complex UL RF implementations.

Proposal-4: To minimize impact to license band spectral efficiency, 1Rx support shall be allowed in 6G unified RAT at least for the lowest-bitrate IoT use-cases, i.e. <=10-20Mbits.

Proposal-5: Study feasibility of sub-PRB allocations in 6G unified RAT, which would be beneficial at least in NTN scenarios.

Proposal-6: Support small number of UL repetitions (max 10dB) for 6G unified RAT.

Proposal-7: Study potential techniques on how to reduce configurability complexity and system overhead (broadcast, capabilities) especially for the RF BW limited UEs in 6G unified RAT

- at least for 6G IoT consider introducing capability profiles (e.g. 3). Features within indicated profile are mandatory for both UE and network.

# References

- RP-250810, Revised SID: Study on 6G Scenarios and requirements, CMCC/Verizon/NTT DOCOMO, INC./Deutsche Telekom