3GPP TSG-RAN  Meeting #108	RP-251342

Prague, Czech,  June 9th – 13th 2025

Agenda Item:	15.4.4

Source:	Ericsson

Title:	Proposed candidate cross-area topics

Document for:	Discussion

# 1	Introduction

During RAN#107, first discussions were held on the RAN4 5G-A package. Discussions are expected to continue at RAN#108 prior to package approval at RAN#109. This contribution is provided for information with our views for the cross-area topics identified at RAN#107.

# 2	RAN4 time unit availability

RP-251341 outlines our view on RAN4 TU for 5G-A. In short, no more than 7.5 TU RF and 7.5 TU RD should be used for both RAN4 impacts of the RAN1-3 led package and all RAN4 led work for 5G-A in release 20. Considering that RAN1-3 led work will already consume up to 6TU RD and 3TU RF, extreme caution must be taken to ensure that work is well understood, reasonable size, relevant and well-motivated.

The proposals in this document should be taken in the context of RAN1-3 led topics, BS RF, UE RF, RRM and demodulation topics and not in isolation.

# 3	NTN

During RAN#107, the following potential areas were discussed for 5G NTN in Rel-20:

NTN enhancements

- NTN HPUE

  - Study whether the high power than PC1 for IoT-NTN and/or NR-NTN, and what high power classes should be considered.

  - FFS whether the proposed high power class can be applied to handheld + FWA or only FWA.

  - Need differentiate NR-NTN and IoT-NTN in terms of power class to be discussed.

- Enhancements for Ku band NTN, including

  - Intra-SAN intra-band CA in Ku band

  - Fixed and other mobile types VSAT (e.g., with small antenna size or LEO only) RF [and RRM] requirements for single carrier

    - For small antenna size, HD-FDD would be needed.

  - Smaller channel bandwidth support (e.g. 3, 5 MHz) and other Rel.19 leftovers

- NTN CA including intra-band and inter-band UL/DL CA for NR-NTN

- NTN mobile VSAT

- Testing method for VSAT

- NTN power domain enhancements

- MSR for in-band IoT-NTN in NR

- Irregular CBW 7MHz for NR-NTN

- Mobility between NR-TN/LTE TN and NR-NTN in the RRC_CONNECTED state.

  - Need check whether RAN2 package includes it.

- Repeater for NTN

NOTE: there are overlapping objectives above and FFS on how to merge them.

It is important to ensure that the NTN workload is focused and take into account that NTN needs to fit within the overall amount of time units available for the RAN4 package, considering RF and RRM implications.

During RAN4#107, HPUE for NTN was identified as being of interest. For NTN HPUE, similar to TN, it is important to identify which types of terminal would be most relevant and likely to be implemented within the 5G timescale.

Consider HPUE for NTN. Scope carefully the UE types to consider.

With regard to CA for NTN, two quite different scenarios have been identified:

- Intra-band contiguous CA for Ku band; this would have the goal of achieving the widest bandwidths that are available for Ku.

- Inter-band and intra-band non-contiguous CA: This would aggregate parts of the L and S band.

In our understanding, both could be relevant, although the demand for the Ku band intra-band contiguous seems to be larger and the task less complex.

Carefully scope CA for NTN according to workload and relevance.

Another set of proposals is support for new types of satellite terminal. When considering new types of terminal, the workload should be considered. Potentially, to enable focused work, a combined and manageable list of realistic VSAT terminal types and HPUE could be constructed to capture essential terminal support for Rel-20.

Consider new terminal types and HPUE together and scope to a reasonable workload.

With regard to the CONNECTED mode mobility between NR/LTE TN and NR-NTN, in our opinion this is an important missing component for NTN integration and should be prioritized. It would need to be addressed within the RAN2 package.

The 7MHz channel bandwidth for NTN is not likely to be a significant amount of specification work and can be included, taking care about the total NTN workload.

7MHz can be accommodated for NTN

# 4	Cross area

During RAN#107, the following cross-topic areas were identified as potential candidates for Rel-20:

Cross RF and baseband topics (Based on limited time, discussions concerning common interest were not addressed for this topic)

- NR FR1 DL fragmented carrier

  - Depending on the conclusion of Rel-19 SI

- Extension of low-low band CA via switching

  - FFS on the scenarios for Rel-20

- Irregular channel bandwidth (flexible channel bandwidth)

  - Check what the new aspects to be evaluated or new solutions to be introduced are? Need consider the other issues, e.g., guard band.

  - FFS whether it should be in 5G-A or 6G.

- 3MHz channel bandwidth for eRedcap devices

  - Minimize the impact on RAN1/RAN2 impacts, if any

  - FFS on whether and what additional features should be supported by 3MHz eRedCap

- Intra-band non-collocated EN-DC/NR-CA deployment

- AI/ML assisted receiver

  - FFS whether it should be in 5G-A or 6G

- ATG evolution

- UAV enhancement

Note: in this section, there is no discussion for

For cross-topic areas, since their nature is to include RF, RRM and potentially demodulation, it is important to consider the impact to each area and the RAN4 workload.

In our understanding, the smallest areas are irregular channel bandwidth and 3MHz for Redcap. For Irregular channel bandwidth, the remaining work is to introduce the 6MHz channel bandwidth and for the next largest bandwidth, variable duplex for some bands.

Introduce 6MHz CBW and variable duplex for next largest CBW.

For the Redcap 3MHz, the work can be kept manageable as long as the Redcap 3MHz is based on existing Rel-17/18 Redcap functionality and maintains the principle that the UE should support all bandwidths up to the maximum 20MHz bandwidth. Introducing further functionality or enhancements beyond what is supported in Rel-17/18 RedCap should be out of scope. This way the impact to the other WGs can be avoided. The requirements shall be developed based on the requirements defined for less than 5 MHz (3MHz carrier) operation for regular NR UEs in less than 5 MHz Rel-18 WI. It is also important to note UE shall operate in 3 MHz carriers only. This way impact to existing deployments can be avoided, also the RAN4 workload can be reduced while it also simplifies the work since the principles of defining the requirements and impacted requirements are already identified. In this case, introducing 2RX 3MHz Redcap is a case of using existing requirements. A small amount of analysis is needed for 1RX.

Introduce 3MHz for Redcap, considering only Rel 17/18 Redcap functionality and supporting all Redcap bandwidths.

For the extension of LB-LB CA with switching, existing work has focussed on the n5_n29 combination with switching between FDD and SDL using a semi-static pattern. Due to the switching between FDD and SDL downlink, the CA does not bring throughput advantages to individual users. Furthermore, due to the semi-static switching pattern, the scheme does not allow for flexibility for scheduling in real traffic and may even limit the throughput benefits and uplink performance. For the n12_n29 combination, it is feasible to switch between FDD+SDL (downlink) and FDD (DL+UL). This, together with a more flexible means of switching would enable increased throughput to individual users due to DL aggregation and flexibility for scheduling in real traffic. Low band FDD-FDD combinations with the UL configured in one band could also be considered, dual RX supported but Rx switching used to improve upon antenna efficiency/tuning.

For LB-LB switching, consider “dual RX” (for e.g. n12_n29) and more dynamic switching.

Regarding fragmented carriers, action for a WI depends on the outcome of the study item. One thing to consider is that it does not make sense to achieve a higher order of carrier aggregation to an individual UE if the performance on the individual carriers is badly degraded. The study needs to establish whether and which band combinations could be achieved with single RX without significant degradation. A potential WI, if it would be useful might need to consider signalling of supported CA combinations and associated capabilities with single RX, configuration of the UE and potentially specific requirements to enable support of higher-order band combinations by means of single-RX chains.

# 5 Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Consider HPUE for NTN. Scope carefully the UE types to consider.

Proposal 2	Carefully scope CA for NTN according to workload and relevance.

Proposal 3	Consider new terminal types and HPUE together and scope to a reasonable workload.

Proposal 4	7MHz can be accommodated for NTN

Proposal 5	Introduce 6MHz CBW and variable duplex for next largest CBW.

Proposal 6	Introduce 3MHz for Redcap, considering only Rel 17/18 Redcap functionality and supporting all Redcap bandwidths.

Proposal 7	For LB-LB switching, consider “dual RX” (for e.g. n12_n29) and more dynamic switching.

Proposed objectives for 3MHz Redcap are as follows:

- Specify RF requirements for Redcap UE operation in 3MHz, based on Rel-17 and Rel-18 Redcap functionality

- Specify RRM requirements for Redcap UE operation in 3MHz, based on Rel-17 and Rel-18 Redcap functionality

- For 2RX, existing non-redcap requirements should be re-used as far as possible.

- For 1RX, adjustments to existing non-Redcap 2RX requirements should be decided as appropriate

- (Performance) specify RRM performance and demodulation requirements for 3MHz Redcap operation, re-using non-Redcap requirements as far as possible, with appropriate adjustments for 1RX.

Proposed objectives for 6 MHz are as follows:

- Specify the generic core requirements to support 6 MHz NR FR1 channel bandwidth [RAN4]

  - Introduce the channel bandwidth 6 MHz

    - Only include 15 kHz SCS

  - Specify the spectrum utilization

  - Specify PC2 and PC3

  - Specify UE RF requirements for UL and DL

  - Specify BS RF requirements for UL and DL

- Specify the band specific core requirements to introduce 6 MHz for NR band TBC. [RAN4]

  - BS and UE Channel bandwidth per operating band

  - UE reference sensitivity

  - When needed:

    - MPR (relative bandwidth criteria)

    - Additional Maximum Power Reduction (A-MPR)

- Enable, if feasible, optional UE support of 6 MHz channel bandwidth from Rel-15 onwards in a release independent manner. [RAN2, RAN4]

  - No impact to 38.307 is expected.

- Once RAN4 has introduced the new channel bandwidth, RAN2 will introduce the necessary signalling for this channel bandwidth. [RAN2]
Note:	This should be similar to the signalling for 35 and 45 MHz channel bandwidths. This signalling should take into account that also additional bands (not considered by this WI) may consider this new channel bandwidth in the future. RAN2 will be informed by RAN4 via LS. No additional RAN2 TUs will be requested for this objective.

- Once RAN4 has introduced the new channel bandwidth, RAN3 will introduce the necessary signalling for this channel bandwidth [RAN3]

Note:	This should be similar to the signalling for 35 and 45 MHz channel bandwidths. This signalling should take into account that also additional bands (not considered by this WI) may consider this new channel bandwidth in the future. RAN3 will be informed by RAN4 via LS. No additional RAN3 TUs will be requested for this objective.

Performance:

Specify necessary UE/BS performance requirements for NR operation in dedicated FDD FR1 spectrum allocations for irregular channel bandwidths, corresponding to the core requirements:

- Specify necessary RRM performance requirements if needed (RAN4)

- Specify, if necessary, UE demodulation performance and CSI reporting requirements for the 6 MHz channel bandwidth (RAN4)

- Specify, if necessary, BS demodulation performance requirements (RAN4)

- Specify necessary BS conformance tests (RAN4)

Proposed objectives for LB-LB switching are as follows:

Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to switch between {case 1, case 3} [RAN4, RAN1]

-	Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2

-	Case 3: Rx on DL aggregation of FDD carrier 1 and SDL carrier 2 and no Tx/Rx on FDD carrier 1

-	Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to switch between {case 4, case 5, case 6, case 7} [RAN4, RAN1]

-	Case 4: Tx/Rx on FDD carrier 1 and no Tx/Rx on FDD carrier 2

-	Case 5: Tx/Rx on FDD carrier 2 and no Tx/Rx on FDD carrier 1

-	Case 6: Tx on FDD carrier 1, Rx on DL aggregation of FDD carrier 1 and FDD carrier 2 and no Tx on FDD carrier 2

-	Case 7: Tx on UL aggregation of FDD carrier 1 and FDD carrier 2, Rx on FDD carrier 2, and no Rx on FDD carrier 1

-	For all switching scenarios, including {case 1, case 2} from the Rel-19 work item NR_LBCA_Sw, define a non-RCC dynamic based procedure, liaising with RAN2 and RAN4 as necessary [RAN1]

-	Specify the switching delay and time mask for carrier switching [RAN4]

-	Specify Tx/Rx collision, interruption, delay requirements and other necessary RRM requirements if identified [RAN4, RAN1]

-	Define the corresponding UE capabilities [RAN4, RAN2, RAN1]

-	Consider the following deployment constraints:

-	The carrier frequency for all cases is <1 GHz

-	Co-located and synchronized network deployment for both carriers

-	Both carriers are in a single TAG

-	SCS 15KHz on both carriers