3GPP TSG RAN Meeting #108                                                RP-251349

Prague, Czech Republic June 09-13, 2025

Title: 	Operator Views on Ambient IoT enhancement in 5G-A Rel-20

Source: 	T-Mobile USA

Agenda item:	15.1.3

Document for:	Discussion and decision

# Introduction

The Rel. 19 work item on ambient IoT is mainly focused on Device Type 1 in Topology 1 in Deployment Scenario 1.  The general scope is summarized in the table below [1]:

| Rel 19 Work Item on Ambient IoT Description |  |
| --- | --- |
| Device Type | Device 1: battery-less with 1 µW power consumption, no signal amplification, RF envelope detector receiver, harvest energy from CW, D2R signal is backscattered signal from the device |
| Deployment Scenario | Deployment scenario 1: both base station and the devices are indoor |
| Topology | Topology 1: Base station communicates with the A-IoT devices directly. A-IoT readers should be part of the base stations. A separate CW emitter transmits signal to the A-IoT device for energy harvesting |
| Spectrum | Use FR1 licensed FDD spectrum. R2D in DL FDD spectrum, D2R in UL FDD spectrum |
| Coexistence with NR | In-band deployment to NR standalone |
| Use Cases | Indoor inventory and indoor command |
| Traffic Type | DO-DTT (Device Originated and Device Terminated Triggered): A device initiates the traffic, and the initiated traffic is triggered by a signaling from the base station to the device. (this is more likely for indoor command use case) |
|  | DT (Device Terminated): A BS initiates the traffic from the base station to the device. |
| Carrier Wave Transmission Waveform | Case 1-4: CW is transmitted from outside the topology. A separate CW emitter transmits signal to the device in UL spectrum. D2R backscattering is in the same carrier as CW for R2D. |
|  | Waveform 1 is used: a single tone unmodulated sinusoid. |
|  | CW emitter is outside the topology |
| Proximity Determination | Solution 1: If the reader successfully receives D2R transmission from the device in response to R2D transmission, then the device is determined as near to the reader |
| Device Availability / Unavailability | Direction 1: Reader does not provide information to a device regarding when the device may become available / unavailable |

Working plan on Rel. 20 WI/SI on ambient IoT has been processed and discussed in the previous meeting.  The proposals are summarized as below [2]:

- Support A-IoT Device 2

- Support additional topologies and deployment scenarios

- Support sensor and positioning use cases

- Support unavailability direction 2

- Support CW (Carrier Wave) inside topology

In this contribution, some details of WI/SI for Rel-20 A-IoT are proposed and discussed.

# Discussion

## Use Cases

- As mentioned in the introduction, Rel. 19 A-IoT is mainly focused on indoor inventory and indoor command use cases.  Both A-IoT enabled base stations and the A-IoT devices are deployed indoors.  In Rel. 19, the position of ambient IoT devices can only be identified based on the proximity of the devices to the locations of A-IoT readers.  This method can achieve the required positioning accuracy for indoor use cases.  It is therefore recommended to work on positioning solutions in Rel. 20.

- Proposal 1: Study feasible positioning solutions for ambient IoT devices

- There are many important outdoor use cases, such as smart grids, automated supply chain distribution and airport terminal luggage tracking, that can benefit from the new ambient IoT technologies to optimize operation efficiency and reduce maintenance costs.  Currently available RFID technologies do not have the capability to serve those outdoor IoT use cases due to very limited communication range and lack of positioning capabilities.  It is therefore essential to include outdoor used cases in Rel.20 WI/SI.

- Proposal 2: The following outdoor use cases should be supported in Rel 20 IoT

- Outdoor inventory (e.g. medical instruments inventory management, airport terminal luggage tracking, automated supply chain distribution)

- Outdoor sensor (e.g. smart grids, forest fire monitoring, smart bridge health monitoring)

- Outdoor positioning (e.g. finding remote lost items, tracking items in transportation)

- Outdoor command (e.g. elderly health care, controller in smart agriculture)

## Devices

In Rel. 19, three types of A-IoT devices are studied [4]:

- Device 1: Battery-less with 1 µW power consumption, no signal amplification, RF envelope detector receiver, harvests energy from CW.  The device does not have a transmitter, D2R signal is backscattered signal from the device

- Device 2a: Batter-less with less than a few tens µW power consumption with signal amplifier for improving signal strength, RF envelope detector receiver, harvests energy from CW.  D2R signal is backscattered signal from the device

- Device 2b: Has battery with less than a few hundred µW power consumption, RF envelope detector with signal amplifier.  D2R signal is transmitted actively from the device.

Only Device 1 is supported in Rel. 19 work item, so consensus has been generally reached on the support of Device 2 in Rel. 20.  Compared to Device 2g, Device 2a does not need battery and has much lower power consumption and complexity, thus has much lower cost.  In addition, Device 2a can achieve longer communication range than A-IoT Device 1 and RFID tags.  Thus, it is important to support Device 2a in Rel. 20.  Device 2b is of course useful in some outdoor use cases where long communication range is required.

Proposal 3: Both Device 2a and Device 2b should be supported in Rel. 20

## Deployment Scenarios and Topologies

Many deployment scenarios and topologies for ambient IoT have been defined in Rel. 18 [3], which are summarized in the table below.  Rel-19 WI is mainly focused on D1T1 scenario, where A-IoT enabled indoor base station communicates with indoor A-IoT directly.  In Rel-20, it is important to investigate the benefits of using an intermediate node (in T2- Topology 2) to enhance coverage and positioning capabilities. Due to the very low complexity of A-IoT devices, it might be difficult to implement complex positioning algorithms directly based on the RF signal from the A-IoT devices.  Instead, A-IoT enabled UEs can be used as an intermediate node and the reference point for positioning the A-IoT devices.  The base station can first localize the position of the A-IoT enabled UE based on 5G positioning technology, then the A-IoT reader inside the UE can localize the A-IoT devices based on the received signal.  This method is especially useful in outdoor use cases, where the required communication range is much larger than indoor use cases.

| Deployment Scenarios |  | Topology |  |
| --- | --- | --- | --- |
| D1 | Both base stations and A-IoT devices are indoors | T1 | A-IoT reader enabled base station communicates with A-IoT devices directly |
|  |  | T2 | Base station communicates with A-IoT devices through an intermediate node. A-IoT reader is integrated into the intermediate node |
|  |  | T3 | An assistant node is used to extend DL (or UL) link budget between the A-IoT enabled base station and A-IoT devices |
| D2 | Base stations are outdoors and A-IoT devices are indoors | T1 | A-IoT reader enabled base station communicates with A-IoT devices directly |
|  |  | T2 | Base station communicates with A-IoT devices through an intermediate node. A-IoT reader is integrated into the intermediate node |
|  |  | T3 | An assistant node is used to extend DL (or UL) link budget between the A-IoT enabled base station and A-IoT devices |
| D3 | A-IoT reader enabled UEs and A-IoT devices are indoors | T4 | A-IoT reader enabled UE communicates with A-IoT devices directly |
| D4 | Both base stations and A-IoT devices are outdoors | T1 | A-IoT reader enabled base station communicates with A-IoT devices directly |
|  |  | T2 | Base station communicates with A-IoT devices through an intermediate node. A-IoT reader is integrated into the intermediate node |
|  |  | T3 | An assistant node is used to extend DL (or UL) link budget between the A-IoT enabled base station and A-IoT devices |

Proposal 4: The following deployment scenarios and topologies should be investigated in Rel.-20:

- D1T2: Both base stations and A-IoT devices are indoors.  A-IoT enabled UEs are used as an intermediate node for coverage extension and positioning assistance.

- D4T2: Both base stations and A-IoT devices are outdoors.  A-IoT enabled UEs are used as an intermediate node for coverage extension and positioning assistance.

- D2T2: Base stations are outdoors and A-IoT devices are indoors. A-IoT enabled UEs are used as an intermediate for coverage extension and positioning assistance.

- D4T1: Both base stations and A-IoT devices are outdoors.  A-IoT enabled base stations communicate with A-IoT devices directly.  The scenario will require a large communication range, thus need Device 2b.

## Carrier Wave Transmission

In Rel. 19 A-IoT work item, the CW (Carrier Wave) is transmitted from outside the topology.  The A-IoT device harvests energy from a separate CW transmitter.  D2R backscattering is in the same carrier as CW for R2D.  For some proposed deployment scenarios under T2 in Section 2.3, it is beneficial to investigate the cases where CW is inside the topology for energy harvesting.  In those scenarios, a CW signal transmitter is integrated in the A-IoT enabled UEs to provide energy harvesting source for A-IoT devices.

Proposal 5: CW inside Topology 2 should be investigated in D1T2, D2T2 and D4T2 deployment scenarios

# Conclusion

Based on the above discussion about Rel-20 A-IoT scope, we have the following proposals:

- Proposal 1: Study feasible positioning solutions for ambient IoT devices

- Proposal 2: The following outdoor use cases should be investigated in Rel 20 IoT

  - Outdoor inventory (e.g. medical instruments inventory management, airport terminal luggage tracking, automated supply chain distribution)

  - Outdoor sensor (e.g. smart grids, forest fire monitoring, smart bridge health monitoring)

  - Outdoor positioning (e.g. finding remote lost items, tracking items in transportation)

  - Outdoor command (e.g. elderly health care, controller in smart agriculture)

- Proposal 3: Both Device 2a and Device 2b should be supported in Rel. 20

- Proposal 4: The following deployment scenarios and topologies should be investigated in Rel.-20:

  - D1T2: Both base stations and A-IoT devices are indoors.  A-IoT enabled UEs are used as an intermediate node for coverage extension and positioning assistance.

  - D4T2: Both base stations and A-IoT devices are outdoors.  A-IoT enabled UEs are used as an intermediate node for coverage extension and positioning assistance.

  - D2T2: Base stations are outdoors and A-IoT devices are indoors. A-IoT enabled UEs are used as an intermediate for coverage extension and positioning assistance.

  - D4T1: Both base stations and A-IoT devices are outdoors.  A-IoT enabled base stations communicate with A-IoT devices directly.  The scenario will require a large communication range, thus need Device 2b.

- Proposal 5: CW inside Topology 2 should be investigated in D1T2, D2T2 and D4T2 deployment scenarios

# References

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

- RP-250802, Moderator’s summary for RAN1 led Rel-20 topics

- TR38.848 V18.0.0. Study on Ambient IoT (Internet of Things) in RAN (Release 18)

- TR38.769 V19.0.0. Study on solutions for Ambient IoT (Internet of Things) (Release 19)