3GPP TSG RAN Meeting #108						           RP-251351

Prague, Czech Republic, June 9-13, 2025

Agenda Item:	15.1.2

Source:	Apple

Title:	Views on Rel-20 5G-Advanced MIMO Phase 6

Document for:	Discussion/Decision

# Introduction

In RAN#107, Slides 7-23 were endorsed in RP-250812 as the chair’s summary for RAN Release 20 5G-Advanced, quoted below

MIMO was listed as one of the candidate for RAN1-led items with estimated TU of ~1. Furthermore, the details on the scope is summarized in RP-250802, quoted below

Based on the contributions submitted to agenda item 15.2.2, the following observations are made. - Overall, the proposals on Rel-20 MIMO can be categorized into the following three areas - Uplink enhancement (items #1, #2, #3, #4, #5) - Downlink CSI enhancement (items #6, #7, #8) - Multi-beam enhancement (items #9, #10) - DMRS enhancement (item #11) - Good level of support on uplink enhancement, downlink CSI enhancement, and DMRS enhancement - However, compared to other proposals, item #5 (UL frequency-selective closed-loop precoding) has relatively large number (6) of companies with negative views - Compared to uplink enhancement, downlink CSI enhancement, and DMRS enhancement, the level of support on multi-beam enhancement was lower or had significant number of companies who had negative views.

In this contribution, we provide our views on Rel-20 5G-Avanced MIMO Phase 6

# Views on Rel-20 5GA MIMO

Overall the previous 5 NR releases, NR MIMO has introduced large amount of features, and many times well above 100 features in some releases. However, many of, or majority of, the NR MIMO features have not been commercially deployed, which makes it reasonable to reconsider the need of Rel-20 5GA MIMO phase 6. On the other side, if we take a look at the features that are deployed in the field, or the features that are critical for the cellular operation especially in high frequency bands, e.g., C-band, etc., MIMO is still the agenda that provides features that have been deployed in the field, with evident performance benefit and robust performance, and enabling the efficient use of large antenna array to improve both the spectral efficiency and link budget. In our view, it is still worthwhile to consider Rel-20 5GA MIMO Phase to address critical or essential issues with very limited scope. Therefore, we have the following proposal

Proposal 1: Consider including MIMO Phase 6 as Rel-20 5G-Advanced RAN1-led scope but with limited scope and limited 1 TU.

Regarding the scope of Rel-20 5G-Advanceed MIMO enhancement, from our perspective, we think there are the following two directions

- Beam management enhancement to reduce the latency and improve the efficiency

- Early CSI report and SRS transmission for both RRC_CONNECTED and RRC_IDLE state

Regarding beam management enhancement, we do observe the following two areas for improvement

- In Rel-19, UE initiated beam management has been designed with the goal to reduce latency. The design focused on the UE initiated beam report via either the dynamically scheduled PUSCH, and configured grant PUSCH, with the common part of UE initiated request via PUCCH. However, Rel-19 does not cover the fast beam change after the UE initiated beam report which is also very crucial to reduce latency in our view. Therefore, it is desirable to support fast TCI switching after the UE initiated beam report in Rel-20.

- Current beam management is designed based on 3 procedures: P1 is used for initial beam acquisition, P2 is used for network beam refinement, and P3 is used for UE beam refinement. In the actual deployment, due to the random rotation of the phase, that UE beam change typically happens more frequently than network beam change. However, one problem is that gNB has no information whether/when to trigger P3, since gNB has no information about the UE beam status. Then UE must rely on periodic SSB or CSI-RS  for P3, which increases the overall latency for UE beam refinement. As explained, it is desirable that the P3 procedure can be triggered by UE. Or more generally, UE can request/trigger aperiodic CSI-RS for beam management enhancement.

Regarding the early CSI report and SRS transmission

- In practice, the device traffic exhibits the following characteristic

  - The traffic is aperiodic and typical packet size is limited, i.e., small packet

- With the above traffic characteristic, to strive a good balance between the user experienced data rate (latency) and device power consumption, it is important to allow fast spectrum adaptation, e.g., SCell activation/deactivation, and fast RRC state transition, e.g., between RRC-CONNECTED and RRC-IDLE/RRC- INACTIVE.

- Based on the above discussion, for both the transition from RRC-IDLE/RRC-INACTIVE to RRC-CONNECTED, and SCell activation, it is desirable to provide more accurate CSI with lower latency that can reduce the latency and improve the resource utilization as this allows more accurate link adaption with lower latency. Moreover, for FDD, early CSI report based on the DL measurement, e.g., SSB and/or CSI-RS can be considered as part of the enhancement; and for TDD, early SRS transmission can be considered exploiting channel reciprocity also as part of the enhancement.

Based on the above discussion, we have the following proposal regarding the scope of Rel-20 5G-Advanced MIMO Phase 6

Proposal 2: For the scope of Rel-20 5G-Advanced MIMO Phase 6, consider the following

- Beam management enhancement including

  - Support of fast TCI switching after the Rel-19 UE initiated beam report

  - Support UE initiated request/triggering of aperiodic CSI-RS

- Early CSI report and SRS transmission for both RRC-CONNECTED and RRC-IDLE state

  - Including the support of both FDD operation (UL CSI report based on DL CSI-RS/SSB measurement) and TDD operation (UL SRS transmission)

# Conclusion

In this contribution, we provide our views on Rel-20 5G-Advacned MIMO Phase 6, with the following proposals

Proposal 1: Consider including MIMO Phase 6 as Rel-20 5G-Advanced RAN1-led scope but with limited scope and limited 1 TU.

Proposal 2: For the scope of Rel-20 5G-Advanced MIMO Phase 6, consider the following

- Beam management enhancement including

  - Support of fast TCI switching after the Rel-19 UE initiated beam report

  - Support UE initiated request/triggering of aperiodic CSI-RS

- Early CSI report and SRS transmission for both RRC-CONNECTED and RRC-IDLE state

  - Including the support of both FDD operation (UL CSI report based on DL CSI-RS/SSB measurement) and TDD operation (UL SRS transmission)

# Reference

[1] RP-250812, Summary for RAN Release 20 5G-Adv, 3GPP TSG RAN Meeting #107, Incheon, Korea, March 12-14, 2025

[2] RP-250802, Moderator's summary for RAN1 led REL-20 topics, 3GPP TSG RAN Meeting #107, Incheon, Korea, March 12-14, 2025