3GPP TSG RAN Meeting #108						                         RP-251352

Prague, Czech Republic, June 9-13, 2025

Agenda Item:	16.1

Source:	Apple

Title:	on RAN plenary study on 6G general

Document for:	Discussion/Decision

# Introduction

In RAN#107, the complete RAN 6G SID, i.e., RP-250810 [1], was approved. The SID covers mainly two aspects of the 6G study, i.e., (1) study on ITU IMT-2030 TPR (Technical Performance Requirement), (2) study of 6G in general. Regarding the study of 6G in general, the followings are the objectives quoted from RP-250810

- Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors. - Develop 3GPP requirements for 6G Radio for improvement of existing services and for new services. - Determine the applicability of legacy services to 6G Radio, and define radio requirements for these, as appropriate. - Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least: - Ensure appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting - Energy efficiency and energy saving: both for network and device - Enhanced spectral efficiency - Enhanced overall coverage, focus on cell-edge performance and UL coverage - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band - Target scalable and forward compatible design for diverse device types - Improved spectrum utilization and operations taking into account diverse spectrum allocations - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs - Aim at a harmonized 6G Radio design for TN and NTN, including their integration - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. - Define a time plan and steer work as appropriate for the RAN WGs during the 6G WG SI to deliver high-level decisions at least on the following areas: - Fundamental 6G radio design aspects: waveform, numerology, channel coding, etc… - Overall high-level aspects of 5G to 6G migration - RAN architecture and interfaces, including RAN-Core interface - Coordination of 6G AI/ML framework

In this contribution, we provide our views on 6G in general.

# Views on 6G in general

In this section, we cover the following aspects of 6G in general

- Services support

- Diverse device type support

## Services support

Service support is an important factor to be considered during the 6G study. Over the course of 3GPP technology evolution, 3GPP has provided specification support for services that have achieved great commercial success, such as MBB (Mobile Broadband). On the other hand, 3GPP also has specified solutions for other services that are not that commercially successful. Most importantly, new services are needed not only to provide more revenues for operators, but also to further improve the user experience. Therefore, it is important that 6G study covers a set of carefully selected, essential, existing services, and new/enhanced services.

For the existing services supported by 3GPP, there are several services that 3GPP has spent considerable amount of time on the design and evolution but achieved no, or very limited, commercial success.  Given the limited time for the 6G first release, it is very crucial for 3GPP to consider only the essential services that have a high chance to be deployed. The services that we believe should not be considered for the 6G first release include sidelink, unlicensed spectrum access, i.e., similar technology as LTE LAA, and NR-U, Broadcast/Multicast, and 6G RAT dependent positioning, etc. Regarding positioning, it is part of ITU IMT-2030 capabilities and is likely to have the corresponding ITU TPR. However, this does not necessarily mean that there is a need to specify 6G RAT dependent positioning in 6G again.

For the new/enhancement services, we think that there are at least three services that 3GPP should consider. The first and foremost is AI/ML, for which 3GPP should investigate the potential performance gain that can be achieved by AI/ML in all layers, in all the WGs, i.e., RAN1/2/3/4. Secondly, ISAC (Integrated Sensing and Communication) is another area that 3GPP can exploit. With the new ISAC channel model ready in 38.901 as the outcome of Rel-19, it is important for 3GPP to investigate the solutions that can allow integrated sensing and communication, and investigate how sensing can be used to improve the user experience, improve the system operation efficiency, etc. Last but not the least, immersive communication, with XR/VR/AR as a good example, is another important topic that 3GPP should cover. Immersive communication has a great potential to significantly improve the user experience, and offer new ways that user can interact with the devices and environments. However, immersive communication is a challenging design as it includes multiple traffic flows with different QoS requirement.

With the above in mind have the following proposal.

Proposal 1: For Rel-20 RAN WG study on 6G, regarding service support

- Do not include the support of following services (not exhaustive list)

- Sidelink

- Unlicensed spectrum access, i.e., similar technology as LTE LAA, and NR-U

- Broadcast, Multicast

- 6G RAT dependent positioning, further discuss how to meet IMT-2030 TPR on positioning, if any

- Features that aim at enhancement without, or with very limited, benefit in practical deployment, e.g.

  - Sub-ms latency enhancement and/or reliability enhancement beyond 99.999%

  - Extremely high peak data rate/spectral efficiency

- Include, at least, support of the following new/enhanced services

- AI/ML

- ISAC (Integrated Sensing and Communication)

- Immersive communication

It is also important to note that, for ISAC, 3GPP mainly covers the channel model in Rel-19, while more important technical discussions including both the performance evaluation and technology design are expected in Rel-20. To carry out the design and evaluation in the RAN WGs, it is important to agree on a set of sensing use cases. In our view use cases that involve the UE as a sensing transmitter and/or receiver and allow for all network nodes as sensing output consumers are essential. This is especially important as proposals in the  R20 5G-A sensing track are very TRP focused with no UE involvement.

Furthermore, it is important to discuss the ISAC key principles of operation and RAN architecture, as those would be quite new and for which there is no 5G baseline. We believe it is important define a clean and usable ISAC architecture, unburdened from the 5G positioning architecture (which saw almost no deployments). To this end, the following ISAC modes of operations should be considered:

- Network-based,

- UE-based, and

- UE-centric.

In our view, the potential use cases for the R20 6G track should include the following:

- Sensing Aided Communications Use cases for both the network and the UE e.g. R19 SA1 ISAC Use Case 5.21

- Privacy focused use cases for sensing data/entity privacy e.g. R19 SA1 ISAC Use Case 5.16

- Public Safety Use cases for both the target and subscriber e.g. R19 SA1 ISAC Use Case 5.27

- Intruder Detection use cases for home e.g. R19 SA1 Use Case 5.1 and 5.6

Proposal 2: For ISAC in the 6G first release, use cases that involve the UE as a sensing transmitter and/or receiver, support network-based, UE-based and UE-centric modes of operation and allow for all network nodes as sensing output consumers are essential. Consider at least the following use cases in TR22.837

- Sensing Aided Communications Use cases for both the network and the UE e.g. R19 SA1 ISAC Use Case 5.21

- Privacy focused use cases for sensing data/entity privacy e.g. R19 SA1 ISAC Use Case 5.16

- Public Safety Use cases for both the target and subscriber e.g. R19 SA1 ISAC Use Case 5.27

- Intruder Detection use cases for home e.g. R19 SA1 Use Case 5.1 and 5.6

## Diverse device type support

It is fair to state that the success of 4G was largely driven by the commercial success of the smart phone and mobile broadband services. On the other hand, the concern on 5G and 6G is also closely related to the lack of new services and device types that can drive the demand for cellular services. As a result, it is very important to support diverse device types for the success of any future 3GPP cellular technology.

Regarding the support of diverse device type, we have the following observation in terms of the lessons learned from 5G

- Introduction of new device types in later releases faces stronger obstacles and less chance for deployment

  - Example 1: RedCap, e.g., wearable devices, introduced in NR Rel-17 and further evolved in Rel-18

  - Example 2: XR device, especially 2Rx XR wearable device, as part of Rel-18 XR and TEI

Therefore, for 6G, it is important to support diverse device types in the first release, at least considering the following

- Covering different device form factors, at least including eMBB smartphones, wearables like watch, XR,  etc.

- Covering different performance requirement/targets for different device types, e.g., user experience data rate, latency, complexity, battery life, coverage, etc.

These are the following aspects that should be considered for the support of diverse device types:

- The definition of diverse device type:

  - Diverse device types can be defined considering that different device types are required to support different sets of mandatory features.

  - Furthermore, for the requirement relaxation for some device types, e.g., #Rx/#Tx, it is not reasonable to have further restriction on other aspects such as support CA/maximum BW, etc.

- The impact of diverse device types on channel bandwidth:

  - Both minimum and maximum channel bandwidth critically impacts the cellular system design. Minimum channel bandwidth impacts the design of features such as initial access, IDLE state design, etc. Maximum channel bandwidth impacts the implementation complexity.

  - For minimum channel bandwidth, if LPWA devices are not considered, we prefer to consider 20MHz. If LPWA devices are considered, we prefer to consider at least 5MHz

  - For maximum channel bandwidth, we consider 200MHz with 8K FFT as a starting point.

- The impact of diverse device type on coverage requirement:

  - Coverage, or MCL (Maximum Coupling Loss), is a very important design objective for cellular systems.

    - In legacy RAT, e.g., NR/LTE, it is considered that the mobile broadband service supports coverage of  ~144dB MCL, IoT solutions like MTC/eMTC support ~154dB MCL, and NB-IoT supports ~164dB MCL

  - We believe the 6G first release should support comprehensive coverage enhancement and ensure adequate coverages for all RRC states, for both DL and UL, and for all channels/signals. In terms of the target MCL, we observe the following need:

    - For wearable devices, e.g., watch, it is reasonable to assume that ~10dB antenna gain loss is needed due to the form factor limitation. Therefore, ~154dB MCL is needed for wearable devices.

    - For NTN, our analysis shows that at least 10dB coverage extension is needed for LEO-600, and more coverage extension is expected for LEO-1200, and GEO. Therefore, NTN also needs at least ~154dB MCL.

With the above discussion, we have the following proposal

Proposal 4:  For the support of diverse device type in 6G first release

- Different device types are required to support different sets/values of mandatory features

- When mandatory feature relaxation is introduced for some device type, e.g., #Rx/#Tx relaxation etc., avoid introducing other unnecessary restrictions, e.g., CA support, maximum BW, etc.

- In terms of the channel bandwidth, from diverse perspective

- For minimum channel bandwidth, consider 20MHz if LPWA device is not included in 6G, otherwise if LPWA device is included, consider at least 5MHz

- For maximum channel bandwidth in FR1/FR3, consider 200MHz with 8K FFT and 30KHz SCS as baseline

- In terms of coverage

- Support comprehensive coverage enhancement to ensure adequate coverages for all RRC states, both DL and UL, and all channels/signals

- Consider ~154dB MCL (Maximum Coupling Loss) as starting point, especially for the following cases

  - Antenna gain loss due to the form factor limitation for wearable devices

  - NTN LEO-600 deployment

# Conclusion

In this contribution, we provide our views on 6G first release in general, with the following proposals

Proposal 1: For Rel-20 RAN WG study on 6G, regarding service support

- Do not include the support of following services (not exhaustive list)

- Sidelink

- Unlicensed spectrum access, i.e., similar technology as LTE LAA, and NR-U

- Broadcast, Multicast

- 6G RAT dependent positioning, further discuss how to meet IMT-2030 TPR on positioning, if any

- Features that aim at enhancement without, or with very limited, benefit in practical deployment, e.g.

  - Sub-ms latency enhancement and/or reliability enhancement beyond 99.999%

  - Extremely high peak data rate/spectral efficiency

- Include, at least, support of the following new/enhanced services

- AI/ML

- ISAC (Integrated Sensing and Communication)

- Immersive communication

Proposal 2: For ISAC in the 6G first release, use cases that involve the UE as a sensing transmitter and/or receiver, support network-based, UE-based and UE-centric modes of operation and allow for all network nodes as sensing output consumers are essential. Consider at least the following use cases in TR22.837

- Sensing Aided Communications Use cases for both the network and the UE e.g. R19 SA1 ISAC Use Case 5.21

- Privacy focused use cases for sensing data/entity privacy e.g. R19 SA1 ISAC Use Case 5.16

- Public Safety Use cases for both the target and subscriber e.g. R19 SA1 ISAC Use Case 5.27

- Intruder Detection use cases for home e.g. R19 SA1 Use Case 5.1 and 5.6

Proposal 3:  For the support of diverse device type in 6G first release

- Different device types are required to support different sets/values of mandatory features

- When mandatory feature relaxation is introduced for some device type, e.g., #Rx/#Tx relaxation etc., avoid introducing other unnecessary restrictions, e.g., CA support, maximum BW, etc.

- In terms of the channel bandwidth, from diverse perspective

- For minimum channel bandwidth, consider 20MHz if LPWA device is not included in 6G, otherwise if LPWA device is included, consider at least 5MHz

- For maximum channel bandwidth in FR1/FR3, consider 200MHz with 8K FFT and 30KHz SCS as baseline

- In terms of coverage

- Support comprehensive coverage enhancement to ensure adequate coverages for all RRC states, both DL and UL, and all channels/signals

- Consider ~154dB MCL (Maximum Coupling Loss) as starting point, especially for the following cases

  - Antenna gain loss due to the form factor limitation for wearable devices

  - NTN LEO-600 deployment

# Reference

[1] RP-250810, Revised SID: Study on 6G Scenarios and requirements, 3GPP TSG RAN Meeting #107, Incheon, Korea, March 12-14, 2025

[2] TR 22.837, Feasibility Study on Integrated Sensing and Communication, 3GPP Technical Specification Group TSG SA