3GPP TSG RAN Meeting #108	RP-251357

Prague, Czech Republic, June 9-13th, 2025

# Source:	Aalyria, Viasat, Inmarsat, ESA

# Title:	New Basket WI on missing NR NTN channel BWs

# Document for:	Approval

Agenda Item:	9.1.4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	Basket WI on missing NR NTN channel BWs

## Acronym: NR_NTN_R19_CHBW

## Unique identifier: 	 XXXXXX

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part |  |

Potential target Release: Rel-19.

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  |  |  |
| No | X |  | X | X | X |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a … {Tick one box. "Feature / Building Block / Work Task" form a hierarchical structure. E.g. no Building Block can be proposed without a corresponding parent Feature. The full structure of all existing Work Items is shown in the 3GPP Work Plan in ftp://ftp.3gpp.org/Information/WORK_PLAN }

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, we define them as work tasks. If you are in doubt, please contact MCC.

### 2.2	Parent Work Item

| Parent Work Items |  |
| --- | --- |
| Unique ID | Title |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one WID. Therefore the table above should just include the feature WI Unique ID and title.

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

NOTE:	Also related or dependent WIs/SIs in other TSGs should be indicated.

## 3	Justification

When NR NTN was first specified in Release 17, certain channel bandwidths, namely CHBW above 20 MHz, were not missed, due to workload or mistaken assumptions on lack of spectrum. However, several bands actually have access to larger blocks of spectrum, and a number of operational reasons also would justify inclusion of the larger channel bandwidths, such as:

- S-band globally: Some operators in S-band have access to the full 30 MHz n256 in certain regions

- R19 L-bands: New R19 n251/n250 Combined L-bands support up to 41 MHz for a single operator in some regions, but lack of Core requirements makes using those CHBW problematic and creates procedure issues.

- Ku band: During the Ku band Work Item definition, great pushback was received in introducing CHBW above 20 MHz for Ku band in its FR1 configuration, however, CHBW up to at least 50 MHz are critical for Ku band operators

- New S-bands: Some countries (e.g. South/Central America) are considering up to 35 MHz of MSS S-band.

- Higher flexibility: Deployment of a larger system BW allows higher flexibility, also to support sub-blocks less than the minimum 3 MHz required for a single standalone channel to be deployed. This also helps improved spectrum efficiency by minimizing guard bands and giving more flexibility in the dynamic allocation of resources.

- Spectrum pooling: Some operators may opt to pool their spectrum and operate a common shared network, thus improving flexibility and spectral efficiency.

- Previous work: 30 MHz channel BW was partially introduced in late R17/18, but with an incomplete set of requirements.  Finalization is needed in order to make it deployable.

In addition, in Release 19, support for HPUE Power Classes including Power Class 2 and Power Class 1 is being finalized, and the general framework has been agreed.  There is strong interest from the satellite community introducing PC2 and PC1 also for the additional bands, as well as a need to align its support to the missing channel BWs.

The preconditions:

For a specified NR NTN band, to propose list of channel bandwidth(s) and associated sub-carrier spacing to be supported. Clause 4.1 will be updated based on approved requests.

It is expected that the Common Core requirements for 25, 35 and 50 MHz channel BWs will be covered by the NR_NTN_Ku_band Work Item, and as such have not been included.

## 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

### 4.1.1	Objective and scope

- Specify, if and where necessary, the generic core requirements to support the missing 30, 40, 45 MHz NR NTN FR1 channel bandwidths [RAN4]

  - Introduce the channel bandwidths of 30, 40, 45 MHz

    - NOTE: It is expected that the Common Core requirements for 25, 35 and 50 MHz channel BWs will be introduced by the R19 NR_NTN_Ku_band Work Item.

  - Include 15, 30 and 60 kHz SCS

  - Specify the spectrum utilization

  - Specify UE RF requirements for UL and DL

  - Specify BS RF requirements for UL and DL

- Specify channel bandwidth – sub-carrier spacing combinations to be supported for each considered band according to Table 4.1.3-1.

  - The channel bandwidth should be one of the following list:

    - FR1: {3 MHz, 5 MHz, 10 MHz, 15 MHz, 20 MHz, 25 MHz, 30 MHz, 35 MHz, 40 MHz, 45 MHz, 50 MHz }. Note that 3 MHz is only possible for bands below 1 GHz.

  - The band should be a specified NR NTN band.

- Introduce support for PC2 and PC1 for the missing channel bandwidths and the corresponding bands in the Basket

- Analyze and specify requirements:

  - Reference sensitivity and associated RB allocation.

  - When needed:

    - MPR (relative bandwidth criteria)

    - Additional Maximum Power Reduction (A-MPR)

    - NS signalling.

### 4.1.2	Way of working

The new request adding support for channel bandwidth in existing NR NTN band should be submitted on RAN4 reflector before TDoc submission deadline the next RAN4 meeting (1 week before the meeting). When requesting the new channel bandwidth, the proponent should identify the expected impairments including A-MPR and capture them in "Additional information" column in Table 4.1.3-1.

The basket WI will then be updated with the new request (section 4.1.3, Table 4.1.3-1) and submitted to next RAN4 meeting for endorsement, before submission to RAN meeting for approval.

When the work is completed, all draft CRs related to one request will be submitted in the same RAN4 meeting to check consistency. If they are endorsed, the basket WI Rapporteur will merge all draft CRs from all requests in big CRs (one per TS specification) .

After the RAN4 meeting preceding a RAN meeting, those big CRs will be sent on RAN4 reflector for email approval (1 week) and, if agreed, they will be submitted to following RAN meeting for approval.

Following figure is summarizing the proposed way of working:

### 4.1.3	Requests overview,

Table 4.1.3-1: Requests tracking

| Band | Channel bandwidth and SCS | Contact name, company | Contact email | Other supporting companies | Justification | Additional information | status |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Note: minimum 3 | Note: Spectrum information and usage that would justify introducing support for the new channel bandwidth(s) in the considered band. | Note: any specific technical challenge should be triggered and highlighted here |  |
| n255 | 25, 30 MHz (15, 30 and 60kHz SCS) | Luca Lodigiani, Aalyria | luca@aalyria.com | Viasat, Inmarsat | Band n255 has a total of 34 MHz of available spectrum, in some regions assigned to a single operator, thus at least channel BW up to 30 MHz should be possible. Support for 3 MHz channel BW with 15 RB was added in Rel-19, but band-specific requirements need to be defined. 3 MHz is critical for certain deployments in regions where spectrum is used by legacy services. |  |  |
| n256 | 25, 30 MHz (15, 30, 60kHz SCS) |  |  |  | Band n256 has 30 MHz of available spectrum, in some regions assigned to a single operator, thus channel BW up to 30 MHz should be possible. |  |  |
| n253 | 3 MHz (15 kHz SCS) | Luca Lodigiani, Aalyria | luca@aalyria.com | Viasat, Inmarsat | n253 has 7 MHz of available spectrum in several regions. Support for 3 MHz channel BW with 15 RB was added in Rel-19, but band-specific requirements need to be defined. 3 MHz is critical for certain deployments in regions where spectrum is used by legacy services. |  |  |
| n251 | 3 MHz DL/UL (15kHz SCS) 25, 30 DL/UL (15, 30 and 60kHz SCS) 35, 40 MHz DL only (15, 30 and 60kHz SCS) | Luca Lodigiani, Aalyria | luca@aalyria.com | Viasat, Inmarsat | Band n251 combines the DLs from n255 and n253 together in a single contiguous DL of 41 MHz, in some regions assigned to a single operator. Therefore, DL channel BWs up to 40 MHz must be supported, with additional Asymmetric channel BW combinations. The UL is limited to a maximum of 34 MHz, therefore, ULs for asymmetric combinations, as well as CHBW symmetric combinations should be limited to 30 MHz. Support for 3 MHz channel BW with 15 RB was added in Rel-19, but band-specific requirements need to be defined. 3 MHz is critical for certain deployments in regions where spectrum is used by legacy services. |  |  |
| n250 | 3 MHz DL/UL (15kHz SCS) 25, 30, 35 and 40 MHz DL only (15, 30 and 60kHz SCS) | Luca Lodigiani, Aalyria | luca@aalyria.com | Viasat, Inmarsat | Band n250 combines the DLs from n255 and n253 together in a single contiguous DL of 41 MHz, in some regions assigned to a single operator. Therefore, DL channel BWs up to 40 MHz must be supported, with additional Asymmetric channel BW combinations. The UL is limited to a maximum of 7 MHz, therefore, ULs for asymmetric combinations, as well as CHBW symmetric combinations should be limited to 5 MHz. Support for 3 MHz channel BW with 15 RB was added in Rel-19, but band-specific requirements need to be defined. 3 MHz is critical for certain deployments in regions where spectrum is used by legacy services. |  |  |

### 4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

- Specify necessary UE/SAN performance requirements for NR NTN for the missing channel bandwidths, corresponding to the core requirements:

- Specify necessary RRM performance requirements if needed (RAN4)

- Specify, if necessary, UE demodulation performance for the missing channel bandwidths (RAN4)

- Specify, if necessary, SAN demodulation performance requirements (RAN4)

- Specify necessary SAN conformance tests (RAN4)

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |

{Note 1: Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TR 38.863 | Basket WI on missing NR NTN channel BWs | TBD | Core Part |
| TS 38.101-5 | Basket WI on missing NR NTN channel BWs | TBD | Core Part |
| TS 38.101-2 | Basket WI on missing NR NTN channel BWs | TBD | Core Part |
| TS 38.108 | Basket WI on missing NR NTN channel BWs | TBD | Core Part |
| TS 38.181 | Basket WI on missing NR NTN channel BWs | TBD | Perf Part |
| TS 38.133 | Basket WI on missing NR NTN channel BWs | TBD | Perf Part |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

## 6	Work item Rapporteur(s)

TBD, TBD ()

## 7	Work item leadership

RAN4

## 8	Aspects that involve other WGs

None identified yet.

NOTE:	For RAN WIs: Section 8 applies only to WGs outside of TSG RAN because RAN WG aspects have to be covered in section 4.

## 9	Supporting Individual Members

Supporting IM name

Aalyria

Viasat

Inmarsat

ESA