3GPP TSG-RAN Meeting #108	RP-251362

Prague, CZ, 9-13 June, 2025

Agenda item:	15.1.5

Source:	Skylo Technologies

Title:	View on Rel-20 NTN for NR Phase 4

Document for:	Discussion and Decision

# Motivation and Justification

Support of 3MHz Channel Bandwidth or less for FR1-NTN bands

In areas with dense terrestrial network (TN) coverage, NR-NTN is not intended to serve as the primary communication layer but rather as a complementary or fallback solution. Based on the research and customer analysis, these areas typically experience coverage gaps, but have minimal throughput demand for  satellite services, making the current minimum NR-NTN channel bandwidth of 5 MHz excessive. Introducing NR-NTN channel bandwidths of 3MHz or less would better align with actual service requirements, avoiding inefficient spectrum usage in environments where high capacity is unnecessary.

The support of 3MHz Channel Bandwidth or less for FR1-NTN bands allows operators to deploy narrower carriers for minimal service footprints without wasting valuable spectrum, particularly in low-demand zones. Furthermore, it supports simpler hardware configurations and more flexible deployment options, especially for satellite payloads and terminals operating under power and bandwidth constraints.

Improved link budget for return link

In 5G NR NTN deployment, the return link is particularly constrained due to the limited transmit power of handheld UEs. UEs are typically unable to dynamically increase their transmit power to compensate for the high and rapidly varying path loss inherent to satellite channels. As a result, improving the spectral energy density of uplink transmission becomes critical. In many scenarios, PUSCH transmission using the standard PRB bandwidth is insufficient to achieve a reliable uplink margin. Therefore, it is proposed to study the feasibility of supporting sub-PRB granularity, such as single-subcarrier or multi-subscriber allocations, for PUSCH transmission in order to enable more power-efficient, narrowband uplink communication under several link budget constraints in NR-NTN scenarios.

Broadcast Frames Optimization:

Current broadcast frame configurations have less adaptability and flexibility. The high broadcast overhead is particularly inefficient for satellite networks, where power is constrained due to reliance on solar energy and limited battery capacity. As a significant portion of downlink bandwidth is used for broadcast signaling, it negatively affects both spectral efficiency and power consumption. To address these challenges, adaptive and flexible broadcast frame transmission mechanisms should be introduced. Such enhancements would enable more efficient use of satellite resources; however, the enhancements should be designed to ensure minimal impact on device connection latency, preserving the user experience.

# Objectives for Rel-20 NTN for NR Phase 4

The following objectives may be considered for potential inclusion in the scope of NTN for NR Phase 4:

- Support for reduced channel bandwidths (e.g. 3MHz or less) for FR1 NTN bands to accommodate low-data-rate and power-constrained use cases

- Support for sub-PRB granularity (e.g. single or multi-subcarrier) allocation in PUSCH transmission to enhance uplink link budget efficiency under severe path loss conditions.

- Mechanisms to enable adaptive and flexible broadcast frames transmission (e.g. adaptive periodicity/frequency)

# Proposal

To accommodate the objectives above in NTN for NR Ph4.