3GPP TSG RAN Meeting #108	RP-251364

Prague, Czech Republic, June 9-13, 2025

Agenda Item:	15.1.5

Source:	NICT

Title:	Views on Non-Terrestrial Networks (NTN) for NR in Rel-20

Document for:	Discussion

1	Introduction

Following the study on NR to support NTN done in Release 15 and 16, enhancements of NR specification to support NTN have been made in Release 17 and 18. Further enhancements are being made in Release 19 under the work item titled Non-Terrestrial Networks (NTN) for NR Phase 3, which is aiming at specifying enhancements of downlink coverage and uplink throughput, signaling of the intended service area of a broadcast service, support of regenerative payload, and Support of Rel-17 RedCap and Rel-18 eRedCap UEs[1]. Regarding downlink coverage enhancements of Release 19, maximum periodicity of SSB transmission has been agreed to be extended from 20ms to 160ms, and repetitions are to be introduced for PDCCH and PDSCH that carry SIBs required before PRACH transmission and following RACH Msgs. However, though SSB can be delivered to more beam footprints by extended SSB periodicity, 100% coverage would not be still achieved under some operating scenarios[2][3]. And, while repetition is the basic measure to improve link budget, its drawback is the inefficient resource usage.

In this contribution, we propose possible justification and objectives for Non-Terrestrial Networks (NTN) for NR Phase 4 work item in Release 20 based on the understanding that it is crucial for commercial success of NTN to provide 100% coverage and improve resource usage efficiency to accommodate more UEs.

2	Discussion and proposed objectives for NR NTN

2.1	Discussion

One of the observation made in RAN1 Release 19 NTN for NR Ph3 work item[2], was that while 100% coverage ratio can be achieved for satellite parameter set 1-1/1-3, which allow 106 simultaneous transmissions, when SSB periodicity is extended to 80ms, for satellite parameter set 1-2, which allow 16 simultaneous transmissions, coverage ratio would be 96.8% when SSB periodicity is extended to 320ms. Since the maximum SSB periodicity was later agreed to be 160ms to minimize specification impact, the coverage ratio is then limited to be 48.4%[4]. This means that more than half of the possible satellite coverage will be left as out of service area, which is undesirable situation considering that being able to provide ubiquitous coverage is one of distinguished feature of NTN compared to TN. The out of service area will also arise for the satellite parameter set 1-1/1-3 if more resources are allocated to high traffic area.

The use of wide beam was one of main topics of Release 19 work item, which requires less SSB transmission opportunities and less radio resources for SIB transmission by delivering them to wider area in one transmission. Though it is thought to be a good approach to increase coverage ratio, enough discussion was not made during Release 19 to introduce specification support for this technology.

The issue of control channel overhead was also identified during the Release 19 discussion. When SSB periodicity is 160ms, 10% of total resource blocks are occupied for control channel for satellite parameter set 1-1/1-3, and it rises to 34% for satellite parameter set 1-2[5][6]. It would be necessary to consider improving radio resource usage efficiency for both control channels and data channels. Since a certain portion of satellite coverage generally includes less traffic area such as ocean or desert, it is thought to be effective to introduce on demand SIB, and on demand SSB if possible, to reduce control channel overhead and accommodate more user data traffic.

To allow handheld terminals to directly connect to the satellite, −5.5dBi of UE antenna gain was assumed and repetition schemes were introduced for PDCCH and PDSCH in the Release 19 work item, which results in low radio resource usage efficiency. It is thought to be worth considering other modes of improving DL coverage that do not sacrifice radio resource usage efficiency such as relay or collaborative modes when they are available.

From the above arguments, we think it is worth considering following topics in NTN for NR Ph4 in Release 20. On demand SIB and relay/collaborative modes are illustrated in Figs 1 and 2.

- Support of the usage of wide beam including relevant signalling for narrow beam resolution

- Reformation of SIB1/19 for NTN to include minimum information for RACH process

- Support of on demand SSB and/or SIB to reduce their transmissions to unnecessary beam footprints while keeping 100% coverage ratio

Fig. 1 On demand SIB transmission for NTN

Relay mode and/or collaboration mode of UEs to reduce repetitions for radio resource usage efficiency and/or improve reachability of notification alert

Fig. 2 Relay mode and collaborative mode

2.2	Proposed justification

According to our view, possible justification for NTN for NR Ph4 in Release 20 can be as follows:

Since the main feature that differentiates NTN from TN is its wide coverage area extending to where TN is difficult to cover physically and/or costly (e.g. ocean, desert, deep mountains, etc.), providing ubiquitous coverage is one of the main motivations to make use of NTN. Therefore, if some areas have to be left uncovered due to the limitation of the standard specification, it could be updated to eliminate such limitation. In addition, because of increasing market interest of NTN and its potential large coverage area, one satellite will have to handle more numbers of UEs. To accommodate more UEs and increase channel resources for data traffic, it is thought to be crucial to reduce control channel overhead.

2.3	Proposed objectives

We propose following objectives to be included for NTN for NR Ph 4 of Release 20 work item:

- Specify enhancements on procedure of SSB and necessary SIB acquisition before UE starts to initiate RACH process.

  - 100% coverage ratio should be achieved for at least SSB delivery.

  - Solution should not sacrifice radio resource usage due to excessive control channel overhead.

  - Solution may utilize outcome of on demand SSB and/or SIB defined in release 19 NES or its adaptation for NTN scenario, or new on demand SSB and/or SIB scheme could be defined for NTN.

- Specify further DL coverage enhancements by utilizing relay or collaboration of UEs for both control channel and data channel

  - Solution should consider the reception of paging or other channels that perform notification alert.

  - Two scenarios can be considered. One is that one UE has better connection to the satellite than other UEs (relay mode), and the other is that multiple UEs collaboratively improve DL reception from the satellite (collaboration mode).

3	Conclusion

Proposal1: To include following justification in NTN for NR Ph 4 of Release 20

Since the main feature that differentiates NTN from TN is its wide coverage area extending to where TN is difficult to cover physically and/or costly (e.g. ocean, desert, deep mountains, etc.), providing ubiquitous coverage is one of the main motivations to make use of NTN. Therefore, if some areas have to be left uncovered due to the limitation of the standard specification, it could be updated to eliminate such limitation. In addition, because of increasing market interest of NTN and its potential large coverage area, one satellite will have to handle more numbers of UEs. To accommodate more UEs and increase channel resources for data traffic, it is thought to be crucial to reduce control channel overhead.

Proposal2: To include following objectives in NTN for NR Ph 4 of Release 20

- Specify enhancements on procedure of SSB and necessary SIB acquisition before UE starts to initiate RACH process.

  - 100% coverage ratio should be achieved for at least SSB delivery.

  - Solution should not sacrifice radio resource usage due to excessive control channel overhead.

  - Solution may utilize outcome of on demand SSB and/or SIB defined in release 19 NES or its adaptation for NTN scenario, or new on demand SSB and/or SIB scheme could be defined for NTN.

- Specify further DL coverage enhancements by utilizing relay or collaboration of UEs for both control channel and data channel

  - Solution should consider the reception of paging or other channels that perform notification alert.

  - Two scenarios can be considered. One is that one UE has better connection to the satellite than other UEs (relay mode), and the other is that multiple UEs collaboratively improve DL reception from the satellite (collaboration mode).

References

- RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3

- R1-2405699, Session notes for 9.11 Non-Terrestrial Networks (NTN) for NR Phase 3 and Internet of Things (IoT) Phase 3

- R1-2502473, Discussion on DL coverage enhancements for NR-NTN, NICT

- R1-2405090, Discussion on NR-NTN downlink coverage enhancement, Mediatek

- R1-2406437, FL Summary #3: NR-NTN downlink coverage enhancements, Thales

- R1-2406434, Discussion on NR NTN Downlink coverage enhancements, Thales