3GPP TSG RAN Meeting #108												RP-251368

Prague, Czech Republic, June 9-13, 2025

Agenda Item:	16.1

Source:	Huawei, HiSilicon

Title:	Consideration on new and enhanced services for 6G

Document for:	Discussion and decision

Introduction

A RAN-level study on “6G Scenarios and requirements” was approved in RAN#107[1]. Regarding the services in the scenario, the objective of this SID includes:

This study item aims to

- investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs.

  - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN

- Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors.

- Develop 3GPP requirements for 6G Radio for improvement of existing services and for new services.

- Determine the applicability of legacy services to 6G Radio, and define radio requirements for these, as appropriate.

- …..

In this contribution, the considerations and views on 6G services are discussed.

Consideration on 6G Radio for new/extended services

The section discusses the new/extended services for 6G. These services could bring new revenue or business in 6G towards to 2030-2040.

Immersive communication service

Base on the 3GPP TR 22.870[2]: "6G Use Cases and Service Requirements", the holographic communication, mixed Reality gaming and other immersive communication services are agreed. These immersive communication use cases define 6G services as integrate augmented, virtual, and mixed realities to create hybrid environments where physical and digital content interact. e.g. the XR traffic involves diverse packet types—control signaling, audio/video streams, motion tracking, haptic feedback, and application data—each with distinct quality-of-service (QoS) requirements.

Proposal 1:  6G RAN design should support high capacity with the required high data rate, low latency and high reliability.

Network-assisted video-based AI inference task offloading for mobile embodied AI

Embodied Artificial Intelligence (e.g. humanoid robots, quadruped robots, L4+ automated driving vehicles, drones, etc) can access the large language model (LLM) in cloud for both data training and inference to handle complicated tasks, according to agreed use case “Network-assisted video-based AI inference task offloading for mobile embodied AI” in 3GPP SA1#110 [3].

The requirement of this use case is summarized in Table 2.1.1-1 with consideration of 3 different configurations, where different quality of video streams are assumed:

- Configuration I is widely used in the existing product;

- Configuration 2 is an upgrade of I with front view cameras (dual camera similar to “human eyes”) replaced with 4K resolution 30Hz refresh rate;

- Configuration 3 is further upgrade with additional 2 cameras covering 360°replaced with 4K resolution 30Hz refresh rate.

Table 2.1.1-1: Requirements for real time video uploading for network-assisted AI inference services

| # Configurations | # I | # II | # III |
| --- | --- | --- | --- |
| # 6 Cameras [NOTE 1] | # 1080p * 6 | # 1080p * 4 + 4K * 2 | # 1080p * 2 + 4K * 4 |
| # UL data rate (Mbps) [NOTE 2] | # 20 | # 60 | # 100 |
| # RAN Latency & Reliability [NOTE 3] | # 15ms & 99% |  |  |
| # NOTE 1: 6 RGB cameras are onboard the “Figure 02” humanoid robot [2]. # NOTE 2: Data Rate = Video resolution*(Bits per color*3) *Refresh rate/Compression ratio. Compression ratio of 240 and 8 bits per color is assumed, thus data rate is around 3 Mbps for 1080p 15Hz, and around 24Mbps for 4K 30Hz. Compression ratio, bits per color and refresh rate for 1080p refers to the similar cases for real-time video uploading of a vehicle as per CCSA YD/T 4778-2024 [3]. # NOTE 3: Value of RAN latency and reliability are reused from use case on video uploading in AR e as defined in TR 38.838[4] can be reused. |  |  |  |

Observation 1: For real-time video uploading for network-assisted AI inference (e.g. for humanoid intelligent robot), up to 100Mbps UL data rate, 15ms RAN UL latency and reliability of 99% will be required.

Immersive XR Gaming

Immersive gaming as defined in section 9.1 of TR 22.870, captures 8K resolution display. However, For XR display devices with angular resolution below around 60 PPD (Pixels Per Degree), a so-called “Screen Door Effect” is expected, whereas users can perceive individual display pixels (i.e. pixel boundaries), creating a mesh-like visual obstruction similar to viewing through a screen door [5].

This optical distortion phenomenon degrades immersive video experiences. To provide users with an ultimate experience, a horizontal Field Of View (FOV) of 210 degrees and a vertical FOV of over 100 degrees are required [6]. Therefore, assuming 60 PPD as threshold, the horizontal resolution should include at least 210 × 60 = 12600 pixels, and the vertical resolution should include at least 100 × 60 = 6000 pixels. This resolution already exceeds that of 12K, and thus preferably, a 16K resolution is desired to meet the pixel density requirements of the human FOV with 60 PPD or higher to ensure seamless immersive video experiences.

The requirement of this use case is summarized in Table 2.1- 2.

Table 2.1.2- 1: Requirements for immersive XR gaming

| DL data rate[NOTE 1] | Resolution | Refresh rate | RAN Latency & Reliability [NOTE 2] |
| --- | --- | --- | --- |
| 100Mbps | 8K (7680×4320） | 90 | 10ms&99% |
| 500Mbps | 16K (15360×8640) | 120 |  |
| NOTE 1: Data Rate = Resolution × Bit Per Pixel × Refresh Rate / Compression Ratio, where compression ratio of 400 and 12 bits per pixel (YUV4:1:1 compression encoding) are considered [8][9]. NOTE 2: As per TR 38.838[4], a minimum RAN latency of 10ms is considered for DL cloud gaming use case, and reliability of 99%. |  |  |  |

Observation 2: For immersive XR gaming, up to 500Mbps DL data rate, 10ms RAN latency, and reliability of 99% will be required.

Operator-assisted automated driving

Driverless taxi represents an emerging business opportunity globally, with projected market penetration exceeding 30% in China by 2030 [7]. Operation of driverless taxi requires network connection, including real-time uploading of external 360° video surveillance of the driverless taxi, for taxi operators and local authorities to monitor and to takeover (by remote safety operator) if needed.

With emerging AI technology evolution, it can be expected that an AI Agent replaces human to monitor the driving status of the driverless Robo taxi, and will assist safe and efficient automated driving on a 24/7 basis, including remote takeover whenever needed. The requirements for this use case are summarized in Table 2.1- 3 with consideration of 3 different scenarios, where different quality of video streams are assumed: I) minimum requirements per CCSA YD/T 4778-2024 [8][9]; II) an upgrade of I with front view camera replaced with 4K resolution and 30Hz refresh rate; III) further upgrade of II with additional back/left/right view cameras replaced with 4K resolution and 30Hz refresh rate.

Table 2.1.3-1: Requirements for real-time video uploading to operator-assisted automated driving

| # Scenario | # I | # II | # III |
| --- | --- | --- | --- |
| # 6 Cameras [NOTE 1] | # 1080p * 6 | # 1080p * 5 + 4K * 1 | # 1080p * 2 + 4K * 4 |
| # UL Data rate (Mbps) [NOTE 1] | # 20 | # 40 | # 100 |
| # RAN Latency & reliability [NOTE 2] | # 15ms and 99% |  |  |
| # NOTE 1: - According to Chinese standard YD/T 4778-2024, 6 video streams with each at least 200kbps frame data rate and 15Hz refresh rate, where compression ratio of 240 and 8 bits per color is for an 1080p RGB camera; UL+DL E2E RTT <=70 or 120ms depending on operational area. - Data Rate = Video resolution*(Bits per color*3) *Refresh rate/Compression ratio. For an improved video quality scenario, resolution of 4K with 30Hz refresh rate is assumed under same compression ratio of 240 and 8 bits per color, thus 3 Mbps is for 1080p 15Hz, and 24Mbps for 4K 30Hz. # NOTE 2: - Value of RAN latency and reliability are reused from use case on video uploading in AR e as defined in TR 38.838 can be reused. |  |  |  |

Observation 3: For real-time video uploading to operator assisted automated driving up to 100Mbps UL data rate, 15 ms RAN UL latency, and reliability of 99% will be required.

Requirement summary for different immersive communication services

In summary, different immersive communication services require different composite requirements in terms of different combination of service data rates and latency for DL and UL, respectively. Some representative cases can be defined for evaluation as given in Table 2.1.4-1

Table 2.1.4-1: Example cases of composite requirement with KPIs

| Examples of composite requirement | Service data rate (Mbits/s) | Latency (ms) | Reliability |
| --- | --- | --- | --- |
| Composite Requirement for Immersive Services (UL) |  |  |  |
| Case 1 | 20Mbps | 15ms | [99%] |
| Case 2 | 100Mbps | 15ms | [99%] |
| Composite Requirement for Immersive Services (DL) |  |  |  |
| Case 3 | 100Mbps | 10ms | [99%] |
| Case 4 | 500Mbps | 10ms | [99%] |

Proposal 2: 6G RAN needs to support different cases of composite requirements with different combinations of data rate, RAN latency and reliability, as in Table 2.1.4-1.

From an end user perspective, smooth user experience means the end-to-end round trip time (RTT) of an interactive service is within the human reaction time. Some statistics of human reaction time distribution is shown in Figure 2.1.4-1. The values are from the Reaction Time Statistics [10], which are derived from over 81 million tests by Human Benchmark, and it shows that the 90% human reaction time falls between 100ms and 300ms. So the target E2E RTT could mainly consider 100~300ms.

The E2E RTT generally consists of two components, i.e. RTT inside 3GPP network and outside 3GPP network, where the RTT inside 3GPP network can be further decoupled as air interface latency and transport network latency, as shown in Figure 2.1.4-2. With some knowledge of the RTT outside 3GPP network as prior, the delay budget of air interface and transport can be jointly considered to meet the overall E2E RTT. By doing so, the network can have room to adjust the latency budget between radio interface and transport layer, according to different situations and do not require to always schedule packets within the most stringent latency requirement.

Figure 2.1.4-1 Decomposition of E2E RTT

In addition to the latency, the reliability itself may also need further study whether the average value is sufficient. For example, according to TR 22.870, the maximum duration of consecutive voice packet loss can impact on the quality of the voice quality; another example according to TR 22.870 is to consider the max allowed packet error rate per frame as there are more new codecs occurring, e. g. Error-tolerant codec.

Proposal 3: 6G design should consider dynamic latency budget among different layers of the network to jointly meet the requirement of E2E RTT.

AI service

As described in ITU-R recommendations under in [xx], AI is considered as a foundational pillar for 6G systems capable of providing services.  According to the 3GPP TR 22.870, in the context of AI-driven 6G systems, it is anticipated that two concepts will be important for shaping the future systems: AI for 6G System and 6G System for AI.

- AI for 6G System refers to the use of AI capabilities to improve system performance.

- 6G System for AI focuses on how the system supports and enables AI applications by leveraging 6G system functionalities.

AI for 6G RAN is mainly to improve the system performance which are more operational requirements, and RAN for AI may bring new opportunities for operators to provide new AI services, therefore in this section, RAN for AI is mainly discussed.

Proposal 4: Net4AI shall be considered in 6G RAN design to better serve various AI services.

6G RAN for AI

In 3GPP 6G use case TR 22.870,

- the AI use case of collaborative AI Agents in section 6.8 is addressed for the new 6G service, where AI Agents can perform tasks for or represent e.g. devices, persons, drones, or cars. These AI Agents may be either implemented in a UE or in the network. In this use case, the hosting of large amounts of AI agent applications shall be supported. Besides, AI-agents communication in section 6.6 and 6G system assisted AI agent service in section 6.7 would be considered for 6G as well.

- the ISAC use case of collaborative robots using digital twinning in section 7.13 is also addressed for the new 6G service, where the Digital Twin (DT) application takes full control of the actions in the physical world without human interactions and it is assumed that all robots act as a traditional User Equipment (UE) and they can reach the DT over the 6G network. In this use case, communication and sensing capabilities should be supported.

Subsequently, when capturing the above new applications in the whole 6G system as below figure 2.2.1-1, considering Digital twin, AI Agent, multiple devices, 6G RAN need to process communication, sensing, and AI capabilities simultaneously and enable deep integration of communication, sensing, and AI related data and capabilities.

* Chain-of-Action (CoA) Data: Data capturing the sequence of perceptions, intentions, actions, and outcomes, crucial for learning and adaptation.

* Distributed AI Agents are collaborative and exchanged data in the network.

Figure 2.2.1-1	Example of enabling AI Agent and collaborative robots using DT applications in 6G

In 5G, various types of non-personal data are generated, which can be categorized as follows:

- RAN channel measurement data, originating from either the base station or the user equipment (UE), based on the measurement of reference signals. These include measurements based on uplink SRS (Sounding Reference Signal), downlink CSI-RS (Channel State Information Reference Signal), DMRS (Demodulation Reference Signal), and SSB (Synchronization Signal Block).

- Data used by SON/MDT (Self-Organizing Networks), which supports RAN automation functionalities such as mobility optimization, load balancing, and anomaly detection.

- AI-related data within the RAN domain, including both training data used for model development, inference data generated by AI models, and performance monitoring data of AI models deployed at UE or network side.

- Sensing data

Currently, these various types of data are collected, managed, and utilized independently using different mechanism. However in reality the data consumer may require multiple types of data, e.g. in XR services in [11], the XR application may require measurement data from sensing and AI. As the current data is reported and managed by independent mechanisms, these data cannot be integrated together. These data might be overlapping, duplicating, using mixed classification etc., which are not efficient to be utilized by network, and also difficult to be provided as a service to the data consumers.

Looking ahead to future 6G systems, there is a clear opportunity to establish an end-to-end, unified data management framework, to enable joint collection, management and distribution of data within 3GPP system, as below example figure 2.2.1-2.

Figure 2.2.1-2	Example of data management of net4AI

This unified approach could support two major targets:

- RAN specific enhancements: Leveraging the integrated data to improve network operation, such as real-time optimization of RAN performance.

- Enable data as a service: E2E transmission scheme and Utilization of diverse data types to support new service and applications. For example, emerging new services such as Agent-to-Agent communication, should be further studied the new traffic models and their impact on RAN design.

Based on the above example and analysis, we propose

Proposal 5: An E2E unified framework between RAN and Core network for data collection, management and monitoring should be supported, considering various types of data, e.g. data related to measurements, AI and sensing etc.

Sensing services

In recommendation ITU-R M.2160-0, integrated sensing and communication (ISAC) is defined as one of the key usage scenarios of IMT-2030. Its typical use cases include navigation, activity detection and movement tracking, environmental monitoring, and provision of sensing data/information of the surroundings for AI, XR, and digital twin applications [12].

In 6G, the following services should be prioritized for study and meeting their requirements, considering the potential for huge markets.

Digital twin services enabled by Environment Object Reconstruction

According to Recommendation ITU-R M.2160, IMT-2030 is expected to be used to replicate the physical world into a digital virtual world as precise real-time representations or digital twins. It is also addressed in Recommendation ITU-T Y.4600 [13], a digital twin is a digital representation of an object of interest and may require different capabilities according to the specific domain of application, such as synchronization between a physical thing and its digital representation, and real-time support.

In the current TR 22.870, several use cases have mentioned digital twin. As the section 7.5 of use case in on environment object reconstruction, 6G RAN sensing is effective to provide a low-cost environmental information to construct digital twin, for enabling smart transportation, smart city and smart home. Moreover, 6G system could obtain the sensing measurement data by RAN sensing, and then utilize AI capability to generate local digital twin including spatial information. Eventually, the global digital twin can be constructed by distributed local digital twins, and then it is to as a real-time network services 3rd party.

- For smart city, the digital twin can provide real-time 3D virtualization of buildings, roads, and other infrastructure elements, which facilitates risk prediction like building collapse and city management like construction project monitoring.

- For intelligent transportation system, digital twin can reconstruct digital counterparts of nearby buildings and enable accurate tracking for traffic participants including vehicles and pedestrians in both line-of-sight and non-line-of-sight areas, which is beneficial for improving the safety and efficiency for autonomous driving and traffic control.

Figure 2.3.1-1	The process from ISAC RF sensing Point-Cloud to Digital Twin

Compared with traditional detection method, 6G RAN can explore the advantages of network and device collaboration to sense targets from different angles, avoiding blind spots or dynamic obstructions in a single sensing link. Also, to construct digital twin and support the other services based on digital twin, 6G needs the sensing capability of [sub meter to meter-level] localization accuracy, [sub meter-level] sensing spatial resolution, sensing detectability ([99-95%] detection/ [1-5%] false alarm probability). Besides, measure different type targets including static and dynamic targets, is one important capability.

These measurement data from wireless sensing would be processed or managed in 6G RAN as below.

Proposal 6:  6G RAN design should support digital twin services with the following capabilities derived from ISAC.

- [sub meter to meter-level] localization accuracy

- [sub meter-level] sensing resolution

- [99-95%] detection probability / [1-5%] false alarm probability

- Measure and [construct] different type targets, including static and dynamic targets

- Sensing measurement data management

    - Road digitalization

Road digitalization, as defined in section 7.6 of TR 22.870, demands to detect and track mobile targets on road with specific localization accuracy, detection probability, and false alarm probability. According to [14], the required localization accuracy is specified as 0.5 meters for vehicles and 1 meter for pedestrians, with detection probability above 95%-99% and false alarm probability below 2-5%. Besides, 6G shall additionally support “sensing target density”, the upper limit of sensing object detection/tracking required by a sensing service, as a new performance metric. In [2], the upper limit of sensing target density for road digitalization is 750 vehicles per 1000 m x 24 m.

Figure 2.3.2-1	Illustration of Road digitalization from TR 22.870

Observation 4: The road digitalization service requires to detect and track mobile targets with at least 1 m localization accuracy.

Proposal 7: 6G RAN needs to support Road digitalization with at least 1m localization accuracy.

    - Health monitoring services

With the aging problem becoming more and more serious, and people paying more and more attention to health issues, smart health monitoring will be a very promising industry. Sensing for health monitoring at home has been proposed since 3GPP Rel-19 in TS 22.137 [14]. It is required to not only detect and track the position of human at home, but also provide some vital signal analysis, e.g., respiration rate, heartbeat rate, etc, based on RAN sensing. Besides, 6G network can also detect some abnormal event (e.g. falling down) based on sensing and then send the corresponding notification to the hospital or his family members. According to TS 22.137[14], the required localization accuracy for indoor human tracking is defined as 1 meter with detection probability above 98% and false alarm rate below 2%. Additionally, the required respiration rate accuracy is defined as 2 times/min.

Proposal 8: 6G RAN needs to support indoor health monitoring service with the requirements of at least [1]m localization accuracy and [2] times/min respiration accuracy.

NTN services

NTN has been specified in 5G. As the complementary services to terrestrial network, NTN is also important to be supported in 6G to complement the coverage and related services where terrestrial network is not available.

In the updated RAN study item, there is a following objective agreed:

  - Aim at a harmonized 6G Radio design for TN and NTN, including their integration

The harmonized radio design would need very clear design targets for NTN, e.g. the coverage requirement for LEO and GEO, the data rate to be supported, the interworking requirement etc. All these aspects will impact the WG level study on specific technical directions.

Proposal 9: 6G RAN should support NTN services. Specific design targets on coverage, data rate etc. needs to be clearly defined.

6G Voice services

As the telecommunications industry begins to shape the standards and architecture for 6G networks, one critical requirement deserves particular attention: the need for native voice support from day one. While data/sensing/AI-centric services and advanced applications continue to drive network evolution, voice communication remains a fundamental service that should be supported in 6G Day 1.

Proposal 10:  6G RAN should support voice service.

Summary

We have discussed the new and extended services and the following are proposals for agreements:

For immersive communication services:

Proposal 1:  6G RAN design should support high capacity with the required high data rate, low latency and high reliability.

Observation 1: For real-time video uploading for network-assisted AI inference (e.g. for humanoid intelligent robot), up to 100Mbps UL data rate, 15ms RAN UL latency and reliability of 99% will be required.

Observation 2: For immersive XR gaming, up to 500Mbps DL data rate, 10ms RAN latency, and reliability of 99% will be required.

Observation 3: For real-time video uploading to operator assisted automated driving up to 100Mbps UL data rate, 15 ms RAN UL latency, and reliability of 99% will be required.

Proposal 2: 6G RAN needs to support different cases of composite requirements with different combinations of data rate, RAN latency and reliability, as in Table 2.1.4-1.

Proposal 3: 6G design should consider dynamic latency budget among different layers of the network to jointly meet the requirement of E2E RTT.

For net4AI services:

Proposal 4: Net4AI shall be considered in 6G RAN design to better serve various AI services.

Proposal 5: An E2E unified framework between RAN and Core network for data collection, management and monitoring should be supported, considering various types of data, e.g. data related to measurements, AI and sensing etc.

For sensing provided services:

Proposal 6:  6G RAN design should support digital twin services with the following capabilities derived from ISAC.

- [sub meter to meter-level] localization accuracy

- [sub meter-level] sensing resolution

- [99-95%] detection probability / [1-5%] false alarm probability

- Measure and [construct] different type targets, including static and dynamic targets

- Sensing measurement data management

Observation 4: The road digitalization service requires to detect and track mobile targets with at least 1 m localization accuracy.

Proposal 7: 6G RAN needs to support Road digitalization with at least 1m localization accuracy.

Proposal 8: 6G RAN needs to support indoor health monitoring service with the requirements of at least [1]m localization accuracy and [2] times/min respiration accuracy.

For other services:

Proposal 9: 6G RAN should support NTN services. Specific design targets on coverage, data rate etc. needs to be clearly defined.

Proposal 9:  6G RAN should support voice service.

References

- RP-250810, “New SID: Study on 6G Scenarios and requirements”, RAN#107, March 2025.

- 3GPP TR 22.870: “Study on 6G Use Cases and Service Requirements”.

- S1-252883, New use case on network-assisted video-based AI inference task offloading for mobile embodied AI, 3GPP SA1#110, May, 2025.

- 3GPP TR 38.838, “Study on XR (Extended Reality) evaluations for NR,”

- Tan G, Lee Y H, Zhan T, et al. Foveated imaging for near-eye displays[J]. Optics express, 2018, 26(19): 25076-25085.

- White Paper for 5G Cloud VR Service Experience Standards, 2019.06

- https://pdf.dfcfw.com/pdf/H301_AP202409121639855666_1.pdf

- https://www.chinesestandard.net/PDF/English.aspx/YDT4778-2024.

- https://std.samr.gov.cn/hb/search/stdHBDetailed?id=2001448F0D0D32F5E06397BE0A0AAD9C

- https://humanbenchmark.com/tests/reactiontime

- S1-252323, Update of use case on low-altitude UAV supervision, 3GPP SA1#110, May, 2025.

- Recommendation ITU-R M.2160-0: "Framework and overall objectives of the future development of IMT for 2030 and beyond".

- Recommendation ITU-T Y.4600: “Requirements and capabilities of a digital twin system for smart cities”

- 3GPP TS 22.137: "Service requirements for Integrated Sensing and Communication; Stage 1".

- RP-232669, Revised WID: NR NTN (Non-Terrestrial Networks) enhancements.