3GPP TSG RAN#108	RP-251369

Prague, CZ, 9th -13th June, 2025

Agenda Item:	16.1

Source:	Huawei, HiSilicon

Title:	Consideration on migration and architecture options for 6G

Document for:	Discussion, Decision

1	Introduction

When it comes to the discussion on a new generation of wireless system, the migration requirements need to be addressed. As happened in Rel-15 for 5G, both standalone (SA) and non-standalone (NSA) options have been specified. For 4G-5G NSA, multiple options have been specified considering different anchor nodes and different bearer types.

In this contribution, we would like to provide our thinking on the migration path from 5G towards 6G.

2	Discussion

2.1	Migration from 4G to 5G

During 5G discussion, standalone, non-standalone, dynamic spectrum sharing (DSS) and carrier aggregation (CA) have been supported since Day 1. Although multiple NSA options are specified, only Option 3 has been commercialized in some countries and regions.

The below deployment options for NSA are supported by 5G specifications [1]:

- Option 3 series (EN-DC): LTE eNB is connected to EPC with non-standalone NR en-gNB

- Option 4 series (NE-DC): NR gNB is connected to 5GC with non-standalone E-UTRA ng-eNB

- Option 7 series (NGEN-DC): LTE ng-eNB is connected to 5GC with non-standalone NR gNB

Figure 1: Specified 5G SA and NSA architectures

Observation 1: With respect to the migration from 4G to 5G, multiple NSA options have been specified but only one option (EN-DC) is commercialized.

2.2	Consideration on 6G deployment

2.2.1    6G Standalone deployment

As a new generation, there is no doubt standalone deployment shall be supported in 6G Day 1. Both 6G Core network and 6G RAN are assumed to be non-backward compatible for legacy 5G UEs, to fully unleash the potential system performance, exploring new services and reducing the cost.

The 5G-6G multi-RAT spectrum sharing (MRSS) shall also be supported in 6G Day 1, considering that the existing spectrum utilized by 5G may also be the frequency band for 6G deployment.

Figure 2: legacy bands and potential new bands for 6G

With standalone deployment, users can benefit from new capabilities and new services supported by 6G system. With MRSS deployment, the spectrum can be multiplexed between 5G and 6G, which is useful for operators to dynamically enhance utilization of spectrum. Therefore both standalone and MRSS are seen essential to be supported in 6G Day 1.

On the other hand, MRSS only provides efficient spectrum utilization on a single carrier without data aggregation gains. During the migration phase from 5G towards 6G, users are expected to have superior user experience from 6G compared to 5G (considering 5G-5G CA penetration is increasing in the real market), and therefore 6G-6G CA needs to be supported as well to achieve the data aggregation benefits since 6G Day 1.

- Proposal 1: 6G standalone, 5G-6G MRSS and 6G-6G CA shall be supported in 6G Day 1.

2.2.2	Consideration on additional migration paths

In addition to the above, whether to have extra migration options seems no common understanding. Some companies are proposing 5G-6G NSA aggregation options at RAN-level, some others are proposing dual stack/dual steer aggregation options at CN-level.

From the sourcing company’s view, such migration options are mainly to consider operators investment pace in different regions, which is more a TSG-level discussion. However, without knowing which potential areas will have new design/enhancements for both 6G RAN and 6G Core network, and how the multi-carrier operation will be designed for 6G, it will be difficult for operators to estimate the cost vs. gains to justify the investment pace.

If the study starts discussing additional migration options at the very beginning, it is hard to exclude any option and a lot of time will be spent in WGs to have detailed solution study. To avoid such a situation, it is seen more suitable to start discussing additional migration requirements and options in the second half of the WG study, when the potential new design and enhancements of 6G system become clearer and operators have a better perception of potential gains from the new system.

- Proposal 2: Start discussing the need of additional migration requirements and candidate options at TSG plenary no earlier than June 2026.

If there is still the need to consider additional migration options at that time, it is suggested to minimize the number of extra options to avoid fragmentation and excessive specification efforts. The technical study aspects should also be identified at RAN plenary, to avoid occupying too much time of technical study in WGs. Impacted WGs, e.g. RAN2, SA2 can then have a short study, e.g. 6 months for 3 meetings, and provide essential technical analysis inputs to TSG for a final decision. The decision can be made at the end of the study, which can be used as input for subsequent WI scoping.

- Proposal 2a: If there is a need for additional migration options, the number of options shall be minimized and the potential aspects for WG study shall be identified at TSG level.

- Proposal 2b: the impacted WGs will be tasked by TSG to perform a subsequent short study to analyse the identified aspects and provide technical analysis inputs to TSG which eventually makes a final decision at the end of the WG study.

3	Proposal

Based on the discussion in the previous sections we propose the following:

Observation 1: With respect to the migration from 4G to 5G, multiple NSA options have been specified but only one option (EN-DC) is commercialized.

- Proposal 1: 6G standalone, 5G-6G MRSS and 6G-6G CA shall be supported in 6G Day 1.

- Proposal 2: Start discussing the need of additional migration requirements and candidate options at TSG plenary no earlier than June 2026.

- Proposal 2a: If there is a need for additional migration options, the number of options shall be minimized and the potential aspects for WG study shall be identified at TSG level.

- Proposal 2b: the impacted WGs will be tasked by TSG to perform a subsequent short study to analyse the identified aspects and provide technical analysis inputs to TSG which eventually makes a final decision at the end of the WG study.

4	References

TS 37.340, Multi-connectivity Overall Description