3GPP TSG-RAN Meeting #108			RP-251416

Prague, Czech, June 9-13, 2025

Agenda item:	16.1

Title:	Discussions on minimum technical performance requirements for IMT-2030 radio interface(s)

Source:	NTT DOCOMO, INC.

Document for:	Discussion

# 1	Introduction

For submission to ITU-R WP 5D from TSG RAN#108 on target values of IMT-2030 technical performance requirements (TPRs), the following workplan towards RAN#108 was provided at RAN#107 meeting [1] and the conference call (CC) #1 was held.

Workplan in TSG RAN for 2Q25 To complete the work on TPRs in RAN#108, it is proposed that the work will progress between RAN#107 and RAN#108 by: - E-mail to circulate input documents and summaries of proposals by using RAN reflector - Two conference calls to consolidate the proposals aiming to facilitate agreement in RAN#108 - CC#1 from 13:00 to 15:00 CET on April 29, 2025 – deadline for contributions on April 24 - CC#2 from 13:00 to 15:00 CET on June 4, 2025

Based on the agreements in CC#1 [2], this contribution provides our updated views on target values for IMT-2030 TPRs. Changes from our previous contribution are highlighted in cyan.

# 2	Discussions

## 2.1	Target values for legacy TPRs from IMT-2020

Table 1 summarizes our initial views on target values for IMT-2030 minimum TPRs inherited from IMT-2020. According to [3], IMT-2020 self-evaluation results of NR significantly exceeded ITU-R requirements for some of TPRs. On the other hand, some of such high performances were achieved by extreme/unrealistic assumptions. Therefore, for legacy TPRs from IMT-2020, the self-evaluation results in [3] as well as assumptions should be carefully reviewed, and target values for IMT-2030 should be derived so that IMT-2030 TPRs show attractive improvements from IMT-2020 with reasonable assumptions.

Table 1: Proposed target values for IMT-2030 minimum TPRs inherited from IMT-2020

| TPR | Ref: IMT-2020 requirement | Proposed target value for IMT-2030 | Comments |
| --- | --- | --- | --- |
| Peak data rate | DL: 20 Gbps UL: 10 Gbps | 1.5x of IMT-2020 | Alternative 1: Not to define a requirement, e.g. Leave the value for peak data rate for the technology proponent to provide Alternative 2: Define a requirement with reasonable value DCM view: It is preferable to define a requirement with reasonable target value. 1.5x is achieved assuming 1024QAM / 256QAM, 8 layers for DL / UL and bandwidth of 500 MHz. |
| Peak spectral efficiency | DL: 30 bps/Hz UL: 15 bps/Hz | 2x of IMT-2020 | DL: [1.5-2.5x] of IMT-2020 UL: [1.5-2.5x] of IMT-2020 DCM view: Assuming 1024QAM / 256QAM and 8 layers for DL / UL |
| User experienced data rate | DL: 100 Mbps UL: 50 Mbps | DL: 1.5x of IMT-2020 UL: [1.5x] of IMT-2020 | For UL: - RAN#107 agreement: For user experienced data rate, further discuss applicable deployment scenarios, and revisit the evaluation methodology before setting values for UL - Proposal 1: Use system level simulation for the evaluation of UL user experienced data rate with no bandwidth scaling taking UE maximum output power into account. DCM view: Assuming up to 256QAM, 4 layers for DL / UL and bandwidth of 400 MHz |
| 5th percentile user spectral efficiency | Indoor Hotspot – eMBB DL: 0.3 bps/Hz UL: 0.21 bps/Hz Dense Urban – eMBB DL: 0.225 bps/Hz UL: 0.15 bps/Hz Rural – eMBB DL: 0.12 bps/Hz UL: 0.045 bps/Hz | 3x of IMT-2020 | Companies are encouraged to provide antenna assumption for different frequency ranges together with the requirements, e.g. 4GHz, 7GHz, 30GHz Applicable deployment scenarios should also be considered. DCM view: Assuming up to 256QAM and 4 layers for DL / UL --------- Indoor Hotspot For 7 GHz, - For 12TRxP TRxP: (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;4,8) UE: (M,N,P,Mg,Ng; Mp,Np) = (1,2,2,1,1; 1,2) For 30 GHz, - For 12TRxP TRxP: (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;2,16) UE: (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 1,2) --------- Dense Urban For 4 GHz, TRxP: (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;4,8) UE: (M,N,P,Mg,Ng; Mp,Np) = (1,2,2,1,1; 1,2) For 7 GHz, TRxP: (M,N,P,Mg,Ng; Mp,Np) = (16,16,2,1,1;4,8) UE: (M,N,P,Mg,Ng; Mp,Np) = (1,2,2,1,1; 1,2) --------- Rural For 700 MHz, TRxP: (M,N,P,Mg,Ng; Mp,Np) = (4,8,2,1,1;1,8) UE: (M,N,P,Mg,Ng; Mp,Np) = (1,2,2,1,1; 1,2) For 4 GHz, TRxP: (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;1,16) UE: (M,N,P,Mg,Ng; Mp,Np) = (1,2,2,1,1; 1,2) |
| Average spectral efficiency | Indoor Hotspot – eMBB DL: 9 bps/Hz/TRxP UL: 6.75 bps/Hz/TRxP Dense Urban – eMBB DL: 7.8 bps/Hz/TRxP UL: 5.4 bps/Hz/TRxP Rural – eMBB DL: 3.3 bps/Hz/TRxP UL: 1.6 bps/Hz/TRxP | 1.5x of IMT-2020 | Assuming up to 256QAM and 4 layers for DL / UL |
| Area traffic capacity | 10 Mbps/m2 @ Indoor Hotspot – eMBB | 1.5x of IMT-2020 @ Indoor Hotspot – IC 0.78 Mbps/m2 @ Dense Urban – IC | Assuming up to 256QAM and 4 layers For Indoor Hotspot – IC, 12 TRxPs and bandwidth of 400 MHz are assumed For IMT-2030, Area traffic capacity should also be evaluated in Dense Urban – IC test environment. Target value is calculated based on the achievable maximum average spectral efficiency in [3] assuming bandwidth of 400 MHz and ISD = 200 m. |
| Latency | User plane: eMBB: 4 ms URLLC: 1 ms Control plane: 20 ms | User plane: IC: 2 ms HRLLC: 0.5 ms Control plane: 10 ms | For user plane, SCSs of 15 / 30 kHz for IC and 60 / 120 kHz for HRLLC are assumed For control plane, enhanced procedure may be required (e.g., 2-step RACH) |
| Connection density | 106 devices/m2 | 1~10x of IMT-2020 | Assuming bandwidth of 180 kHz, SCS of 15 kHz and packet size of 32 bytes within packet delay of 10 s Assumption of bandwidth should be discussed |
| Energy efficiency | - | TBD |  |
| Reliability | 1-10-5 for 32 bytes in 1 ms | 1-10-7 | Assuming packet size of 32 bytes, required maximum time of 1 ms and SU-MIMO with rank 1 Assumptions of packet size and required maximum time should be discussed |
| Mobility | Indoor Hotspot – eMBB 1.5 bps/Hz @ 10 km/h Dense Urban – eMBB 1.12 bps/Hz @ 30 km/h Rural – eMBB 0.8 bps/Hz @ 120 km/h 0.45 bps/Hz @ 500 km/h | Mobility class: Same as IMT-2020 Spectral efficiency: 2x of IMT-2020 | Assuming up to 256QAM and 2 layers |
| Mobility interruption time | 0 ms | Same as IMT-2020 |  |
| Bandwidth | At least 100 MHz | At least 400 MHz | RAN#107 agreement: set a target value of 400MHz (same definition as IMT-2020) (same definition as IMT-2020) Agreed in RAN#107 |

Proposal 1: To adopt the target values in Table 1 for legacy TPRs from IMT-2020.

For 5th percentile user spectral efficiency, the following was agreed in CC#1.

Companies are encouraged to provide antenna assumption for different frequency ranges together with the requirements, e.g. 4GHz, 7GHz, 30GHz Applicable deployment scenarios should also be considered.

Our views on antenna assumptions for different frequency ranges are described in “Comments” of Table 1. The antenna configuration is indicated as (M, N, P, Mg, Ng; Mp, Np), where M and N are the number of vertical, horizontal antenna elements within a panel, P is number of polarizations, Mg is the number of panels in a column, Ng is the number of panels in row; and Mp and Np are the number of vertical, horizontal TXRUs within a panel and polarization. For Indoor Hotspot and Dense Urban, by adopting 7 GHz band, the number of antenna elements can be increased compared to 4 GHz band and therefore, it can be expected to achieve around 3 times higher 5th percentile user spectral efficiency than IMT-2020. For Rural, according to [3], more than 3 times higher 5th percentile user spectral efficiency than IMT-2020 requirement can be achieved by using antenna configurations described in Table 1 under 700 MHz and 4 GHz bands.

For energy efficiency, the evaluation in IMT-2020 was conducted based on the sleep ratio and sleep duration for network and Device. Network and Device can achieve high sleep ratio and long sleep duration in unloaded case.

At the latest ITU-R WP 5D meeting, energy efficiency can be categorised into network energy efficiency and device energy efficiency, and preliminary drafting on the associated definition was conducted as follows [4]:

The requirement is the relative energy savings between the selected load case(s) and a reference case. Proponents should report the power model used for the evaluation, as well as impacts on communication performance associated with the evaluated power saving technologies, see [IMT-2030 EVAL] for details.

According to [4], fully loaded IMT-2030 case is assumed as the reference case to evaluate the relative energy savings. However, in that case, the energy efficiency improvements in IMT-2030 from IMT-2020 may not be clear. Therefore, for the evaluation of energy efficiency in 3GPP, it should be discussed how to clarify the improvement from IMT-2020 to IMT-2030.

Proposal 2: For the evaluation of energy efficiency in 3GPP, it should be discussed how to clarify the improvement from IMT-2020 to IMT-2030.

## 2.2	New TPRs of IMT-2030

### 2.2.1	Positioning

ITU-R WP 5D is discussing positioning accuracy as the positioning-related TPR, which is defined as the difference between the calculated horizontal/vertical position and the actual horizontal/vertical position of a UE/device.

In [5], the following requirement for positioning accuracy is defined in NR Release 16:

-	Horizontal positioning error < 3m for 80% of UEs in indoor deployment scenarios

-	Vertical positioning error < 3m for 80% of UEs in indoor deployment scenarios

-	Horizontal positioning error < 10m for 80% of UEs in outdoor deployments scenarios

-	Vertical positioning error < 3m for 80% of UEs in outdoor deployment scenarios

According to the evaluation results in [5], it is possible to achieve higher horizontal accuracy compared to the above requirement. Therefore, for IMT-2030, target values of the positioning accuracy can be considered based on the evaluation results of horizontal accuracy in [5] with reasonable level of enhancement, and it should be discussed whether the evaluation of vertical accuracy is necessary or not. The target values for positioning related TPRs are proposed as shown in Table 2.

Table 2: Target values for positioning accuracy (for 80% of UEs/devices)

| Test environment | Horizontal accuracy | Vertical accuracy |
| --- | --- | --- |
| Indoor | < 1 m | [meter-level] |
| Outdoor | < 5 m | [meter-level] |

Proposal 3: To adopt the target values in Table 2 for positioning of IMT-2030 and discuss whether the evaluation of vertical accuracy is necessary or not.

### 2.2.2	Sensing-related capabilities

For sensing-related capabilities, in ITU-R WP 5D meeting, the following performance metrics/requirements were selected.

□	Detectability (Detection / False Alarm Probability) – Detection probability, PD, is the probability of correctly detecting the object status (e.g., presence, category, and state), and False alarm probability, PFA, is the associated probability that an object status is detected given that the object is not present.

□	Localization Accuracy – Localization accuracy refers to the closeness of the measured sensing result of position of the target object to its true position value and is the difference between the calculated location and the actual location of an object.

□	Velocity Accuracy – Velocity accuracy is defined as the difference between the estimated velocity and the actual velocity of the sensing target.

□	Sensing Resolution – Sensing resolution is defined as the minimum difference of the measured value (e.g, range, location) of two target objects, in order to distinguish these two target objects.

For consideration of sensing-related capabilities, sensing task, test environment, and target values of the above requirements should be discussed based on [6] as starting point. Table 4 summarizes the requirements indicated in [6].

Table 4: Target values of sensing-related capabilities

| Sensing task | Test environment | Detection probability [%] | False alarm probability [%] | Localization accuracy [m] | Velocity accuracy [m/s] | Sensing resolution [m] |
| --- | --- | --- | --- | --- | --- | --- |
| Object detection and tracking | Indoor Outdoor | 95~99.9 | 2~5 | 0.5~10 | 0.1~15 | 0.5~10 |
| Environment monitoring | TBD | 95~99.9 | 3 | Horizontal: 10 Vertical: 0.2 | - | - |
| Motion monitoring | TBD | 95 | 5 | 0.2 | 0.1 | 0.375 |

Proposal 4: For sensing-related capabilities, sensing task, test environment, and target values of the requirements should be discussed based on TS 22.137 as starting point.

### 2.2.3	AI-related capabilities

In RAN#107 meeting, it was agreed to define AI-related capabilities as qualitative TPR evaluated by inspection [7]. Preliminary drafting on the definition of AI-related capabilities was conducted in the latest ITU-R WP 5D meeting as follows [4]:

Applicable AI-related capabilities refer to the ability to provide certain functionalities throughout IMT-2030 to support AI enabled applications. The RIT should support AI-related capabilities, which refer to the ability for the RIT/SRIT to support mechanisms and/or signalling/ related to AI functionalities over the radio interface (between UE and Network). This should include support for one or more of the following: - Distributed data processing - Distributed learning - AI computing - AI model execution - AI model inference - or other AI capabilities of the RITs/SRITs (to be reported by the proponent)

For AI-related capabilities as qualitative TPR, its definition should be discussed based on the above preliminary draft as starting point.

Proposal 5: For AI-related capabilities as qualitative TPR, its definition should be discussed based on the preliminary draft in the latest ITU-R WP 5D working document as starting point.

### 2.2.4	Joint / Composite requirements

The latest ITU-R WP 5D meeting made the preliminary draft on the definition of joint / composite requirement on data rate, latency, reliability, and capacity. For consideration of its target values in Immersive Communication (IC) usage scenario, the evaluation results for XR application in [8] can be used as starting point of discussion. Our views on target values for joint / composite requirement are summarized in Table 6 and 7.

Table 6: DL target values for joint / composite requirements for IC usage scenario

| Test environment | Data rate (Mbps) | Latency (ms) | Reliability | Number of satisfied users (/Cell) |
| --- | --- | --- | --- | --- |
| Dense Urban-IC | 30 | 10 | 99% | 4~11 |
|  |  | 15 |  | 5~15 |
|  | 45 | 10 |  | 2~7 |
|  |  | 15 |  | 6 |
| Indoor Hotspot-IC | 30 | 10 | 99% | 4~9 |
|  |  | 15 |  | 8~11 |
|  | 45 | 10 |  | 4~6 |
|  | 60 | 10 |  | 2~4 |

Table 7: UL target values for joint / composite requirements for IC usage scenario

| Test environment | Data rate (Mbps) | Latency (ms) | Reliability | Number of satisfied users (/Cell) |
| --- | --- | --- | --- | --- |
| Dense Urban-IC | 0.2 | 10 | 99% | 20~240 |
|  | 10 | 15 |  | 5 |
|  |  | 30 |  | 2~9 |
|  | 20 | 30 |  | 3 |
| Indoor Hotspot-IC | 0.2 | 10 | 99% | 20~240 |
|  | 10 | 30 |  | 4~9 |

Proposal 6: To adopt the target values in Table 6 and 7 for joint / composite requirement in IC usage scenario.

### 2.2.5	Other items

For coverage and resilience, our views on the moderator proposals are listed as follows.

| Moderator proposals [2] | DCM’s view |
| --- | --- |
| Proposal 2-5: For coverage, further discuss the following three options: - Option 1: define coverage as a quantitative TPR - Option 2: do not define coverage as a TPR, use link budget evaluation approach (same as IMT-2020) - Option 3: define extra connectivity as a qualitative TPR, which was revised from a part of coverage | Since coverage is operator’s network deployment and operation matter, we prefer Option 2 |
| Proposal 2-6: For resilience, further discuss the following two options: - Option 1: define resilience as a qualitative TPR - Option 2: do not define resilience as a TPR, these can be covered by submission template for IMT-2030. | Since resilience is operator’s network deployment and operation matter, we prefer Option 2 |

Proposal 7: It is preferable not to define coverage and resilience as TPRs.

# 3	Conclusion

In this contribution, the following proposals are provided.

Proposal 1: To adopt the target values in Table 1 for legacy TPRs from IMT-2020.

Proposal 2: For the evaluation of energy efficiency in 3GPP, it should be discussed how to clarify the improvement from IMT-2020 to IMT-2030.

Proposal 3: To adopt the target values in Table 2 for positioning of IMT-2030 and discuss whether the evaluation of vertical accuracy is necessary or not.

Proposal 4: For sensing-related capabilities, sensing task, test environment, and target values of the requirements should be discussed based on TS 22.137 as starting point.

Proposal 5: For AI-related capabilities as qualitative TPR, its definition should be discussed based on the preliminary draft in the latest ITU-R WP 5D working document as starting point.

Proposal 6: To adopt the target values in Table 6 and 7 for joint / composite requirement in IC usage scenario.

Proposal 7: It is preferable not to define coverage and resilience as TPRs.

# 4	References

[1] RP-250820, Direct Submission to ITU-R from TSG RAN#108 on IMT-2030 TPR values, CMCC, Verizon, NTT DOCOMO, Deutsche Telekom, Novamint

[2] RT-250024, Moderator's summary for IMT-2030 TPRs in CC#1, CMCC

[3] TR 37.910

[4] Annex 5.9 to Doc. 5D/563, Working document towards a preliminary draft new Report ITU-R M.[IMT-2030.TECH PERF REQ] – Minimum requirements related to technical performance for IMT-2030 radio interface(s), ITU-R WP 5D

[5] TR 38.855

[6] TS 22.137

[7] RP-250819, Moderator's summary for IMT-2030 TPRs, CMCC

[8] TR 38.838