3GPP TSG RAN#108	RP-251418

Prague, Czeck Republic, 9th -13th June 2025

Agenda Item:	16.2

Source:	NICT

Title:	NICT Views on Release 20 RAN WG SIs for 6G

Document for:	Discussion

1	Introduction

This document discusses new RAN WG SIs for 6G in Release 20.

This contribution provides a list of potential study items for 3GPP RAN to consider based on contributions of 3GPP 6G Workshop and NICT’s original thoughts.

We believe that it is important for 3GPP RAN to consider not only degree of performance improvement, but also possible complexities, required time slot(s), TCO, energy efficiency and overlaps of topics etc.

2	Candidate topics for Release 20 6G Study Item

2.1	A list of candidate topics for Release 20 6G Study Item

A list of candidate topics for Release 20 6G Study item for 3GPP RAN to evaluate is summarized in the table 1.

Table 1 A list of candidate topics for Release 20 6G Study Item with a priority

| Topics | Note |
| --- | --- |
| Channel modelling and numerology for cm-wave | Derive numerology to enforth FR3 and additional channel modelling if necessary. |
| 5G/6G spectrum sharing (MRSS) | Very important ensuring smooth migration from 5G to 6G |
| Spectrum sharing for FR3 (among TN-NTN, 3GPP NW-non 3GPP NW) | In FR3, coexistence with non-3GPP systems may be necessary. |
| Integrated communication and sensing | Enable new use cases using 3GPP radio resouces as well as for data collection for optimization of 3GPP NW operation. |
| Energy efficiency of NW and UE | For example, signal separation for idle and connected mode. Important for sustainable evolution. |
| Extreme massive MIMO | Data throughput is to be increased by increased received signal power and less interferece. |
| Cell-free massive MIMO (Distributed MIMO deployment) | Potential for unified quality within the coverage ranging from macro to pico cell deployment. |
| AI for NW | For efficient NW management/operation with simpler and lower cost and shorter time |
| Air interface design by AI | Potential for better signal constellation, less pilot signal. |
| NTN enhancement | Performance improvement is expected by coordinated resoure/beam/scheduling management and proper signaling. Can be in conjunction with NTN-TN unification. |
| NTN-TN unification | Potential to provide ubiquitous coverage without user awareness of the system being used. |
| PHY/MAC layer basic function for eMBB, NTN, ISAC and IoT unified design | PHY/MAC for various scenarios can have unified design for simplicity of the specification, while they can have individual adaptation for better performance. |
| Time synchronization and positioning | For more robust and better accuracy in various environments |

3GPP RAN to evaluate topics listed in the table 1 for 6G RAN study items.

3GPP RAN to evaluate topics for study items considering not only degree of performance improvement, but also possible complexities, time slot(s), TCO, energy efficiency and overlaps of topics, etc.

3	Proposals

- 3GPP RAN to evaluate topics listed in the table 1 for 6G RAN study items.

- 3GPP RAN to evaluate topics for study items considering not only degree of performance improvement, but also possible complexities, time slot(s), TCO, energy efficiency and overlaps of topics, etc.

4	References

6GWS-250123 NICT’s view on 6G – RAN Perspective, 3GPP 6G Workshop, March 10th-11th, 2025

6GWS-250244 Meeting Report for 3GPP 6G Workshop