3GPP TSG RAN Meeting #108							      	       RP-251436

Prague, Czech Republic, June 9-13, 2025

Source 	: CAICT

Title 	: Discussions on AI/ML for NR Air Interface enhancements

Agenda Item	: 15.1.1

Document for	: Discussion / Decision

- Introduction

According to the conclusion of last plenary, Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface are proposed by lots of companies to continue in 5G-A R20 [1]. Based on the conclusions from last WGs meetings, this paper provides some discussions on the justifications and objectives of AI/ML for NR air interface enhancements in R20.

- Discussions

2.1 Justifications

The application of AI/ML techniques to NR air interface has been further studied in FS_NR_AIML_air_Ph2[2], including further study on CSI compression (two-sided model) covering aspects on improved trade-off between performance and complexity/overhead and alleviating/resolving issues related to inter-vendor training collaborations.

During the study, it was observed that promising performance gains can be achieved for Case 0/2/3 with compromised complexity/overhead. For inter-vendor training collaborations, both Direction A and Direction C are further studied. RAN1 concludes that both Direction A and Direction C are feasible. For Direction A, RAN1 concludes that the feasible sub-options are sub-option 4-1 and sub-option 3a-1, and in case of sub-option 3a-1, with and without target CSI sharing from NW side [3].

Observation 1: According to FS_NR_AIML_air_Ph2 study outcome, AI/ML based CSI compression should be specified in Rel-20.

  - Objectives

The specification works for AI/ML based CSI compression include different objectives across different working groups. As the leading work group, RAN1 has finished the study works in June and RAN2/RAN4 will continue the study till September.

The following recommendations are achieved in RAN1 for WI:

Based on the study, for addressing inter-vendor collaboration complexity, RAN1 identifies the following specification impacts for supporting sub-option 3a-1 (including with target CSI and without target CSI), sub-option 4-1, and Direction C

- Reference model / structure specification (for sub-option 3a-1 and Direction C)

  - Note: The so called fully specified reference model discussed in RAN1 for Direction C is assumed to be equivalent to the to-be-specified model, if specified, for performance requirements in RAN4.

  - Note: The model structure(s) for the reference encoder in inter-vendor collaboration sub-option 3a-1 can be same as/equivalent to the structure of the RAN4 defined/specified (if defined/specified for testability) reference encoder.

    - To reduce the workload of the potential normative work scope, the model structure(s) for the reference encoder in inter-vendor collaboration sub-option 3a-1 is(are) same as/equivalent to the structure of the RAN4 defined/specified (if defined/specified for testability) reference encoder.

  - Note: RAN1 has studied and agreed on a scalable model structure for feasibility study during Rel-19 study, and considers that can serve as a starting point for the model structure specification for the potential normative work.

- Format and contents of parameter/dataset/information exchange: dataset (for 4-1 and 3a-1 with target CSI)/ encoder parameter (for 3a-1) / additional information (for 3a-1 and 4-1).

- The workload can be reduced by identifying the common part among different aspects.

Based on the study, RAN1 recommends the following with potential RAN1 specification impacts common to all inter-vendor collaboration options.

- Recommend Case 0

  - Case 2 and 3 can be considered for further enhancement built upon the specification of case 0.

- Recommend following specification impacts for the two-sided CSI compression use case

  - Model pairing procedure including ID and applicability reporting

  - Inference aspects including target CSI type, measurement and report configuration, CQI / RI determination, payload determination, quantization configuration / codebook, UCI mapping, CSI processing criteria and timeline, priority rules for CSI reports

  - NW and UE side data collection for training,

    - Target CSI format

    - Note: The framework defined in BM and CSI prediction use cases could be reused.

  - Specify performance monitoring, if needed

For the supporting of sub-option 3a-1 (including with target CSI and without target CSI), sub-option 4-1, and Direction C, all these three inter-vendor collaboration options should be specified. The reference model / structure could be specified in RAN4 for performance requirements to minimize the workload in RAN1. Format and contents of parameter/dataset/information exchange for option 3a-1 and 4-1 could also be specified jointly.

Proposal 1: Inter-vendor collaboration sub-option 3a-1 (including with target CSI and without target CSI), sub-option 4-1, and Direction C should be specified. The reference model/structure for sub-option 3a-1 and Direction C is specified in RAN4.

For the supporting of use cases, Case 0 has been recommended. In addition to Case 0, Case 3 includes time domain prediction at UE side to further enhance system performance while Case 2 minimizes CSI feedback overhead with historic information from both UE and NW side. Ideally, all three cases could be specified to fully explore the potential of AI-based CSI compression. With the constraint of TU budget, if only one case could be introduced, Case 3 should be considered due to lower workload.

Proposal 2: Case 0/2/3 could be specified in R20. If down-selection is operated between Case 2 and Case 3, Case 3 is proposed to be specified.

- Conclusion

In summary, the following observation and proposals is provided:

Observation 1: According to FS_NR_AIML_air_Ph2 study outcome, AI/ML based CSI compression should be specified in Rel-20.

Proposal 1: Inter-vendor collaboration sub-option 3a-1 (including with target CSI and without target CSI), sub-option 4-1, and Direction C should be specified. The reference model/structure for sub-option 3a-1 and Direction C is specified in RAN4.

Proposal 2: Case 0/2/3 could be specified in R20. If down-selection is operated between Case 2 and Case 3, Case 3 is proposed to be specified.

Reference

[1] RP-250040 Rel-20 5G-Advanced RAN vice Chair

[2] RP-250792 Revised WID on AI/ML for NR air interface Qualcomm Innovation Center Inc

[3] Chair notes RAN1#121