3GPP TSG RAN Plenary #108			RP-251472

Prague, Czech Republic, June 9-13, 2025

Agenda item:		16.1

Source:	China Telecom

Title:	Views on IMT-2030 Technical Performance Requirements (TPRs)

Document for:		Discussion

Introduction

In the last 3GPP TSG RAN meeting #107, the IMT-2030 TRPs have been discussed with the following proposals agreed in [1].

Agreement Proposal 1: Define positioning accuracy as a quantitative TPR. Proposal 2: Define Sensing-related capabilities as a TPR. Proposal 3: define AI-related capabilities as qualitative TPR evaluated by inspection. Proposal 4: Define energy efficiency as a TPR. Proposal 5: For user experienced data rate, further discuss applicable deployment scenarios, and revisit the evaluation methodology before setting values for UL. Proposal 6: For bandwidth TPR, it is proposed to set a target value of 400MHz (same definition as IMT-2020) - TBD on other TPR values Proposal 7: Define TPR (e.g. joint/composite requirement) related to data rate, latency, reliability, capacity etc. as a candidate IMT-2030 TPR. - Applicable usage scenario: immersive communication

In order to submit LS to ITU-R WP5D with providing the inputs on the minimum technical performance requirements for IMT-2030 radio interface technologies before the submission deadline on June 12, two conference calls were planned to progress between RAN#107 and RAN#108 in [2].

- CC#1 from 14:00 to 16:00 CEST on April 29, 2025 – deadline for contributions on April 24

- CC#2 from 14:00 to 16:00 CEST on June 4, 2025

The first conference call has been made with the following agreements highlighted in green [3].

| ITU IMT-2030 TPR | IMT-2020 Value | Proposed Values | Agreements in CC#1 |
| --- | --- | --- | --- |
| Peak data rate (Gbit/s) | DL: 20 UL: 10 | [1~10x] of IMT-2020 | Alternative 1: Not to define a requirement, e.g. Leave the value for peak data rate for the technology proponent to provide Alternative 2: Define a requirement with reasonable value |
| Peak spectral efficiency (bit/s/Hz) | DL: 30 UL: 15 | [1~3x] of IMT-2020 | DL: [1.5-2.5x] of IMT-2020 UL: [1.5-2.5x] of IMT-2020 |
| User experienced data rate (Mbit/s) | DL: 100 UL: 50 | DL: [1.25~10x] of IMT-2020 UL: [1.25~4x] of IMT-2020 | For UL: - RAN#107 agreement: For user experienced data rate, further discuss applicable deployment scenarios, and revisit the evaluation methodology before setting values for UL - Proposal 1: Use system level simulation for the evaluation of UL user experienced data rate with no bandwidth scaling taking UE maximum output power into account. |
| 5th percentile user spectral efficiency (bit/s/Hz) | Indoor Hotspot-eMBB: - DL: 0.3 - UL: 0.21 Dense Urban - eMBB: - DL: 0.225 - UL: 0.15 Rural-eMBB: - DL: 0.12 - UL: 0.045 | [1.5~5x] of IMT-2020 | Companies are encouraged to provide antenna assumption for different frequency ranges together with the requirements, e.g. 4GHz, 7GHz, 30GHz Applicable deployment scenarios should also be considered. |

The following proposals are agreed in CC#1 for IMT-2030 TPRs:

Proposal 2-3: Do not define security, interoperability as IMT-2030 TPRs, these can be covered by submission template for IMT-2030.

Proposal 2-4: Sustainability is covered by energy efficiency, do not define separate TPR for sustainability.

Based on the above progress, we will focus on providing views on the remaining issues on the potential TPRs in this contribution.

Views on legacy TPRs

The potential target values of candidate legacy TPRs have been discussed with a listed table in the moderator's summary for IMT-2030 TPRs in CC#1, which can be considered as a starting point for further discussion [3].

| ITU IMT-2030 TPR |  | IMT-2020 Value | Proposed Values | Updated Moderator proposal |
| --- | --- | --- | --- | --- |
| Peak data rate (Gbit/s) |  | DL: 20 UL: 10 | [1~10x] of IMT-2020 | Alternative 1: Not to define a requirement, e.g. Leave the value for peak data rate for the technology proponent to provide Alternative 2: Define a requirement with reasonable value |
| Peak spectral efficiency (bit/s/Hz) |  | DL: 30 UL: 15 | [1~3x] of IMT-2020 | DL: [1.5-2.5x] of IMT-2020 UL: [1.5-2.5x] of IMT-2020 |
| User experienced data rate (Mbit/s) |  | DL: 100 UL: 50 | DL: [1.25~10x] of IMT-2020 UL: [1.25~4x] of IMT-2020 | For UL: - RAN#107 agreement: For user experienced data rate, further discuss applicable deployment scenarios, and revisit the evaluation methodology before setting values for UL - Proposal 1: Use system level simulation for the evaluation of UL user experienced data rate with no bandwidth scaling taking UE maximum output power into account. |
| 5th percentile user spectral efficiency (bit/s/Hz) |  | Indoor Hotspot-eMBB: - DL: 0.3 - UL: 0.21 Dense Urban - eMBB: - DL: 0.225 - UL: 0.15 Rural-eMBB: - DL: 0.12 - UL: 0.045 | [1.5~5x] of IMT-2020 | Companies are encouraged to provide antenna assumption for different frequency ranges together with the requirements, e.g. 4GHz, 7GHz, 30GHz Applicable deployment scenarios should also be considered. |
| Average spectral efficiency (bit/s/Hz) |  | Indoor Hotspot - eMBB: - DL: 9; - UL: 6.75 Dense Urban - eMBB: - DL: 7.8; - UL: 5.4 Rural - eMBB: - DL: 3.3; - UL: 1.6 | [1.5~5x] of IMT-2020 | [3x] of IMT-2020 |
| Area traffic capacity (Mbit/s/m²) |  | DL: 10 | [1~10x] of IMT-2020 | [5x] of IMT-2020 |
| latency | User plane latency (ms) | eMBB: 4 URLLC: 1 | eMBB: [1~4] HRLLC: [0.1~1] | IC: [2] HRLLC: [1] |
|  | Control plane latency (ms) | 20 | [5~20] | [10] |
| Connection density (devices/km²) |  | 10⁶ | [1~100x] of IMT-2020 | [10x] of IMT-2020 |
| Reliability |  | - 10⁻⁵ | [1~100x] of IMT-2020 | [1-10-5] |
| Mobility (bit/s/Hz @ km/h) |  | Indoor Hotspot-eMBB: 1.5 @10 km/h Dense Urban- eMBB: 1.12 @30 km/h Rural-eMBB: 0.8 @120 km/h 0.45 @500 km/h | 1x of IMT-2020~ 1000/1200km/h | Keep the velocities defined for IMT-2020. Should new velocity be added? (up to 1000/1200km/h for airplane) Should 50% spectrum efficiency be increased for the velocity defined in IMT-2020? e.g. [1.5x] of IMT-2020 |
| Mobility interruption time (ms) |  | 0 | 0 ms | [0 ms] |
| Bandwidth (MHz) |  | 100 | 200MHz or 400MHz | RAN#107 agreement: set a target value of 400MHz (same definition as IMT-2020) (same definition as IMT-2020) |

  - 2.1 Peak data rate

During the discussion about peak data rate for IMT-2030, there are two alternatives. One is not to define a requirement and leave the value for the technology proponent to provide. The value could span a broad range for different technologies adopted for various IMT-2030 scenarios. The other one is to define a requirement with reasonable value. As a common understanding, the peak data rate is calculated based on peak spectral efficiency and bandwidth, aiming to provide the maximum achievable data rate under ideal conditions to represent promising visions for future IMT-2030. We think a reasonable value is needed, which is helpful to gather the confidence of IMT-2030 and gives guidance for potential technology selection in 3GPP and spectrum allocation in ITU-R. The peak spectral efficiency has consensus with 1.5-2.5 times higher than IMT-2020. Based on the RAN#107 agreement, the target value of bandwidth is 400MHz. Hence, we propose to set the target value of peak data rate to be 5 times higher than IMT-2020, which can be seen as a moderately reachable goal.

Proposal 1: Support the value of peak data rate as 5 times higher than IMT-2020, i.e., 100 Gbps for DL and 50 Gbps for UL.

Generally, we think the reasonable TPRs are essential for 6G study and subsequent 6G commercial operation. For legacy TPRs in IMT-2030, e.g., data rate, we think the target values should be pragmatic to meet 6G requirements. Based on our contribution submitted in the last RAN plenary meeting, we updated the proposed legacy TPRs with target values highlighted in yellow for IMT-2030, with considering the current progress in [3].

Table 1: The proposed legacy TPRs with target values for IMT-2030

| Index | Items | Proposed target values for IMT-2030 | Note |
| --- | --- | --- | --- |
| 1 | Peak data rate | DL: 100Gbps, UL: 50bps | 5 x IMT-2020 |
| 2 | Peak spectrum efficiency | DL: 45~75bps/Hz, UL: 22.5~37.5bps/Hz | 1.5~2.5 x IMT-2020 |
| 3 | User experienced data rate | DL: 300Mbps, UL: 150Mbps | 3 x IMT-2020 |
| 4 | 5% percentile user spectrum efficiency | Indoor Hotspot: 0.45~0.9 (DL) /0.315~0.63 (UL) Dense Urban: 0.3375~0.675 (DL) /0.225~0.45 (UL) Rural: 0.18~0.36 (DL) /0.0675~0.135 (UL) | 1.5~3 x IMT-2020 |
| 5 | Average spectrum efficiency | Indoor Hotspot: 13.5~27 (DL) /10.125~20.25 (UL) Dense Urban: 11.7~23.4 (DL)/8.1~16.2 (UL) Rural: 4.95~9.9 (DL)/2.4~4.8 (UL) | 1.5~3 x IMT-2020 |
| 6 | Sustainability/Energy efficiency | TBD |  |
| 7 | Area traffic capacity | 50 Mbit/s/m2 | 5 x IMT-2020 |
| 8 | User plane latency | HRLLC: 0.1~0.5ms |  |
| 9 | Control plane latency | 10ms | 1/2 x IMT-2020 |
| 10 | Connection density | 107~108 devices/km2 | 10~100 x IMT-2020 |
| 11 | Reliability | 10-6~10-7 | 10~100 x IMT-2020 |
| 12 | Mobility | Stationary: 0 km/h Pedestrian: 0 km/h to 10 km/h Vehicle: 10 km/h to 120 km/h High speed vehicle: 120 km/h to 700 km/h Ultra speed aerial vehicle: 700 km/h to 1000 km/h. SINR: TBD |  |
| 13 | Mobility interruption time | 0ms |  |
| 14 | Bandwidth | 400MHz |  |

Proposal 2: Adopt the proposed legacy TPRs with target values for IMT-2030. The remaining values and evaluation methodology can be further study.

Views on other TPRs

For new TPRs, the AI-related capabilities and sensing-related capabilities are very close to use cases and implementation. It may be hard to have quantitative targets or unified evaluation criteria. We prefer to identify essential scenarios first and use necessary inspection for evaluation if needed. Based on the previous discussion, there are some agreements on other TPRs, e.g., positioning, sensing-related capabilities, AI-related capabilities, security, interoperability, sustainability.

Agreement Proposal 1: Define positioning accuracy as a quantitative TPR. Proposal 2: Define Sensing-related capabilities as a TPR. Proposal 3: define AI-related capabilities as qualitative TPR evaluated by inspection. Proposal 2-3: Do not define security, interoperability as IMT-2030 TPRs, these can be covered by submission template for IMT-2030. Proposal 2-4: Sustainability is covered by energy efficiency, do not define separate TPR for sustainability.

  - 3.1 Sensing-related capabilities

The sensing-related capabilities have been agreed to be defined as a TPR. As the low-altitude economy is rapidly developing and is becoming an emerging industry, many cities in China have planned several low altitude logistics routes for trial. Sensing-related capabilities refer to the ability to provide functionalities in the radio interface including range/velocity/angle estimation, object detection, localization, imaging, mapping, etc., which are essential for 6G scenarios and technologies. We propose five kinds of candidate items for sensing, including False alarm probability/Missed detection probability, Localization accuracy, Velocity accuracy, Sensing resolution and Maximum sensing service latency. The sensing resolution refers to the minimum difference between any two distinguishable target objects in the measured aspects of range, velocity and angle.

Proposal 3: Adopt the following items as sensing-related capabilities.

- False alarm probability/Missed detection probability

- Localization accuracy

- Velocity accuracy

- Sensing resolution

- Maximum sensing service latency

However, the target values for each candidate item are closely related to test environments. Hence, it would be reasonable to first determine the typical test environments for sensing. The detailed assumptions for each test environment can be further discussed. Based on the speed and type of sensing target objects, there are three kinds of test environments proposed, i.e., ISAC-UAV, ISAC-Automotive and ISAC-Human. The descriptions are as follows.

ISAC_UAV: A test environment targeting at sensing the medium or high speed vehicle, e.g., UAVs.

ISAC_Automotive: A test environment targeting at sensing the low and medium speed vehicle, e.g., smart vehicles, UGVs.

ISAC_Human: A test environment targeting at sensing the human motions or target objects which are stationary or low speed.

Hence, we have the following proposal.

Proposal 4: Adopt the following test environments for sensing scenario. The detailed assumptions can be further discussed.

- ISAC_UAV: A test environment targeting at sensing the medium or high speed vehicle, e.g., UAVs.

- ISAC_Automotive: A test environment targeting at sensing the low and medium speed vehicles, e.g., smart vehicles, UGVs.

- ISAC_Human: A test environment targeting at sensing human motions or target objects which are stationary or low speed.

Regarding the preliminary consideration on evaluation methodology for sensing scenarios, we think it would be reasonable to reuse 5G-A ISAC as much as possible, especially the achieved progress on ISAC channel model and assumptions for identified typical scenarios. In our understanding, sensing should be evaluated under ISAC test environments using system-level simulation or system-level simulation followed by link-level simulation. Relevant sensing performance metrics and identified TPRs can be acquired from sensing signal processing and evaluation. Hence, we have the following proposal.

Proposal 5: Reuse 5G-A ISAC channel model and assumptions as much as possible, with adopting system-level simulation or system-level simulation followed by link-level simulation under ISAC test environments.

  - 3.2 AI-related capabilities

Regarding AI-related capabilities, it has been agreed to defined as qualitative TPR evaluated by inspection. The qualitative items can be inspected via comparing AI enabled cases and no AI cases. The consumption of AI functionalities over the radio interface including computing, power consumption and latency should be inspected and considered. We propose at least three kinds of items should be defined as AI-related capabilities, i.e., AI service computing, AI service accuracy, AI service latency. The other AI capabilities can be further discussed.

Proposal 6: At least adopt the following items as AI-related capabilities, other AI capabilities can be further discussed.

- AI service computing

- AI service accuracy

- AI service latency

  - 3.3 Coverage

During the first conference call discussion, there was a proposal on coverage, however, no consensus was made [3].

Proposal 2-5: For coverage, further discuss the following three options:

- Option 1: define coverage as a quantitative TPR

- Option 2: do not define coverage as a TPR, use link budget evaluation approach (same as IMT-2020)

- Option 3: define extra connectivity as a qualitative TPR, which was revised from a part of coverage

We prefer Option 2 and it can reuse link budget evaluation methodology in TR 38.830 [4], aiming to reduce workload. Coverage refers to the ability to provide access to communication services for users in a desired service area. It can be defined as the cell edge distance of a single cell through link budget analysis. The defined MPL (Maximum Path Loss) can be chosen as a performance metric to provide direct distance information for various IMT-2030 network deployments. Hence, we have the following proposal.

Proposal 7: Do not define coverage as a TPR, reuse link budget evaluation methodology and MPL in TR 38.830 with necessary modifications if any.

Conclusions

In this contribution, we have the following proposals:

Proposal 1: Support the value of peak data rate as 5 times higher than IMT-2020, i.e., 100 Gbps for DL and 50 Gbps for UL.

Proposal 2: Adopt the proposed legacy TPRs with target values for IMT-2030. The remaining values and evaluation methodology can be further study.

Table 1: The proposed legacy TPRs with target values for IMT-2030

| Index | Items | Proposed target values for IMT-2030 | Note |
| --- | --- | --- | --- |
| 1 | Peak data rate | DL: 100Gbps, UL: 50bps | 5 x IMT-2020 |
| 2 | Peak spectrum efficiency | DL: 45~75bps/Hz, UL: 22.5~37.5bps/Hz | 1.5~2.5 x IMT-2020 |
| 3 | User experienced data rate | DL: 300Mbps, UL: 150Mbps | 3 x IMT-2020 |
| 4 | 5% percentile user spectrum efficiency | Indoor Hotspot: 0.45~0.9 (DL) /0.315~0.63 (UL) Dense Urban: 0.3375~0.675 (DL) /0.225~0.45 (UL) Rural: 0.18~0.36 (DL) /0.0675~0.135 (UL) | 1.5~3 x IMT-2020 |
| 5 | Average spectrum efficiency | Indoor Hotspot: 13.5~27 (DL) /10.125~20.25 (UL) Dense Urban: 11.7~23.4 (DL)/8.1~16.2 (UL) Rural: 4.95~9.9 (DL)/2.4~4.8 (UL) | 1.5~3 x IMT-2020 |
| 6 | Sustainability/Energy efficiency | TBD |  |
| 7 | Area traffic capacity | 50 Mbit/s/m2 | 5 x IMT-2020 |
| 8 | User plane latency | HRLLC: 0.1~0.5ms |  |
| 9 | Control plane latency | 10ms | 1/2 x IMT-2020 |
| 10 | Connection density | 107~108 devices/km2 | 10~100 x IMT-2020 |
| 11 | Reliability | 10-6~10-7 | 10~100 x IMT-2020 |
| 12 | Mobility | Stationary: 0 km/h Pedestrian: 0 km/h to 10 km/h Vehicle: 10 km/h to 120 km/h High speed vehicle: 120 km/h to 700 km/h Ultra speed aerial vehicle: 700 km/h to 1000 km/h. SINR: TBD |  |
| 13 | Mobility interruption time | 0ms |  |
| 14 | Bandwidth | 400MHz |  |

Proposal 3: Adopt the following items as sensing-related capabilities.

- False alarm probability/Missed detection probability

- Localization accuracy

- Velocity accuracy

- Sensing resolution

- Maximum sensing service latency

Proposal 4: Adopt the following test environments for sensing scenario. The detailed assumptions can be further discussed.

- ISAC_UAV: A test environment targeting at sensing the medium or high speed vehicle, e.g., UAVs.

- ISAC_Automotive: A test environment targeting at sensing the low and medium speed vehicles, e.g., smart vehicles, UGVs.

- ISAC_Human: A test environment targeting at sensing human motions or target objects which are stationary or low speed.

Proposal 5: Reuse 5G-A ISAC channel model and assumptions as much as possible, with adopting system-level simulation or system-level simulation followed by link-level simulation under ISAC test environments.

Proposal 6: At least adopt the following items as AI-related capabilities, other AI capabilities can be further discussed.

- AI service computing

- AI service accuracy

- AI service latency

Proposal 7: Do not define coverage as a TPR, reuse link budget evaluation methodology and MPL in TR 38.830 with necessary modifications if any.

- References

- RP-250819, Moderator's summary for IMT-2030 TPRs, 3GPP TSG RAN Meeting #107, Incheon, Korea, March 12-14, 2025.

- RP-250820, Direct Submission to ITU-R from TSG RAN#108 on IMT-2030 TPR values, 3GPP TSG RAN Meeting #107, Incheon, Korea, March 12-14, 2025.

- RT-250024, Moderator's summary for IMT-2030 TPRs in CC#1, April 29, 2025.

- TR 38.830, Study on NR coverage enhancements (Release 17), Dec. 2020.