3GPP TSG RAN Meeting #108						                                         RP-251476

Prague, Czech Republic, June 9-13, 2025

## Status Report to TSG

Agenda item:			9.4.1.1

| WI / SI Name | # Introduction of another NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_NTN_Sband |  |  |  |  |
| Unique ID | 1040129 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-250488 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: 6/2025 | Performance part: 6/2025 | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: 100% | Performance Part: 100% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Laehteensuo, Toni |
|  | Company | Qualcomm Incorporated |
|  | Email | tlaehtee@qti.qualcomm.com |

# 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

# 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

## 2.1	RAN1

### 2.1.1	Agreements

### 2.1.2	Remaining Open issues

## 2.2	RAN2

### 2.2.1	Agreements

### 2.2.2	Remaining Open issues

## 2.3	RAN3

### 2.3.1	Agreements

### 2.3.2	Remaining Open issues

## 2.4	RAN4

### 2.4.1	Agreements

#### 2.4.1.1	RAN4 #114bis

Agreements from WF R4-2505235 [16]

#### SS Raster

  - For band n252, specify only SS Raster Case A for the time being. Introduction or Case B or Case C can be considered at later stage based on explicit demand from an operator.

#### UE A-MPR

  - Consider the following tables as preliminary A-MPR values for -30dBm/MHz and -40dBm/MHz protection criteria.

A-MPR regions for -30 dBm/MHz

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5 MHz | - | - | - | - |
| 10MHz | < 2010 | ≤ 1.44 | ≤ 2.7 | A1 |
|  |  | ≤ 1.44 | > 2.7, < 4.5 | A2 |
|  |  | ≤ 1.44 | ≥ 4.5 | A1 |
|  |  | ≥ 1.44 | ≥ 5.4 | A2 |
| 15MHz | < 2012.5 | ≤ 3.24 | ≤ 2.7 | A1 |
|  |  | ≤ 3.24 | > 2.7, < 4.5 | A2 |
|  |  | ≤ 3.24 | ≥ 4.5 | A1 |
|  |  | ≥ 3.24 | ≥ 5.4 | A2 |
|  | 2012.5 | ≤ 1.44 | ≤ 2.7 | A1 |
|  |  | ≤ 1.44 | > 2.7, < 5.4 | A2 |
|  |  | ≤ 1.44 | ≥ 5.4 | A1 |
|  |  | ≥ 1.44 | ≥ 7.2 | A2 |
| 20 MHz | 2010 | ≤ 4.68 | ≤ 3.6 | A1 |
|  |  | ≤ 4.68 | > 3.6, ≤ 5.4 | A2 |
|  |  | ≤ 4.68 | > 5.4 | A1 |
|  |  | > 4.68 | ≥ 7.2 | A2 |

A-MPR for -30 dBm/MHz

| Modulation/Waveform | A1 | A2 |
| --- | --- | --- |
|  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 4 | ≤ 2.5 |
| DFT-s-OFDM QPSK | ≤ 4.5 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 5 | ≤ 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 4 |
| CP-OFDM QPSK | ≤ 7 | ≤ 4.5 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 4.5 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 4.5 |

A-MPR regions for -40 dBm/MHz

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | < 2007.5 | ≥ 0 | ≥ 2.16 | A1 |
| 10MHz | < 2010 | ≤ 1.44 | ≥ 0 | A2 |
|  |  | > 1.44 | ≥ 3.6 | A1 |
|  |  | > 7.74 | ≤ 0.72 | A1 |
|  | 2010 ≤ Fc ≤ 2015 | ≥ 0 | ≥ 4.5 | A1 |
| 15MHz | < 2012.5 | ≤ 3.24 | ≥ 0 | A3 |
|  |  | > 3.24 | ≥ 4.5, ≤ 7.2 | A1 |
|  |  | > 3.24 | > 7.2 | A2 |
|  |  | > 10.8 | ≤ 0.72 | A1 |
|  | 2012.5 | ≤ 1.44 | ≥ 0 | A2 |
|  |  | > 1.44 | ≥ 6.48 | A1 |
|  |  | > 12.6 | ≤ 0.72 | A1 |
| 20 MHz | 2010 | ≤ 4.68 | ≥ 0 | A3 |
|  |  | > 4.68 | ≥ 4.5, ≤ 7.2 | A1 |
|  |  | > 4.68 | > 7.2 | A2 |
|  |  | > 12.6 | ≤ 0.72 | A1 |

A-MPR for -40 dBm/MHz

| Modulation/Waveform | A1 | A2 | A3 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 4 | ≤ [8] | ≤ [11] |
| DFT-s-OFDM QPSK | ≤ 4.5 | ≤ [8] | ≤ [11] |
| DFT-s-OFDM 16 QAM | ≤ 5 | ≤ [8] | ≤ [11] |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ [8] | ≤ [11] |
| CP-OFDM QPSK | ≤ 7 | ≤ [10] | ≤ [12] |
| CP-OFDM 16 QAM | ≤ 7 | ≤ [10] | ≤ [12] |
| CP-OFDM 64 QAM | ≤ 7 | ≤ [10] | ≤ [12] |

#### UE REFSENS

  - Consider the following table with the REFSENS values for further discussions

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| 15 | -99.5 | -96.3 | -94.5 | -93.3 |
| 30 |  | -96.6 | -94.6 | -93.5 |
| 60 |  | -97.0 | -94.9 | -93.7 |

#### Duplexer Configuration

  - RAN WG4 does not make any assumption on a specific duplexer implementation

  - RAN WG4 agrees that the OOB blocking requirements are decoupled from the implementation assumption of UE duplexer.

  - Capture these considerations in TR 38.863 and TR 36.764

#### Out-of-band Blocking

  - Take option 5 as a baseline

  - Do not preclude the implementation supporting only n252 and not supporting n65.

  - The following out-of-band blocking requirements are agreed (based on option 5 above)

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n252 | Finterferer (CW) | MHz | -110 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -155 < f – FDL_low ≤ -110 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 155 or FDL_high + 85 ≤ f ≤ 12750 |

#### Protection of GNSS

  - Band n252/b252 will have new NS_06N, NS_07N and NS_08N flags defined

  - GNSS protection requirement will be included in new NS flags

  - NOTE: the agreements above apply for NR-NTN and IoT-NTN

#### SAN SS Raster

  - For band n252, specify only SS Raster Case A for the time being. Introduction or Case B or Case C can be considered at later release based on explicit demand from an operator.

Following draft CRs were endorsed:

R4-2503441	DraftCR for TS 38.108, Introduction on system parameters for SAN supporting New S band	CATT

R4-2503735	draftCR to TS38.108 Introduction of NR-NTN S-band	ZTE Corporation, Sanechips

R4-2504623	Draft CR to TS 38.108: Clause 5.2 Operating bands	THALES

#### 2.4.1.2	RAN4 #115

Following CRs were agreed:

R4-2508010	CR to TR 38.863: Introduction of NR-NTN band n252

R4-2508055	CR to TS 38.101-5: Introduction of band n252

R4-2508011	CR on TS 38.108 Introduction of NR NTN band n252

R4-2508034	CR to TS 38.133: Introduction of a new NTN S-band n252

R4-2507149	CR to TS 38.181: Introduction of NR-NTN band n252

Additional agreements in WF R4-2508054 [34]

#### UE REFSENS

bands n252 and n256 apply the same REFSENS requirements for all CBW.

#### Out-of-band Blocking

The agreement in the previous meeting which is option 1 is confirmed.

#### SAN Conformance

With the exception of regional requirement, the introduction of the n252 band will not have impact on SAN conformance specifications.

### 2.5.2	Remaining Open issues

None.

### 2.5.3	Remaining Open issues with cross-WG dependencies

None.

## 2.5	RAN5

### 2.5.1	Agreements

### 2.5.2	Remaining Open issues

### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

### 2.6.1	Agreements

### 2.6.2	Remaining Open issues

# 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

## 3.1	SAx/CTs

### 3.1.1	Agreements with cross-TSG impacts

### 3.1.2	Remaining Open issues with cross-TSG impacts

# 4.	References

## RAN4#114bis

- R4-2503176	Further input on UE power back-off while operating in 2000-2020 MHz UL range of the NTN band n252	Apple

- R4-2503177	Introduction of the NTN S-band to TS 38.101-5	Apple

- R4-2503439	Discussion on SAN RF requirements for the NTN new S-band	CATT

- R4-2503440	Discussion on UE RF requirements for the NTN new S-band	CATT

- R4-2503441	DraftCR for TS 38.108, Introduction on system parameters for SAN supporting New S band	CATT

- R4-2503565	Views on open issues of n252/B252 UE RF requirements	EchoStar, Boost Mobile Network

- R4-2503566	DraftCR to TR 38.863 to capture agreement for n252 introduction	EchoStar, Boost Mobile Network

- R4-2503697	Discussion on the NTN S band definition	Huawei, HiSilicon

- R4-2503735	draftCR to TS38.108 Introduction of NR-NTN S-band	ZTE Corporation, Sanechips

- R4-2503800	Draft CR to TS 38.133 Introduction of a new NTN S-band	ZTECorporation,Sanechips

- R4-2504387	Discussion on NR-NTN n252 UE RF requirements	MediaTek (Hefei) Inc.

- R4-2504544	A-MPR and OOB blocking for n252	Qualcomm Inc.

- R4-2504623	Draft CR to TS 38.108: Clause 5.2 Operating bands	THALES

- R4-2505125	Introduction of the NTN S-band to TS 38.101-5	Apple

- R4-2505126	DraftCR to TR 38.863 to capture agreement for n252 introduction	EchoStar, Boost Mobile Network

- R4-2505235	WF on NR-NTN and IoT-NTN bands	Apple

## RAN4#115

- R4-2505413	Further input on UE power back-off while operating in 2000-2020 MHz UL range of the NTN band n252	Apple

- R4-2505414	CR to TS 38.101-5: Introduction of band n252	Apple

- R4-2505479	Discussion on NR-NTN n252 UE RF REFSENS	MediaTek (Hefei) Inc.

- R4-2506035	Discussion on UE RF requirements for the NTN new S-band	CATT

- R4-2506521	n252 A-MPR simulations and proposal	Nokia

- R4-2507711	CR to TR 38.863: Introduction of NR-NTN band n252	EchoStar, Boost Mobile Network

- R4-2507752	A-MPR for n252	Qualcomm Incorporated

- R4-2508010	CR to TR 38.863: Introduction of NR-NTN band n252	EchoStar, Boost Mobile Network

- R4-2508012	CR to TS 38.101-5: Introduction of band n252	Apple

- R4-2508055	CR to TS 38.101-5: Introduction of band n252	Apple, Aalyria Technologies, Boost Mobile Network, Echostar, Qualcomm Inc.

- R4-2508120	CR to TS 38.101-5: Introduction of band n252	Apple, Aalyria Technologies, Boost Mobile Network, Echostar, Qualcomm Inc., T-Mobile

- R4-2507697	CR on TS 38.108 Introduction of NR NTN band n252	THALES, ZTE Corporation, CATT, EchoStar, Boost Mobile Network, Aalyria

- R4-2508011	CR on TS 38.108 Introduction of NR NTN band n252	THALES, ZTE Corporation, CATT, EchoStar, Boost Mobile Network, Aalyria

- R4-2507207	Draft CR to TS 38.133 Introduction of a new NTN S-band	ZTECorporation,Sanechips

- R4-2508034	CR to TS 38.133: Introduction of a new NTN S-band n252	ZTE Corporation, Sanechips, EchoStar, Boost Mobile Network

- R4-2506034	Discussion on SAN conformance requirements for the NTN new S-band	CATT

- R4-2507149	CR to TS 38.181: Introduction of NR-NTN band n252	EchoStar, Boost Mobile Network

- R4-2508054	WF on NR_IoT_NTN_Bands	Aalyria

- R4-2508120	 CR to TS 38.101-5: Introduction of band n252, Apple, Aalyria Technologies, Boost Mobile Network, Echostar, Qualcomm Inc., T-Mobile