3GPP TSG RAN Meeting #108						RP-251478

Prague, Czech Republic, June 9-13, 2025

Agenda Item:		16.2

Source:		           Rakuten Mobile views on 6G RAN WG SIs for Rel-20

Title:				Our views on 6G RAN WG Study item

Document for:		Discussion

## 1	Purpose

This contribution outlines Rakuten Mobile’s views on the scope and key priorities to be addressed in the upcoming WG-level 6G RAN Study Item. The purpose is present a consolidated proposal identifying areas that we believe merit investigation in Rel-20, aligned with 6G Workshop conclusions and broader industry consensus.

### 2	Key Design Proposals

- Service-aligned scalability: 6G to support for high-performance, low-latency applications and device type like XR and AI inference, alongside efficient designs for IoT & LPWA within same specifications framework, minimize optionality for each device type.

- Enhanced uplink and edge coverage: 6G to address coverage gaps in FR2 and future FR3 bands through high-power UE classes, TRP cooperation, and low-latency uplink enhancements.

- Energy efficiency baseline: 6G based on energy efficient signaling frameworks including AI-driven DRX/WUS, sparse control and broadcast signals, Support for low level energy monitoring in RAN.

- AI-native architecture: Define lifecycle management, fallback safeguards, inference placement, and explainability KPIs across RAN.

- RAN Architecture : Keep 5G gNB architecture in 6G as baseline which support deployment flexibilty.

- Migration and co-existence readiness: Include MRSS based spectrum reuse, Study Dual Registration and Inter RAT DC Scenarios, and logical gNB alignment across legacy and new bands.

- TN and NTN Harmonization including Spectrum Sharing.

- Study RAN & Core Interface enhancement (CU-UP & UPF).

- Keep Rel-20 studies scope open and exploratory with check point basde convergence.

### 3	Working Group level Proposals

### RAN1 — Fundamental Radio Design for 6G

- Waveform, modulation, coding: Pick a single baseline early in Study phase.

- UL Coverage and MIMO: shorten Msg3/Msg5 delay; add high-power UE options evaluate cell-free / multi-TRP reception for stronger uplink at edge.

- Spectrum aggregation & MRSS: MRSS study for efficient 5G/6G sharing.

- RIS support: model RIS links and define minimum configuration signalling.

- Energy efficient signalling: Study Energy efficient Phy layer design including Sparse SSB/SIB cycles, wake-up..

- AI-Native PHY: Agree on a short-list of high-value use cases by the mid-term checkpoint of the release.

### RAN 2 — Protocol & Procedures for 6G

- User-plane design: Study further enhancements to support high-rate, low latency, low jitter XR and AI traffic.

- Control-plane design: Study control plane enhancements to support diverse Use cases.

- RRC procedures: Study enhancements for RRC procedures to support 6G use case and performance improvments.

- Model life-cycle: Registration, update, and fall-back procedures for AI functions other AI aspects in RAN2 domain.

- Service QoS labels: Scheduling enhancements such as per-service tags (voice, XR, AI, IoT).

### RAN 3 — RAN Architecture & Interfaces for 6G

- Cloud-native CU/DU: F1 & E1 enhancements to support 6G use cases. Maintain 5G disaggregated architecture as baseline.

- RAN-Core interface: Study enhancements related to RAN & Core interface including CU-UPF integration.

- Dual-registration & migration: Two Tier Study on Inter& Intra RAT DC as well as Dual-Registration to understand the value proposition (Gains) and impact analysis (including limits).

- NTN integration: Study further enhancements related to TN-NTN integration, including Spectrum sharing between TN & NTN.

- Interworking: Study MRSS for multi vendor deployments.

### RAN 4 — RF, RRM & Conformance requirements for 6G

- RF requirements: New device type study. (XR, High power UE etc)

- Spectrum masks & guard-bands: Investigate and study Gap requirements for MRSS.

- FR3 test plans: OTA cases for 7–24 GHz, including RIS-aided paths.

- AI receiver testing: verify model update, roll-back and performance under fading.

# 4. Conclusion

This paper sets out the rationale and goals for a new 6G RAN working-group Study Item, targeted for approval at 3GPP RAN #108.