3GPP TSG-RAN Meeting 108	RP-251494

Prague, Czech Republic, June 9-13, 2025

Agenda Item:	15.2.1

Source:	Huawei, HiSilicon

Title:	Discussion for AI/ML based mobility

Document for:	Discussion

# 1	Introduction

In the last TSG-RAN meeting, many companies have proposed their expectations with respect to the scope of AI/ML for mobility WI in Rel-20. In this contribution, we discuss the FFS points from RAN#107 meeting, considering the latest progress in RAN2 meetings.

# 2	Discussion

## 2.1 Analysis for spatial domain prediction

In the previous RAN2 meeting, both the time domain and frequency domain prediction have been discussed, while the spatial domain prediction was pending. Consequently, in the RAN#107 meeting, it has been concluded that at least the temporal domain prediction and frequency domain prediction will be supported, while whether to include spatial domain prediction in the WID is pending RAN2 study progress. Further, in the last RAN2#130 meeting, the following agreements have been made [1]:

Agreements 1 The scenario of intra-frequency spatial domain prediction (in cell dimension) is not considered by the UE-side model in 5G. RAN2 assumes that a network-sided model requires no specification impacts. Intra-frequency spatial domain prediction (in cell dimension) involves measuring one or more cells as input to the model to derive L3 filtered cell-level measurements for the same time instance of another cell(s). 2 The specification impact of intra-frequency spatial domain prediction in beam dimension will be studied by RAN2 for both UE-side and network-side model.

We can observe that RAN2 agreed that cell level spatial domain prediction should be limited to NW-sided model. An assumption is that this has no specifications impact and no further study/work is needed. Meanwhile, for beam level spatial domain prediction, RAN2 aims to study both the UE-sided and NW-sided model. Further during the RAN2#130 meeting, there were actually some agreements made which consider spatial domain already, i.e.:

- The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain • PW length (for temporal case A) • Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional). MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted beam pattern (optional). MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted frequency (For inter-freq domain prediction)

Similarly, for NW-sided model, the agreements are agnostic to whether temporal or spatial domain prediction is utilized:

Agreements

- For network-side RRM measurement prediction, the legacy RRM measurement configuration and RRM measurement reporting framework can be used.   For L1-filtered beam-level RSRP reporting can be configured by setting co-efficient to zero.   FFS if there are specification impacts to support subcase 1 and 3 (if supported) and interference for cell level prediction.

- To support cell/beam level prediction, one enhancements is to report RRM measurement results per cell at multiple time instances in one measurement report for NW-sided model.

Therefore, we propose RAN to include intra-frequency spatial domain prediction in beam level in Rel-20 WI scope.

Observation 1: RAN2 has agreed to study intra-frequency spatial domain prediction in beam level and made some progress on this for both UE-sided model and NW-sided model.

Proposal 1: RAN to include intra-frequency spatial domain prediction in beam level in Rel-20 WI scope for both UE-sided model and NW-sided model.

## 2.2 Analysis for cell-level and beam-level prediction

Another leftover issue from last RAN plenary meeting is whether to consider both cell level and beam level prediction in the WI phase. This topic was discussed during RAN2#129bis meeting with the following conclusion:

L3 Beam-level prediction specification impact will be studied

Such conclusion was reached as current simulation results presented within both AIML for air interface SI/WI and in AIML for mobility SI prove that beam level prediction is feasible. Also, as clarified in [3], reporting of beam-level measurements together with cell-level measurements is crucial for mobility performance, as it is necessary to enable CFRA access to the target cell.

Observation 2: RAN2 confirmed that support of beam-level RRM measurement prediction is feasible and essential from the perspective of handover performance.

During RAN2#130 meeting, RAN2 also made several agreements with respect to beam-level prediction support, e.g.:

a) UE can be configured to log, at a certain logging periodicity, one or more of the following: - L3 cell level measurements, - L3 beam level measurements, - L1-filtered beam level measurements (if sub-case 1 and 3 is supported) - Cell ID (FFS CGI of serving cell. If CGI is unavailable, or for neighboring cells the UE logs PCI-ARFCN as a fallback) - Time info (if as agreed by AI/ML and/or if needed)

1 For network-side RRM measurement prediction, the legacy RRM measurement configuration and RRM measurement reporting framework can be used. For L1-filtered beam-level RSRP reporting can be configured by setting co-efficient to zero. FFS if there are specification impacts to support subcase 1 and 3 (if supported) and interference for cell level prediction. 2 To support cell/beam level prediction, one enhancements is to report RRM measurement results per cell at multiple time instances in one measurement report for NW-sided model.

Other than that, a common understanding in RAN2 is that the prediction configuration and reporting will be based on the existing RRM measurement framework, which already supports reporting of both cell-level and beam-level measurements.

Besides, in RAN #107 plenary meeting, a note that concerning the NW-sided models for RRM prediction is left [4]:

Conclusions: Only the following objectives are considered as baseline: - Specify support for RRM measurement prediction for both UE and network sided models [RAN2]: - Specify signalling and protocol aspect to enable reporting inference outcome of UE sided model for RRM measurement prediction - Specify signalling and protocol aspect to enable reporting information from UE for inference operation of network sided model for RRM measurement prediction - At least the use cases studied so far in RAN2 SI are supported: Temporal domain prediction and Frequency domain prediction. FFS Spatial domain pending RAN2 Study progress. FFS Cell level vs beam level prediction NOTE: network sided models can be concluded in RAN#108 after RAN2 discussions

So far, RAN2 #130 meeting [1] also agreed that for the NW-sided RRM measurement prediction, the legacy RRM measurement configuration and reporting framework can be used as baseline, with potential enhancements, e.g. to report RRM measurement results per cell at multiple time instances in one measurement report. Therefore, we propose RAN to remove the note and include NW-sided models in RRM prediction.

Agreements 1 For network-side RRM measurement prediction, the legacy RRM measurement configuration and RRM measurement reporting framework can be used. For L1-filtered beam-level RSRP reporting can be configured by setting co-efficient to zero. FFS if there are specification impacts to support subcase 1 and 3 (if supported) and interference for cell level prediction. 2 To support cell/beam level prediction, one enhancements is to report RRM measurement results per cell at multiple time instances in one measurement report for NW-sided model.

Based on the above analyses, we propose the following:

Proposal 2: Both cell-level and beam-level RRM measurement prediction should be included in the normative phase for both UE-sided and NW-sided models. Remove the NOTE about NW-sided models for RRM prediction in conclusions.

## 2.3 RAN4 work scope

In the SID objectives of AI/ML mobility in Rel-19, RAN4 was asked to analyse the following items

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4]

  - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed

  - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.)

  - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability.

  - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

In Rel-20, RAN4 should finalize the testing framework and procedures for the typical one-sided models in AI mobility. The followings could be reference, and the details may be further adjusted based on Rel-19 progress.

- Relation to legacy requirements

- Performance monitoring and LCM aspects considering use-case specifics

- Generalization aspects

- UE processing capability and limitations

- Post-deployment validation due to model change/drift

Proposal 3: In Rel-20, RAN4 should finalize the testing framework and procedures for the typical one-sided models in AI/ML for mobility.

# 3. Conclusions

In this paper, we provide our further considerations for AI/ML based mobility study in Rel-20 and give the following observation and proposals:

Observation 1: RAN2 has agreed to study intra-frequency spatial domain prediction in beam level and made some progress on this for both UE-sided model and NW-sided model.

Observation 2: RAN2 confirmed that support of beam-level RRM measurement prediction is feasible and essential from the perspective of handover performance.

Proposal 1: RAN to include intra-frequency spatial domain prediction in beam level in Rel-20 WI scope for both UE-sided model and NW-sided model.

Proposal 2: Both cell-level and beam-level RRM measurement prediction should be included in the normative phase for both UE-sided and NW-sided models. Remove the NOTE about NW-sided models for RRM prediction in conclusions.

Proposal 3: In Rel-20, RAN4 should finalize the testing framework and procedures for the typical one-sided models in AI/ML for mobility.

# References

R2_130_Chair_minutes_2025-05-23_final.

3GPP TR 38.744 V0.0.4.

R2-2501789, Way forward on study of beam-level prediction, vivo, Ericsson, ZTE, Huawei, HiSilicon, CATT, NTT DOCOMO, CMCC, Samsung, Interdigital

RP-250803, Summary on R20 5GA RAN2 Topics.