3GPP TSG RAN Meeting #108			RP-251496

Prague, Czech Republic, June 9-13, 2025

Agenda item:	16.1

Title:	Views on 6G Scenarios and Requirements

Source:	SK Telecom

Document for:	Discussion

1. Introduction

At RAN#107, the revised SID on 6G scenarios and requirements has been approved with updating the objectives as follows [1]:

This study item aims to - investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs. - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN - Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors. - Develop 3GPP requirements for 6G Radio for improvement of existing services and for new services. - Determine the applicability of legacy services to 6G Radio, and define radio requirements for these, as appropriate. - Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least: - Ensure appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting - Energy efficiency and energy saving: both for network and device - Enhanced spectral efficiency - Enhanced overall coverage, focus on cell-edge performance and UL coverage - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band - Target scalable and forward compatible design for diverse device types - Improved spectrum utilization and operations taking into account diverse spectrum allocations - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs - Aim at a harmonized 6G Radio design for TN and NTN, including their integration - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. - Define a time plan and steer work as appropriate for the RAN WGs during the 6G WG SI to deliver high-level decisions at least on the following areas: - Fundamental 6G radio design aspects: waveform, numerology, channel coding, etc… - Overall high-level aspects of 5G to 6G migration - RAN architecture and interfaces, including RAN-Core interface - Coordination of 6G AI/ML framework Note: The detailed time plan for the high-level decisions above will be decided at RAN#108 at the approval of the 6G RAN WG SI and is to be aligned with the RAN SI progress.

In this contribution, we aim at providing our view on 6G scenarios and requirements which should be taken into account for rel-20 6G study.

2. Discussion

2.1 6G Scenarios

Basically, 6G deployment scenarios should well support usage scenarios for 6G identified in 6G frame recommendation approved by the ITU-R in 2023 [2]. We think that the deployment scenarios identified for 5G in TR 38.913 [3] can be mostly reused with modifications such as ISD, carrier frequency, aggregated system bandwidth, BS/UE antenna configuration, user distribution, maximum mobility speed and other factors if deemed necessary. We think, at least, some of deployment scenarios need to be considered for 6G such as dense urban, rural, urban macro, indoor hotspot, high speed/highway, and satellite scenario. Of course, the deployment scenarios for 6G to be specified during the 6G RAN study are for evaluation purpose, and there is no mandating to deploy any of the option(s).

- The deployment scenarios identified for 5G in TR 38.913 can be considered as baseline for 6G deployment scenarios with modifications such as ISD, carrier frequency, aggregated system bandwidth, BS/UE antenna configuration, user distribution, maximum mobility speed and other factors if deemed necessary.

- At least, dense urban, rural, urban macro, indoor hotspot, high speed/highway, and satellite scenario can be considered as starting point for 6G deployment scenarios.

Regarding carrier frequency, the frequency spectrum being considered for potential 6G use (including those identified in WRC-23) can be further considered (e.g., 4.4-4.8 GHz, 6.425-7.125 GHz, 7.125-8.425 GHz, and so on) for the 6G deployment scenarios where applicable. Correspondingly, the aggregated system bandwidth can be adjusted properly depending on the band. Also, more advanced BS/UE antenna configuration needs to be further studied especially considering E-MIMO deployments around 7 GHz spectrum. In some of deployment scenarios, user distribution and maximum mobility speed may need updates to accommodate each of usage scenarios and requirements to be evaluated.

For the integrated sensing and communication (ISAC) usage scenario, the scenarios agreed in ISAC CM SI in RAN1 can be considered as starting point.

For AI and communication usage scenario, we do not see huge necessity to have separate deployment scenarios but prefer to have test environment common with other proper usage scenario (e.g., Immersive Communication).

- For ISAC usage scenario, the scenarios agreed in ISAC CM SI in RAN1 can be considered as starting point.

- For AI and communication usage scenario, we prefer to have test environment common with other proper usage scenario (e.g., Immersive communication).

2.2 6G Requirements

From our perspective, at least the following requirements should be considered for 6G RAN study in rel-20 (and also for further normative work).

- Improved user experience, TCO reduction, and creation of new revenue streams shall be taken into account for designing 6G radio.

- 6G radio shall be based on the principles that minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

- 6G radio shall be energy-efficient for both network and UE.

- 6G radio shall provide enhanced spectral efficiency compared with baseline.

  - Note: FFS on baseline

- 6G radio shall provide enhanced overall coverage while maintaining performance.

  - Note: For ensuring re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band, proper evaluation should be conducted during SI.

- 6G radio shall strive for smooth migration and simple architecture option.

  - At least, 6G radio shall support a standalone mode with multi-RAT spectrum sharing (MRSS) with 5G and carrier aggregation.

    - Note: Regarding how to realize this, the relevant study on specification impact shall be conducted.

  - Additionally, all other migration options being discussed shall be studied in rel-20 SI, and 6G radio shall support the option(s) only if justified based on the outcome of SI from both RAN and SA.

- 6G radio shall support interworking with 5G. Additionally, whether/how to support interworking with 4G shall be studied in rel-20 SI.

- 6G radio shall support network sharing (i.e. MOCN, MORAN and relevant enhancements if any).

- 6G radio shall support FR1, FR2 and also FR3.

- 6G radio shall support IDLE and CONNECTED mode mobility between TN and NTN at least with 5G. Additionally, whether/how to support IDLE and CONNECTED mode mobility between TN and NTN with 4G shall be studied in rel-20 SI.

- 6G radio shall support MIMO with providing higher spectral efficiency while ensuring coverage comparable to 5G.

  - The overall framework (inc.  CSI reporting mechanism and MIMO codebook) shall be studied for aiming at compactness and efficiency while providing performance.

- 6G radio shall support use of AI for efficient/effective RAN operation including performance enhancement, NW optimization, and energy saving.

  - How to ensure secure, efficient, and manageable model transfer and delivery shall be studied in rel-20 SI.

- 6G radio shall support enabling AI applications especially considering the orchestration of computing resources (e.g., xPU) shared by RAN and AI workload.

- 6G radio shall support sensing capabilities such as object detection, localization, and tracking, while being tightly integrated with RAN functionalities and AI-driven processing.

  - All sensing modes shall be studied in rel-20 SI, and then 6G radio shall support the mode(s) justified based on the outcome of RAN WG SI.

- 6G radio shall aim at providing the functionalities necessary for security, integrity and privacy.

- 6G radio shall support repeater with improving the positioning accuracy, minimizing the power consumption, enabling separate power control from access link, and enabling to provide the statistics per repeater.

- Any feature/functionality of 6G radio should be evaluated under practical assumption/configuration.

  - Note: The performance benefit can be over-estimated by evaluation with unrealistic assumption typically not used in commercial network(e.g.,5ms CSI-RS periodicity).

- The requirements in section 2.2 should be considered for 6G RAN study in rel-20.

3. Conclusion

In this contribution, the following proposals are provided.

- The deployment scenarios identified for 5G in TR 38.913 can be considered as baseline for 6G deployment scenarios with modifications such as ISD, carrier frequency, aggregated system bandwidth, BS/UE antenna configuration, user distribution, maximum mobility speed and other factors if deemed necessary.

- At least, dense urban, rural, urban macro, indoor hotspot, high speed/highway, and satellite scenario can be considered as starting point for 6G deployment scenarios.

- For ISAC usage scenario, the scenarios agreed in ISAC CM SI in RAN1 can be considered as starting point.

- For AI and communication usage scenario, we prefer to have test environment common with other proper usage scenario (e.g., Immersive communication).

- The requirements in section 2.2 should be considered for 6G RAN study in rel-20.

References

- RP-250810, “Revised SID: Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom, RAN#107.

- ITU-R M.2160-0, “Framework and overall objectives of the future development of IMT for 2030 and beyond”, Nov. 2023.

- 3GPP TR 38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies”.