3GPP TSG RAN Meeting #108								RP-251518

Prague, Czech Republic, June 9-13, 2025

Agenda Item:	15.1.1

Source:		CEWiT

Title:		Left over items on AIML for positioning accuracy enhancement case 3a

Document for:	Discussion

## 1.	Introduction

In Rel. 19 the Work Item (WI) on artificial intelligence/machine learning for NR air interface has the following objectives on AI/ML positioning accuracy enhancements [1]:

Provide specification support for the following aspects: - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

Out of the cases, case 1, case 3a and case 3b were the first priorities in Rel 19 discussion in RAN 1, RAN 2 and RAN 3. Work of case 1 and case 3b is completing as per expectation but for the case 3a progress is limited by the cross TSG coordination mainly with SA2.

The RAN 1 and RAN 3 agreements w.r.t to AIML assisted positioning case 3a are captured in Annex 1. Annex 1 gives the history of the progress in the Rel 19 for case 3a.

## 2.	RAN 3 progress and remaining work on case 3a

As part of the work on use case 3a for AI/ML based positioning accuracy enhancement, included in the WI NR_AIML_air-Core, RAN 1 and RAN3 is working on solutions. RAN 1 is working mainly on the measurements as input (part A) and output (Part B) to the AIML model for training and monitoring purpose.  RAN 3 has discussed on two solutions to retrieve positioning data at the NG-RAN viz solution 1 and solution 2. mentioned in the Annex 2.

Solution 1 is for the positioning information collection from the UEs whose positioning is underway (RAN3 calls it as opportunistic method). For solution 2, there will be an explicit trigger for the positioning information collection from the UEs served by the NG-RAN (RAN3 calls it proactive method). From a RAN3 perspective, both solutions are desirable and expected to standardize both solutions to provide better control at NG-RAN for the model training and monitoring in case 3a.

Observation 1: For AIML for positioning accuracy improvement, both solutions (solution 1 and solution 2) are desirable. It is expected to standardize both solutions to provide better control at NG-RAN for model training and monitoring in case 3a. Solution 2 is standalone whereas solution 1 being exploring ongoing positioning if available.

These solutions have SA2 impact. SA2 has already frozen the Rel 19 work and has very minimum time for the cross TSG work. RAN 3 sent the LS to SA2 as R3-252485 seeking feedback and feasibility on the solutions. Based on the initial discussion, SA2 sent the following reply in S2-2505943.

For solution 1 it may be desirable to document related LMF procedures in TS 23.273. There are SA2 impacts to support the solution but SA2 could address those impacts as alignment with RAN in Rel-19. For solution 2, SA2 concludes that this solution is not feasible in current frozen Rel-19. SA2 is currently discussing for data collection for training models to be used by the LMF for positioning (RAN case 3b) whether a user consent check and/or a LCS privacy profile check is required. SA2 is anticipating that similar checks could also be required for case 3a (both for solution 1 and solution 2).

SA2 clearly mentioned that solution 2 is not possible to complete in the Rel 19 and need to be completed in Rel 20 to make AIML positioning case 3a to completely specify.

Observation 2: Solution 1 being opportunistic method does not provide control to NG-RAN for the model training and monitoring. Therefore, defining solution 2 is important to complete case 3a related work.

Observation 3: Based on the reply-LS from SA2, it is evident that though SA2 can specify the solution 2, it is not possible to complete in rel. 19 as SA2 rel 19 specification are frozen. Therefore, it should be carried forward to Rel 20 5G-A.

Proposal 1: Left over items of AIML for positioning accuracy detection case 3a one of the two options should be considered,

Option 1: For Left over items of AIML for positioning accuracy detection case 3a, RAN should provide guidance to RAN 3 to complete the work in Rel 19. RAN 3 can provide exception sheet, if necessary, in next plenary to extend the work.

Option 2: Left over items of AIML for positioning accuracy detection case 3a in the Rel 20 AI/ML for Air Interface work item.

- The work can be restricted to RAN 3 and TSG SA especially SA2. Scope of the work can be restricted to solution 2 finalization.

## 3.	Rel 20 objectives for AIML assisted positioning case 3a

If the option1 is selected, then current objective of Rel 19 from [1] is sufficient and need not update it. Based on the progress in the next RAN 3 meeting work extension can be discussed.

If the option 2 is selected, then the objective of the leftover of AI/ML positioning accuracy enhancements AIML assisted positioning case 3a could be as below.

Provide specification support for the following aspects: - Positioning accuracy enhancements, AI/ML assisted positioning case 3a [RAN3/SA2]: - Support proactive method to retrieve positioning data at the NG-RAN for model training and monitoring purposes.

This work can be restricted to RAN 3 with TU budget of 1. For SA 2 work 1 TU should be sufficient but this can be coordinated with SA2 during joint session.

Proposal 2: If the work of the AI/ML positioning accuracy enhancements AIML assisted positioning case 3a leftover items, to be completed in Rel 20 then, following objective should be included in the AI/ML for Air Interface WI:

Provide specification support for the following aspects:

- Positioning accuracy enhancements, AI/ML assisted positioning case 3a [RAN3/SA2]:

  - Support proactive method to retrieve positioning data at the NG-RAN for model training and monitoring purposes.

Proposal 3: The TU allocation should be restricted to RAN 3 and it can be TU = 1. For SA 2 work TU budget can be coordinated with SA2 during joint session.

## 4.	Conclusion

This contribution proposes to consider the leftover item of AIML based positioning accuracy improvement particularly for AIML assisted positioning case 3a in Rel 20. Contribution provides the history and justification of the work happened for case 3a. Following are the observations and proposals.

Observation 1: For AIML for positioning accuracy improvement, both solutions (solution 1 and solution 2) are desirable. It is expected to standardize both solutions to provide better control at NG-RAN for model training and monitoring in case 3a. Solution 2 is standalone whereas solution 1 being exploring ongoing positioning if available.

Observation 2: Solution 1 being opportunistic method does not provide control to NG-RAN for the model training and monitoring. Therefore, defining solution 2 is important to complete case 3a related work.

Observation 3: Based on the reply-LS from SA2, it is evident that though SA2 can specify the solution 2, it is not possible to complete in rel. 19 as SA2 rel 19 specification are frozen. Therefore, it should be carried forward to Rel 20 5G-A.

Proposal 1: Left over items of AIML for positioning accuracy detection case 3a one of the two options should be considered,

- Option 1: For Left over items of AIML for positioning accuracy detection case 3a, RAN should provide guidance to RAN 3 to complete the work in Rel 19. RAN 3 can provide exception sheet, if necessary, in next plenary to extend the work.

- Option 2: Left over items of AIML for positioning accuracy detection case 3a in the Rel 20 AI/ML for Air Interface work item.

The work can be restricted to RAN 3 and TSG SA especially SA2. Scope of the work can be restricted to solution 2 finalization.

Proposal 2: If the work of the AI/ML positioning accuracy enhancements AIML assisted positioning case 3a leftover items, to be completed in Rel 20 then, following objective should be included in the AI/ML for Air Interface WI:

Provide specification support for the following aspects:

- Positioning accuracy enhancements, AI/ML assisted positioning case 3a [RAN3/SA2]:

  - Support proactive method to retrieve positioning data at the NG-RAN for model training and monitoring purposes.

Proposal 3: The TU allocation should be restricted to RAN 3 and it can be TU = 1. For SA 2 work TU budget can be coordinated with SA2 during joint session.

## 5.	References

[1] RP-250792- "Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface".

[2] R3-252485 - LS reply on AI/ML Positioning Case 3b

[3] S2-2505943 Reply LS on AI/ML Positioning Case 3a

[4] RAN3 #127-bis Meeting Report Wuhan, China, 07/04/2025 - 11/04/2025

[5] R3-252306 summary of Offline discussion on AIPHY

[6] DRAFT Meeting Report for S2 meeting: #S2-169 Ordinary meeting

## Annex 1:

### 1.1 RAN 1 agreements:

******** RAN1#116 ******** Agreement For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. • If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. • If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. • Note: details of the report are pending further discussion. ******** RAN1#116bis ******** Agreement For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB. Agreement For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: • channel measurement • quality indicator of channel measurement • time stamp of channel measurement Part B: • ground truth label (or its approximation) • quality indicator of label • time stamp of label Note: "Part A" and "Part B" terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement Agreement For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: • LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope. ******** RAN1#118bis ******** Agreement For training data collection of AI/ML based positioning, the quality indicator of timing information in Part A when reported is: • When applicable, the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455; FFS: details on how to associate quality indicator to timing information Agreement From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: • (Mandatory) timing information; • (Optional) Quality of the timing information; ◦ Existing IE “Timing Measurement Quality” can be reused. • (Mandatory) Time stamp. FFS: LOS/NLOS indicator. Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3. ******** RAN1#119 ******** Conclusion For measurement report of AI/ML assisted positioning Case 3a, regarding the report of LOS/NLOS indicator, • LOS/NLOS indicator can’t be reported independently from other measurements ******** RAN1#120 ******** Agreement For AI/ML based positioning, • When channel measurement is to be reported/sent, time stamp of channel measurement refers to the time instance when the channel measurement is performed. • When label is to be reported/sent, time stamp of label is the time instance for which the label is valid. Conclusion For AI/ML based positioning Case 3a, regarding the time stamp in a measurement report from gNB to LMF, • Existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective. Agreement • For training data collection of AI/ML based positioning 3a/3b, if Part A and Part B are generated by different entities, for pairing between a Part A entry and a Part B entry, the following is needed: • The time stamp of Part A (if Part A is transmitted) and the time stamp of Part B (if Part B is transmitted). • FFS: other information is not precluded (e.g., if Part B is valid for a duration) Note: Purpose such as “training data collection” will not necessarily be specified in RAN1 specifications. RAN1 send an LS to RAN3 and RAN2 to inform the above. ******** RAN1#120bis ******** Agreement For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes timing information, support the following for providing label and quality indicator of label, • The corresponding label is provided by LMF to gNB in the format of timing information. “Timing Measurement Quality” in 38.455 can serve as quality indicator of the label. Note: It is assumed that user data privacy of non-PRU UE is preserved. Conclusion For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes the LOS/NLOS indicator, • The corresponding label is provided from LMF to the gNB by using the existing IE LOS-NLOS-Indicator-r17. There is no need of a separate field for quality indicator of LOS/NLOS indicator. ******** RAN1#121 ******** Agreement For Case 3a, the FFS in RAN1#118bis agreement is resolved by updating the agreement as follows. Agreement (RAN1#118bis) From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: • (Mandatory) timing information; • (Optional) Quality of the timing information; ◦ Existing IE “Timing Measurement Quality” can be reused. • (Mandatory) Time stamp. • FFS: LOS/NLOS indicator. • (Optional) LOS/NLOS indicator with legacy format Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3. Note: It is up to RAN3 to decide whether the field of LOS/NLOS indicator should be removed or kept. Agreement On top of the RAN1#118bis agreement, Agreement (RAN1#118bis) For training data collection of AI/ML based positioning, the quality indicator of timing information in Part A when reported is: • When applicable, the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455; ◦ FFS: details on how to associate quality indicator to timing information

### 1.2 RAN 3 Agreements:

RAN3#125bis The existing LCS framework is re-used for AI/ML positioning. Case 3b: Re-using the Measurement procedure defined in NRPPa to support requesting and reporting the related measurements to LMF for Case 3b. The details of the related measurements are still pending and subject to further discussion in RAN1. NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF. The details of the measurements are still pending and subject to further discussion in RAN1. Case 3a: The AI/ML model (inference and training functions) is at the gNB. LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN. FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design. How the LMF obtains the ground truth label and related data information is outside of RAN3 scope. NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1). The details of the measurements are still pending and subject to further discussion in RAN1. WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model. RAN3#126 Turn WA to agreement: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model. Signalling of measurements as the inference output for case 3a can re-use the existing procedures for positioning measurement specified over NRPPa. WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed. FFS how the gNB requests the LMF and whether this request triggers positioning for the UE being requested for part B information (as defined by RAN1). FFS on the protocol used (NRPPa or NGAP) for the data collection request. FFS how the UE is identified. Common understanding is that the existing Time Stamp IE, Measurement Quality IE and LoS/NLoS indicator IE in the TRP measurement result can be re-used. FFS if LMF should be aware that the reported measurements are AI/ML generated. RAN3#127 Turn the “WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.” to an agreement. For case 3a, it should be possible to collect data in both an “opportunistic” (gNB collects available data from ongoing positioning sessions) and “proactive” (gNB triggers data collection at the LMF, e.g., which may trigger positioning) manner. For case 3a, it should be possible for the gNB to collect data for both served and non-served UEs. WA: For case 3a data collection, Part A is known internally by the gNB and is not necessarily signalled outside the gNB. RAN3#127bis Case 3a data collection: An NRPPA based solution could be used to collect UE location information in an opportunistic way, i.e. from UEs that are being positioned. FFS on Whether to introduce an NGAP based procedure to collect UE location information in a proactive way, i.e. from UEs that are being served by the NG-RAN. Pending feedback from SA2. RAN3#128 Define the Positioning Data Collection Needed IE as a new bitmap. The requested information in the bitmap is timing measurement (UL-TDOA, gNB Rx-Tx Time Difference) together with optional LoS/NLoS indicator. FFS if UE location can be requested. The Positioning Data Collection Needed IE is introduced in the MEASUREMENT REPORT message. Introduce POSITIONING DATA COLLECTION REPORT message as class 2 procedure including Positioning Data Information IE and RAN/LMF Measurement IDs Introduce an optional Positioning Data Unavailable IE (FFS on encoding) when the Positioning Data Information IE is absent in the POSITIONING DATA COLLECTION REPORT message. The codepoints indicating the reason. RAN3 confirms that Performance monitoring is done in the RAN in case 3a. RAN3 confirms the WA: For case 3a data collection, Part A is known internally by the gNB and is not signalled over any network interface. Introduce the new flag “Inferred measurement” in the TRP Measurement Result to align with RAN1 conclusion

## Annex 2:

### 2.1 Proposed solution 1 in RAN 3 LS:

Solution 1: Positioning Information collection for UEs under positioning

- The LMF sends the NRPPa MEASUREMENT REQUEST message to one or more gNBs according to positioning procedures.

- The gNB(s) determines that data collection is needed for the UE being positioned.

- The gNB(s) sends the NRPPa MEASUREMENT RESPONSE message to the LMF and indicates that data collection is needed for the UE being positioned.

Note: Steps 1 to 3 may occur while the LMF performs one or more of the positioning procedures described in clauses 6.11.1, 6.11.2 and 6.11.3 of TS 23.273 [35].

- The LMF sends the (new) NRPPa POSITIONING DATA COLLECTION REPORT message to the gNB(s) which indicated that positioning data collection is needed for the UE being positioned. The message includes information related to UE location and correlation information that enables the gNB to correlate the information related to UE location with information related to UL measurements (e.g., LMF Measurement ID and RAN Measurement ID).

### 2.1 Proposed solution 2 in RAN 3 LS:

Solution 2: Positioning Information collection for UEs served by the NG-RAN which are not being positioned.

- The gNB wishes to receive positioning information, consisting of labels (e.g. UE location information) to form a data set for training. Based on gNB implementation, the gNB decides when/if to trigger a (new) NG: POSITIONING INFORMATION REQUEST message to the serving AMF of the served UE(s).

- The POSITIONING INFORMATION REQUEST message is an NGAP non-UE associated message containing e.g. the NGAP UE IDs of the UEs for which positioning information is required or other criteria.

- The AMF receives the request message from the NG-RAN node. If the requested information is already available at AMF as part of UE positioning context, the AMF responds directly with the message in step 6. Otherwise, it selects an LMF to request positioning of each UE for Positioning Information collection purposes. 
The AMF triggers a Positioning Information Request towards the LMF (it could consist of the (legacy) Nlmf_Location_DetermineLocation service operation) to request the current location of the UE.

- The LMF performs one or more of the positioning procedures described in clause 6.11.1, 6.11.2 and 6.11.3 of TS 23.273.

- The LMF returns the requested information towards the AMF. The message Nlmf_Location_DetermineLocation Response might be reused.

- The AMF returns the requested label(s) to the NG-RAN node in a POSITIONING INFORMATION RESPONSE message. The AMF may return instead a failure message with appropriate cause value if the procedure does not succeed.

- The gNB combines the received label with the TRP measurements to form the data needed.