3GPP TSG RAN Meeting #108	RP-251527

Prague, Czech Republic, June 9-13, 2025

Agenda item:	15.3.1

Source:	Nokia

Title:	AI/ML for NG-RAN in Rel-20

Document for:	Discussion

# 1	Introduction

At RAN#107, the following way forward was endorsed for AI/ML for NG-RAN:

From slide 18 of [1]: - Rel-20 AI/ML for NG-RAN should be based on current 5G architecture and interfaces. - The objectives of AI/ML for NG-RAN in Rel-20 are listed below: - Study the following new AI/ML based use cases and down-scope (1 or 2 use cases) in next plenary [RAN3]: - QoE Optimization; - Network energy saving - Mobility (including Multiple-hop target node UE trajectory) - Note: Other Leftovers depend on Rel-19 outcome, if any. - Note: Coordination based on LSs with other working groups, when needed. - 6 months for the Rel-20 Study Item on AI/ML for NG-RAN, followed by 12 months for the subsequent Work Item.

In this paper, we provide our views on the scope and objectives for AI/ML for NG-RAN in Rel-20.

# 2	Discussion

In 5GA workshop, different use case cases were discussed in the context of AI/ML for NG-RAN for Rel-20 and particular emphasis was given to use cases that address “real and critical commercial deployment needs”. NG-RAN AI/ML work has addressed several use cases in Rel-18 and Rel-19. In Rel-19 work item phase the topics of Energy Saving and Multi-hop UE Trajectory were dropped. This down-scoping was done in order to adapt the work item load to the available TUs, and not due to missing importance or relevance of those topics.  In our analysis of Rel-20 work item scope, we therefore think that we should focus on important use cases with real commercial value. We think that this criterion is met for Mobility Optimization and Energy Saving use cases. Another principle that we think should remain valid is that the Rel-20 NG-RAN AI/ML study/work should be RAN3 led and should avoid UE impacts.

## 2.1	Mobility Optimization use case

In Rel-18, UE Trajectory prediction is limited to the cells of the target NG-RAN node and it is calculated on a cell-level granularity. In addition, Predicted UE Trajectory over the cells of the target NG-RAN node is limiting since it does not exploit the full potential of predicted trajectory. It would be very useful for a node to receive trajectory information not only over its own cells but also over the cells of other neighbouring NG-RAN nodes. In this way, it can better determine how traffic is expected to depart from its coverage.

In Rel-19 NG-RAN AI/ML study item, we looked into multi-hop trajectory prediction and corresponding multi-hop measured UE Trajectory, but the topic was dropped due to necessary down-scoping.

In addition, the current predicted trajectory utilizes information only from connected mode UEs. However, we can assume that mobility patterns for idle mode and connected mode UEs are the same and therefore, additional idle or inactive mode input could be used for UE Trajectory prediction. This can improve the input data availability for training a UE Trajectory prediction model.

Furthermore, in Rel-18 UE Trajectory prediction is limited to cell-based UE Trajectory. In our view, finer granularity of predicted UE trajectory in terms of beams will enable improved per-beam decisions in terms of mobility and resource allocation.

We believe that the discussions in RAN3 in Rel-18 work item phase, as well as in Rel-19 study item phase, on the Mobility Enhancements use case were extensive and therefore, this topic should be concluded in Rel-20.

Proposal 1:	Rel-20 enhancements for the mobility optimization use case should include UE Trajectory prediction enhancements, i.e. support multi-gNB, RRC_IDLE/RRC_INACTIVE states, and beam-level (predicted) UE Trajectory.

## 2.2	Energy Saving use case

The AI/ML Energy saving solution in Rel-18 assumes that a node wishing to initiate an action (cell activation/deactivation) is able to determine the energy cost at the target node that this action will incur, in particular due to the related offloading of UEs. The energy cost measurement is directly related to an energy consumption measurement at the node. With the rise of electricity prices and the environmental concerns, there are more and more efforts to consider sustainability and renewable energy. Thus, we believe that in Rel-20, RAN3 should include an objective to consider the effect of renewable energy sources on the AI/ML Energy Saving decisions e.g., handovers.

In addition, the energy cost metric is defined on a node level granularity and hence the energy cost reported by the NG-RAN node receiving offloaded traffic doesn’t necessarily reflect the actual cost pertaining to the offloading action, but will in many cases include energy cost resulting from other handovers to/from other neighbouring nodes.

In Rel-19 study item phase, RAN3 attempted to address finer granularity Energy Cost. Even though no conclusion could be drawn on this topic, a number of possible approaches to improve Energy Cost granularity were discussed such as Energy Cost per group of cells based on energy consumption measurements associated to the hardware serving the group of cells, Energy Cost per cell based on energy consumption estimations, Energy Cost per one or more handovers based on energy consumption estimations, Energy Cost per gNB-DU based on energy consumption measurements associated to the hardware serving the gNB-DU. Thus, we believe that in Rel-20, RAN3 should include an objective to address finer granularity Energy Cost.

Proposal 2:	Rel-20 enhancements for the energy saving use case should include considerations of renewable energy sources in the AI/ML Energy Saving decisions and Energy Cost of finer granularity.

### 2.3	Leftovers from the on-going Rel-19 work

Depending on the progress of Rel-19 some aspects may not be possible to fully complete on time, e.g., Continuous MDT. Also, it may not be possible to cover all aspects of the new Rel-19 use cases, e.g. energy efficiency aspects should be considered for all features, but time might not allow to handle such aspects. One such example is for the AI/ML Coverage and Capacity Optimization (CCO) use case.

We think that those topics should be considered part of Rel-20 work item, with decision taken at the end of Rel-19.

Proposal 3:	RAN to determine at the end of Rel-19 any leftovers from currently ongoing Rel-19 work that should be included in Rel-20 work item phase.

### 2.4	Others

Another use case that has been proposed for Rel-19 and is now also proposed for Rel-20 is QoE Optimization. The available RVQoE metrics are at the moment limited to application layer buffer level and playout delay for media startup. We think that those metrics are not suitable for AI/ML optimization and introducing new metrics in the end of 5G would have UE impacts, limiting the number of UEs that may possibly implement the feature. For instance, it is almost impossible for the RAN to associate an application layer buffer level to a RAN related action. This is because a gNB does not have information about how the application layer sends the data, whether it uses a smaller buffer or a longer buffer instead. Thus, in our view AI/ML QoE Optimization is more relevant for 6G use cases, where additional RVQoE metrics could be exploited.

# 3	Potential work item objectives

1)	Enhanced support of Rel-18 and Rel-19 use cases [RAN3]:

-	Mobility optimization, including:

-	UE Trajectory prediction enhancements, namely multi-hop UE Trajectory prediction across gNBs, across RRC states, and beam level granularity of (predicted) UE Trajectory.

-	Energy saving, including:

-	Renewable energy sources in the AI/ML Energy Saving decisions, and Energy Cost of finer granularity.

2)	Support of leftovers from the Rel-19 AI/ML for NG-RAN [RAN3].

Note: The Rel-19 leftovers to be included in Rel-20 (if any) will be identified at RAN#109.

# 4	Conclusions

In this contribution, we discussed the potential objectives for the Rel-20 AI/ML for NG-RAN WI, following the guidance endorsed at RAN#107 in [1]. We make the following proposals:

Proposal 1:	Rel-20 enhancements for the mobility optimization use case should include UE Trajectory prediction enhancements, i.e. support multi-gNB, RRC_IDLE/RRC_INACTIVE states, and beam-level (predicted) UE Trajectory.

Proposal 2:	Rel-20 enhancements for the energy saving use case should include considerations of renewable energy sources in the AI/ML Energy Saving decisions and Energy Cost of finer granularity.

Proposal 3:	RAN to determine at the end of Rel-19 any leftovers from currently ongoing Rel-19 work that should be included in Rel-20 work item phase.

Based on the above proposals, we have provided a list of work item objectives in section 3. Thus, the following is further proposed:

Proposal 4:		Adopt the objectives in section 3 for the Rel-20 AI/ML for NG-RAN work item.

# References

- RP-250812 Summary for RAN Release 20 5G-Adv, RAN Vice Chairs, RAN1, RAN2, RAN3, RAN4 Chairs