3GPP TSG RAN Meeting #108 	RP-251531

Prague, Czech, June 9-13, 2025

Source: 	Huawei, HiSilicon, Verizon, China Telecom, China Unicom, Vodafone, Qualcomm

Title: 	Discussion on SBFD-SBFD adjacent channel operation

Agenda Item:	9.3.1.2

Document for:	Discussion/Decision

Background

After the comprehensive study carried out by the Rel-18 SI for SBFD, the BS RF requirements for SBFD have been extensively discussed with good progress since the first meeting of the Rel-18 WI for SBFD. However, the RF requirements for SBFD-SBFD co-ex case for FR1 WA or MR BS is not included in Rel-19 WI. In this contribution we provide our consideration and proposal for the important deployment scenario.

Discussion

At last RAN4 meeting RAN4#115, the following way forward is approved on the deployment scenarios for FR1 SBFD BS. Although RAN4 identified the importance of the deployment scenarios on SBFD-SBFD co-ex, due to the work load and limited time in Rel-19, RAN4 agreed not to introduce the specific requirements for SBFD-SBFD co-ex case for FR1 WA or MR BS in Rel-19, and SBFD-SBFD co-ex case can be considered for Rel-20 [2].

Whether SBFD-SBFD co-ex case should be in Rel-19 scope - Way forward - RAN4 do not introduce the specific requirements for SBFD-SBFD co-ex case for FR1 WA or MR BS in Rel-19. SBFD-SBFD co-ex case can be considered for Rel-20.

We think SBFD-SBFD co-ex case and corresponding RF requirements should be included in the Rel-20 5G-Advanced RAN4 Package. The justifications are discussed as below.

Firstly, for TDD-SBFD co-ex, ACIR enhancement is limited by ACLR of TDD BS. Limited ACIR enhancement cannot help much for better co-existence performance. Co-existence is only feasible for 100% grid-shift, which is a quite limited deployment scenario.

We think the potential SBFD deployment scenarios especially for FR1 WA could be quite limited if RAN4 only focus on SBFD-TDD co-existence and define the RF requirements accordingly. Following analyses are mainly based on the Rel-18 SBFD SI outcomes.

- Per the conclusions for Case 3 SBFD UL co-existence evaluation in TR 38.858 clause 11.3, the throughput degradation is increased with grid-shift lower than 100%.

Case 3 SBFD UL co-existence conclusions

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro Scenario 1 | FR1 | Under baseline assumptions, SBFD UL throughput degradation is observed only for cell edge throughput, and minor degradation but acceptable to some companies for average throughput. With other assumptions (higher BS Tx power and lower grid-shifts), the degradation is increased for cell edge throughput and average throughput. |

But it should be noted that the 100% grid-shift might be too ideal for macro BS comparing with real deployment.

Secondly, for lower grid shift, e.g. 10%/20%/50%, for co-existence the required ACIR enhancement is far exceed the legacy BS ACIR, whose enhancement cannot exceed 1.2 dB in FR1 based on the limitation of the legacy BS ACLR simulation assumption.

More specifically, we reran the RAN4 co-ex simulation with 10%/20%/50% grid-shift to see how much UL throughputs degradation can be reduced with the help from the increased ACIR. Note that even for 50% grid-shift the baseline UL throughput loss i.e. applying the existing 42.5dB ACIR determined by 45dB BS ACLR and 46dB BS ACS, is 100% at cell-edge and 16.72% at cell-average, respectively.

|  | 10% grid shift UL throughput loss |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACIR(dB) UL Thr. Loss @X%-tile |  | 42.5 (baseline) | 52.5 | 62.5 | 65 | 67.5 | 70 | 72.5 | 75 | 77.5 | 80 |
| @5% (cell-edge) |  | 100% | 100% | 100% | 100% | 100% | 100% | 12.98% | 8.24% | 5.22% | 2.71% |
| @50% (cell-average) |  | 33.2% | 13.17% | 4.51% | 3.39% | 2.52% | 1.99% | 1.54% | 1.04% | 0.67% | 0.49% |

|  | 20% grid shift UL throughput loss |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACIR(dB) UL Thr. Loss @X%-tile |  | 42.5 (baseline) | 52.5 | 62.5 | 65 | 67.5 | 70 | 72.5 |
| @5% (cell-edge) |  | 100% | 100% | 22.18% | 18.31% | 9.96% | 4.29% | 2.23% |
| @50% (cell-average) |  | 22.88% | 10.09% | 2.64% | 1.91% | 1.40% | 0.98% | 0.64% |

|  |  |  | 50% grid shift UL throughput loss |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACIR(dB) UL Thr. Loss @X%-tile |  | 42.5 (baseline) |  | 50 | 52.5 | 55 | 57.5 | 60 |
| @5% (cell-edge) |  | 100% |  | 100% | 100% | 9.08% | 5.34% | 1.96% |
| @50% (cell-average) |  | 16.72% |  | 3.51% | 2.61% | 1.94% | 1.27% | 0.77% |

From the above table, it is found that,

If the grid shift is set to 10%, around 36 dB improvement on ACIR can be enough to guarantee <5% UL throughput loss.

If the grid shift is set to 20%, around 28 dB improvement on ACIR can be enough to guarantee <5% UL throughput loss.

If the grid shift is set to 50%, around 16 dB improvement on ACIR can be enough to guarantee <5% UL throughput loss.

However, for SBFD-SBFD co-ex, ACIR can be improved thanks to the nature of SBFD design (e.g. uplink subband some distance away from inter-operator band), so better co-existence performance can be achieved.

For a new wide TDD band without existing deployment of legacy TDD base stations, it would be more beneficial to deploy SBFD BS to increase UL coverage and reduce latency to realize the full potential of SBFD technology. In this case improved adjacent channel co-existence between an SBFD BS with another SBFD BS would be feasible by enhancing the ACIR, i.e. defining additional requirements both transmitter side and receiver side. As shown in below figure, thanks to the nature of SBFD design, for protection of SBFD UL of operator B, the SBFD DL of operator B can be adopted as guard band for filter implementation of adjacent operator A. It would not impose additional implementation difficulty for taking the advantage of narrow band filter.

In conclusion, we think SBFD-SBFD co-existence for FR1 macro and micro BS is an important deployment scenario and should be included in Rel-20 WI.

Proposal: It is proposed to continue the standard work to define RF requirements for SBFD-SBFD adjacent operator scenario in Rel-20 5G-Advanced WI.

    - Define FR1 Macro/Micro RF requirements for SBFD-SBFD adjacent operator scenario, considering the expected spectrum allocation and sub-band sizes, and relevant co-existence and co-location deployment scenarios

      - TX unwanted emission

      - RX in-band blocking

Conclusion

In this contribution we discussed the deployment scenarios for FR1 SBFD BS.

Proposal: It is proposed to continue the standard work to define RF requirements for SBFD-SBFD adjacent operator scenario in Rel-20 5G-Advanced WI.

    - Define FR1 Macro/Micro RF requirements for SBFD-SBFD adjacent operator scenario, considering the expected spectrum allocation and sub-band sizes, and relevant co-existence and co-location deployment scenarios

      - TX unwanted emission

      - RX in-band blocking

- References

- TR 38.858.

- R4-2508713, Way Forward for [115][307] NR_duplex_evo_BSRF