3GPP TSG-RAN Meeting #108                                                RP-251540

Prague, CZ, Jun 9-13, 2025

Source: 	Huawei, HiSilicon

Title: 	Proposals for RAN4-led Rel-20 Candidate topics impacting both RF/OTA and Demod/RRM

Agenda Item:	15.4.4

Document for:	Discussion/Decision

Introduction

In RAN#107 the cross RF and baseband topics were summarized [1] as below.

Cross RF and baseband topics (Based on limited time, discussions concerning common interest were not addressed for this topic) - NR FR1 DL fragmented carrier - Depending on the conclusion of Rel-19 SI - Extension of low-low band CA via switching - FFS on the scenarios for Rel-20 - Irregular channel bandwidth (flexible channel bandwidth) - Check what the new aspects to be evaluated or new solutions to be introduced are? Need consider the other issues, e.g., guard band. - FFS whether it should be in 5G-A or 6G. - 3MHz channel bandwidth for eRedcap devices - Minimize the impact on RAN1/RAN2 impacts, if any - FFS on whether and what additional features should be supported by 3MHz eRedCap - Intra-band non-collocated EN-DC/NR-CA deployment - AI/ML assisted receiver - FFS whether it should be in 5G-A or 6G - ATG evolution - UAV enhancement

In this contribution we share our view particularly on the topic of “extension of low-low band CA via switching”.

Discussion

In Rel-19, the following scenarios have been discussed for Low-low band CA via switching when the WID was proposed and also during the RAN4 discussions [2], [3].

- Scenario #1: Switching between FDD Tx/Rx and SDL Rx from UE perspective.

  - Band combinations: CA_n5A-n29A, CA_n12A-n29A, CA_n14A-n29A, CA_n28A-n67A, CA_n29A-n71A

- Scenario #2: Switching between FDD Tx/Rx and FDD Rx+SDL Rx from UE perspective, i.e., dual DL

  - Band combinations: CA_n5A-n29A, CA_n12A-n29A, CA_n14A-n29A, CA_n28A-n67A, CA_n29A-n71A

- Scenario #3: Switching between FDD Tx/Rx and FDD Tx/Rx

  - Band combinations: CA_n26A-n106A

Only the scenario #1 is addressed in Rel-19 due to limited time and the switching is based on a semi-static pattern based on RRC configuration. In Rel-20, the scenario #2 and #3 were proposed and the dynamic switching scheme with more flexibility was also proposed. The reason not to support simultaneous Tx-Rx cross bands or not to support the traditional CA is that the bands are (partially) overlapping or there is no gap between bands such that it is very challenging to implement the proper filter.

Besides the above scenarios, we also think the switching scenarios between FDD Tx/Rx and FDD Tx/Rx, or between TDD and TDD, or between FDD and TDD are also important, which is not limited by baseband capability rather than by RF filter design. However, in our view the similar switching scheme as for low-low band switching can be reused.

In the future network, the operators would deploy more bands/CCs in a site, e.g., more than 4 bands, to boost the capacity. But most likely, the UE baseband capability of both DL and UL may not be able to match such big number of bands especially for low and middle end of UEs, because the UE is sensitive to the cost and power consumption and the traffic of wireless service may not be rapidly increased. Considering the actual traffic requirements in the near future, 3DL or 4 DL capability for high end UE and 2DL (200MHz) capability for low and middle end UEs would still be prevail in our view.

Then the question is how to optimize the performance of 2DL, or 3/4DL capable UE in the network with more than 4 bands/CCs. In our view, the answer would be to do the switching between Cells. If the UE can switch between the cells, the network can do the dynamic load balancing well and thus the UPT could be improved compared to UE not being capable of switching.

As shown in figure 1, we compared the performance of UE capabilities

- Capability #1: UE capable of only 1 DL CC, and being configured and fixed on one DL CC.

- Capability #2: UE capable of selecting 1 DL CC out of 2 CCs

- Capability #3: UE capable of selecting 2 DL CCs out of 4 CCs

- Capability #4: UE capable of 4DL CA

From the system level simulations, we observes that the UPT performance of UE capability #2 is much better than single carrier capability, i.e., capability #1. The performance of capability #3 is almost the same as full CA capability, i.e., capability #4, under the assumed traffic model.

The component of such scheme would be the switching within a pair of bands. For example, take band A+B+C+D for an example. If we can pair A+B and pair C+D, then the switching could be conducted between A and B or between C and D. Then to some extent, selection 2 DL CCs out of 4 CCs could be implementable. Due to the TU limitation, we may not have chance to enable the selection across all the configured bands, but at least it would be beneficial to use the band pair switching as an component to enhance the performance for low/middle end UE or even for high-end UE when the CC# numbers of BS is higher than the UE capability.

Thus from our side, the whole list of scenarios for inter-band CA via switching is as follows

- Scenario #1: Switching between FDD Tx/Rx and SDL Rx from UE perspective.

  - Band combinations: CA_n5A-n29A, CA_n12A-n29A, CA_n14A-n29A, CA_n28A-n67A, CA_n29A-n71A

- Scenario #2: Switching between FDD Tx/Rx and FDD Rx+SDL Rx from UE perspective, i.e., dual DL

  - Band combinations: CA_n5A-n29A, CA_n12A-n29A, CA_n14A-n29A, CA_n28A-n67A, CA_n29A-n71A

- Scenario #3: Switching between FDD Tx/Rx and FDD Tx/Rx

  - Band combinations: CA_n26A-n106A

- Scenario #4: Switching between FDD Tx/Rx and TDD Tx/Rx from UE perspective.

  - Band combinations: TBD

- Scenario #5: Switching between TDD Tx/Rx and TDD Tx/Rx from UE perspective

  - Band combinations: TBD

According to the previous discussions, most companies were focusing on Scenario #2 to address RF challenging. Due to time limit, we are not against focusing on a certain of scenarios according to operators demand. But when RAN1 starts their work on either dynamic scheduling scheme or enhancement of switching pattern, we would like to propose to design the switching scheme in a band agnostic way rather than focusing on FDD-SDL case only.

Besides, it would not be desirable to design the scheme for FDD-SDL only in Rel-20 and afterwards when FDD-FDD or TDD-TDD cases are required in the future release, the additional scheme will be designed again. We prefer a complete and non-fragmented solution. So we propose that

Proposal: Specify the dynamic switching scheme or extend the switching scheme in a band agnostic way to cover the FDD-SDL, FDD-FDD, FDD-TDD and TDD-TDD cases in one release, at least for RAN1 scheme.

Summary

In this contribution, we share our view on the Rel-20 candidate WI proposal of band-band switching, we propose

Proposal: Specify the dynamic switching scheme or extend the switching scheme in a band agnostic way to cover the FDD-SDL, FDD-FDD, FDD-TDD and TDD-TDD cases in one release, at least for RAN1 scheme.

- References

[1] RP-250817, Moderator's summary for RAN4 led REL-20 topics, RAN4 Chair, RAN#107

[2] R4-2508119, Draft TR for 38.768 LB-CA via switching, Apple, RAN4#115

[3] RP-250569, Motivation for low-band carrier aggregation via switching enhancement in Rel-20, Telus, Bell Mobility, Murata, AT7T, BT plc, Apple, RAN#107.

- Annex: system simulation assumptions

| ISD | 400 m |
| --- | --- |
| Topology | 7 sites, 21 sectors |
| Bands | 3 bands, 4CCs |
| Modulation | Up to 256QAM |
| CSI periodicity | 5ms |
| MIMO | SU-MIMO only |
| UE number per cell | 10 users |
| Indoor users: outdoor | 8:2 |
| Traffic model | FTP model: 1 packet per 5s，packet-size：0.5MByte |
| DL switching gap | 35us |