3GPP TSG-RAN Meeting #108                                                 RP-251543

Prague, CZ, Jun 9-13, 2025

Source: 	Huawei, HiSilicon, Deutsche Telekom AG

Title: 	MPR reduction for upper 6GHz by ACLR relaxation

Agenda Item:	15.4.2

Document for:	Discussion/Decision

Introduction

This contribution is a companion paper of [1] to elaborate the motivation and scope of the proposal.

Discussion

Review of a R17 SI

A release 17 SI of “Study on International Mobile Telecommunications (IMT) parameters for 6.425 - 7.025 GHz, 7.025 - 7.125 GHz and 10.0 - 10.5 GHz” performed co-existence study. Note that the SI is called FS_NR_IMT_param hereafter in this contribution.

According to the corresponding TR 38.921 for FS_NR_IMT_param, it was agreed to specify 26 dB ACLR for the 6.425~7.125 GHz frequency range assuming UE PC3. This value, however, has not yet been incorporated into TS38.101-1, even though the study results for BS side was updated accordingly.

Observation 1: TR 38.921 for FS_NR_IMT_param agreed to specify not 30 dB, but rather 26 dB ACLR for the 6.425~7.125 GHz frequency range assuming UE PC3, while it has not been incorporated into TS38.101-1.

Relation between ACLR and MPR

MPR may be needed depending on relation between various configured/scheduled parameters such as the number of RBs, its position, MCS, waveform and gating factors such as ACLR, SEM, EVM, etc.

For lower order modulation(s) like QPSK, ACLR can be a gating factor for outer RBs allocation especially for wider RBs allocations. Thus, relaxing ACLR can reduce MPR for those cases. According to our initial analysis, it would be possible to reduce MPR by up to 0.5~1.0 dB for QPSK DFT-s-OFDM and up to 1.0~1.5 dB for QPSK CP-OFDM, respectively. See the detailed simulation results in Annex.

Observation 2: 4 dB ACLR relaxation would enable MPR to be reduced by 0.5~1.0 dB for QPSK DFT-s-OFDM and 1.0~1.5 dB for QPSK CP-OFDM, respectively.

Views on 6.425~7.125 GHz

In 6G workshop held in March 2025, many of the companies who shared views on spectrum showed the importance of upper 6 GHz incl. 6.425~7.125 GHz, where some of the t-docs showed necessity to compensate for increased path loss due to higher frequency range than the existing below 6 GHz spectrum.

Observation 3: The importance of reducing MPR for upper 6GHz spectrum has been well acknowledged.

Apart from PC2 or higher, given that co-existence for PC3 between IMT systems for this frequency range was completed in the R17 SI of FS_NR_IMT_param, there is no need to wait for introducing more appropriate ACLR for PC3 together with more appropriate, i.e., reduced MPR. The framework for this specific scenario and associated work process as well as the requirements can be reused when the co-existence study for higher power of UE is completed.

Observation 4: ACLR of 26 dB with reduced MPR for PC3 is introduced into R20, while new ACLR for higher power is introduced when the co-existence studies with higher power than PC3 are completed.

Proposal: Introduce a relaxed ACLR of 26 dB together with reduced MPR in R20 with consideration of the observations.

Summary

We obtained following observations and proposal.

Observation 1: TR 38.921 for FS_NR_IMT_param agreed to specify not 30 dB, but rather 26 dB ACLR for the 6.425~7.125 GHz frequency range assuming UE PC3, while it has not been incorporated into TS38.101-1.

Observation 2: 4 dB ACLR relaxation would enable MPR to be reduced by 0.5~1.0 dB for QPSK DFT-s-OFDM and 1.0~1.5 dB for QPSK CP-OFDM, respectively.

Observation 3: The importance of reducing MPR for upper 6GHz spectrum has been well acknowledged.

Observation 4: ACLR of 26 dB with reduced MPR for PC3 is introduced into R20, while new ACLR for higher power is introduced when the co-existence studies with higher power than PC3 are completed.

Proposal: Introduce a relaxed ACLR of 26 dB together with reduced MPR for PC3 in R20 with consideration of the observations.

- References

[1] RP-250284, Views on Rel-20 RAN4-led topics, Huawei, HiSilicon, RAN#107

- Annex 1 (QPSK)

| ACLR (dB) | CP-OFDM | DFT-s-OFDM |
| --- | --- | --- |
| 30 |  |  |
| 29 |  |  |
| 28 |  |  |
| 27 |  |  |
| 26 |  |  |

- Annex 2 (256QAM)

| ACLR (dB) | CP-OFDM | DFT-s-OFDM |
| --- | --- | --- |
| 30 2nd |  |  |
| 26 2nd |  |  |

For reference: Table 6.2.2-1 Maximum power reduction (MPR) for power class 3

| Modulation | MPR (dB) |  |  |
| --- | --- | --- | --- |
|  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| Pi/2 BPSK w/ Rel-15 DMRS | ≤ 3.51 | ≤ 1.21 | ≤ 0.21 |
|  | ≤ 0.52,3 | ≤ 0.52 | 02,4 |
| Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.52,3 | 02 | 02,4 |
| QPSK | ≤ 1 |  | 05 |
| 16 QAM | ≤ 2 |  | ≤ 1 |
| 64 QAM | ≤ 2.5 |  |  |
| 256 QAM | ≤ 4.5 |  |  |
| QPSK | ≤ 3 |  | ≤ 1.5 |
| 16 QAM | ≤ 3 |  | ≤ 2 |
| 64 QAM | ≤ 3.5 |  |  |
| 256 QAM | ≤ 6.5 |  |  |