3GPP TSG RAN Meeting #108								RP-251556

Prague, Czech Republic, June 9-13, 2025

Agenda Item:	16.1

Source:	Xiaomi

Title:	Discussion on minimum TPR for IMT-2030

Document for:	Discussion and Decision

- Introduction

In RAN#106 meeting, 14 legacy TPRs of IMT-2020 are agreed as candidates IMT-2030 TPRs and an LS has been sent to ITU-R.

In RAN#107, some new candidate TPRs were agreed to be defined, including positioning accuracy, sensing-related capabilities, AI-related capabilities, Energy efficiency and joint/composite requirement [1]. The following proposals are agreed:

Proposal 1: Define positioning accuracy as a quantitative TPR Proposal 2: Define Sensing-related capabilities as a TPR. Proposal 3: define AI-related capabilities as qualitative TPR evaluated by inspection. Proposal 4: Define energy efficiency as a TPR. Proposal 7: Define TPR (e.g. joint/composite requirement) related to data rate, latency, reliability, capacity etc. as a candidate IMT-2030 TPR.

Furthermore, the values and some associated key assumptions of legacy TPRs were discussed and the following agreements were made:

Proposal 5: For user experienced data rate, further discuss applicable deployment scenarios, and revisit the evaluation methodology before setting values for UL Proposal 6: For bandwidth TPR, it is proposed to set a target value of 400MHz (same definition as IMT-2020) - TBD on other TPR values

In conference call 1 after RAN#107, further agreements on the values of peak data rate, peak spectral efficiency, user experience data rate, 5th percentile user spectral efficiency were made, and it was agreed that security, interoperability, and sustainability are not defined as separate IMT-2030 TPR [2].

In this contribution, the remaining details of minimum TPRs are discussed.

- Values of legacy TPR

Peak data rate & peak spectrum efficiency

In Conference call 1 after RAN#107, two alternatives were agreed to determine the value of peak data rate [2]:

- Alternative 1: Not to define a requirement, e.g., Leave the value for peak data rate for the technology proponent to provide

- Alternative 2: Define a requirement with reasonable value

Also, the following agreement was made for value of peak spectrum efficiency:

- DL: [1.5-2.5x] of IMT-2020

- UL: [1.5-2.5x] of IMT-2020

The value of the two TPRs are highly related, as the value of the peak data rate can be calculated by multiplexing the value of peak spectrum efficiency with the bandwidth. For peak spectrum efficiency, assuming that the overhead of 6G system is similar as that of 5G system, the enhancement comes mainly from the increase of modulation order and the number of layers.

In TR 37.910 [3], the 3GPP evaluation assumption for DL spectrum efficiency is 8 layers and 256 QAM. Assuming with max. coding rate 0.9258, the corresponding calculated spectrum efficiency is 46.7 with 100MHz BW and 30KHz SCS, which is higher than the minimum spectrum efficiency TPR value of IMT-2020.

Assuming the same configuration for IMT-2030, and if the modulation order is increased from 256QAM to 1024 QAM, the spectrum efficiency will be 58.4, which is still lower than 2 times of the spectrum efficiency value of IMT-2020. Therefore, to support 2 times of peak spectrum efficiency of IMT-2020, either 16 layers shall be assumed or 2048 QAM shall be assumed.

Either 16 layers or 2048 QAM cannot be the typical configuration for 6G system, and thus it can be risky to set the value of peak spectrum of IMT-2030 to be 2 times of that of IMT-2020. Therefore, 1.5 times of IMT-2020 can be a more reasonable value for the peak spectrum efficiency of IMT-2030.

Proposal 1: The value for peak spectrum efficiency of IMT-2030 is 1.5 times of that of IMT-2020

On the value of peak data rate, from recommendation M.2160 [4], a rather wide range of values have been proposed, i.e. [1~10x] of IMT-2020. Determining a reasonable value for this TPR would be difficult. In addition, almost all companies agree that the minimum TPR value of peak data rate would be a theoretical value which can never been achieved in the realistic. Therefore, Alternative 1 would be preferred if companies cannot achieve consensus on the exact value for this TPR.

Proposal 2: Not to define a requirement for peak data rate, e.g., leave the value for peak data rate for the technology proponent to provide

5th percentile user spectral efficiency & Average spectral efficiency

In the conference call after RAN#107, companies were encouraged to provide antenna assumption for different frequency ranges together with the requirements, e.g., 4GHz, 7GHz, and 30GHz. Also, the applicable scenarios should be considered [2].

(a) Macro layer of dense urban					(b) Indoor hotspot

Figure 1. DL performance of average spectral efficiency and 5th percentile user spectral efficiency

In Figure 1, the DL performance gain of average spectral efficiency and 5th percentile user spectral efficiency in macro layer of dense urban and indoor hotspot scenarios are shown, with different antenna elements and antenna port assumptions. Non-ideal CSI and channel estimation are assumed. 4GHz carrier frequency is assumed for NR as the baseline performance, and 7GHz carrier frequency is assumed for 6G system. At each TRxP, each antenna port corresponds with 4 antenna elements in macro layer of dense urban and 1 antenna element in indoor hotspot, respectively; At each UE, 4 antenna elements driven by 4 TXRUs are assumed.

It can be observed that more than 2.5 times of 5th percentile user spectral efficiency can be achieved, if the antenna configuration of TRxP is increased from 32 ports (corresponding to 128 antenna elements in macro layer of dense urban or 32 antenna elements in indoor hotspot) to 256 ports (corresponding to 1024 antenna elements in macro layer of dense urban or 32 antenna elements in indoor hotspot). With this assumption, 2 times of IMT-2020 can be considered a reasonable value for IMT-2030.

Proposal 3: The value of 5th percentile user spectral efficiency of IMT-2030 is set as 2 times of that of IMT-2020.

From Figure 1, it can also be observed that more than 2.5 times of average spectral efficiency can be achieved, if the antenna configuration of TRxP is increased from 32 ports (corresponding to 128 antenna elements in macro layer of dense urban or 32 antenna elements in indoor hotspot) to 256 ports (corresponding to 1024 antenna elements in macro layer of dense urban or 32 antenna elements in indoor hotspot). With this assumption, 2 times of IMT-2020 can be considered a reasonable value for IMT-2030.

Proposal 4: The value of average spectral efficiency of IMT-2030 is set as 2 times of that of IMT-2020.

Area traffic capacity

Area traffic capacity is the total traffic throughput served per geographic area (in Mbit/s/m2), which can be expressed as

Carea = ρ × W × SEavg

where W denote the channel bandwidth and ρ the TRxP density (TRxP/m2). Assuming that the same TRxP density as 5G, 2 times of improvements on average spectral efficiency, and 2 times increase on channel bandwidth, totally 4 times improvements on area traffic capacity can be expected.

User plane latency

User plane latency is defined as the one-way time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface in either uplink or downlink in the network for a given service in unloaded conditions, assuming the mobile station is in the active state.

The requirement of user plane latency for IMT-2030 can be 1ms for Immersive communication, and 0.5ms for HRLLC communications.

Control plane latency

Control plane latency refers to the transition time from a most “battery efficient” state (e.g., Idle state) to the start of continuous data transfer (e.g., Active state).

The control plane latency for Immersive communication shall be reduced from IMT-2020, the minimum requirement of which can be 5-10ms for IMT-2030.

Connection density

Connection density is the total number of devices fulfilling a specific quality of service (QoS) per unit area (per km2).

The minimum requirement for connection density for IMT-2030 can be 106-107 devices/km2.

Reliability

Reliability is the success probability of transmitting a layer 2/3 packet within a required maximum time. For IMT-2020, the minimum requirement for the reliability is 1-10−5 success probability of transmitting a layer 2 PDU (protocol data unit) of 32 bytes within 1 ms.

For IMT-2030, similar requirement of success probability, i.e., 1-10-5, can be considered with larger PDU or smaller latency constraints.

Mobility

Mobility is the maximum mobile station speed at which a defined QoS can be achieved (in km/h).

For IMT-2030, mobile station with speed of up to 1000 km/h can be supported.

Mobility interruption time

Mobility interruption time is the shortest time duration supported by the system during which a user terminal cannot exchange user plane packets with any base station during transitions.

For IMT-2030, the minimum requirement for mobility interruption time shall be 0 ms.

In summary, the proposed TPR values are summarized as below:

Table 1: Summary of remaining TPR values for IMT-2030

| TPR items | IMT-2020 | IMT-2030 |
| --- | --- | --- |
| Peak data rate (Gbit/s) | DL: 20; UL: 10 | Not to define a requirement, e.g. Leave the value for peak data rate for the technology proponent to provide |
| Peak spectral efficiency (bit/s/Hz) | DL: 30; UL: 15 | 1.5 times of IMT-2020 |
| User experienced data rate (Mbit/s) | Dense Urban: DL: 100; UL: 50 | TBD |
| 5th percentile user spectral efficiency (bit/s/Hz) | InH: DL: 0.3; UL: 0.21 Dense Urban: DL: 0.225; UL: 0.15 Rural: DL: 0.12; UL: 0.045 | 2 times of IMT-2020 |
| Average spectral efficiency (bit/s/Hz/TRxP) | InH: DL: 9; UL: 6.75 Dense Urban: DL: 7.8; UL: 5.4 Rural: DL: 3.3; UL: 1.6 | 2 times of IMT-2020 |
| Area traffic capacity (bit/s/m2) | InH: 10 M | 4 times of IMT-2020 |
| User plane latency (ms) | 4 for eMBB, 1 for URLLC | 1 for IC, 0.5 for HRLLC |
| Control plane latency (ms) | 20 | 5-10 |
| Connection density (devices per km2) | 106 | 106-107 |
| Reliability | 1-10-5 | 1-10-5 |
| Mobility (km/h) | 500 | 1000 |
| Mobility interruption time (ms) | 0 | 0 |

Proposal 5: Consider values in Table 1 as the candidate values for remaining minimum TPRs of IMT-2030

- Definitions and values of Sensing-related TPR

For integrated Sensing and Communications (ISAC) scenario, quantitative TPR items for sensing-related capabilities needs to be defined.

The following TPR items can be considered:

- Missed detection probability and False alarm probability

Missed detection probability is defined as the probability of not detecting the presence of a target object when the target object is present, and False alarm probability is defined as the probability of falsely detecting the presence of a target object when the target object is not present.

- Sensing accuracy

The sensing accuracy can be defined as accuracy on position (m) and/or accuracy on velocity (m/s). The accuracy on position is defined as the difference of the sensed position of the target object to its true position value; and the accuracy on velocity is defined as the difference of the sensed velocity of the target object to its true velocity value. Sensing accuracy on position can be further derived into a horizontal accuracy and a vertical accuracy.

- Sensing resolution

Sensing resolution can be defined as the minimum difference in the measured magnitude of target objects to be allowed to detect objects in different magnitude, e.g., range resolution (m), velocity resolution (m/s).

Proposal 6: Quantitative TPR items for sensing related capabilities should be defined, including Missed detection probability and False alarm probability, Sensing accuracy, and Sensing resolution.

The TS 22.137 [5] can be referred for the value ranges of the interested TPR items. Note: Exact value for a TPR item is further dependent on the use case.

The minimum requirement for missed detection probability and false alarm probability can be:

- Missed detection probability is 1-5%

- False alarm probability is 1-5%

The minimum requirement for sensing accuracy can be:

- Range accuracy: 0.5-1m for horizontal and 0.5-1m for vertical, with confidential level 95%

- Velocity accuracy: 1-1.5m/s for horizontal and 1-1.5m/s for vertical, with confidential level 95%

The minimum requirement for sensing resolution can be:

- Range resolution: 0.5-1m

- Velocity resolution: 0.5-1m/s

- Other 6G capabilities

Coverage

In the conference call after RAN#107, the following three options are discussed [2]:

- Option 1: define coverage as a quantitative TPR

- Option 2: do not define coverage as a TPR, use link budget evaluation approach (same as IMT-2020)

- Option 3: define extra connectivity as a qualitative TPR, which was revised from a part of coverage

Link budget evaluation approach has been widely used to evaluate the coverage performance in both IMT-2020 and 3GPP, therefore, the same method can also be used for IMT-2030.

Proposal 7: Do not define coverage as a TPR, and use link budget evaluation approach

Resilience

In the conference call after RAN#107, the following two options are discussed [2]:

- Option 1: define resilience as a qualitative TPR

- Option 2: do not define resilience as a TPR, these can be covered by submission template for IMT-2030.

Although resilience is an important capability for 6G system, it would be quite difficult to define the requirement of or to evaluate the resilience of a given system. Therefore, it is preferred to not define resilience as a TPR.

Proposal 8: Do not define resilience as a TPR

- Conclusion

In this contribution, the remaining issues of minimum TPR for IMT-2030 are discussed, and the following proposals are proposed:

Proposal 1: The value for peak spectrum efficiency of IMT-2030 is 1.5 times of that of IMT-2020

Proposal 2: Not to define a requirement for peak data rate, e.g., Leave the value for peak data rate for the technology proponent to provide

Proposal 3: The value of 5th percentile user spectral efficiency of IMT-2030 is set as 2 times of that of IMT-2020.

Proposal 4: The value of average spectral efficiency of IMT-2030 is set as 2 times of that of IMT-2020.

Proposal 5: Consider values in Table 1 as the candidate values for remaining minimum TPRs of IMT-2030

Proposal 6: Quantitative TPR items for sensing related capabilities should be defined, including Missed detection probability and False alarm probability, Sensing accuracy, and Sensing resolution.

Proposal 7: Do not define coverage as a TPR, and use link budget evaluation approach

Proposal 8: Do not define resilience as a TPR

- References

- RP-250819, Moderator's summary for IMT-2030 TPRs, Moderator (CMCC), RAN#107, March 12-14, 2025

- RT-250024, Moderator's summary for IMT-2030 TPRs in CC#1, Moderator (CMCC), CC#1 for IMT-2030 TPR discussion, April 29, 2025

- 3GPP TR 37.910, Study on self evaluation towards IMT-2020 submission

- Recommendation ITU-R M.2160-0, Framework and overall objectives of the future development of IMT for 2030 and beyond, Nov. 2023

- 3GPP TS 22.137, Service requirements for Integrated Sensing and Communication; Stage 1 (Release 19), V19.1.0, 2024