3GPP TSG RAN Meeting #108								RP-251560

Prague, Czech Republic, June 9-13, 2025

Agenda Item:	16.1

Source:	Xiaomi

Title:	Discussion on 6G scenarios and requirements

Document for:	Discussion and Decision

- Introduction

In RAN#107 meeting, in addition to the objectives on TPR study, the following objectives were added in the updated SID [1]:

- Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors. - Develop 3GPP requirements for 6G Radio for improvement of existing services and for new services. - Determine the applicability of legacy services to 6G Radio, and define radio requirements for these, as appropriate. - Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least: - Ensure appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting - Energy efficiency and energy saving: both for network and device - Enhanced spectral efficiency - Enhanced overall coverage, focus on cell-edge performance and UL coverage - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band - Target scalable and forward compatible design for diverse device types - Improved spectrum utilization and operations taking into account diverse spectrum allocations - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs - Aim at a harmonized 6G Radio design for TN and NTN, including their integration - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. - Define a time plan and steer work as appropriate for the RAN WGs during the 6G WG SI to deliver high-level decisions at least on the following areas: - Fundamental 6G radio design aspects: waveform, numerology, channel coding, etc… - Overall high-level aspects of 5G to 6G migration - RAN architecture and interfaces, including RAN-Core interface - Coordination of 6G AI/ML framework

In this contribution, our views on 6G deployment scenarios, RAN requirements and potential guidance from RAN to WGs study are provided.

- Deployment scenarios

Six usage scenarios have been identified for IMT-2030 [2], including Immersive Communication, Hyper Reliable and Low-Latency Communication (HRLLC), Massive Communication, Integrated Sensing and Communication, Artificial Intelligence (AI) and Communication, and Ubiquitous Connectivity.

To avoid introducing too many deployment scenarios, some deployment scenarios for immersive communications can be reused for HRLLC or Massive Communications scenario. On the other hand, qualitative TPRs will be defined for AI and communication scenario, and thus the deployment scenario for AI and communication may not be necessary. For ubiquitous connectivity usage scenario, NTN is a very useful and efficient scenario to provide services for those areas where the terrestrial service is not available. Therefore, in this section the discussion will focus on deployment scenarios for immersive communications, integrated sensing and communications, and NTN.

  - For immersive communications

In NR, a variety of deployment scenarios are defined based on the practical applications and use cases, e.g., indoor hotspot, Dense Urban, Rural, Urban macro (UMa) [3]. For 6G design, existing 5G mid-band is targeted to be reused for site grid for 6G deployments in at least around 7 GHz, therefore, those NR deployment scenarios can be reused in 6G studies. However, 7GHz carrier frequency shall be additionally supported for these scenarios. The number of BS antenna elements shall also be increased, e.g. to 1024.

With respect to immersive communications, which is envisaged as a further enhancement of enhanced mobile broadband (eMBB) in IMT-2020 systems and is expected to provide extremely high data rate, the scenarios of UMa, UMi, and indoor office can be considered for immersive communications in 6G studies.

Proposal 1: The scenarios of indoor hotspot, Dense Urban, Urban Macro, and Rural, can be considered for immersive communications in 6G studies.

- 7GHz carrier frequency shall be supported

- The number of BS antenna elements shall be increased

  - For integrated sensing and communications

Integrated Sensing And Communication is a key usage scenario for 6G. It extends communication oriented 3GPP system with the new sensing capabilities for broader business opportunities. 30+ use cases are proposed in SA1 TS 22.137, which can be categorized into three groups, i.e., object detection and tracking, environment monitoring and motion monitoring. Other advanced use cases are suggested by different proponents too, e.g., environment reconstruction.

The Rel-19 study on ISAC channel model is focusing on object detection/tracking. A batch of sensing scenarios categorized by sensing targets are proposed in Rel-19, i.e., UAV sensing scenarios, automotive sensing scenarios, human sensing scenarios, AGV sensing scenarios, and object creating hazard on road/railways sensing scenarios. The above sensing scenarios can be further grouped into sensing scenario UMi/UMa/RMa/UMi-AV/UMa-AV/RMa-AV/InH/InF/Urban grid/Highway/HST/Indoor room, where each sensing scenario can support one or multiple target types. The sensing scenarios defined in Rel-19 ISAC SI can be used as starting point for 6G studies.

Proposal 2: The sensing scenarios defined in Rel-19 SI on ISAC CM, i.e., UAV sensing scenarios, automotive sensing scenarios, human sensing scenarios, AGV sensing scenarios, and object creating hazard on road/railways sensing scenarios can be starting point for 6G studies.

As to a specific sensing scenario, it is defined based on the existing communication scenario(s) according to the Rel-19 study item on ISAC CM. The topology of TRPs in a sensing scenario is the same as the respective communication scenario. At least a subset of TRPs and UEs can be used as sensing transmitter/receiver. The sensing target is then dropped in an area within the cell layout. One or multiple sensing target types can be considered in a simulation drop for a sensing scenario. All sensing modes, i.e., TRP monostatic, TRP-TRP bistatic, UE monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic (Uu based) can be considered, including corporative sensing or multi-static for better sensing performance. Sensing evaluation, especially tracking would require the implementation of spatial consistency. Further, an aerial UE may fly in 3D direction which necessitate 3D spatial consistency in the UAV scenarios. Finally, a type-2 EO can be considered in e.g., Urban grid sensing scenario to better model the background environment. All the above features require more calculations and result in even larger simulation time. In conclusion, a method which reduces the evaluation complexity, e.g., by limiting the max number of targets needs to be considered.

Proposal 3: All sensing modes, i.e., TRP monostatic, TRP-TRP bistatic, UE monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic (Uu based) can be considered in 6G studies.

Proposal 4: Corporative sensing or multi-static can be included in 6G studies.

Proposal 5: A method which reduces the evaluation complexity, e.g., by limiting the max number of targets needs to be considered.

The number of combinations of (sensing scenario, target type) are too large, it is therefore reasonable to consider defining several representative deployment scenarios of a reduced set of combinations.

Given the market/interest of low-altitude economy, one or multiple sensing scenarios for UAV use cases can be prioritized, e.g., UMa-AV (channel model calibrated in Rel-19 SI). Assuming TRP monostatic sensing mode will be studied in 5GA Rel-20, other sensing modes or corporation between all sensing modes can be considered in 6G. The example attributes of deployment scenario for UAV use cases are provided in Table 1.

Table 1: Example attributes for sensing scenarios for UAV use cases

| Attributes |  | Values or assumptions |
| --- | --- | --- |
| Sensing scenario |  | UMa-AV for sensing |
| Carrier Frequency |  | Around 4GHz, Around 30GHz |
| Bandwidth for sensing |  | Around 4GHz: Up to 200MHz Around 30GHz: Up to 1GHz |
| BS | Layout | Hexagonal grid, 19 macro sites, 3 sectors per site |
|  | ISD | ISD = 500m |
|  | Antenna height | 25m |
|  | Antenna elements | Up to 256 Tx and Rx antenna elements |
| UE | Outdoor/indoor | 100% Outdoor |
|  | LOS/NLOS | LOS/NLOS |
|  | Height | 1.5m |
|  | Distribution | 10 users per TRxP |
|  | Mobility | 3km/h |
|  | UE antenna elements | Up to 8 Tx and Rx antenna elements |
| Sensing target | Type | UAV |
|  | LOS/NLOS | LOS and NLOS |
|  | Outdoor/indoor | Outdoor |
|  | Mobility | Horizontal: {0 km/h, Uniform distribution between 0 and 180km/h} Vertical: {0, 20, 40} km/h Trajectory: straight-line |
|  | Distribution | Horizontal: N targets uniformly distributed {within one cell, per cell}, N = {0, 1, 2, 3, 4, 5} Vertical: Uniform between 1.5m and 300m. |
|  | Orientation | Random in horizontal domain |
|  | Physical characteristics | Size: UAV with {small size, large size} defined in Rel-19 |

Human intruder detection or health care are important use cases, which motivate the prioritization of one or more human sensing scenarios. Preferably, indoor scenario is considered as a typical scenario. The example attributes of indoor deployment scenario for human related used cases, i.e., InH, is provided in Table 2.

Table 2: Example attributes for InH for human related use cases

| Attributes |  | Values or assumptions |
| --- | --- | --- |
| Sensing scenario |  | Indoor hotspot for sensing |
| Carrier Frequency |  | Around 4GHz, Around 30GHz |
| Bandwidth for sensing |  | Around 4GHz: Up to 200MHz Around 30GHz: Up to 1GHz |
| BS | Layout | 120mx50mx3m |
|  | ISD | ISD = 20m |
|  | Antenna height | 3m |
|  | Antenna elements | Up to 256 Tx and Rx antenna elements |
| UE | Outdoor/indoor | 100% indoor |
|  | LOS/NLOS | LOS/NLOS |
|  | Height | 1m |
|  | Distribution | 10 users per TRxP |
|  | Mobility | 3km/h |
|  | UE antenna elements | Up to 8 Tx and Rx antenna elements |
| Sensing target | Type | Human |
|  | LOS/NLOS | LOS and NLOS |
|  | Outdoor/indoor | Indoor |
|  | Mobility | Uniform distribution between 0km/h and 3km/hr Trajectory: straight-line |
|  | Distribution | N targets uniformly distributed over the horizontal area of the convex hull of the TRP deployment, N = {0, 1, 2, 3, 4, 5} |
|  | Orientation | Random over the horizontal area |
|  | Physical characteristics | Adult Pedestrian: 0.5m x 0.5m x 1.75m |

Further, to better support automatic driving, early accident indication and intruder detection on road, one or multiple sensing scenarios for automotive use cases shall be considered. Sensing scenario Urban gird is preferred compared to Highway, since more complicated automotive scenarios can be modeled in Urban grid, e.g., the crossroad, the blocking/shadowing of building, etc. The example attributes of deployment scenario for automotive use cases are provided in Table 3.

Table 3: Example attributes for Urban grid for automotive use cases

| Attributes |  | Values or assumptions |  |
| --- | --- | --- | --- |
| Sensing scenario |  | Urban grid for sensing |  |
| Carrier Frequency |  | Around 6GHz, Around 30GHz |  |
| Bandwidth for sensing |  | Around 6GHz: Up to 200MHz Around 30GHz: Up to 1GHz |  |
| BS | Layout | Macro only |  |
|  | ISD | ISD = 500m |  |
|  | Antenna height | 25m |  |
|  | Antenna elements | Up to 256 Tx and Rx antenna elements |  |
| UE | Outdoor/indoor | 100% indoor |  |
|  | LOS/NLOS | LOS/NLOS |  |
|  | Height | 1m |  |
|  | Distribution | Based on dropping in TR37.885 |  |
|  | Mobility | Based on TR37.885 |  |
|  | UE antenna elements | Vehicle Tx/Rx: Up to 8 Tx Pedestrian Tx/Rx: Up to 8 Tx |  |
| Sensing target, i.e., UAV | Type | Vehicle | Human |
|  | LOS/NLOS | LOS and NLOS (including NLOSv) | LOS and NLOS |
|  | Outdoor/indoor | Outdoor | Outdoor |
|  | Mobility | Based on TR37.885 | Uniform distribution between 0km/h and 10km/h Trajectory: straight-line |
|  | Distribution | Based on dropping in TR37.885 | Based on dropping in TR37.885 |
|  | Orientation | Lane direction in horizontal plane | Random over the horizontal area |
|  | Physical characteristics | Type 1/2 (passenger vehicle) Vehicle type distribution per TR 37.885 | Adult Pedestrian: 0.5m x 0.5m x 1.75m |
| Environment Objects |  | Optional: Type-2 EO - 4 walls modelled as Type-2 EO, per building of size 413m x 230m x 20m. |  |

Proposal 6: Prioritization of certain representative deployment scenarios of a reduced set of combinations of (sensing scenario, target type) can be considered, including,

- One UAV sensing scenario, i.e., UMa-AV

- One indoor human sensing scenario, i.e., InH,

- One automotive sensing scenario, i.e., Urban grid, with vehicle and pedestrian can be STX/SRX and the target

Proposal 7: Table 1, 2, 3 are considered for the example attributes of the representative deployment scenarios for ISAC.

Finally, depending the RAN discussion, if any advanced use cases can be included in 6G study, more sensing scenarios can be considered accordingly. The advanced use cases may require an update on the Rel-19 ISAC CM too.

Proposal 8: If any advanced use case is included in 6G study, new sensing scenario may be defined. The ISAC channel model may need an update correspondingly.

  - For 6G NTN

Similar to 5G, the deployment scenario for NTN is to provide services for those areas where the terrestrial service is not available and also for services that can be more efficiently supported by NTN such as broadcasting service. For 6G, NTN is supposed to provide more diverse services than 5G with the ability to provide much higher data throughput. NTN is also supposed to supported different device types, e.g. handheld, mobile VSAT, GNSS-less device, IoT device. Table 4 lists some of the attributes.

Table 4: Examples for Satellite Deployment

| Attributes | Deployment-1 | Deployment-2 |
| --- | --- | --- |
| Carrier Frequency | FR1 | FR2/FR3 |
| Duplexing | FDD, [TDD] | FDD, [TDD] |
| Satellite architecture | Transparent Regenerative [1] | Transparent Regenerative [1] |
| Typical satellite system positioning in the 5G architecture | Access network | Access network |
| Aggregated Bandwidth (DL + UL) | Up to [40 MHz] | Up to [500MHz] |
| Satellite Orbit | GEO, MEO, LEO multi-satellite[2] | LEO, MEO, GEO multi-satellite[2] |
| UE Distribution | 100% Outdoors | 100% Outdoors |
| UE Mobility | Fixed, Portable, Mobile | Fixed, Portable, Mobile |
| UE Type | Handheld, VSAT, GNSS-less device, IoT device | Handheld, VSAT, GNSS-less device, IoT device |

[1] Full-BS on board, CN onboard and DU on board are supported for regenerative payload.

[2] Multi-satellite scenario including multi-satellite with same orbit and multi-satellite with different orbit are considered

Proposal 9: Deployment scenario for satellite shall be supported, and attributes in Table 4 is considered as the starting point.

- 6G requirements

  - Lean system design

The next generation of cellular system should be friendly to both network and UE implementation, on top of satisfying requirements and scenarios from IMT-2030. From the lesson of 5G, excessive flexibility on configuration and multiple options for same functionality significantly increase magnitude of complexity for products. In addition, the lack of essential enhancement with strong commercialization in 5G day 1 makes specification more fragmented, which makes specification even more complicated. Hence, it is very important to pursue a lean system design in order to facilitate commercialization, achieve better customer experience and make a more successful generation. From our perspective, the following principles should be considered:

- Make a harmonized system supporting different device type, e.g., co-existence of normal UE and IoT device.

- Take techniques/scenarios with strong commercial need into account at the very beginning, e.g., UL coverage enhancement, energy saving.

- Support AI and ISAC natively without fragmentation, e.g., involve AI/ISAC in physical procedure instead of make a separate one with major difference.

- Avoid multiple options for same functionality as much as possible, e.g., repetition mechanisms, HARQ-ACK codebook, CSI report type.

- Simplify physical channel/signal design with less options in terms of configuration and pattern, e.g., PUCCH format, PDCCH structure.

- Optimize physical procedure without excessive complexity, e.g., UCI multiplexing, PDCCH monitoring, initial access.

- Simplified network architecture with minimal coupling between network nodes of same RAT and different RATs, e.g. MR-DC architecture is not considered.

- Unified and lean protocol design for control plane and user plane protocols with reduced processing delay, signaling overhead and standardization complexity.

Proposal 10: the following principles should be considered for lean system design:

- Make a harmonized system supporting different device type, e.g., co-existence of normal UE and IoT device.

- Take techniques/scenarios with strong commercial need into account at the very beginning, e.g., UL coverage enhancement, energy saving.

- Support AI and ISAC natively without fragmentation, e.g., involve AI/ISAC in physical procedure instead of make a separate one with major difference.

- Avoid multiple options for same functionality as much as possible, e.g., repetition mechanisms, HARQ-ACK codebook, CSI report type.

- Simplify physical channel/signal design with less options in terms of configuration and pattern, e.g., PUCCH format, PDCCH structure.

- Optimize physical procedure without excessive complexity, e.g., UCI multiplexing, PDCCH monitoring, initial access.

- Simplified network architecture with minimal coupling between network nodes of same RAT and different RATs, e.g. MR-DC architecture is not considered.

- Unified and lean protocol design for control plane and user plane protocols with reduced processing delay, signalling overhead and standardization complexity

  - IoT design

Internet-of-Things (IoT) is developed for many years and the relative branches of technologies have been evolving for several releases in 3GPP. In 4G LTE and 5G NR, IoT technologies includes NB-IoT, LTE MTC, LTE Cat. 1/1b/4, NR RedCap, NR eRedCap. 6G Massive communication is considering about IoT Technic evolution from 5G to 6G. In 4G and 5G, IoT technologies were design from Day-2, which means that the basic system is firstly designed to support normal UE, and then the enhancement, such as reduce complexity and low power consumption, was designed based on the basic system to support IoT UE. This procedure can provide a stable platform when designing IoT system, but it leads to heavily delay on the commercialization of IoT technology in each generation. In order to support IoT in 6G Day-1, one possible solution is to have a common system design which is fully compatible of both normal UE and low capability UE.

The requirement on 6G IoT can consider several aspects including bandwidth, duplex operation, peak data rate and energy aspect.

Bandwidth: For NR RedCap and eRedCap, the UE RF bandwidth is 20 MHz. For LTE NB-IoT, the bandwidth is 180 kHz and LTE MTC consumes 1.4 MHz bandwidth. Considering about the IoT evolution from 5G to 6G, a bandwidth limitation of 3~5 MHz is considered for Day-1 design, and negative impact is not expected to the system design and performance on normal UE.

Duplex operation: IoT UE usually has simple architecture and very low complexity, therefore, half-duplex is expected for 6G IoT.

Peak data rate: The peak data rate of eRedCap is 10 Mbps for both UL and DL. Considering the reduced bandwidth of 6G IoT of 3~5 MHz, a reasonable lower peak data rate can be expected. Higher data rate can provide beneficial performance of IoT system, too low data rate may not satisfy the 6G expectation and requirement on diverse scenarios. 5~10 Mbps can be studied for 6G IoT design, and an even higher data rate can also be considered.

Energy aspect: 5G works on the power saving for UE and network in several releases. Power consumption relative aspects are always popular topic in IoT. 6G should also consider about how to minimize the power consumption of IoT device. Furthermore, energy charging, such as RF resource, can be considered and studied as a supplementary method to extend the battery life of IoT devices.

Proposal 11: To satisfy diverse IoT requirements, following aspects need to be considered:

- Maximum bandwidth ([3M] or [5M]) for the lowest-end IoT device can be studied, but it shall not impact the performance of commercial UE;

- Half-duplex should be considered for 6G IoT;

- Peak data rate, e.g., 5~10 Mbps can be studied for 6G IoT design to satisfy diverse requirements from industry, and avoid the impact on system performance;

- Energy charging, such as RF resource, can be considered and studied as a supplementary method to extend the battery life of IoT devices.

  - harmonized radio design for TN and NTN

In 5G phase, the support of the NTN features started late from Rel-17 which impose heavy standard effort to support efficient NTN operation. The next generation cellular system should be able to support harmonized radio design for TN and NTN from the very beginning. To achieve this, the following aspects can be considered:

- Unified air interface design between TN and NTN considering the following aspects

  - Unified random access mechanism design

  - Support both TDD and FDD in NTN

  - Support various UE types in NTN, e.g., handheld, IoT device especially consider the GNSS-less device

- Support dynamic spectrum sharing between TN and NTN which enables efficient spectrum utilization.

- Support flexible TN-NTN interworking, e.g., TN-NTN, NTN-TN, NTN-NTN mobility

Proposal 12: To achieve harmonized radio design for TN and NTN, the following aspects can be considered:

- Unified air interface design between TN and NTN considering the following aspects

  - Unified random access mechanism design

  - Support both TDD and FDD in NTN

  - Support various UE types in NTN, e.g., handheld, IoT device especially consider the GNSS-less device

- Support dynamic spectrum sharing between TN and NTN which enables efficient spectrum utilization.

- Support flexible TN-NTN interworking, e.g., TN-NTN, NTN-TN, NTN-NTN mobility

  - AI

AI/ML based processing could be utilized in various network domains to improve the performance. For example, AI/ML could be utilized to improve the radio transmission/receiving performance in air interface and facilitate the network management in various network entities. And thus 6G system should support flexible deployment of AI-based processing in various network domains to unleash the power of AI/ML everywhere in 6G system.

Proposal 13:  6G system should support flexible deployment of AI/ML-based processing in different network domains

One key issue of AI/ML algorithm is generalization. When the data distribution of inference is different from that of training, performance degradation may happen. In this case, mechanisms should be supported in 6G to enable the consistency checking between training and inference to monitor the inference performance. Once inconsistency is detected, proper action can be taken to guarantee stable performance.

Proposal 14:  Consistency check between training and inference should be supported to guarantee stable performance

Due to the generalization issue, AI/ML-based processing is not expected to achieve outstanding performance everywhere. In some cases, non-AI/ML based processing is more appropriate. Switch between non-AI/ML based processing and AI/ML based processing is necessary. In addition, considering the dynamic change of wireless environment, such switch should be sufficiently fast and smooth.

Proposal 15: 6G system should support smooth switch between non-AI/ML based processing and AI/ML based processing

AI/ML-based processing is kind of computation which would consume power of devices. With the increased deployment of AI/ML-based processing on device side, the power consumption of AI/ML would take a larger and larger portion. Considering this situation, mechanisms to maintain high energy efficiency of AI/ML processing is critical to devices’ battery life

Proposal 16: Mechanisms to maintain high energy efficiency of AI/ML processing should be supported

AI/ML model can be deployed at NW or UE side. For NW side model, the data for training/performance monitoring may be collected and reported by UE in some cases, e.g. beam management and mobility. However, the data collection would consume UE power and storage. Similar to MDT, User consent should be respected. For example, user should be allowed to indicate the preference about performing data collection for NW side model.

Proposal 17: User consent should be respected regarding NW side data collection.

In 5G-A, LCM is defined at RAN and SA respectively. In 6G, it’s preferred to have a unified LCM framework across RAN and SA.

Besides the radio resource, AI/ML resource, i.e. model, data and computation, is also critical for AI/ML operation. In 5G-A, model and computation handling is up to implementation. The efficiency may be improved if 6G system can manage model, data and computation resource.

Several AI/ML use cases were studied in 5G-A. AI/ML model can be deployed at UE and/or NW. Depending on the use case, one-sided or two-sided model may be required. Two-sided model requires coordination between UE and NW, which may be more complicated than one-sided model. Although use cases requiring two-sided model may not be supported at the beginning of 6G, it’s important to consider the support of both one-sided and two-sided model at the beginning of 6G from protocol and architecture design point of view for forward compatibility.

Proposal 18: 6G system shall support forward compatible unified LCM.

- RAN Guidance for WG study

As agreed in RAN#107, the detailed time plan for the high -level decisions will be decided at RAN#108.

- Define a time plan and steer work as appropriate for the RAN WGs during the 6G WG SI to deliver high-level decisions at least on the following areas: - Fundamental 6G radio design aspects: waveform, numerology, channel coding, etc… - Overall high-level aspects of 5G to 6G migration - RAN architecture and interfaces, including RAN-Core interface - Coordination of 6G AI/ML framework Note: The detailed time plan for the high-level decisions above will be decided at RAN#108 at the approval of the 6G RAN WG SI and is to be aligned with the RAN SI progress.

Besides the areas listed in the SID [1], we think the guidance/high-level decisions on following areas are also needed:

- Supported use cases for AI

Use case, design principle, etc., for AI can be discussed under 6G basic system design without the need of dedicated agenda at the beginning. However, the checkpoint is preferred in RANP on the progress of the study, and high-level decisions in RANP would be helpful if there are controversial issues during the AI study, especially the supported use cases.

- Supported scenarios for ISAC channel model study

As discussed in 2.2, 30+ use cases are proposed in SA1 TS 22.137, focusing on object detection and tracking, environment monitoring, motion monitoring, environment reconstruction and gesture, etc. The object detection/tracking has been considered in Rel-19 ISAC channel model study. For the rest scenarios, additional efforts on channel model study may need to be considered. To save the RAN1 time, it would be good to have high-level decision in RANP on which scenarios should be considered in RAN1 channel model study.

- The design principle for devise devices, and the requirement of lowest end IOT device

Based on the experience of 4G and 5G. There is different way to define device type, LTE category based (e.g., Cat 1) and NR minimum mandatory features based (e.g., RedCap). We need to consider what is the best way to define device type in 6G, and what principle shall be used.

In addition, there is clear requirement to support LPWA like IoT device in 6G. However, companies have different view on the minimum system capability for the lowest-end device, e.g., mandatory max BW [3M], [5M] or [20M], mandatory Modulation, mandatory Rx/Tx, mandatory duplex, etc. Considering these metrics will impact the basic 6G system design, e.g., SSB, use case, design principle, minimum set of capabilities, e.g., BW, etc. shall be confirmed as early as possible. High-level decision in RANP would help the discussion in WGs.

Proposal 19: Following areas require high-level decision in RANP:

- Supported use cases for AI

- Supported scenarios for ISAC channel model study

- Use case, design principle, minimum set of capabilities for lowest end IOT device

Different timeline would be preferred for different areas. For instance, the use case, design principle, minimum set of capabilities for lowest end IOT device will impact the basic system decision which requires the guidance from RANP as early as possible, e.g., 3 or 6 months.

However, for the use cases of AI, ISAC, more time is needed. We may set the longer time for checkpoint in RANP, e.g., 9 months, upon the completion of 6G RAN SI.

Proposal 20: set 6 months’ timeline for the high-level decision in RANP on use case, design principle, minimum set of capabilities for lowest end IOT device.

Proposal 21: set 9 months’ timeline for the high-level decision in RANP on other areas.

- Conclusion

In this contribution, the deployment scenario, requirements and potential RAN guidance are discussed. The following proposals are proposed:

Proposal 1: The scenarios of indoor hotspot, Dense Urban, Urban Macro, and Rural, can be considered for immersive communications in 6G studies.

- 7GHz carrier frequency shall be supported

- The number of BS antenna elements shall be increased

Proposal 2: The sensing scenarios defined in Rel-19 SI on ISAC CM, i.e., UAV sensing scenarios, automotive sensing scenarios, human sensing scenarios, AGV sensing scenarios, and object creating hazard on road/railways sensing scenarios can be starting point for 6G studies.

Proposal 3: All sensing modes, i.e., TRP monostatic, TRP-TRP bistatic, UE monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic (Uu based) can be considered in 6G studies.

Proposal 4: Corporative sensing or multi-static can be included in 6G studies.

Proposal 5: A method which reduces the evaluation complexity, e.g., by limiting the max number of targets needs to be considered.

Proposal 6: Prioritization of certain representative deployment scenarios of a reduced set of combinations of (sensing scenario, target type) can be considered, including,

Proposal 7: Table 1, 2, 3 are considered for the example attributes of the representative deployment scenarios for ISAC.

Proposal 8: If any advanced use case is included in 6G study, new sensing scenario may be defined. The ISAC channel model may need an update correspondingly.

Proposal 9: Deployment scenario for satellite shall be supported, and attributes in Table 4 is considered as the starting point.

Proposal 10: the following principles should be considered for lean system design:

- Make a harmonized system supporting different device type, e.g., co-existence of normal UE and IoT device.

- Take techniques/scenarios with strong commercial need into account at the very beginning, e.g., UL coverage enhancement, energy saving.

- Support AI and ISAC natively without fragmentation, e.g., involve AI/ISAC in physical procedure instead of make a separate one with major difference.

- Avoid multiple options for same functionality as much as possible, e.g., repetition mechanisms, HARQ-ACK codebook, CSI report type.

- Simplify physical channel/signal design with less options in terms of configuration and pattern, e.g., PUCCH format, PDCCH structure.

- Optimize physical procedure without excessive complexity, e.g., UCI multiplexing, PDCCH monitoring, initial access.

- Simplified network architecture with minimal coupling between network nodes of same RAT and different RATs, e.g. MR-DC architecture is not considered.

- Unified and lean protocol design for control plane and user plane protocols with reduced processing delay, signalling overhead and standardization complexity

Proposal 11: To satisfy diverse IoT requirements, following aspects need to be considered:

- Maximum bandwidth ([3M] or [5M]) for the lowest-end IoT device can be studied, but it shall not impact the performance of commercial UE;

- Half-duplex should be considered for 6G IoT;

- Peak data rate, e.g., 5~10 Mbps can be studied for 6G IoT design to satisfy diverse requirements from industry, and avoid the impact on system performance;

- Energy charging, such as RF resource, can be considered and studied as a supplementary method to extend the battery life of IoT devices.

Proposal 12: To achieve harmonized radio design for TN and NTN, the following aspects can be considered:

- Unified air interface design between TN and NTN considering the following aspects

  - Unified random access mechanism design

  - Support both TDD and FDD in NTN

  - Support various UE types in NTN, e.g., handheld, IoT device especially consider the GNSS-less device

- Support dynamic spectrum sharing between TN and NTN which enables efficient spectrum utilization.

- Support flexible TN-NTN interworking, e.g., TN-NTN, NTN-TN, NTN-NTN mobility

Proposal 13:  6G system should support flexible deployment of AI/ML-based processing in different network domains

Proposal 14:  Consistency check between training and inference should be supported to guarantee stable performance

Proposal 15: 6G system should support smooth switch between non-AI/ML based processing and AI/ML based processing

Proposal 16: Mechanisms to maintain high energy efficiency of AI/ML processing should be supported

Proposal 17: User consent should be respected regarding NW side data collection.

Proposal 18: 6G system shall support forward compatible unified LCM.

Proposal 19: Following areas require high-level decision in RANP:

- Supported use cases for AI

- Supported scenarios for ISAC channel model study

- Use case, design principle, minimum set of capabilities for lowest end IOT device

Proposal 20: set 6 months’ timeline for the high-level decision in RANP on use case, design principle, minimum set of capabilities for lowest end IOT device.

Proposal 21: set 9 months’ timeline for the high-level decision in RANP on other areas.

- References

- RP-250810, Revised SID: Study on 6G Scenarios and requirements, CMCC, Verizon, NTT DOCOMO, Inc., Deutsche Telecom, Mar. 12-14, 2025

- Recommendation ITU-R M.2160-0, Framework and overall objectives of the future development of IMT for 2030 and beyond, Nov. 2023

- 3GPP TR 38.913, Study on Scenarios and Requirements for Next Generation Access Technologies