3GPP TSG RAN WG1 #121	R1-2505058

# St Julian’s, Malta, May 19 – 23, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.211 | CR | 0575 | rev | - | Current version: | 18.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of IoT_NTN_TDD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_TDD-Core |  |  |  |  |  | Date: |  |  | 2025-06-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of IoT_NTN_TDD |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support of frame structure 1 in NTN TDD pattern |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for IoT_NTN_TDD |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

CCE	Control Channel Element

CDD	Cyclic Delay Diversity

CRS	Cell-specific Reference Signal

CSI	Channel-State Information

DCI	Downlink Control Information

DM-RS	Demodulation Reference Signal

ECCE	Enhanced Control Channel Element

EPDCCH	Enhanced Physical Downlink Control CHannel

EREG	Enhanced Resource-Element Group

MPDCCH	MTC Physical Downlink Control Channel

MWUS	MTC Wake-Up Signal

NCCE	Narrowband Control Channel Element

NPBCH	Narrowband Physical Broadcast CHannel

NPDCCH	Narrowband Physical Downlink Control CHannel

NPDSCH	Narrowband Physical Downlink Shared CHannel

NPRACH	Narrowband Physical Random Access CHannel

NPUSCH	Narrowband Physical Uplink Shared CHannel

NPRS	Narrowband Positioning Reference Signal

NPSS	Narrowband Primary Synchronization Signal

NSSS	Narrowband Secondary Synchronization Signal

NRS	Narrowband Reference Signal PBCH	Physical Broadcast CHannel

NTN	Non-Terrestrial Networks

PCFICH	Physical Control Format Indicator CHannel

PDCCH	Physical Downlink Control CHannel

PDSCH	Physical Downlink Shared CHannel

PHICH	Physical Hybrid-ARQ Indicator CHannel

PMCH	Physical Multicast CHannel

PRACH	Physical Random Access CHannel

PRB	Physical Resource Block

PRG	Precoding Resource Block Group

PRS	Positioning Reference Signal

PSBCH	Physical Sidelink Broadcast CHannel

PSCCH	Physical Sidelink Control CHannel

PSDCH	Physical Sidelink Discovery CHannel

PSSCH	Physical Sidelink Shared CHannel

PUCCH	Physical Uplink Control CHannel

PUSCH	Physical Uplink Shared CHannel

REG	Resource-Element Group

RSS	Resynchronization Signal

SCCE	Short Control Channel Element

SCG	Secondary Cell Group

SPDCCH	Short Physical Downlink Control CHannel

SPUCCH	Short Physical Uplink Control CHannel

SREG	Short Resource-Element Group

SRS	Sounding Reference Signal

VRB	Virtual Resource Block

# 4	Frame structure

Throughout this specification, unless otherwise noted, the size of various fields in the time domain is expressed as a number of time units  seconds.

Downlink, uplink and sidelink transmissions are organized into radio frames with  duration. 
Three radio frame structures are supported:

-	Type 1, applicable to FDD and NTN-TDD only,

-	Type 2, applicable to TDD only,

-	Type 3, applicable to LAA secondary cell operation only.

NOTE: LAA secondary cell operation only applies to frame structure type 3.

Transmissions in multiple cells can be aggregated where up to 31 secondary cells can be used in addition to the primary cell. Unless otherwise noted, the description in this specification applies to each of the up to 32 serving cells. In case of multi-cell aggregation, different frame structures can be used in the different serving cells.

## 4.1	Frame structure type 1 for FDD

Frame structure type 1 is applicable to both full duplex and half duplex FDD only. Each radio frame is  long and consists of 10 subframes of length , numbered from 0 to 9. Subframe  in frame  has an absolute subframe number  where  is the system frame number.

For subframes using Δf=2.5 kHz, , or , subframe  is defined as two slots,  and , of length  each.

For subframes using , subframe  is defined as one slot, , of length .

For transmissions using Δf=182944Ts≈0.37 kHz, a slot has a length of 92160Ts=3 ms. There are 13 slots, numbered in increasing order from 0 to 12, in a 40 ms period starting at nf mod 4=0 with slot 0 starting at 30720Ts in the 40 ms period.

For subframes using , the subframe can further be divided into six subslots according to Table 4.1-1. Downlink subslot pattern 1 is applied if the number of symbols used for PDCCH is equal to 1 or 3 and downlink subslot pattern 2 is applied if the number of symbols used for PDCCH is equal to 2. For system bandwidths , subslot transmission is not supported in case 4 symbols used for PDCCH.

For FDD, 10 subframes, 20 slots, or up to 60 subslots are available for downlink transmission and 10 subframes, 20 slots, or up to 60 subslots are available for uplink transmissions in each 10 ms interval. Uplink and downlink transmissions are separated in the frequency domain. In half-duplex FDD operation, the UE cannot transmit and receive at the same time while there are no such restrictions in full-duplex FDD.

Figure 4.1-1: Frame structure type 1 (assuming Δfϵ2.5, 7.5, 15 kHz).

Table 4.1-1: SC-FDMA/OFDM symbols in different subslots of subframe i

| Subslot number | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| Slot number | 2i |  |  | 2i+1 |  |  |
| Uplink subslot pattern | 0, 1, 2 | 3, 4 | 5, 6 | 0, 1 | 2, 3 | 4, 5, 6 |
| Downlink subslot pattern 1 | 0, 1, 2 | 3, 4 | 5, 6 | 0, 1 | 2, 3 | 4, 5, 6 |
| Downlink subslot pattern 2 | 0, 1 | 2, 3, 4 | 5, 6 | 0, 1 | 2, 3 | 4, 5, 6 |

## 4.4	Frame structure type 1 for NTN-TDD

Frame structure type 1 is applicable to NTN-TDD in band 249. Each radio frame is Tf=307200Ts=10 ms long and consists of 10 subframes of length 30720Ts=1 ms, numbered from 0 to 9. Subframe i in frame nf has an absolute subframe number nsfabs=10nf+i where nf is the system frame number.

The frame structure for NTN-TDD , at the uplink time synchronization reference point defined in clause 16.1.2 of TS 36.213 [4] consists of 8 consecutive downlink subframes, followed by 50 consecutive guard period subframes, followed by 8 consecutive uplink subframes in each 90 ms interval.

-	The UE shall not assume any signal or channel being transmitted in subframes other than downlink subframes 3, 4, 5, 6, 7, 8, 9, and 0 across two consecutive radio frames.

-	The UE shall not transmit any signal or channel on a subframe other than the 8 consecutive uplink subframes.

#### 10.0.1.1	Frame structure type 1

Frame structure type 1 is applicable to FDD and NTN-TDD operation only.

#### 10.1.3.6	Mapping to physical resources

Each NPUSCH codeword can be mapped to one or more than one resource units, , as given by clause 16.5.1.2 of TS 36.213 [4], each of which shall be transmitted  times.

The block of complex-valued symbols  shall be multiplied with the amplitude scaling factor  in order to conform to the transmit power specified in [4], and mapped in sequence starting with  to subcarriers assigned for transmission of NPUSCH. The mapping to resource elements  corresponding to the subcarriers assigned for transmission and not used for transmission of reference signals, shall be in increasing order of first the index , then the index, starting with the first slot in the assigned resource unit.

After mapping to slots, the  slots shall be repeated  additional times, before continuing the mapping of  to the following slot, where

For NPUSCH Format 1 and 2 on frame structure type 2 with ,

-	the NPUSCH transmission is carried out in the first set of  slots spanning over two contiguous uplink subframes not overlapping with any uplink subframe configured as invalid;

-	for TDD configuration 1 and 4, if the starting position for the NPUSCH is indicated as the second of the two contiguous uplink subframes, the NPUSCH transmission is postponed until the start of two consecutive uplink subframes.

If a mapping to  slots or a repetition of the mapping contains a resource element which overlaps with

-	any configured NPRACH resource according to nprach-ParametersList in SystemInformationBlockType2-NB, or

-	any configured NPRACH resource according to nprach-ParametersList given by ul-ConfigList in SystemInformationBlockType22-NB and if the UE indicates multiCarrier-NPRACH as supported, or

-	any configured NPRACH resource according to nprach-ParametersList given by ul-ConfigListMixed in SystemInformationBlockType22-NB and if the UE indicates multiCarrier-NPRACH and mixedOperationMode as supported, or

-	any configured NPRACH resource according to nprach-ParametersListFmt2 in SystemInformationBlockType2-NB and if the UE indicates nprach-Format2 as supported, or

-	any configured NPRACH resource according to nprach-ParametersListFmt2 given by ul-ConfigList in SystemInformationBlockType23-NB and if the UE indicates multiCarrier-NPRACH and nprach-Format2 as supported, or

-	any configured NPRACH resource according to nprach-ParametersListFmt2 given by ul-ConfigListMixed in SystemInformationBlockType23-NB and if the UE indicates multiCarrier-NPRACH, mixedOperationMode and nprach-Format2 as supported, or

-	any configured NPRACH resource according to nprach-ParametersListTDD in SystemInformationBlockType2-NB, or

-	any configured NPRACH resource according to nprach-ParametersListTDD in SystemInformationBlockType22-NB and if the UE indicates multiCarrier-NPRACH as supported, or

-	any configured NPRACH resource configured for Early Data Transmission and if the NPUSCH transmission is during an Early Data Transmission procedure [12, Clause 7.3b],

then,

-	for  the NPUSCH transmission in overlapped slots is postponed until the next  slots not overlapping with any configured NPRACH resource.

-	for  the NPUSCH transmission in overlapped  slots is postponed until the next  slots starting with the first slot satisfying ns mod 2=0 nsmod 2=0and not overlapping with any configured NPRACH resource.

NPRACH gaps as defined in clause 10.1.6.1 are not part of the NPRACH resource. For frame structure type 2, the valid uplink subframes which are not used for NPRACH transmission when it is not possible to map G symbol groups back-to-back are not part of the NPRACH resource. The mapping of  is then repeated until  slots have been transmitted. After transmissions and/or postponements due to NPRACH of  time units, for frame structure type 1 for FDD, a gap of  time units shall be inserted where the NPUSCH transmission is postponed. The portion of a postponement due to NPRACH which coincides with a gap is counted as part of the gap.

When higher layer parameter npusch-AllSymbols is set to false, resource elements in SC-FDMA symbols overlapping with a symbol configured with SRS according to srs-SubframeConfig shall be counted in the NPUSCH mapping but not used for transmission of the NPUSCH. When higher layer parameter npusch-AllSymbols is set to true, all symbols are transmitted.

If higher layer parameter resourceReservationConfigUL is configured, then in case of NPUSCH format 1 transmission associated with C-RNTI or SPS C-RNTI using UE-specific NPDCCH search space with the Resource reservation field in the DCI set to 1 including NPUSCH format 1 transmission without a corresponding NPDCCH, or in case of NPUSCH format 2 transmission associated with C-RNTI using UE-specific NPDCCH search space,

-	In a subframe for  or a slot for that is overlapping with any fully reserved uplink subframe as defined in clause 16.5 in [4],

-	for , the NPUSCH transmission is postponed until the next NB-IoT uplink subframe that is not fully reserved.

-	for , the NPUSCH transmission in the slot is postponed until the next slot spanning over two contiguous uplink subframes not overlapping with any uplink subframe that is fully reserved.

-	In a subframe for  or a slot for that is not overlapping with any fully reserved uplink subframe, any SC-FDMA symbols overlapping with reserved symbols shall be counted in the NPUSCH mapping but not used for transmission of the NPUSCH.

For a UE communicating over NTN, after transmissions (and/or postponements due to NPRACH) of Nsegmentprecompensation time units, for frame structure type 1 for FDD, a transmission gap of Ngapprecompensation time units shall be counted for the NPUSCH resource mapping but not used for transmission of the NPUSCH according to the UE capability ntn-SegmentedPrecompensationGaps-r17 as specified in 3GPP TS 36.331 [9]. The quantity Nsegmentprecompensation is provided by higher layers, and the quantity of Ngapprecompensation is configured by higher layers based on the UE capability if signalled.

#### 10.1.6.1	Time and frequency structure

The physical layer random access preamble is based on single-subcarrier frequency-hopping symbol groups. A symbol group is illustrated in Figure 10.1.6.1-1, consisting of a cyclic prefix of length  and a sequence of  identical symbols with total length. The total number of symbol groups in a preamble repetition unit is denoted by P. The number of time-contiguous symbol groups is given by G.

The parameter values for frame structures 1 and 2 are listed in Tables 10.1.6.1-1 and 10.1.6.1-2, respectively.

Figure 10.1.6.1-1: Random access symbol group

Table 10.1.6.1-1: Random access preamble parameters for frame structure type 1

| Preamble format |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 0 | 4 | 4 | 5 |  |  |
| 1 | 4 | 4 | 5 |  |  |
| 2 (note 1) | 6 | 6 | 3 |  | 3∙24576Ts |

Note 1: Not used for NTN TDD operation with frame structure 1.

Table 10.1.6.1-2: Random access preamble parameters for frame structure type 2

| Preamble format | Supported uplink-downlink configurations |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 1, 2, 3, 4, 5 | 2 | 4 | 1 | 4768Ts |  |
| 1 | 1, 4 | 2 | 4 | 2 |  |  |
| 2 | 3 | 2 | 4 | 4 |  |  |
| 0-a | 1, 2, 3, 4, 5 | 3 | 6 | 1 |  |  |
| 1-a | 1, 4 | 3 | 6 | 2 |  |  |

The preamble consisting of P symbol groups shall be transmitted  times.

For NTN TDD operation with frame structure 1, a preamble repetition unit shall be fully transmitted within the 8 consecutive uplink subframes of the 90 ms interval described in clause 4.4, whereas the transmission of a preamble repetition unit that is partially or not within the 8 conseccutive uplink subframes shall be postponed until the next 8 consecutive uplink subframes of the 90 ms interval.

For frame structure type 2, when an invalid uplink subframe overlaps the transmission of G symbol groups without a gap, the G symbol groups are dropped. For frame structure type 2, the transmission of G symbol groups are aligned with the subframe boundary.

The transmission of a random-access preamble, if triggered by the MAC layer, is restricted to certain time and frequency resources.

A NPRACH configuration provided by higher layers contains the following:

-	NPRACH resource periodicity  (nprach-Periodicity),

-	frequency location of the first subcarrier allocated to NPRACH  (nprach-SubcarrierOffset),

-	number of subcarriers allocated to NPRACH  (nprach-NumSubcarriers),

-	number of starting sub-carriers allocated to UE initiated random access  (nprach-NumCBRA-StartSubcarriers),

-	number of NPRACH repetitions per attempt  (numRepetitionsPerPreambleAttempt),

-	NPRACH starting time  (nprach-StartTime),

-	Fraction for calculating starting subcarrier index for the range of NPRACH subcarriers reserved for indication of UE support for multi-tone msg3 transmission  (nprach-SubcarrierMSG3-RangeStart).

NPRACH transmission can start only  time units after the start of a radio frame fulfilling . For frame structure type 1, after transmissions of  time units for preamble formats 0 and 1, or 16∙6(TCP+TSEQ) time units for preamble format 2, a gap of  time units shall be inserted.

NPRACH configurations where  are invalid.

The NPRACH starting subcarriers allocated to UE initiated random access are split in two sets of subcarriers,  and , where the second set, if present, indicate UE support for multi-tone msg3 transmission.

The frequency location of the NPRACH transmission is constrained within  sub-carriers, and within NscRA=36  subcarriers when preamble format 2 as described in Table 10.1.6.1-1 is configured. Frequency hopping shall be used within the 12 subcarriers and 36 subcarriers when preamble format 2 as described in Table 10.1.6.1-1 is configured, where the frequency location of the ith symbol group is given by  where . The quantity nscRAi depends on the frame structure.

For frame structure type 1:

-	if G=4, P=4 for preamble formats 0 and 1 as described in Table 10.1.6.1-1:

where  with  being the subcarrier selected by the MAC layer from , and the pseudo random sequence  is given by clause 7.2. The pseudo random sequence generator shall be initialised with .

-	if G=6, P=6 for preamble format 2 as described in Table 10.1.6.1-1:

nSCRAi=nSCRA0+f(i/6)modNscRAi mod 6=0 and i>0nSCRAi-1+1i mod 6=1, 5 and nSCRAi-1 mod 2=0nSCRAi-1-1i mod 6=1, 5 and nSCRAi-1 mod 2=1nSCRAi-1+3i mod 6=2, 4 and nSCRAi-13 mod 2=0nSCRAi-1-3i mod 6=2, 4 and nSCRAi-13 mod 2=1nSCRAi-1+18i mod 6=3 and nSCRAi-1<18nSCRAi-1-18i mod 6=3 and nSCRAi-1≥18

ft=ft-1+n=10t+110t+9cn2n-10t+1modNscRA-1+1modNscRA

f-1=0

where  with  being the subcarrier selected by the MAC layer from , and the pseudo random sequence  is given by clause 7.2. The pseudo random sequence generator shall be initialised with .

For frame structure type 2:

-	if G=2, P=4 for preamble formats 0, 1, and 2 as described in Table 10.1.6.1-2:

nSCRAi=             Yi mod 8=0, 2 and i>0nSCRAi-1+1i mod 8=1 and nSCRAi-1=0, 2, 4, 6, 8, 10nSCRAi-1-1i mod 8=1 and nSCRAi-1=1, 3, 5, 7, 9, 11nSCRAi-1+6i mod 8=3 and nSCRAi-1=0, 1, 2, 3, 4, 5nSCRAi-1-6i mod 8=3 and nSCRAi-1=6, 7, 8, 9, 10, 11 2Y2+1i mod 8=4 and nSCRAi-4=0, 2, 4, 6, 8, 10 2Y2i mod 8=4 and nSCRAi-4=1, 3, 5, 7, 9, 11nSCRAi-1-1i mod 8=5 and nSCRAi-1=1, 3, 5, 7, 9, 11nSCRAi-1+1i mod 8=5 and nSCRAi-1=0, 2, 4, 6, 8, 10Y mod 6+6i mod 8=6 and nSCRAi-4=0, 1, 2, 3, 4, 5Y mod 6i mod 8=6 and nSCRAi-4=6, 7, 8, 9, 10, 11nSCRAi-1-6i mod 8=7 and nSCRAi-1=6, 7, 8, 9, 10, 11nSCRAi-1+6i mod 8=7 and nSCRAi-1=0, 1, 2, 3, 4, 5

Y= ñSCRA0+f(i/2)modNscRA

ft=ft-1+n=10t+110t+9cn2n-10t+1modNscRA-1+1modNscRA

f-1=0

where  with  being the subcarrier selected by the MAC layer from , and the pseudo random sequence  is given by clause 7.2. The pseudo random sequence generator shall be initialised with .

-	if G=3, P=6 for preamble formats 0-a, 1-a, as described in Table 10.1.6.1-2:

nSCRAi=nSCRA0+f(i/3)modNscRAi mod 6=0, 3 and i>0nSCRAi-1+1i mod 6=1 and nSCRAi-1=0, 2, 4, 6, 8, 10nSCRAi-1-1i mod 6=2 and nSCRAi-2=0, 2, 4, 6, 8, 10nSCRAi-1-1i mod 6=1 and nSCRAi-1=1, 3, 5, 7, 9, 11nSCRAi-1+1i mod 6=2 and nSCRAi-2=1, 3, 5, 7, 9, 11nSCRAi-1+6i mod 6=4 and nSCRAi-1=0, 1, 2, 3, 4, 5nSCRAi-1-6i mod 6=5 and nSCRAi-2=0, 1, 2, 3, 4, 5nSCRAi-1-6i mod 6=4 and nSCRAi-1=6, 7, 8, 9, 10, 11nSCRAi-1+6i mod 6=5 and nSCRAi-2=6, 7, 8, 9, 10, 11

ft=ft-1+n=10t+110t+9cn2n-10t+1modNscRA-1+1mod NscRA

f-1=0

where  with  being the subcarrier selected by the MAC layer from , and the pseudo random sequence  is given by clause 7.2. The pseudo random sequence generator shall be initialised with .