3GPP TSG RAN WG1 #121	        		   			R1-2504969

# St Julian’s, Malta, May 17th – 21st, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0706 | rev |  | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of enhancements of network energy savings for NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR_enh-Core |  |  |  |  |  | Date: |  |  | 2025-06-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of enhancements of network energy savings for NR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of enhancements of network energy savings for NR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of enhancements of network energy savings for NR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4 (new), 7, 8.1, 10, 10.1, 11.1, 11.5, 11.6 (new), 23 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.211, TS 38.212, TS 38.321, TS 38.331 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## *** Unchanged parts are omitted ***

## 4.4	Activation/deactivation of SS/PBCH block transmissions on a secondary cell

A UE can be indicated transmission of first SS/PBCH blocks with frequency location [9, TS 38.104] provided by absoluteFrequencySSB on an SCell. The UE can be indicated, by od-ssb-config [12, TS 38.331] or by a MAC CE [11, TS 38.321], activation of transmission for second SS/PBCH blocks in a configured DL BWP of the SCell [19, TS 38.300]. The UE can be indicated, by a MAC CE, adaptation of parameters for the activated transmission of the second SS/PBCH blocks when the SCell is activated. A number of half frames with transmission of the second SS/PBCH blocks is indicated by a MAC CE from values provided by od-ssb-nrofBurst, if provided; otherwise, the transmission of the second SS/PBCH blocks occurs until it is deactivated by od-ssb-config or a MAC CE [11, TS 38.321], where

-	the physical cell identity of the second SS/PBCH blocks is indicated by od-ssb-physCellId, if provided; otherwise, by physCellId in ServingCellConfigCommon

-	the indexes of transmitted second SS/PBCH blocks are indicated by a MAC CE from candidate values provided by od-ssb-PositionsInBurst, if provided; otherwise, by ssb-PositionsInBurst

-	the frequency location of the second SS/PBCH blocks is indicated by od-absoluteFrequencySSB, if provided; otherwise, by absoluteFrequencySSB

-	the SCS configuration of the second SS/PBCH blocks is indicated by od-ssbSubcarrierSpacing, if provided; otherwise, by ssbSubcarrierSpacing

-	the power of the second SS/PBCH blocks is indicated by od-ss-PBCH-BlockPower, if provided; otherwise, by ss-PBCH-BlockPower

-	the periodicity of the transmission of the second SS/PBCH blocks are indicated by a MAC CE from candidate values by od-ssb-Periodicity

-	the half frames for the transmission of the second SS/PBCH blocks are determined based on an indication by a MAC CE

-	the transmission of the second SS/PBCH blocks is in frames with SFN determined from SFN+SFN_offset⋅10 mod P=0, where P is the periodicity for the transmission of the second SS/PBCH blocks, and SFN_offset is the indicated SFN offset by the MAC CE from candidate values by od-ssb-sfn-Offset, if provided; else, SFNoffset=0. An index of a half frame with transmission of the second SS/PBCH blocks in a corresponding frame is indicated by the MAC CE from candidate values by od-ssb- halfFrameIndex, if provided; else the index is 0

When the activation or adaptation of the second SS/PBCH blocks transmission is by a MAC CE, respectively, and with reference to slots for PUCCH transmission, the UE expects that the transmission of the second SS/PBCH blocks according to the indicated parameters starts from a first slot including the candidate SS/PBCH block occasion corresponding to the first transmitted second SS/PBCH block index and located in a first half frame within the half frames for the transmissions of the second SS/PBCH blocks, that is at least m+3 Nslotsubframe,μ+1 slots after slot n, where n is a slot when a PDSCH reception providing the MAC CE ends, respectively, n+m is a slot indicated for PUCCH transmission with HARQ-ACK information for the PDSCH reception as described in clause 9.2.3, and Nslotsubframe,μ is a number of slots per subframe for the SCS configuration μ of the PUCCH transmission as defined in [4, TS 38.211].

When the deactivation of the second SS/PBCH blocks transmission is by a MAC CE, and with reference to slots for PUCCH transmission, the UE expects that the transmission of the second SS/PBCH blocks according to the indicated parameters terminates from the end of

-	a slot n+T, if the slot n+T is not within a first slot to a last slot with activated transmission of second SS/PBCH blocks in a half frame, where T=m+3 Nslotsubframe,μ+1, n is a slot when a PDSCH reception providing the MAC CE ends, n+m is a slot indicated for PUCCH transmission with HARQ-ACK information for the PDSCH reception as described in clause 9.2.3, and Nslotsubframe,μ is a number of slots per subframe for the SCS configuration μ of the PUCCH transmission as defined in [4, TS 38.211]

-	the first slot including the candidate SS/PBCH block corresponding to the index of the last transmitted second SS/PBCH block that is not earlier than the slot n+T, if the slot n+T is within a first slot to a last slot with activated transmission of second SS/PBCH blocks in a half frame

When the UE is not provided absoluteFrequencySSB for the SCell, the UE does not expect the transmission of the second SS/PBCH blocks provided by od-ssb-config to be deactivated while the SCell is activated.

When the first SS/PBCH blocks in a configured DL BWP can be used to obtain SIB1 and the frequency location of the first SS/PBCH blocks corresponds to the GSCN of a synchronization raster entry, the UE expects

-	a frequency location of the second SS/PBCH blocks to be different from the frequency location of the first SS/PBCH blocks and not to correspond to the GSCN of a synchronization raster entry

-	frequency resources of the second SS/PBCH blocks not to overlap with frequency resources of the first SS/PBCH blocks

-	the second SS/PBCH blocks to be within the configured DL BWP as the first SS/PBCH blocks

-	the second SS/PBCH blocks are not used to obtain SIB1

When the first SS/PBCH blocks in a configured DL BWP cannot be used to obtain SIB1, the UE expects

-	a same frequency location for the second SS/PBCH blocks and for the first SS/PBCH blocks

-	a same PBCH payload, other than the SFN index and the half frame index, for a first SS/PBCH block, from the first SS/PBCH blocks, and for a second SS/PBCH block, from the second SS/PBCH blocks, with same SS/PBCH block index as the first SS/PBCH block

The UE may assume that a first SS/PBCH block from the first SS/PBCH blocks and a second SS/PBCH block from the second SS/PBCH blocks are quasi co-located with respect to Doppler shift, Doppler spread, average gain, average delay, delay spread and, when applicable, spatial RX parameters, when they have a same SS/PBCH block index.

The UE may assume that SS/PBCH blocks from the second SS/PBCH blocks are quasi co-located with respect to Doppler shift, Doppler spread, average gain, average delay, delay spread and, when applicable, spatial RX parameters, when they have a same SS/PBCH block index.

## *** Unchanged parts are omitted ***

# 7	Uplink Power control

Uplink power control determines a power for PUSCH, PUCCH, SRS, and PRACH transmissions.

A UE does not expect to simultaneously maintain more than four pathloss estimates per serving cell for all PUSCH/PUCCH/SRS transmissions as described in clauses 7.1.1, 7.2.1, and 7.3.1, except for SRS transmissions configured by SRS-PosResourceSet as described in clause 7.3.1. If the UE is provided a number of RS resources for pathloss estimation for PUSCH/PUCCH/SRS transmissions that is larger than 4, the UE maintains for pathloss estimation RS resources corresponding to RS resource indexes qd as described in clauses 7.1.1, 7.2.1, and 7.3.1. If an RS resource updated by MAC CE, as described in clauses 7.1.1, 7.2.1 and 7.3.1, is one from the RS resources the UE maintains for pathloss estimation for PUSCH/PUCCH/SRS transmissions, the UE applies the pathloss estimation based on the RS resources starting from the first slot that is after slot k+3∙Nslotsubframe,  μ+2μ∙kmac where k is the slot where the UE would transmit a PUCCH or PUSCH with HARQ-ACK information for the PDSCH providing the MAC CE, μ  is the SCS configuration for the PUCCH or PUSCH, respectively, that is determined in the slot when the MAC CE command is applied and kmac is a number of slots for SCS configuration μ=0 provided by kmac or kmac=0 if kmac is not provided.

A PUSCH/PUCCH/SRS/PRACH transmission occasion i is defined by a slot index ns,fμ within a frame with system frame number SFN, a first symbol S within the slot, and a number of consecutive symbols L. For a PUSCH transmission with repetition Type B, a PUSCH transmission occasion is a nominal repetition [6, TS 38.214].

In the remaining of this clause, if a UE is provided TCI-State in dl-OrJointTCI-StateList or TCI-UL-State or CandidateTCI-State or CandidateTCI-UL-State indicated in the LTM Cell Switch Command MAC CE, and for each indicated one or two TCI-State or TCI-UL-State or CandidateTCI-State or CandidateTCI-UL-State of a PUSCH, PUCCH, or SRS transmission occasion as described in [6, TS 38.214]

-	in clauses 7.1.1, 7.2.1, and 7.3.1, the RS index qd for obtaining the downlink pathloss estimate for PUSCH, PUCCH, and SRS transmission is provided by pathlossReferenceRS-Id-r17 associated with or included in the indicated TCI-State or TCI-UL-State except for SRS transmission that is not provided followUnifiedTCI-StateSRS, or by pathlossReferenceRS-Id included in CandidateTCI-State or CandidateTCI-UL-State indicated in the LTM Cell Switch Command MAC CE

-	in clause 7.1.1, if p0AlphaSetforPUSCH is provided, the values of PO_UE_PUSCH,b,f,cj, αb,f,cj, and the PUSCH power control adjustment state l are provided by p0AlphaSetforPUSCH associated with the indicated TCI-State or TCI-UL-State, or by p0AlphaSetforPUSCH associated with the CandidateTCI-State or CandidateTCI-UL-State indicated in the LTM Cell Switch Command MAC CE

-	in clause 7.2.1, if p0AlphaSetforPUCCH is provided, the values of PO_UE_PUCCHqu and the PUCCH power control adjustment state l are provided by p0AlphaSetforPUCCH associated with the indicated TCI-State or TCI-UL-State, or by p0AlphaSetforPUCCH associated with the CandidateTCI-State or CandidateTCI-UL-State indicated in the LTM Cell Switch Command MAC CE

-	in clause 7.3.1, if p0AlphaSetforSRS is provided,

-	if followUnifiedTCI-StateSRS is provided for a SRS resource set, the values of  PO_UE_SRS,b,f,cqs, αSRS,b,f,cqs, and SRS power control adjustment state l are provided by p0AlphaSetforSRS associated with the indicated TCI-State or TCI-UL-State, or by p0AlphaSetforSRS associated with the CandidateTCI-State or CandidateTCI-UL-State indicated in the LTM Cell Switch Command MAC CE

-	else, if followUnifiedTCI-StateSRS is not provided for a SRS resource set and for a SRS resource from the SRS resource set, the values of PO_UE_SRS,b,f,cqs, αSRS,b,f,cqs, and SRS power control adjustment state l are provided by p0AlphaSetforSRS associated with TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set and a RS index qd for obtaining a pathloss estimate for the SRS transmission is provided by pathlossReferenceRS-Id-r17 associated with or included in the TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set

PO_SRS,b,f,cqs is the sum of the component PO_UE_SRS,b,f,cqs and a component p0 provided by SRS-ResourceSet corresponding to the SRS resource set.

In the remaining of this clause, if a UE obtains a downlink pathloss estimate based on reception of second SS/PBCH blocks, as described in Clause 4.4, ss-PBCH-BlockPower is replaced by od-ss-PBCH-BlockPower.

In the remaining of this clause, if a PDCCH reception by a UE includes two PDCCH candidates from corresponding search space sets, as described in clause 10.1

-	a PDCCH monitoring occasion is the union of the PDCCH monitoring occasions for the two PDCCH candidates

-	the end of the PDCCH reception is the end of the PDCCH candidate that ends later

The PDCCH reception includes the two PDCCH candidates also when the UE is not required to monitor one of the two PDCCH candidates as described in clauses 10 (except clause 10.4), 11.1, 11.1.1 and 17.2.

## *** Unchanged parts are omitted ***

## 8.1	Random access preamble

Physical random access procedure for a UE is triggered upon request of a PRACH transmission by higher layers or by a PDCCH order or LTM Cell Switch Command MAC CE in clause 6.1.3.75 [11, TS 38.321] for a cell. A configuration by higher layers for a PRACH transmission includes the following:

-	A configuration for PRACH transmission on the cell [4, TS 38.211].

-	A preamble index, a preamble SCS, PPRACH,target, a corresponding RA-RNTI when applicable [11, TS 38.321], and a PRACH resource for the cell.

-	A number of Npreamblerep>1 preamble repetitions for the PRACH transmission if the UE would transmit the PRACH with repetitions.

A UE transmits a PRACH on a cell using the selected PRACH format with transmission power PPRACH,b,f,c(i), as described in clause 7.4, on the indicated PRACH resource or on a determined set of Npreamblerep resources using a same spatial filter in case of Npreamblerep preamble repetitions.

For Type-1 random access procedure, a UE can be provided, by addl-RACH-Config-Adaptation in RACH-ConfigCommon, parameters for determining time resources and frequency resources for PRACH transmission [4, TS 38.211]. When a PRACH occasion associated with addl-RACH-Config-Adaptation has same frequency resource and same time resource as a PRACH occasion in RACH-ConfigCommon that is not associated with addl-RACH-Config-Adaptation, the former PRACH occasion is not valid and is not considered in the procedures in this clause.

For valid PRACH occasions associated with addl-RACH-Config-Adaptation in RACH-ConfigCommon, the UE can be additionally provided a PRACH mask index, by prach-SubsetMask-Index-Adaptation, that indicates one or more association periods per Kmask association pattern periods according to Table 8.1-0, where Kmask is provided by KforAPPForPRACHsubsetMask.

Table 8.1-0: Mapping of mask index to association periods per Kmask association pattern periods

| Mask Index | Association periods (APs) per Kmask association pattern periods (APPs) |
| --- | --- |
| 0 | First half of APs in Kmask APPs |
| 1 | First quarter of APs in Kmask APPs |
| 2 | First eighth of APs in Kmask APPs |
| 3 | First sixteenth of APs in Kmask APPs |

Valid PRACH occasions associated with addl-RACH-Config-Adaptation, and additionally in association periods indicated by prach-SubsetMask-Index-Adaptation, if provided, are indicated as available for PRACH transmission based on an indication in a DCI format 1_0 with CRC scrambled by a P-RNTI or a C-RNTI [5, TS 38.212]. For indication by DCI format 1_0 with CRC scrambled by the P-RNTI, the PRACH occasions are available for a duration provided by validity-DurationForAddlRACHAdaptation, starting from the first frame of the SI modification period [12, TS 38.331] that includes a PDCCH monitoring occasion where the UE receives a PDCCH providing the DCI format 1_0 with CRC scrambled by the P-RNTI.

In the following, for determining valid PRACH occasions, if a UE is not provided SS/PBCH block indexes by ssb-PositionsInBurst in ServingCellConfigCommon and is provided od-ssb-Config, reference to SS/PBCH blocks is for the union of SS/PBCH blocks provided by each combination of od-ssb-PositionsInBurst and od-ssb-Periodicity in od-ssb-Config. If the UE is provided SS/PBCH block indexes by ssb-PositionsInBurst in ServingCellConfigCommon and is provided od-ssb-Config, reference to SS/PBCH blocks is for the union of SS/PBCH blocks provided by ssb-Periodicity and ssb-PositionsInBurst in ServingCellConfigCommon and by each combination of od-ssb-PositionsInBurst and od-ssb-Periodicity in od-ssb-Config.

In the following, for determining an association between PRACH occasions and SS/PBCH block indexes, if a UE is not provided SS/PBCH block indexes by ssb-PositionsInBurst in ServingCellConfigCommon and is provided od-ssb-Config, reference to SS/PBCH block indexes provided by ssb-PositionsInBurst in ServingCellConfigCommon is for the union of SS/PBCH block indexes provided by each od-ssb-PositionsInBurst in od-ssb-Config. If the UE is provided SS/PBCH block indexes by ssb-PositionsInBurst in ServingCellConfigCommon and is provided od-ssb-Config, reference to SS/PBCH block indexes provided by ssb-PositionsInBurst in ServingCellConfigCommon is for the union of SS/PBCH block indexes provided by ssb-PositionsInBurst in ServingCellConfigCommon and by each od-ssb-PositionsInBurst in od-ssb-Config.

For Type-1 random access procedure, a UE is provided a number N of SS/PBCH block indexes associated with one PRACH occasion and a number R of contention based preambles per SS/PBCH block index per valid PRACH occasion either by ssb-perRACH-OccasionAndCB-PreamblesPerSSB or by ssb-perRACH-OccasionAndCB-PreamblesPerSSB-r19.

For Type-2 random access procedure with common configuration of PRACH occasions with Type-1 random access procedure, a UE is provided a number N of SS/PBCH block indexes associated with one PRACH occasion by ssb-perRACH-OccasionAndCB-PreamblesPerSSB and a number Q of contention based preambles per SS/PBCH block index per valid PRACH occasion by msgA-CB-PreamblesPerSSB-PerSharedRO. The PRACH transmission can be on a subset of PRACH occasions associated with a same SS/PBCH block index within an SSB-RO mapping cycle for a UE provided with a PRACH mask index by msgA-SSB-SharedRO-MaskIndex according to [11, TS 38.321].

For Type-2 random access procedure with separate configuration of PRACH occasions with Type-1 random access procedure, a UE is provided a number N of SS/PBCH block indexes associated with one PRACH occasion and a number R of contention based preambles per SS/PBCH block index per valid PRACH occasion by msgA-SSB-PerRACH-OccasionAndCB-PreamblesPerSSB when provided; otherwise, by ssb-perRACH-OccasionAndCB-PreamblesPerSSB.

For a random access procedure associated with a feature combination indicated by FeatureCombinationPreambles, a UE is provided a number N of SS/PBCH block indexes associated with one PRACH occasion by ssb-perRACH-OccasionAndCB-PreamblesPerSSB or msgA-SSB-PerRACH-OccasionAndCB-PreamblesPerSSB when provided and a number S of contention based preambles per SS/PBCH block index per valid PRACH occasion by startPreambleForThisPartition and numberOfPreamblesPerSSB-ForThisPartition. The PRACH transmission can be on a subset of PRACH occasions associated with a same SS/PBCH block index within an SSB-RO mapping cycle for a UE provided with a PRACH mask index by ssb-SharedRO-MaskIndex according to [11, TS 38.321].

For Type-1 random access procedure, or for Type-2 random access procedure with separate configuration of PRACH occasions from Type 1 random access procedure, if N<1, one SS/PBCH block index is mapped to 1N consecutive valid PRACH occasions and R contention based preambles with consecutive indexes associated with the SS/PBCH block index per valid PRACH occasion start from preamble index 0. If N≥1, R contention based preambles with consecutive indexes associated with SS/PBCH block index n, 0≤n≤N-1, per valid PRACH occasion start from preamble index n⋅NpreambletotalN where Npreambletotal is provided by totalNumberOfRA-Preambles for Type-1 random access procedure, or by msgA-TotalNumberOfRA-Preambles for Type-2 random access procedure with separate configuration of PRACH occasions from a Type 1 random access procedure, and is an integer multiple of N.

For Type-2 random access procedure with common configuration of PRACH occasions with Type-1 random access procedure, if N<1, one SS/PBCH block index is mapped to 1N consecutive valid PRACH occasions and Q contention based preambles with consecutive indexes associated with the SS/PBCH block index per valid PRACH occasion start from preamble index R. If N≥1, Q contention based preambles with consecutive indexes associated with SS/PBCH block index n, 0≤n≤N-1, per valid PRACH occasion start from preamble index n⋅NpreambletotalN+R, where Npreambletotal is provided by totalNumberOfRA-Preambles for Type-1 random access procedure.

For link recovery, a UE is provided N SS/PBCH block indexes associated with one PRACH occasion by ssb-perRACH-Occasion in BeamFailureRecoveryConfig. For a dedicated RACH configuration provided by RACH-ConfigDedicated, if cfra is provided, a UE is provided N SS/PBCH block indexes associated with one PRACH occasion by ssb-perRACH-Occasion in occasions. For the PRACH transmission on a candidate cell [38.213, clause 21], a UE is provided N SS/PBCH block indexes associated with one PRACH occasion by ssb-PerRACH-Occasion-r18 in EarlyUL-SyncConfig. If N<1, one SS/PBCH block index is mapped to 1N consecutive valid PRACH occasions. If N≥1, all consecutive N SS/PBCH block indexes are associated with one PRACH occasion.

SS/PBCH block indexes provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon or in SSB-MTC-AdditionalPCI or in LTM-SSB-Config are mapped to valid PRACH occasions in the following order where the parameters are described in [4, TS 38.211]. The mapping of SS/PBCH block indexes to valid PRACH occasions is separate for valid PRACH occasions determined by RACH-ConfigCommon and for valid PRACH occasions determined by addl-RACH-Config-Adaptation.

-	First, in increasing order of preamble indexes within a single PRACH occasion

-	Second, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions

-	Third, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot

-	Fourth, in increasing order of indexes for PRACH slots

An association period, starting from frame 0, for mapping SS/PBCH block indexes to PRACH occasions is the smallest integer number in the set determined by the PRACH configuration period according to Table 8.1-1 such that NTxSSB SS/PBCH block indexes are mapped at least once to the PRACH occasions within the association period, where a UE obtains NTxSSB from the value of ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon or in SSB-MTC-AdditionalPCI or in LTM-SSB-Config. If after an integer number of SS/PBCH block indexes to PRACH occasions mapping cycles within the association period there is a set of PRACH occasions or PRACH preambles that are not mapped to NTxSSB SS/PBCH block indexes, no SS/PBCH block indexes are mapped to the set of PRACH occasions or PRACH preambles. An association pattern period includes one or more association periods and is determined so that a pattern between PRACH occasions and SS/PBCH block indexes repeats at most every 160 msec. PRACH occasions not associated with SS/PBCH block indexes after an integer number of association periods, if any, are not used for PRACH transmissions.

Table 8.1-1: Mapping between PRACH configuration period and SS/PBCH block to PRACH occasion association period

| PRACH configuration period (msec) | Association period (number of PRACH configuration periods) |
| --- | --- |
| 10 | {1, 2, 4, 8, 16} |
| 20 | {1, 2, 4, 8} |
| 40 | {1, 2, 4} |
| 80 | {1, 2} |
| 160 | {1} |

For a PRACH transmission by a UE triggered by a PDCCH order or an LTM cell switch command MAC CE, the PRACH mask index field, if the value of the random access preamble index field is not zero, indicates the PRACH occasion for the PRACH transmission where the PRACH occasions are associated with the SS/PBCH block index indicated by the SS/PBCH block index field of the PDCCH order or the LTM cell switch command MAC CE and, if any, a cell indicator field in PDCCH order [5, TS 38.212] or a Target Configuration ID field in LTM cell switch command MAC CE [11, TS 38.321] indicates a cell for the PRACH transmission. If the UE is provided Kcell,offset by cellSpecificKoffset, the PRACH occasion is after slot n+2μ∙Kcell,offset where n is the slot of the UL BWP for the PRACH transmission that overlaps with the end of the PDCCH order reception assuming TTA=0, and μ is the SCS configuration for the PRACH transmission. If the PDCCH reception for the PDCCH order includes two PDCCH candidates from two linked search space sets based on searchSpaceLinkingId, as described in clause 10.1, the last symbol of the PDCCH reception is the last symbol of the PDCCH candidate that ends later. The PDCCH reception includes the two PDCCH candidates also when the UE is not required to monitor one of the two PDCCH candidates as described in clauses 10 (except clause 10.4), 11.1, 11.1.1 and 17.2.

For a PRACH transmission triggered by higher layers, if ssb-ResourceList is provided, the PRACH mask index is indicated by ra-ssb-OccasionMaskIndex which indicates the PRACH occasions for the PRACH transmission where the PRACH occasions are associated with the selected SS/PBCH block index.

The PRACH occasions are mapped consecutively per corresponding SS/PBCH block index. The indexing of the PRACH occasion indicated by the mask index value is reset per mapping cycle of consecutive PRACH occasions per SS/PBCH block index. The UE selects for a PRACH transmission the PRACH occasion indicated by PRACH mask index value for the indicated SS/PBCH block index in the first available mapping cycle.

For the indicated preamble index, the ordering of the PRACH occasions is

-	First, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions

-	Second, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot

-	Third, in increasing order of indexes for PRACH slots

For a PRACH transmission with Npreamblerep preamble repetitions, a set consists of Npreamblerep valid PRACH occasions that are consecutive in time, use same frequency resources, and are associated with same one or more SS/PBCH block index(es), and each SS/PBCH block index is associated with same preamble indexes in all valid PRACH occasions within the set.

For a PRACH transmission triggered by higher layers, if ssb-ResourceList is provided, the PRACH mask index is indicated by ra-ssb-OccasionMaskIndex which indicates the PRACH occasions for the PRACH transmission where the PRACH occasions are associated with the selected SS/PBCH block index.

The PRACH occasions are mapped consecutively per corresponding SS/PBCH block index. The indexing of the PRACH occasion indicated by the mask index value is reset per mapping cycle of consecutive PRACH occasions per SS/PBCH block index. The UE selects for a PRACH transmission the PRACH occasion indicated by PRACH mask index value for the indicated SS/PBCH block index in the first available mapping cycle.

For the indicated preamble index, the ordering of the PRACH occasions is

-	First, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions

-	Second, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot

-	Third, in increasing order of indexes for PRACH slots

For a PRACH transmission with Npreamblerep preamble repetitions, a set consists of Npreamblerep valid PRACH occasions that are consecutive in time, use same frequency resources, and are associated with same one or more SS/PBCH block index(es), and each SS/PBCH block index is associated with same preamble indexes in all valid PRACH occasions within the set.

For a PRACH transmission with preamble repetitions, a time period, starting from frame 0, is the smallest integer number of association pattern periods such that at least one set of valid PRACH occasions for each of the NTxSSB SS/PBCH block indexes can be determined within the time period for all configured number of preamble repetitions for each supported feature combination provided in each RACH-ConfigCommon. The set(s) of valid PRACH occasions for each configured number of preamble repetitions repeats every time period.

Within a time period, for set(s) of Npreamblerep valid PRACH occasions for a PRACH transmission with Npreamblerep preamble repetitions

-	the first valid PRACH occasion of the first set is the first valid PRACH occasion

-	the first valid PRACH occasion of subsequent sets, if any, is determined according to an ordering of valid PRACH occasions

-	first, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions

-	second, in increasing order of time resource indexes for time multiplexed PRACH occasions

where, for each frequency resource index for frequency multiplexed PRACH occasions

-	the first valid PRACH occasion of the first set is the first valid PRACH occasion

-	the first valid PRACH occasion of subsequent sets, if any,

-	is after msg1-RepetitionTimeOffsetROGroup consecutive valid PRACH occasions in time from the first valid PRACH occasion of the previous set, where each PRACH occasion is associated with same SS/PBCH block index(es) and each SS/PBCH block index is associated with same preambles, if msg1-RepetitionTimeOffsetROGroup is provided

-	is after the PRACH occasions for the previous set, if msg1-RepetitionTimeOffsetROGroup is not provided

For a PRACH transmission with preamble repetitions in CFRA procedure, msg1-RepetitionTimeOffsetROGroup is determined by the FeatureCombinationPreambles indicating msg1-Repetitions with same value as msg1-RepetitionNum provided by RACH-ConfigDedicated.

For a PRACH transmission triggered upon request by higher layers, a value of ra-OccasionList [12, TS 38.331], if csirs-ResourceList is provided, indicates a list of PRACH occasions for the PRACH transmission where the PRACH occasions are associated with the selected CSI-RS index indicated by csi-RS. The indexing of the PRACH occasions indicated by ra-OccasionList is reset per association pattern period.

For paired spectrum or supplementary uplink band all PRACH occasions are valid.

## *** Unchanged parts are omitted ***

# 10	UE procedure for receiving control information

## *** Unchanged parts are omitted ***

For monitoring of a PDCCH candidate by a UE, if the UE

-	has received ssb-PositionsInBurst in SIB1 and has not received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell, and

-	does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and

-	at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in SIB1,

the UE is not required to monitor the PDCCH candidate.

For monitoring of a PDCCH candidate by a UE, if the UE

-	has received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell, and

-	does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and

-	at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in ServingCellConfigCommon,

the UE is not required to monitor the PDCCH candidate.

For monitoring of a PDCCH candidate by a UE, if the UE

-	has received an indication for transmission of second SS/PBCH blocks on a serving cell as described in Clause 4.4, and

-	at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, from the second SS/PBCH blocks, corresponding to a SS/PBCH block index provided as described in Clause 4.4,

the UE is not required to monitor the PDCCH candidate.

For monitoring of a PDCCH candidate by a UE, if the UE

-	has received ssb-PositionsInBurst in SSB-MTCAdditionalPCI for a serving cell, and

-	at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in SSB-MTCAdditionalPCI with same physical cell identity as the one associated with a RS having same quasi-collocation properties as a CORESET for the PDCCH candidate,

the UE is not required to monitor the PDCCH candidate.

A UE is not required to monitor PDCCH candidates for a Type0/0A/0B/1/1A /2/2A -PDCCH CSS set when the active TCI state for a corresponding CORESET is not associated with physCellId in ServingCellConfigCommon.

## *** Unchanged parts are omitted ***

## 10.1	UE procedure for determining physical downlink control channel assignment

-	a Type3-PDCCH CSS set configured by

-	SearchSpace in PDCCH-Config with searchSpaceType = common for DCI formats with CRC scrambled by INT-RNTI, SFI-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, TPC-SRS-RNTI, CI-RNTI, cellDTRX-RNTI, or ssbPeriodicityIndication-RNTI and, only for the primary cell, C-RNTI, MCS-C-RNTI, CS-RNTI(s), or PS-RNTI, or

-	SearchSpace in pdcch-ConfigMulticast for DCI formats with CRC scrambled by G-RNTI, or G-CS-RNTI, or

-	searchSpaceMCCH and searchSpaceMTCH on a secondary cell for a DCI format 4_0 with CRC scrambled by a MCCH-RNTI or a G-RNTI for broadcast, and

-	a USS set configured by

-	SearchSpace or by SearchSpaceExt-v1800 in PDCCH-Config with searchSpaceType = ue-Specific for DCI formats with CRC scrambled by C-RNTI, MCS-C-RNTI, SP-CSI-RNTI, CS-RNTI(s), SL-RNTI, SL-CS-RNTI, SL Semi-Persistent Scheduling V-RNTI, or NCR-RNTI

## *** Unchanged parts are omitted ***

## 11.1	Slot configuration

## *** Unchanged parts are omitted ***

For a set of symbols of a slot that are indicated to a UE as flexible by tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated if provided, the UE does not expect to receive both dedicated higher layer parameters configuring transmission from the UE in the set of symbols of the slot and dedicated higher layer parameters configuring reception by the UE in the set of symbols of the slot.

For operation on a single carrier in unpaired spectrum, for a set of symbols of a slot indicated to a UE for reception of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or by ssb-PositionsInBurst in ServingCellConfigCommon or as described in Clause 4.4 or, if the UE is not provided dl-OrJointTCI-StateList, by ssb-PositionsInBurst in SSB-MTCAdditionalPCI associated to physical cell ID with active TCI states for PDCCH or PDSCH, or for a set of symbols of a slot corresponding to SS/PBCH blocks configured for L1 beam measurement/reporting, the UE does not transmit PUSCH, PUCCH, PRACH in the slot if a transmission would overlap with any symbol from the set of symbols and the UE does not transmit SRS in the set of symbols of the slot. The UE does not expect the set of symbols of the slot to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, when provided to the UE.

If a UE

-	is configured with multiple serving cells and is provided with directionalCollisionHandling-r16 = 'enabled' for a set of serving cell(s) among the multiple serving cells, and

-	indicates support of half-DuplexTDD-CA-SameSCS-r16 capability, and

-	is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells,

for a set of symbols of a slot that are indicated to the UE for reception of SS/PBCH blocks in a first cell of the multiple serving cells by ssb-PositionsInBurst in SystemInformationBlockType1 or by ssb-PositionsInBurst in ServingCellConfigCommon or as described in Clause 4.4 or, if the UE is not provided dl-OrJointTCI-StateList, by ssb-PositionsInBurst in SSB-MTCAdditionalPCI associated to physical cell ID with active TCI states for PDCCH or PDSCH, or for a set of symbols of a slot corresponding to SS/PBCH blocks configured for L1 beam measurement/reporting, the UE does not transmit PUSCH, PUCCH, or PRACH in the slot if a transmission would overlap with any symbol from the set of symbols, and the UE does not transmit SRS in the set of symbols of the slot in

-	any of the multiple serving cells if the UE is not capable of simultaneous transmission and reception as indicated by simultaneousRxTxInterBandCA among the multiple serving cells, and

-	any one of the cells corresponding to the same band as the first cell, irrespective of any capability indicated by simultaneousRxTxInterBandCA.

For a set of symbols of a slot corresponding to a valid PRACH occasion and Ngap symbols before the valid PRACH occasion, as described in clause 8.1, the UE does not receive PDCCH, PDSCH, or CSI-RS in the slot if a reception would overlap with any symbol from the set of symbols. The UE does not expect the set of symbols of the slot to be indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

## *** Unchanged parts are omitted ***

## 11.5	Adaptation of cell operation

A UE configured for operation on a serving cell according to one or both of a cell DTX operation and a cell DRX operation by cellDTRX-DCI-Config for the serving cell [11, TS 38.321], can be additionally provided by dci-Format2-9 a Type3-PDCCH CSS set to monitor PDCCH for detection of DCI format 2_9 with CRC scrambled by a cellDTRX-RNTI as described in clause 10.1 during Active Time [11, TS 38.321], and a location in DCI format 2_9 by positionInDCI-cellDTRX of a cell DTX/DRX indication field for the serving cell and/or a NES-mode indication field for the PCell

## *** Unchanged parts are omitted ***

## 11.6	Periodicity adaptation for reception of SS/PBCH blocks on a secondary cell

A UE can be provided, by addl-ssb-Periodicity, a set of periodicities for reception of SS/PBCH blocks on an SCell.

The UE can be additionally provided, by dci-Format2-9, a Type3-PDCCH CSS set to monitor PDCCH for detection of DCI format 2_9 with CRC scrambled by a ssbPeriodicityIndication-RNTI as described in clause 10.1, and a location in DCI format 2_9 by positionInDCI-ssbPeriod an SS/PBCH block reception periodicity indication field for the SCell [5, TS 38.212].

If the UE is provided one periodicity value by addl-ssb-Periodicity

-	a '0' value for the SS/PBCH block reception periodicity indication field indicates that the SS/PBCH block reception periodicity is provided by ssb-periodicityServingCell, and

-	a '1' value for the SS/PBCH block reception periodicity indication field indicates that the SS/PBCH block reception periodicity is provided by addl-ssb-Periodicity

If the UE is provided first and second periodicity values by addl-ssb-Periodicity

-	a '00' value for the SS/PBCH block reception periodicity indication field indicates that the SS/PBCH block reception periodicity is provided by ssb-periodicityServingCell,

-	a '01' value for the SS/PBCH block reception periodicity indication field indicates that the SS/PBCH block reception periodicity is provided by the first periodicity value, and

-	a '10' value for the SS/PBCH block reception periodicity indication field indicates that the SS/PBCH block reception periodicity is provided by the second periodicity value

When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in periodicity for reception of SS/PBCH blocks on a second serving cell, the UE expects that the transmission of SS/PBCH blocks according to the indicated periodicity on the second serving cell starts from a first slot on the second serving cell that includes the candidate SS/PBCH block occasion corresponding to the first transmitted SS/PBCH block provided by ssb-PositionsInBurst in a first half frame for the indicated periodicity that

-	is within a set of half frames with a half frame index provided by addl-ssb-halfFrameIndex, and has an SFN offset, relative to SFN 0, provided by addl-ssb-Offset, when the indicated periodicity is provided by addl-ssb-Periodicity, or

-	is within a set of half frames determined by the UE, when the indicated periodicity is provided by ssb-periodicityServingCell

and does not begin before the beginning of slot m+d on the active DL BWP of the first serving cell, where d is a number of slots for the SCS of the active DL BWP of the first serving cell in Table 11.5-1.

*** Unchanged parts are omitted ***

# 23	UE procedure to request SIB1 reception

Unless otherwise mentioned, the higher layer parameters in this clause and in referenced clauses are provided by SIB1-RequestConfig on a first cell.

A UE can be provided, by NES_CellId, a physical cell identity of a second cell and an ARFCN by ARFCN-ValueNR for SS/PBCH block receptions on the second cell. When

-	the UE receives an SS/PBCH block on the second cell, and

-	kSSB>23 for FR1 or kSSB>11 for FR2 is indicated by the SS/PBCH block on the second cell, and

-	conditions for PRACH transmission associated with the SS/PBCH block on the second cell are satisfied [12, TS 38.331],

the UE is not required to monitor PDCCH on the second cell to detect the DCI format 1_0 with CRC scrambled by the SI-RNTI and can transmit a PRACH associated with the SS/PBCH block on the second cell based on:

-	a timing adjustment indicated by n-TimingAdvanceOffset, if provided, as described in Clause 4.2

-	a power determined as described in Clause 7.4

-	a procedure determined as in Clause 8.1 based on Type-1 random access procedure

where kSSB for determining the common resource block [4, TS 38.211] is provided by k-ssb.

In response to the PRACH transmission, the UE monitors PDCCH on the second cell to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a window controlled by ra_ResponseWindow, as described in Clause 8.2. The UE monitors PDCCH according to a Type1-PDCCH CSS set provided by ra-SearchSpace, if provided; else provided by SearchSpaceZero as described in Clause 10.1, in a CORESET provided by controlResourceSetZero.

If the higher layers at the UE identify a RAPID associated with a corresponding PRACH transmission from the UE in a PDSCH reception scheduled by the DCI format 1_0 with CRC scrambled by the RA-RNTI, the higher layers indicate to the physical layer to monitor PDCCH on the second cell to detect a DCI format 1_0 with CRC scrambled by the SI-RNTI according to a Type0-PDCCH CSS set provided by SearchSpaceZero. If the UE is provided sib1-pdcchRestrictionToPRACHAssociatedSSBs, the UE monitors PDCCH only in monitoring occasions associated with the SS/PBCH block; otherwise, the UE assumes that PDCCH is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB. The UE monitors PDCCH to detect the DCI format 1_0 with CRC scrambled by the SI-RNTI after a number of slots provided by od-sib1-windowStartOffset from the start of the first slot of the window controlled by ra_ResponseWindow, and for a number of slots provided by od-sib1-WindowDuration.

After the PRACH transmission, the UE is not required to monitor PDCCH on the second cell to detect the DCI format 1_0 with CRC scrambled by the SI-RNTI prior to a reception of a PDSCH providing a RAPID associated with a corresponding PRACH transmission scheduled by the DCI format 1_0 with CRC scrambled by the RA-RNTI.

*** Unchanged parts are omitted ***