3GPP TSG-RAN WG1 Meeting #121	R1-2504995

St Julian's, Malta, May 19th – 23rd, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0675 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Multi-carrier enhancements for NR Phase 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MC_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-06-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of enhancements for Multi-carrier enhancements for NR Phase 3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of multi-PUSCH scheduling using DCI format 0_3 and multi-PDSCH scheduling using DCI format 1_3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multi-PUSCH scheduling using DCI format 0_3 and multi-PDSCH scheduling using DCI format 1_3 not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.2.1, 5.1.2.1.1, 5.1.3.2, 5.1.4.2, 5.1.5, 5.2.3, 6.1, 6.1.2.1, 6.1.2.1.1, 6.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.212, TS 38.213 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<omitted text>

#### 5.1.2.1	Resource allocation in time domain

When the UE is scheduled to receive PDSCH by a DCI, the Time domain resource assignment field value m for the scheduled PDSCH on the serving cell provides a row index m + 1 to a resource allocation table. The determination of the used resource allocation table is defined in Clause 5.1.2.1.1. The indexed row defines the slot offset K0, the start and length indicator SLIV, or directly the start symbol S and the allocation length L, and the PDSCH mapping type to be assumed in the PDSCH reception.

Given the parameter values of the indexed row:

-	The slot allocated for the PDSCH is Ks, where , if UE is configured with ca-SlotOffset for at least one of the scheduled and scheduling cell, and Ks = , otherwise, and where n is the slot with the scheduling DCI, and K0 is based on the numerology of PDSCH, and  and are the subcarrier spacing configurations for PDSCH and PDCCH, respectively, and

-	Nslot, offset, PDCCHCA and μoffset,PDCCH are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset, for the cell receiving the PDCCH respectively, Nslot, offset, PDSCHCA and μoffset,PDSCH are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDSCH, as defined in clause 4.5 of [4, TS 38.211].

-	The reference point S0 for starting symbol S is defined as:

-	if configured with referenceOfSLIVDCI-1-2, and when receiving PDSCH scheduled by DCI format 1_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI with K0=0, and PDSCH mapping Type B, the starting symbol S is relative to the starting symbol S0 of the PDCCH monitoring occasion where DCI format 1_2 is detected; when the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], the PDCCH candidate that starts later in time is used for the purpose of determining the starting symbol S0;

-	otherwise, the starting symbol S is relative to the start of the slot using S0=0.

-	The number of consecutive symbols L counting from the starting symbol S allocated for the PDSCH are determined from the start and length indicator SLIV:

if  then

else

where, and

-	the PDSCH mapping type is set to Type A or Type B as defined in Clause 7.4.1.1.2 of [4, TS 38.211].

The UE shall consider the S and L combinations defined in table 5.1.2.1-1 satisfying  for normal cyclic prefix and  for extended cyclic prefix as valid PDSCH allocations:

Table 5.1.2.1-1: Valid S and L combinations

| PDSCH mapping type | Normal cyclic prefix |  |  | Extended cyclic prefix |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | S | L | S+L | S | L | S+L |
| Type A | {0,1,2,3} (Note 1) | {3,…,14} | {3,…,14} | {0,1,2,3} (Note 1) | {3,…,12} | {3,…,12} |
| Type B | {0,…,12} | {2,…,13} | {2,…,14} | {0,…,10} | {2,4,6} | {2,…,12} |
| Note 1: S = 3 is applicable only if dmrs-TypeA-Position = 3 |  |  |  |  |  |  |

When configured with SCS m = 5 or m = 6, the UE does not expect to be scheduled with more than one unicast PDSCH in a slot, by a single DCI scheduling multiple PDSCHs or by multiple DCIs, where multiple DCIs are not associated with CORESETs having different coresetpoolIndex.

When receiving PDSCH scheduled by DCI format 1_1, 1_2 or 1_3 in PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, or scheduled by DCI format 1_1 or 1_2 in PDCCH with CRC scrambled by CS-RNTI with NDI=1, if the UE is configured with pdsch-AggregationFactor in pdsch-config, the same symbol allocation is applied across the pdsch-AggregationFactor consecutive slots. When receiving PDSCH scheduled by DCI format 1_1 or 1_2 in PDCCH with CRC scrambled by CS-RNTI with NDI=0, or PDSCH scheduled without corresponding PDCCH transmission using sps-Config and activated by DCI format 1_1 or 1_2, the same symbol allocation is applied across the pdsch-AggregationFactor, in sps-Config if configured, or across the pdsch-AggregationFactor in pdsch-config otherwise, consecutive slots. The UE may expect that the TB is repeated within each symbol allocation among each of the pdsch-AggregationFactor consecutive slots and the PDSCH is limited to a single transmission layer. For PDSCH scheduled by DCI format 1_1 or 1_2 in PDCCH with CRC scrambled by CS-RNTI with NDI=0, or PDSCH scheduled without corresponding PDCCH transmission using sps-Config and activated by DCI format 1_1 or 1_2, the UE is not expected to be configured with the time duration for the reception of pdsch-AggregationFactor repetitions, in sps-Config if configured, or across the pdsch-AggregationFactor in pdsch-config otherwise, larger than the time duration derived by the periodicity P obtained from the corresponding sps-Config. The redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, …pdsch-AggregationFactor -1, is determined according to table 5.1.2.1-2 and "rvid indicated by the DCI scheduling the PDSCH" in table 5.1.2.1-2 is assumed to be 0 for PDSCH scheduled without corresponding PDCCH transmission using sps-Config and activated by DCI format 1_1 or 1_2.

When receiving PDSCH scheduled by DCI format 4_1, or 4_2 in PDCCH with CRC scrambled by G-RNTI for multicast, if the UE is configured with pdsch-AggregationFactor in the MBS-RNTI-SpecificConfig associated with the corresponding G-RNTI for multicast, the same symbol allocation is applied across the pdsch-AggregationFactor consecutive slots. When receiving PDSCH scheduled by DCI format 4_1 or 4_2 for multicast reception in PDCCH with CRC scrambled by G-CS-RNTI, or PDSCH without corresponding PDCCH transmission using associated SPS-Config and activated by the DCI format 4_1 or 4_2 in PDCCH with CRC scrambled by G-CS-RNTI, the same symbol allocation is applied across the pdsch-AggregationFactor in associated SPS-Config if configured, or across repetitionNumber in PDSCH-TimeDomainResourceAllocation in pdsch-ConfigMulticast if provided by an entry indicated by the 'Time domain resource assignment' field of the activating DCI, or 1 otherwise, consecutive slots. The redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, …pdsch-AggregationFactor -1, is determined according to table 5.1.2.1-2 and "rvid indicated by the DCI scheduling the PDSCH" in table 5.1.2.1-2 is assumed to be 0 for PDSCH scheduled without corresponding PDCCH transmission using SPS-Config and activated by DCI format 4_1 or 4_2. When receiving PDSCH scheduled by DCI format 4_1 in PDCCH with CRC scrambled by G-RNTI for multicast in RRC_INACTIVE state, if the UE is configured with pdsch-AggregationFactor in the PDSCH-ConfigPTM-r18 for multicast in RRC_INACTIVE state, the same symbol allocation is applied across the pdsch-AggregationFactor consecutive slots, and the redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, …pdsch-AggregationFactor -1, is determined according to table 5.1.2.1-2. When receiving PDSCH scheduled by DCI format 4_0 in PDCCH with CRC scrambled by G-RNTI for broadcast, if the UE is configured with pdsch-AggregationFactor in the PDSCH-ConfigPTM, the same symbol allocation is applied across the pdsch-AggregationFactor consecutive slots, and the redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, …pdsch-AggregationFactor -1, is determined according to table 5.1.2.1-2.

When receiving PDSCH scheduled by DCI in PDCCH with CRC scrambled by G-CS-RNTI for multicast reception or G-RNTI, if the DCI field 'Time domain resource assignment' indicates an entry which contains repetitionNumber in PDSCH-TimeDomainResourceAllocation in the pdsch-ConfigMulticast, or pdsch-Config-MTCH, or PDSCH-Config-Broadcast, the same SLIV is applied for all PDSCH transmission occasions across the repetitionNumber consecutive slots. When receiving PDSCH scheduled without corresponding PDCCH transmission using associated SPS-Config and activated by DCI in PDCCH with CRC scrambled by G-CS-RNTI for multicast reception, if the DCI field 'Time domain resource assignment' of the activating DCI indicates an entry which contains repetitionNumber in PDSCH-TimeDomainResourceAllocation in the pdsch-ConfigMulticast, the same SLIV is applied for all PDSCH transmission occasions across the repetitionNumber consecutive slots. The redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, …repetitionNumber -1, is determined according to table 5.1.2.1-2 and "rvid indicated by the DCI scheduling the PDSCH" in table 5.1.2.1-2 is assumed to be 0 for PDSCH scheduled without corresponding PDCCH transmission using sps-Config and activated by DCI format with CRC scrambled by G-CS-RNTI.

If a UE is configured with higher layer parameter repetitionNumber or if the UE is configured by repetitionScheme set to one of ' fdmSchemeA', ' fdmSchemeB' and 'tdmSchemeA', the UE does not expect to be configured with pdsch-AggregationFactor for the same PDSCH.

If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH or pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 in which one or more rows contain multiple SLIVs for PDSCH, the UE does not expect to be configured with higher layer parameter repetitionNumber in pdsch-TimeDomainAllocationListForMultiPDSCH or pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3.

If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH in which one or more rows contain multiple SLIVs for PDSCH on a DL BWP of a serving cell, the UE does not apply pdsch-AggregationFactor in PDSCH-config, if configured, to DCI format 1_1 on the DL BWP of the serving cell. If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 in which one or more rows contain multiple SLIVs for PDSCH on a DL BWP of a serving cell, the UE does not apply pdsch-AggregationFactor in PDSCH-config, if configured, to DCI format 1_3 on the DL BWP of the serving cell.

If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH or pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 in which one or more rows contain multiple SLIVs for PDSCH on a DL BWP of a serving cell, when any two DL DCIs end in the same symbol and at least one of the DCIs schedules multiple PDSCHs, the UE does not expect that the scheduled PDSCH(s) by the two DCIs  have overlapping spans, where the span associated with a DCI is defined from the beginning of the first scheduled PDSCH or up to the end of the last scheduled PDSCH.

If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH in which one or more rows contain multiple SLIVs for PDSCH on a DL BWP of a serving cell within a PUCCH group, the UE does not expect to be configured with higher layer parameter ScheduledCell-ListDCI-1-3 on any serving cell within the PUCCH group.

Table 5.1.2.1-2: Applied redundancy version when pdsch-AggregationFactor or repetitionNumber is present

| rvid indicated by the DCI scheduling the PDSCH | rvid to be applied to nth transmission occasion |  |  |  |
| --- | --- | --- | --- | --- |
|  | n mod 4 = 0 | n mod 4 = 1 | n mod 4 = 2 | n mod 4 = 3 |
| 0 | 0 | 2 | 3 | 1 |
| 2 | 2 | 3 | 1 | 0 |
| 3 | 3 | 1 | 0 | 2 |
| 1 | 1 | 0 | 2 | 3 |

A PDSCH reception in a slot of a multi-slot PDSCH reception is omitted according to the conditions in clause 11.1 and clause 17.2 of [6, TS 38.213].

The UE is not expected to receive a PDSCH with mapping type A in a slot, if the PDCCH scheduling the PDSCH was received in the same slot and was not contained within the first three symbols of the slot. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], if the two PDCCH candidates scheduling the PDSCH with mapping Type A were received in the same slot as the PDSCH, both PDCCH candidates are expected to be contained within the first three symbols of the slot.

The UE is not expected to receive a PDSCH with mapping type B in a slot, if the first symbol of the PDCCH scheduling the PDSCH was received in a later symbol than the first symbol indicated in the PDSCH time domain resource allocation. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], the UE is not expected to receive a PDSCH with mapping type B in a slot, if the first symbol of the PDCCH candidate that starts later in time scheduling the PDSCH was received in a later symbol than the first symbol indicated in the PDSCH time domain resource allocation.

When the UE is configured with minimumSchedulingOffsetK0 in an active DL BWP it applies a minimum scheduling offset restriction indicated by the 'Minimum applicable scheduling offset indicator' field in DCI format 0_1, 0_3, 1_1 or 1_3, if the same field is available. When the UE is configured with minimumSchedulingOffsetK0 in an active DL BWP and it has not received 'Minimum applicable scheduling offset indicator' field in DCI format 0_1, 0_3, 1_1 or 1_3, the UE shall apply a minimum scheduling offset restriction indicated based on 'Minimum applicable scheduling offset indicator' value '0'. When the minimum scheduling offset restriction is applied the UE is not expected to be scheduled with a DCI in slot n to receive a PDSCH scheduled with C-RNTI, CS-RNTI or MCS-C-RNTI with K0 smaller than  K0min⋅2μ'2μ, where K0min and μ are the applied minimum scheduling offset restriction and the numerology of the active DL BWP of the scheduled cell when receiving the DCI in slot n, respectively, and μ' is the numerology of the new active DL BWP in case of active DL BWP change in the scheduled cell and is equal to μ, otherwise. The minimum scheduling offset restriction is not applied when PDSCH transmission is scheduled with C-RNTI, CS-RNTI or MCS-C-RNTI in common search space associated with CORESET0 and default PDSCH time domain resource allocation is used, in the search space set provided by recoverySearchSpaceId when monitoring PDCCH as described in [6, TS 38.213] or when PDSCH transmission is scheduled with SI-RNTI, MSGB-RNTI or RA-RNTI. The application delay of the change of the minimum scheduling offset restriction is determined in Clause 5.3.1.

The UE is not expected to be configured with referenceOfSLIVDCI-1-2 for serving cells configured for cross-carrier scheduling with a scheduling cell of a different downlink SCS configuration.

When a UE is configured by the higher layer parameter repetitionScheme set to 'tdmSchemeA' and indicated DM-RS port(s) within one CDM group in the DCI field 'Antenna Port(s)', the number of PDSCH transmission occasions is derived by the number of TCI states indicated by the DCI field 'Transmission Configuration Indication' of the scheduling DCI for the UE not configured with dl-OrJointTCI-StateList, or by the number of indicated TCI states when the UE configured with dl-OrJointTCI-StateList.

-	If two TCI states are indicated by the DCI field 'Transmission Configuration Indication' for a UE not configured with dl-OrJointTCI-StateList, or both indicated TCI states applicable to PDSCH for a UE configured with dl-OrJointTCI-StateList, the UE is expected to receive two PDSCH transmission occasions, where the first TCI state is applied to the first PDSCH transmission occasion and resource allocation in time domain for the first PDSCH transmission occasion follows Clause 5.1.2.1. The second TCI state is applied to the second PDSCH transmission occasion, and the second PDSCH transmission occasion shall have the same number of symbols as the first PDSCH transmission occasion. If the UE is configured by the higher layers with a value K in StartingSymbolOffsetK, it shall determine that the first symbol of the second PDSCH transmission occasion starts after K symbols from the last symbol of the first PDSCH transmission occasion. If the value K is not configured via the higher layer parameter StartingSymbolOffsetK, K  = 0 shall be assumed by the UE. The UE is not expected to receive more than two PDSCH transmission layers for each PDSCH transmission occasion. For two PDSCH transmission occasions, the redundancy version to be applied is derived according to Table 5.1.2.1-2, where n=0, 1 applied respectively to the first and second TCI state. The UE expects the PDSCH mapping type indicated by DCI field 'Time domain resource assignment' to be mapping type B, and the indicated PDSCH mapping type is applied to both PDSCH transmission occasions.

-	Otherwise, the UE is expected to receive a single PDSCH transmission occasion, and the resource allocation in the time domain follows Clause 5.1.2.1.

When a UE configured by the higher layer parameter PDSCH-config that indicates at least one entry contains repetitionNumber in PDSCH-TimeDomainResourceAllocation,

-	If two TCI states are indicated by the DCI field 'Transmission Configuration Indication' for a UE not configured with dl-OrJointTCI-StateList, or both indicated TCI states applicable to PDSCH for a UE configured with dl-OrJointTCI-StateList, together with the DCI field 'Time domain resource assignment' indicating an entry which contains repetitionNumber in PDSCH-TimeDomainResourceAllocation and DM-RS port(s) within one CDM group in the DCI field 'Antenna Port(s)', the same SLIV is applied for all PDSCH transmission occasions across the repetitionNumber consecutive slots, the first TCI state is applied to the first PDSCH transmission occasion and resource allocation in time domain for the first PDSCH transmission occasion follows Clause 5.1.2.1.

When the value indicated by repetitionNumber in PDSCH-TimeDomainResourceAllocation equals to two, the second TCI state is applied to the second PDSCH transmission occasion. When the value indicated by repetitionNumber in PDSCH-TimeDomainResourceAllocation is larger than two, the UE may be further configured to enable cyclicMapping or sequenticalMapping in tciMapping.

-	When cyclicMapping is enabled, the first and second TCI states are applied to the first and second PDSCH transmission occasions, respectively, and the same TCI mapping pattern continues to the remaining PDSCH transmission occasions.

-	When sequenticalMapping is enabled, first TCI state is applied to the first and second PDSCH transmission occasions, and the second TCI state is applied to the third and fourth PDSCH transmission occasions, and the same TCI mapping pattern continues to the remaining PDSCH transmission occasions.

The UE may expect that each PDSCH transmission occasion is limited to two transmission layers. For all PDSCH transmission occasions associated with the first TCI state, the redundancy version to be applied is derived according to Table 5.1.2.1-2, where n is counted only considering PDSCH transmission occasions associated with the first TCI state. The redundancy version for PDSCH transmission occasions associated with the second TCI state is derived according to Table 5.1.2.1-3, where additional shifting operation for each redundancy version rvs is configured by higher layer parameter sequenceOffsetforRV and n is counted only considering PDSCH transmission occasions associated with the second TCI state.

Table 5.1.2.1-3: Applied redundancy version for the second TCI state when sequenceOffsetforRV is present

| rvid indicated by the DCI scheduling the PDSCH | rvid to be applied to nth transmission occasion with second TCI state |  |  |  |
| --- | --- | --- | --- | --- |
|  | n mod 4 = 0 | n mod 4 = 1 | n mod 4 = 2 | n mod 4 = 3 |
| 0 | (0+ rvs) mod 4 | (2+ rvs) mod 4 | (3+ rvs) mod 4 | (1+ rvs) mod 4 |
| 2 | (2+ rvs) mod 4 | (3+ rvs) mod 4 | (1+ rvs) mod 4 | (0+ rvs) mod 4 |
| 3 | (3+ rvs) mod 4 | (1+ rvs) mod 4 | (0+ rvs) mod 4 | (2+ rvs) mod 4 |
| 1 | (1+ rvs) mod 4 | (0+ rvs) mod 4 | (2+ rvs) mod 4 | (3+ rvs) mod 4 |

-	If one TCI state is indicated by the DCI field 'Transmission Configuration Indication' for a UE not configured with dl-OrJointTCI-StateList, or one indicated TCI states applicable to PDSCH for a UE configured with dl-OrJointTCI-StateList, together with the DCI field 'Time domain resource assignment' indicating an entry which contains repetitionNumber in PDSCH-TimeDomainResourceAllocation and DM-RS port(s) within one CDM group in the DCI field 'Antenna Port(s)', the same SLIV is applied for all PDSCH transmission occasions across the repetitionNumber consecutive slots, the first PDSCH transmission occasion follows Clause 5.1.2.1, the same TCI state is applied to all PDSCH transmission occasions. The UE may expect that each PDSCH transmission occasion is limited to two transmission layers. For all PDSCH transmission occasions, the redundancy version to be applied is derived according to Table 5.1.2.1-2, where n is counted considering PDSCH transmission occasions.

-	Otherwise, the UE is expected to receive a single PDSCH transmission occasion, and the resource allocation in the time domain follows Clause 5.1.2.1.

For pdsch-TimeDomainAllocationListForMultiPDSCH or pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 in pdsch-Config each PDSCH has a separate SLIV, mapping type and K0. The number of scheduled PDSCHs is signalled by the number of indicated SLIVs in the row of the pdsch-TimeDomainAllocationListForMultiPDSCH signalled in DCI format 1_1 or in the row of the pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 signalled in DCI format 1_3.

If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH in which one or more rows contain multiple SLIVs for PDSCH on a DL BWP of a serving cell, and the UE is indicated re-transmission of PDSCH corresponding to a DL SPS by DCI format 1_1, the UE does not expect that the number of indicated SLIVs in the row of the pdsch-TimeDomainAllocationListForMultiPDSCH by the DCI is more than one.

If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH or pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 in which one or more rows contain multiple SLIVs for PDSCH on a DL BWP of a serving cell, the UE does not expect to be scheduled with one or multiple PDSCH receptions by a single DCI format 1_1 or DCI format 1_3, respectively, where each PDSCH reception overlaps with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided.

##### 5.1.2.1.1	Determination of the resource allocation table to be used for PDSCH

Table 5.1.2.1.1-1, Table 5.1.2.1.1-1A and Table 5.1.2.1.1-1B define which PDSCH time domain resource allocation configuration to apply. Either a default PDSCH time domain allocation A, B or C according to tables 5.1.2.1.1-2, 5.1.2.1.1-3, 5.1.2.1.1-4 and 5.1.2.1.1-5 is applied, or the higher layer configured pdsch-TimeDomainAllocationList or pdsch-TimeDomainAllocationListForMultiPDSCH or pdsch-TimeDomainAllocationListDCI-1-2 or pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 is applied. For operation with shared spectrum channel access in frequency range 1, as described in [16, TS 37.213], UE reinterprets S and L in row 9 of Table 5.1.2.1.1-2 as S=6 and L=7.

Table 5.1.2.1.1-1: Applicable PDSCH time domain resource allocation for DCI formats 1_0, 1_1, 4_0, 4_1 and 4_2

| RNTI | PDCCH search space | SS/PBCH block and CORESET multiplexing pattern | PDSCH-ConfigCommon includes pdsch-TimeDomainAllocationList | PDSCH-Config includes pdsch-TimeDomainAllocationList | pdsch-ConfigMCCH / pdsch-ConfigMTCH includes pdsch-TimeDomainAllocationList Or pdsch-ConfigMulticast includes pdsch-TimeDomainAllocationList | PDSCH-Config includes pdsch-TimeDomainAllocationListForMultiPDSCH | PDSCH time domain resource allocation to apply |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SI-RNTI | Type0 common | 1 | - | - | - | - | Default A for normal CP |
|  |  | 2 | - | - | - | - | Default B |
|  |  | 3 | - | - | - | - | Default C |
| SI-RNTI | Type0A common | 1 | No | - | - | - | Default A |
|  |  | 2 | No | - | - | - | Default B |
|  |  | 3 | No | - | - | - | Default C |
|  |  | 1,2,3 | Yes | - | - | - | Pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
| RA-RNTI, MSGB-RNTI, TC-RNTI | Type1 common | 1,2,3 | No | - | - | - | Default A |
|  |  | 1,2,3 | Yes | - | - | - | Pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
| P-RNTI | Type2 common | 1 | No | - | - | - | Default A |
|  |  | 2 | No | - | - | - | Default B |
|  |  | 3 | No | - | - | - | Default C |
|  |  | 1,2,3 | Yes | - | - | - | Pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
| MCCH-RNTI | Type 0/0B/3 common for broadcast | 1 | No | - | No | - | Default A |
|  |  | 2 | No | - | No | - | Default B |
|  |  | 3 | No | - | No | - | Default C |
|  |  | 1,2,3 | Yes | - | No | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
|  |  | 1,2,3 | No/Yes | - | Yes | - | pdsch-TimeDomainAllocationList provided in pdsch-ConfigMCCH |
| Multicast-MCCH-RNTI | Type 0/0B common for multicast | 1 | No | - | No | - | Default A |
|  |  | 2 | No | - | No | - | Default B |
|  |  | 3 | No | - | No | - | Default C |
|  |  | 1,2,3 | Yes | - | No | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
|  |  | 1,2,3 | No/Yes | - | Yes | - | pdsch-TimeDomainAllocationList provided in pdsch-ConfigMCCH for multicast for RRC_INACTIVE |
| G-RNTI for broadcast | Type 0/0B/3 common for broadcast | 1 | No | - | No | - | Default A |
|  |  | 2 | No | - | No | - | Default B |
|  |  | 3 | No | - | No | - | Default C |
|  |  | 1,2,3 | Yes | - | No | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
|  |  | 1,2,3 | No/Yes | - | Yes | - | pdsch-TimeDomainAllocationList provided in pdsch-ConfigMTCH, if configured, otherwise pdsch-TimeDomainAllocationList provided in pdsch-ConfigMCCH |
| G-RNTI for multicast in RRC_INACTIVE | Type 0/0B common for multicast | 1 | No | - | No | - | Default A |
|  |  | 2 | No | - | No | - | Default B |
|  |  | 3 | No | - | No | - | Default C |
|  |  | 1,2,3 | Yes | - | No | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
|  |  | 1,2,3 | No/Yes | - | Yes | - | pdsch-TimeDomainAllocationList provided in pdsch-ConfigMTCH-r18, if configured, otherwise pdsch-TimeDomainAllocationList provided in pdsch-ConfigMCCH for multicast for RRC_INACTIVE (Note 1a) |
| C-RNTI, MCS-C-RNTI, CS-RNTI | Any common search space associated with CORESET 0 | 1, 2, 3 | No | - | - | - | Default A |
|  |  | 1, 2, 3 | Yes | - | - | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
| C-RNTI, MCS-C-RNTI, CS-RNTI | Any common search space not associated with CORESET 0 UE specific search space | 1,2,3 | No | No | - | - | Default A |
|  |  | 1,2,3 | Yes | No | - | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
|  |  | 1,2,3 | No/Yes | Yes | - | - | pdsch-TimeDomainAllocationList provided in PDSCH-Config |
|  |  | 1,2,3 | No/Yes | - | - | Yes | pdsch-TimeDomainAllocationListForMultiPDSCH provided in PDSCH-Config (Note 2) |
| G-RNTI for multicast, G-CS-RNTI | Type 3 common search space for multicast | 1,2,3 | No | - | No | - | Default A |
|  |  | 1,2,3 | Yes | - | No | - | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon (Note 1) |
|  |  | 1,2,3 | No/Yes | - | Yes | - | pdsch-TimeDomainAllocationList provided in pdsch-ConfigMulticast (Note 1) |
| Note 1: For a UE that supports multicast, the same TDRA table applies to all G-RNTIs and G-CS-RNTIs (configured for multicast in RRC_CONNECTED) if configured on a given serving cell. Note 1a: For a UE that supports multicast in RRC_INACTIVE, the same TDRA table applies to all G-RNTIs (configured for multicast in RRC_INACTIVE) if configured on a given serving cell. Note 2: If pdsch-TimeDomainAllocationListForMultiPDSCH is provided, it is applicable to DCI format 1_1 only. |  |  |  |  |  |  |  |

Table 5.1.2.1.1-1A: Applicable PDSCH time domain resource allocation for DCI format 1_2

| PDSCH-ConfigCommon includes pdsch-TimeDomainAllocationList | PDSCH-Config includes pdsch-TimeDomainAllocationList | PDSCH-Config includes pdsch- TimeDomainAllocationListDCI-1-2 | PDSCH time domain resource allocation to apply |
| --- | --- | --- | --- |
| No | No | No | Default A |
| Yes | No | No | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
| No/Yes | Yes | No | pdsch-TimeDomainAllocationList provided in PDSCH-Config |
| No/Yes | No/Yes | Yes | pdsch-TimeDomainAllocationListDCI-1-2 provided in PDSCH-Config |

Table 5.1.2.1.1-1B: Applicable PDSCH time domain resource allocation for DCI format 1_3

| PDSCH-ConfigCommon includes pdsch-TimeDomainAllocationList | PDSCH-Config includes pdsch-TimeDomainAllocationList | PDSCH-Config includes pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 | PDSCH time domain resource allocation to apply |
| --- | --- | --- | --- |
| No | No | No | Default A |
| Yes | No | No | pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon |
| No/Yes | Yes | No | pdsch-TimeDomainAllocationList provided in PDSCH-Config |
| No/Yes | No/Yes | Yes | pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 provided in PDSCH-Config |

Table 5.1.2.1.1-2: Default PDSCH time domain resource allocation A for normal CP

| Row index | dmrs-TypeA-Position | PDSCH mapping type | K0 | S | L |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | Type A | 0 | 2 | 12 |
|  | 3 | Type A | 0 | 3 | 11 |
| 2 | 2 | Type A | 0 | 2 | 10 |
|  | 3 | Type A | 0 | 3 | 9 |
| 3 | 2 | Type A | 0 | 2 | 9 |
|  | 3 | Type A | 0 | 3 | 8 |
| 4 | 2 | Type A | 0 | 2 | 7 |
|  | 3 | Type A | 0 | 3 | 6 |
| 5 | 2 | Type A | 0 | 2 | 5 |
|  | 3 | Type A | 0 | 3 | 4 |
| 6 | 2 | Type B | 0 | 9 | 4 |
|  | 3 | Type B | 0 | 10 | 4 |
| 7 | 2 | Type B | 0 | 4 | 4 |
|  | 3 | Type B | 0 | 6 | 4 |
| 8 | 2,3 | Type B | 0 | 5 | 7 |
| 9 | 2,3 | Type B | 0 | 5 | 2 |
| 10 | 2,3 | Type B | 0 | 9 | 2 |
| 11 | 2,3 | Type B | 0 | 12 | 2 |
| 12 | 2,3 | Type A | 0 | 1 | 13 |
| 13 | 2,3 | Type A | 0 | 1 | 6 |
| 14 | 2,3 | Type A | 0 | 2 | 4 |
| 15 | 2,3 | Type B | 0 | 4 | 7 |
| 16 | 2,3 | Type B | 0 | 8 | 4 |

<omitted text>

#### 5.1.3.2	Transport block size determination

In case the higher layer parameter maxNrofCodeWordsScheduledByDCI in PDSCH-config indicates that two codeword transmission is enabled, then one of the two transport blocks is disabled by DCI format 1_1 or 1_3 if IMCS = 26 and if rvid = 1 for the corresponding transport block. In case the higher layer parameter maxNrofCodeWordsScheduledByDCI in pdsch-ConfigMulticast indicates that two codeword transmission is enabled, then one of the two transport blocks is disabled by DCI format 4_2 if IMCS = 26 and if rvid = 1 for the corresponding transport block. When the UE is configured with higher layer parameter pdsch-TimeDomainAllocationListForMultiPDSCH or pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3, either the first or the second transport block of all scheduled PDSCHs is disabled by the DCI format 1_1 or DCI format 1_3 if IMCS = 26 and if rvid = 2 for the corresponding transport block of all scheduled PDSCHs. If both transport blocks are enabled, transport block 1 and 2 are mapped to codeword 0 and 1 respectively. If only one transport block is enabled, then the enabled transport block is always mapped to the first codeword.

<omitted text>

#### 5.1.4.2	PDSCH resource mapping with RE level granularity

The procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 1_2, by applying the parameters of aperiodicZP-CSI-RS-ResourceSetsToAddModListDCI-1-2 instead of aperiodic-ZP-CSI-RS-ResourceSetsToAddModList. The procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 1_3.

The procedures for PDSCH scheduled by PDCCH with DCI format 1_0 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 4_1 and the procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 4_2, by applying the parameters of aperiodicZP-CSI-RS-ResourceSetsToAddModList in pdsch-ConfigMulticast instead of aperiodic-ZP-CSI-RS-ResourceSetsToAddModList in PDSCH-Config.

A UE may be configured with any of the following higher layer parameters:

-	REs indicated by the 'RateMatchPatternLTE-CRS' in lte-CRS-ToMatchAround in ServingCellConfig or ServingCellConfigCommon configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH.

-	REs indicated by 'RateMatchPatternLTE-CRS' in lte-CRS-PatternList1-r16 or lte-CRS-PatternList3-r18 in ServingCellConfig configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH.

-	For the UE for broadcast reception or multicast reception in RRC_INACTIVE_state, REs indicated by 'RateMatchPatternLTE-CRS' in pdsch-ConfigMCCH or pdsch-ConfigMTCH configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for broadcast PDSCH or multicast PDSCH reception in RRC_INACTIVE_state. The total number of RateMatchPatternLTE-CRS for broadcast reception or multicast reception in RRC_INACTIVE_state that a UE can be configured with is the same as for unicast in Rel-15.

-	Each RateMatchPatternLTE-CRS configuration contains v-Shift consisting of LTE-CRS-vshift(s), nrofCRS-Ports consisting of LTE-CRS antenna ports 1, 2 or 4 ports, carrierFreqDL representing the offset in units of 15 kHz subcarriers from (reference) point A to the LTE carrier centre subcarrier location, carrierBandwidthDL representing the LTE carrier bandwidth, and may also configure mbsfn-SubframeConfigList representing MBSFN subframe configuration. A UE determines the CRS position within the slot according to Clause 6.10.1.2 in [15, TS 36.211], where slot corresponds to LTE subframe.

-	If the UE is configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet and is also configured by the higher layer parameter lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16 in ServingCellConfig, the following REs are declared as not available for PDSCH:

-	if the UE is configured with crs-RateMatch-PerCoresetPoolIndex, REs indicated by the CRS pattern(s) in lte-CRS-PatternList1-r16 if the PDSCH is associated with coresetPoolIndex set to '0', or the CRS pattern(s) in lte-CRS-PatternList2-r16 if PDSCH is associated with coresetPoolIndex set to '1';

-	otherwise, REs indicated by lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16, in ServingCellConfig.

-	If the UE is not configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet, and if the UE is configured by higher layer parameter lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 in ServingCellConfig, REs indicated by lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are declared as not available for PDSCH.

-	If the UE is configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet and is also configured by the higher layer parameter lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 in ServingCellConfig, the following REs are declared as not available for PDSCH:

-	if the UE is configured with crs-RateMatch-PerCoresetPoolIndex, REs indicated by the CRS pattern(s) in lte-CRS-PatternList3-r18 if the PDSCH is associated with coresetPoolIndex set to '0', or the CRS pattern(s) in lte-CRS-PatternList4-r18 if PDSCH is associated with coresetPoolIndex set to '1';

-	otherwise, REs indicated by lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18, in ServingCellConfig.

-	Within a BWP, the UE can be configured with one or more ZP CSI-RS resource set configuration(s) for aperiodic, semi-persistent and periodic time-domain behaviours (higher layer parameters aperiodic-ZP-CSI-RS-ResourceSetsToAddModList,  sp-ZP-CSI-RS-ResourceSetsToAddModList and p-ZP-CSI-RS-ResourceSet respectively comprised in PDSCH-Config), with each ZP CSI-RS resource set consisting of at most 16 ZP CSI-RS resources (higher layer parameter ZP-CSI-RS-Resource) in numerology of the BWP. The REs indicated by p-ZP-CSI-RS-ResourceSet are declared as not available for PDSCH. The REs indicated by sp-ZP-CSI-RS-ResourceSetsToAddModList and aperiodic-ZP-CSI-RS-ResourceSetsToAddModList are declared as not available for PDSCH when their triggering and activation are applied, respectively. The following parameters are configured via higher layer signaling for each ZP CSI-RS resource configuration:

-	zp-CSI-RS-ResourceId in ZP-CSI-RS-Resource determines ZP CSI-RS resource configuration identity.

-	nrofPorts in CSI-RS-ResourceMapping defines the number of CSI-RS ports, where the allowable values are given in Clause 7.4.1.5 of [4, TS 38.211].

-	cdm-Type in CSI-RS-ResourceMapping defines CDM values and pattern, where the allowable values are given in Clause 7.4.1.5 of [4, TS 38.211].

-	resourceMapping in ZP-CSI-RS-Resource defines the OFDM symbol and subcarrier occupancy of the ZP CSI-RS resource within a slot that are given in Clause 7.4.1.5 of [4, TS 38.211].

-	periodicityAndOffset in ZP-CSI-RS-Resource defines the ZP-CSI-RS periodicity and slot offset for periodic/semi-persistent ZP CSI-RS.

-	For the UE in RRC_CONNECTED mode for multicast reception, p-ZP-CSI-RS-ResourceSet can be configured in pdsch-ConfigMulticast for GC-PDSCH rate matching, subject to UE capability. The REs indicated by p-ZP-CSI-RS-ResourceSet are declared as not available for GC-PDSCH. The REs indicated by p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config for unicast do not apply for GC-PDSCH and the REs indicated by p-ZP-CSI-RS-ResourceSet configured in pdsch-ConfigMulticast for multicast do not apply for unicast PDSCH. The total number of periodic ZP-CSI-RS-Resources that a UE can be configured with is the same as for unicast in Rel-16. If p-ZP-CSI-RS-ResourceSet is configured in both PDSCH-Config and pdsch-ConfigMulticast, it is subject to UE capability whether the p-ZP-CSI-RS-ResourceSet configured in pdsch-ConfigMulticast can be different from the p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config.

-	For the UE in RRC_CONNECTED mode for multicast reception, sp-ZP-CSI-RS-ResourceSet can be configured in pdsch-ConfigMulticast for GC-PDSCH rate matching, subject to UE capability. The REs indicated by sp-ZP-CSI-RS-ResourceSet are declared as not available for GC-PDSCH when their triggering and activation delivered by unicast PDSCH are applied. The REs indicated by sp-ZP-CSI-RS-ResourceSet configured in PDSCH-Config for unicast do not apply for GC-PDSCH and the REs indicated by sp-ZP-CSI-RS-ResourceSet configured in pdsch-ConfigMulticast for multicast do not apply for unicast PDSCH. The total number of semi-persistent ZP-CSI-RS-Resources that a UE can be configured with is the same as for unicast.

The UE may be configured with a DCI field for triggering the aperiodic ZP CSI-RS. A list of ZP-CSI-RS-ResourceSet(s), provided by higher layer parameter aperiodic-ZP-CSI-RS-ResourceSetsToAddModList in PDSCH-Config, is configured for aperiodic triggering. The maximum number of aperiodic ZP-CSI-RS-ResourceSet(s) configured per BWP is 3. The bit-length of DCI field ZP CSI-RS trigger depends on the number of aperiodic ZP-CSI-RS-ResourceSet(s)configured (up to 2 bits). Each non-zero codepoint of 'ZP CSI-RS' trigger in DCI format 1_1 triggers one aperiodic 'ZP-CSI-RS-ResourceSet' in the list aperiodic-ZP-CSI-RS-ResourceSetsToAddModList by indicating the aperiodic ZP CSI-RS resource set ID. The DCI codepoint '01' triggers the resource set with 'ZP-CSI-RS-ResourceSetId' set to '1', the DCI codepoint '10' triggers the resource set with 'ZP-CSI-RS-ResourceSetId' set to '2', and the DCI codepoint '11' triggers the resource set with 'ZP-CSI-RS-ResourceSetId' set to '3'. Codepoint '00' is reserved for not triggering aperiodic ZP CSI-RS. When receiving PDSCH scheduled by DCI format 1_0 or PDSCHs with SPS activated by DCI format 1_0, the REs corresponding to configured resources in aperiodic-ZP-CSI-RS-ResourceSetsToAddModList or in aperiodicZP-CSI-RS-ResourceSetsToAddModListDCI-1-2 are available for PDSCH.

When the UE is configured with multi-slot and single-slot PDSCH scheduling or pdsch-TimeDomainAllocationListForMultiPDSCH or pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3, the triggered aperiodic ZP CSI-RS is applied to all the slot(s) of the PDSCH(s) scheduled or the PDSCHs with SPS activated by the PDCCH containing the trigger.

For a UE configured with a list of semi-persistent ZP-CSI-RS-ResourceSet(s) provided by higher layer parameter sp-ZP-CSI-RS-ResourceSetsToAddModList:

-	when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, as described in clause 6.1.3.19 of [10, TS 38.321], for ZP CSI-RS resource(s), the corresponding action in [10, TS 38.321] and the UE assumption on the PDSCH RE mapping corresponding to the activated ZP CSI-RS resource(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+2μ2μKmac∙kmac where  is the SCS configuration for the PUCCH and μKmac is the subcarrier spacing configuration for kmac with a value of 0 for frequency range 1 and for FR2-NTN, and kmac is provided by K-Mac or kmac=0 if K-Mac is not provided.

-	when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the deactivation command, as described in clause 6.1.3.19 of [10, TS 38.321], for activated ZP CSI-RS resource(s), the corresponding action in [10, TS 38.321] and the UE assumption on cessation of the PDSCH RE mapping corresponding to the de-activated ZP CSI-RS resource(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+2μ2μKmac∙kmac where  is the SCS configuration for the PUCCH and μKmac is the subcarrier spacing configuration for kmac with a value of 0 for frequency range 1 and for FR2-NTN, and kmac is provided by K-Mac or kmac=0 if K-Mac is not provided.

<omitted text>

### 5.1.5	Antenna ports quasi co-location

The UE can be configured with a list of up to M TCI-State configurations within the higher layer parameter PDSCH-Config to decode PDSCH according to a detected PDCCH with DCI intended for the UE and the given serving cell, where M depends on the UE capability maxNumberConfiguredTCIstatesPerCC. Each TCI-State contains parameters for configuring a quasi co-location relationship between one or two downlink reference signals and the DM-RS ports of the PDSCH, the DM-RS port of PDCCH or the CSI-RS port(s) of a CSI-RS resource. The quasi co-location relationship is configured by the higher layer parameter qcl-Type1 for the first DL RS, and qcl-Type2 for the second DL RS (if configured). For the case of two DL RSs, the QCL types shall not be the same, regardless of whether the references are to the same DL RS or different DL RSs. The quasi co-location types corresponding to each DL RS are given by the higher layer parameter qcl-Type in QCL-Info and may take one of the following values:

-	'typeA': {Doppler shift, Doppler spread, average delay, delay spread}

-	'typeB': {Doppler shift, Doppler spread}

-	'typeC': {Doppler shift, average delay}

-	'typeD': {Spatial Rx parameter}

The UE can be configured with a list of up to 128 TCI-State configurations, within the higher layer parameter dl-OrJointTCI-StateList in PDSCH-Config for providing a reference signal for the quasi co-location for DM-RS of PDSCH and DM-RS of PDCCH in a BWP/CC, for CSI-RS, and to provide a reference signal with qcl-Type set to 'typeD', if applicable, for determining UL TX spatial filter for dynamic-grant and configured-grant based PUSCH and PUCCH resource in a BWP/CC, and SRS.

If the TCI-State or TCI-UL-State configurations are absent in a BWP of the CC, the UE can apply the TCI-State or TCI-UL-State configurations from a reference BWP of a reference CC configured by unifiedTCI-StateRef. The UE is not expected to be configured with tci-StatesToAddModList, SpatialRelationInfo or PUCCH-SpatialRelationInfo, except SpatialRelationInfoPos in a CC in a band, if the UE is configured with dl-OrJointTCI-StateList or ul-TCI-StateList in any CC in the same band. The UE can assume that when the UE is configured with tci-StatesToAddModList in any CC in the CC list configured by simultaneousTCI-UpdateList1-r16, simultaneousTCI-UpdateList2-r16, simultaneousSpatial-UpdatedList1-r16, or simultaneousSpatial-UpdatedList2-r16, the UE is not configured with dl-OrJointTCI-StateList or ul-TCI-StateList in any CC within the same band in the CC list.

The UE receives an activation command, as described in clause 6.1.3.14 of [10, TS 38.321], or 6.1.3.47 of [10, TS 38.321], used to map up to 8 TCI states and/or pairs of TCI states, with one TCI state for DL channels/signals and/or one TCI state for UL channels/signals to the codepoints of the DCI field 'Transmission Configuration Indication' for one or for a set of CCs/DL BWPs. When a set of TCI state IDs are activated for a set of CCs/DL BWPs and if applicable, for a set of CCs/UL BWPs, where the applicable list of CCs is determined by the indicated CC in the activation command, the same set of TCI state IDs are applied for all DL and/or UL BWPs in the indicated CCs. If the activation command maps TCI-State(s) and/or TCI-UL-State(s) to only one TCI codepoint, the UE shall apply the indicated TCI-State(s) and/or TCI-UL-State(s) to one or to a set of CCs /DL BWPs, and if applicable, to one or to a set of CCs /UL BWPs once the indicated mapping for the one single TCI codepoint is applied as described in [11, TS 38.133].

When the bwp-id or cell for QCL-TypeA/D source RS in a QCL-Info of the TCI state is not configured, the UE assumes that QCL-TypeA/D source RS is configured in the CC/DL BWP where TCI state applies.

When tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, a UE configured with dl-OrJointTCI-StateList with activated TCI-State or ul-TCI-StateList with activated TCI-UL-State receives DCI format 1_1/1_2/1_3 providing indicated TCI-State(s) and/or TCI-UL-State(s) for a CC or all CCs in the same CC list configured by simultaneousU-TCI-UpdateList1-r17, simultaneousU-TCI-UpdateList2-r17, simultaneousU-TCI-UpdateList3-r17, simultaneousU-TCI-UpdateList4-r17. The DCI format 1_3 provides indicated TCI state(s) and/or TCI-UL-State(s) for the CC(s) in a scheduledCellListDCI-1-3 if the UE is scheduled by the DCI format 1_3 to receive PDSCH at least on one serving cell in the scheduledCellListDCI-1-3. The DCI format 1_1/1_2 can be with or without, if applicable, DL assignment. If the DCI format 1_1/1_2 is without DL assignment, the UE can assume the following:

-	CS-RNTI is used to scramble the CRC for the DCI

-	The values of the following DCI fields are set as follows:

-	RV = all '1's

-	MCS = all '1's

-	NDI = 0

-	Set to all '0's for FDRA Type 0, or all '1's for FDRA Type 1, or all '0's for dynamicSwitch (same as in Table 10.2-4 of [6, TS 38.213]).

After a UE receives an initial higher layer configuration of dl-OrJointTCI-StateList where more than one TCI-State can be used as an indicated TCI state and before application of an indicated TCI state from the configured TCI states:

-	The UE assumes that DM-RS of PDSCH and DM-RS of PDCCH that are not received during the RACH procedure, and the CSI-RS applying the indicated TCI state are quasi co-located with the reference signal(s) in the CandidateTCI-State indicated in the LTM Cell Switch Command MAC CE [10, 38.321] if applicable, otherwise.

-	The UE assumes that DM-RS of PDSCH and DM-RS of PDCCH and the CSI-RS applying the indicated TCI state are quasi co-located with the SS/PBCH block the UE identified during the initial access procedure

After a UE receives an initial higher layer configuration of dl-OrJointTCI-StateList where more than one TCI-State can be used as an indicated TCI state or an initial higher layer configuration of ul-TCI-StateList where more than one TCI-UL-State can be used as an indicated TCI state and before application of an indicated TCI state from the configured TCI states:

-	The UE determines the UL TX spatial filter, if applicable, for dynamic-grant based PUSCH that is not transmitted during the RACH procedure and configured-grant based PUSCH and PUCCH that are not transmitted during the RACH procedure, and for SRS applying the indicated TCI state, from the CandidateTCI-State or CandidateTCI-UL-State indicated in the LTM Cell Switch Command MAC CE [10, 38.321] if applicable, otherwise.

-	The UE assumes that the UL TX spatial filter, if applicable, for dynamic-grant and configured-grant based PUSCH and PUCCH, and for SRS applying the indicated TCI state, is the same as that for a PUSCH transmission scheduled by a RAR UL grant or a MsgA PUSCH transmission during the initial access procedure

After a UE receives a higher layer configuration of dl-OrJointTCI-StateList where more than one TCI-State can be used as an indicated TCI state as part of a Reconfiguration with sync procedure as described in [12, TS 38.331] and before applying an indicated TCI state from the configured TCI states:

-	The UE assumes that DM-RS of PDSCH and DM-RS of PDCCH, and the CSI-RS applying the indicated TCI state are quasi co-located with the SS/PBCH block or the CSI-RS resource the UE identified during the random access procedure initiated by the Reconfiguration with sync procedure as described in [12, TS 38.331].

After a UE receives a higher layer configuration of dl-OrJointTCI-StateList where more than one TCI-State can be used as an indicated TCI state or a higher layer configuration of ul-TCI-StateList where more than one TCI-UL-State can be used as an indicated TCI state as part of a Reconfiguration with sync procedure as described in [12, TS 38.331] and before applying an indicated TCI state from the configured TCI states:

-	The UE assumes that the UL TX spatial filter, if applicable, for dynamic-grant and configured-grant based PUSCH and PUCCH, and for SRS applying the indicated TCI state, is the same as that for a PUSCH transmission scheduled by a RAR UL grant or a MsgA PUSCH transmission during random access procedure initiated by the Reconfiguration with sync procedure as described in [12, TS 38.331].

If a UE receives a higher layer configuration of dl-OrJointTCI-StateList where only one TCI-State can be used as an indicated TCI state, the UE obtains the QCL assumptions from that TCI state for DM-RS of PDSCH and DM-RS of PDCCH, and the CSI -RS applying the indicated TCI state.

If a UE receives a higher layer configuration of dl-OrJointTCI-StateList where only one TCI-State can be used as an indicated TCI state or a higher layer configuration of ul-TCI-StateList where only one TCI-UL-State can be used as an indicated TCI state, the UE determines an UL TX spatial filter, if applicable, from that TCI state for dynamic-grant and configured-grant based PUSCH and PUCCH, and SRS applying the indicated TCI state.

When a UE configured with dl-OrJointTCI-StateList would transmit a PUCCH with positive HARQ-ACK or a PUSCH with positive HARQ-ACK corresponding to the DCI carrying the TCI State indication and without DL assignment, or corresponding to one or more PDSCH(s) scheduled by the DCI carrying the TCI State indication, and if the indicated TCI State(s) is/are different from the previously indicated one(s), the indicated TCI-State(s) and/or TCI-UL-State(s) should be applied starting from the first slot that is at least  beamAppTime symbols after the last symbol of the PUCCH or the PUSCH, and if the UE receives more than one indicated TCI state for a CC/BWP to be applied starting from the first slot that is at least  beamAppTime symbols after the last symbol of the PUCCH or the PUSCH, the indicated TCI state carried in the latest DCI, for the corresponding coresetPoolIndex value when applicable, in time corresponding to positive HARQ-ACK value is applied. The first slot and the  beamAppTime symbols are both determined on the active BWP with the smallest SCS among the BWP(s) from the CCs applying the indicated TCI-State(s) or TCI-UL-State(s) that are active at the end of the PUCCH or the PUSCH carrying the positive HARQ-ACK. The indicated TCI-State(s) or TCI-UL-State(s) is/are based on the activated TCI states in each slot where the UE applies the indicated TCI-State(s) or TCI-UL-State(s) to DL channel(s)/signal(s) or UL channel(s)/signal(s).

When a UE is configured with dl-OrJointTCI-StateList and is having one indicated TCI-state, and if the UE is configured with unifiedTCI-StateType is set as ‘separate’, and if the UE receives a TCI codepoint mapped with either of {TCI-State, TCI-UL-State}, the UE shall update the one indicated {TCI-State, TCI-UL-State} and maintain the other {TCI-State, TCI-UL-State} that is not updated by the received TCI codepoint.

When a UE is configured with dl-OrJointTCI-StateList and is having two indicated TCI-states, if the UE receives a TCI codepoint mapped with a sub-set of first and second TCI-State(s) and/or a sub-set of first and second TCI-UL-State(s), the UE shall update the first/second TCI-State(s) and/or first/second TCI-UL-State(s) mapped to the TCI codepoint, when applicable, and keep the previously indicated first/second TCI-State(s) and/or first/second TCI-UL-State(s) that is/are not updated by the TCI codepoint.

If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH in which one or more rows contain multiple SLIVs for PDSCH on a DL BWP of a serving cell, and the UE is receiving a DCI carrying the TCI-State indication and without DL assignment, the UE does not expect that the number of indicated SLIVs in the row of the pdsch-TimeDomainAllocationListForMultiPDSCH by the DCI is more than one.

If the UE is configured with PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet, the UE receives an activation command associated with each coresetPoolIndex, as described in clause 6.1.3.14 of [10, TS 38.321] or 6.1.3.47 of [10, TS 38.321], used to map up to 8 TCI states and/or pairs of TCI states, with one TCI state for DL channels/signals and/or one TCI state for UL channels/signals to the codepoints of the DCI field 'Transmission Configuration Indication' in one CC/DL BWP. When a set of TCI state IDs are activated for a coresetPoolIndex, the activated TCI states corresponding to one coresetPoolIndex is associated with the serving cell physical cell ID and activated TCI states corresponding to another coresetPoolIndex can be associated with another physical cell ID, if the UE is further configured with SSB-MTC-AddtionalPCI.

When a UE supports two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' the UE may receive an activation command, as described in clause 6.1.3.24 of [10, TS 38.321], the activation command is used to map up to 8 combinations of one or two TCI states to the codepoints of the DCI field 'Transmission Configuration Indication'. The UE is not expected to receive more than 8 TCI states in the activation command.

If the UE is provided a set of serving cells by mc-DCI-SetOfCellsToAddModList-r18, the UE does not expect to receive an activation command mapping two TCI-States and/or two TCI-UL-States to only one TCI codepoint, or to be provided PDCCH-Config that is associated with two different values of coresetPoolIndex for scheduling on a serving cell from the set of serving cells.

When a UE configured with dl-OrJointTCI-StateList supports tci-JointTCI-UpdateMultiActiveTCI-PerCC-r18, the UE may receive an activation command, as described in clause 6.1.3.70 of [10, TS 38.321], the activation command is used to map up to 8 sets of TCI states to the codepoints of the DCI field 'Transmission Configuration Indication' for one or for a set of CCs/DL BWPs, and if applicable, for one or for a set of CCs/UL BWPs, where each set is comprised of up to two TCI state(s) for DL and UL signals/channels.

When a UE configured with dl-OrJointTCI-StateList supports tci-SeparateTCI-UpdateMultiActiveTCI-PerCC-r18, the UE may receive an activation command, as described in clause 6.1.3.71 of [10, TS 38.321], the activation command is used to map up to 8 sets of TCI states to the codepoints of the DCI field 'Transmission Configuration Indication' for one or for a set of CCs/DL BWPs, and if applicable, for one or for a set of CCs/UL BWPs, where each set is comprised of up to two TCI state(s) for DL channels/signals and up to two TCI state(s) for UL channels/signals.

When the DCI field 'Transmission Configuration Indication' is present in DCI format 1_2 and when the number of codepoints S in the DCI field 'Transmission Configuration Indication' of DCI format 1_2 is smaller than the number of TCI codepoints that are activated by the activation command, as described in clauses 6.1.3.14, 6.1.3.24, 6.1.3.47, 6.1.3.70 and 6.1.3.71 of [10, TS 38.321], only the first S activated codepoints are applied for DCI format 1_2.

When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, the indicated mapping between TCI states and codepoints of the DCI field 'Transmission Configuration Indication' should be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+2μ2μKmac∙kmac where  is the SCS configuration for the PUCCH and μKmac is the subcarrier spacing configuration for kmac with a value of 0 for frequency range 1 and for FR2-NTN, and kmac is provided by K-Mac or kmac=0 if K-Mac is not provided. If tci-PresentInDCI is set to 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET scheduling the PDSCH, and the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than timeDurationForQCL if applicable, after a UE receives an initial higher layer configuration of TCI states and before reception of the activation command,

-	the UE assumes that DM-RS of ports of PDSCH of a serving cell are quasi co-located with the reference signal(s) in the CandidateTCI-State indicated in the LTM Cell Switch Command MAC CE [10, 38.321], except during RACH procedure for RACH-based LTM, if applicable, otherwise

-	the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the SS/PBCH block determined in the initial access procedure with respect to qcl-Type set to 'typeA', and when applicable, also with respect to qcl-Type set to 'typeD'.

If a UE is configured with the higher layer parameter tci-PresentInDCI that is set as 'enabled' for the CORESET scheduling a PDSCH, the UE assumes that the TCI field is present in the DCI format 1_1 or format 1_3 of the PDCCH transmitted on the CORESET. If a UE is configured with the higher layer parameter tci-PresentDCI-1-2 for the CORESET scheduling the PDSCH, the UE assumes that the TCI field with a DCI field size indicated by tci-PresentDCI-1-2 is present in the DCI format 1_2 of the PDCCH transmitted on the CORESET. If a UE is configured with the higher layer parameter tci-PresentInDCI that is set as 'enabled' for the CORESET scheduling the multicast PDSCH, the UE assumes that the TCI field is present in the DCI format 4_2 of the PDCCH transmitted on the CORESET. If the PDSCH is scheduled by a DCI format not having the TCI field present, and the time offset between the reception of the DL DCI and the corresponding PDSCH of a serving cell is equal to or greater than a threshold timeDurationForQCL if applicable, where the threshold is based on reported UE capability [13, TS 38.306], for determining PDSCH antenna port quasi co-location, the UE assumes that the TCI state or the QCL assumption for the PDSCH is identical to the TCI state or QCL assumption whichever is applied for the CORESET used for the PDCCH transmission within the active BWP of the serving cell.

When a UE is configured with both sfnSchemePDCCH and sfnSchemePDSCH scheduled by DCI format 1_0 or by DCI format 1_1/1_2, if the time offset between the reception of the DL DCI and the corresponding PDSCH of a serving cell is equal to or greater than a threshold timeDurationForQCL if applicable:

-	if the UE supports sfn-DefaultDL-BeamSetup-r17 for DCI scheduling without TCI field, the UE assumes that the TCI state(s) or the QCL assumption(s) for the PDSCH is identical to the TCI state(s) or QCL assumption(s) whichever is applied for the CORESET used for the reception of the DL DCI within the active BWP of the serving cell regardless of the number of active TCI states of the CORESET. If the UE does not support sfn-SchemeA-DynamicSwitching-r17 or sfn-SchemeB-DynamicSwitching-r17, the UE should be activated with the CORESET with two TCI states.

-	else if the UE does not support sfn-DefaultDL-BeamSetup-r17 for DCI scheduling without TCI field, the UE shall expect TCI field present when scheduled by DCI format 1_1/1_2.

When a UE is configured with sfnSchemePDSCH and sfnSchemePDCCH is not configured, when scheduled by DCI format 1_1/1_2, if the time offset between the reception of the DL DCI and the corresponding PDSCH of a serving cell is equal to or greater than a threshold timeDurationForQCL if applicable, the UE shall expect TCI field present.

For PDSCH scheduled by DCI format 1_0 or by DCI format 1_1/1_2 without TCI field, when a UE is configured with sfnSchemePDCCH set to 'sfnSchemeA' and sfnSchemePDSCH is not configured, and there is no TCI codepoint with two TCI states in the activation command, and if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal or larger than the threshold timeDurationForQCL if applicable and the CORESET which schedules the PDSCH is indicated with two TCI states, the UE assumes that the TCI state or the QCL assumption for the PDSCH is identical to the first TCI state or QCL assumption which is applied for the CORESET used for the PDCCH transmission within the active BWP of the serving cell.

If a UE is not provided dl-OrJointTCI-StateList-r17, and if a PDSCH is scheduled by a DCI format having the TCI field present, the TCI field in DCI in the scheduling component carrier points to the activated TCI states in the scheduled component carrier or DL BWP, the UE shall use the TCI-State according to the value of the 'Transmission Configuration Indication' field in the detected PDCCH with DCI for determining PDSCH antenna port quasi co-location. The UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) in the TCI state with respect to the QCL type parameter(s) given by the indicated TCI state if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, where the threshold is based on reported UE capability [13, TS 38.306]. For a single slot PDSCH, the indicated TCI state(s) should be based on the activated TCI states in the slot with the scheduled PDSCH. For a multi-slot PDSCH or the UE is configured with higher layer parameter pdsch-TimeDomainAllocationListForMultiPDSCH or pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3, the indicated TCI state(s) should be based on the activated TCI states in the first slot with the scheduled PDSCH(s), and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH(s). When the UE is configured with CORESET associated with a search space set for cross-carrier scheduling and the UE is not configured with enableDefaultBeamForCCS, or when the UE is configured with CORESET associated with a search space set for DCI format 1_3 and the UE is not configured with enabledDefaultBeamFormultiCellScheduling, the UE expects tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, and if one or more of the TCI states configured for the serving cell scheduled by the search space set contains qcl-Type set to 'typeD', the UE expects the time offset between the reception of the detected PDCCH in the search space set and a corresponding PDSCH is larger than or equal to the threshold timeDurationForQCL.

Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2 in RRC connected mode, if the UE is not provided dl-OrJointTCI-StateList-r17, and if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI state for the serving cell of scheduled PDSCH contains qcl-Type set to 'typeD',

-	the UE may assume that the DM-RS ports of PDSCH(s) of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. In this case, if the qcl-Type is set to 'typeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers).

-	If a UE is configured with enableDefaultTCI-StatePerCoresetPoolIndex and the UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in different ControlResourceSets,

-	the UE may assume that the DM-RS ports of PDSCH associated with a value of coresetPoolIndex of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId among CORESETs, which are configured with the same value of coresetPoolIndex as the PDCCH scheduling that PDSCH, in the latest slot in which one or more CORESETs associated with the same value of coresetPoolIndex as the PDCCH scheduling that PDSCH within the active BWP of the serving cell are monitored by the UE. In this case, if the 'QCL-TypeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol and they are associated with same value of coresetPoolIndex, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers).

-	If a UE is configured with enableTwoDefaultTCI-States, and at least one TCI codepoint indicates two TCI states, the UE may assume that the DM-RS ports of PDSCH or PDSCH transmission occasions of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states. When the UE is configured by higher layer parameter repetitionScheme set to 'tdmSchemeA' or is configured with higher layer parameter repetitionNumber, and the offset between the reception of the DL DCI and the first PDSCH transmission occasion is less than the threshold timeDurationForQCL, the mapping of the TCI states to PDSCH transmission occasions is determined according to clause 5.1.2.1 by replacing the indicated TCI states with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states based on the activated TCI states in the slot with the first PDSCH transmission occasion. In this case, if the 'QCL-TypeD' in both of the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers)

-	If a UE is not configured with sfnSchemePDSCH, and the UE is configured with sfnSchemePDCCH set to 'sfnSchemeA' and there is no TCI codepoint with two TCI states in the activation command and the CORESET associated with a monitored search space with the lowest CORESET ID in the latest slot is indicated with two TCI states, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the first TCI state of two TCI states indicated for the CORESET. In this case, if the qcl-Type is set to 'typeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET with single active TCI state. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers).

-	In all cases above, if none of configured TCI states for the serving cell of scheduled PDSCH is configured with qcl-Type set to 'typeD', the UE shall obtain the other QCL assumptions from the indicated TCI state(s) for its scheduled PDSCH irrespective of the time offset between the reception of the DL DCI and the corresponding PDSCH.

Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2 in RRC connected mode, if the UE is provided dl-OrJointTCI-StateList-r17, and if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI state for the serving cell of scheduled PDSCH contains qcl-Type set to 'typeD', regardless of configuration of followUnifiedTCI-State,

-	if the indicated TCI state is associated with the PCI of the serving cell, the indicated TCI state is applied to PDSCH reception.

-	if the indicated TCI state is associated with a PCI different from the serving cell, the UE may assume that the DM-RS ports of PDSCH(s) of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. In the CA case, if the 'QCL-TypeD' of the PDSCH DM-RSs from respective CCs in a band are different in a slot, the QCL-TypeD assumption of the PDSCH DM-RS in the CC with lowest CC ID in the band is applied to all the PDSCH DM-RSs in the CCs in the band. In this case, if the qcl-Type is set to 'typeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers).

Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2 in RRC connected mode, for a UE that is provided dl-OrJointTCI-StateList-r17, if a PDSCH of a serving cell is scheduled by a CORESET that does not follow the indicated TCI state, and if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, the indicated TCI state is applied to the PDSCH of the serving cell..

If the PDCCH carrying the scheduling DCI is received on one component carrier, and a PDSCH scheduled by that DCI is on another component carrier:

-	The timeDurationForQCL is determined based on the subcarrier spacing of the scheduled PDSCH. If µPDCCH < µPDSCH an additional timing delay d2μPDSCH2μPDCCH is added to the timeDurationForQCL, where d is defined in 5.2.1.5.1a-1, otherwise d is zero;

-	When the UE is configured with enableDefaultBeamForCCS, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, or if the DL DCI does not have the TCI field present, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH in the active BWP of the scheduled cell.

A UE that has indicated a capability beamCorrespondenceWithoutUL-BeamSweeping set to 'supported', as described in [13, TS 38.306], can determine a spatial domain filter to be used while performing the applicable channel access procedures described in [16, TS 37.213] prior to a UL transmission on the channel as follows:

-	if UE is indicated with an SRI corresponding to the UL transmission, the UE may use a spatial domain filter that is same as the spatial domain transmission filter associated with the indicated SRI,

-	if UE is configured with SRS-spatialRelationInfo for the UL transmission, the UE may use a spatial domain filter that is same as the spatial domain filter associated with referenceSignal in the corresponding SRS-spatialRelationInfo,

-	if UE is configured with TCI-State in dl-OrJointTCI-StateList or TCI-UL-State in ul-TCI-StateList, the UE may use a spatial domain filter that is same as the spatial domain receive filter the UE may use to receive the DL reference signal associated with the indicated TCI state.

When the PDCCH reception includes two PDCCH from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining the time offset between the reception of the DL DCI and the corresponding PDSCH, the PDCCH candidate that ends later in time is used. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the configuration of tci-PresentInDCI or tci-PresentDCI-1-2, the UE expects the same configuration in the first and second CORESETs associated with the two PDCCH candidates; and if the PDSCH is scheduled by a DCI format not having the TCI field present and if the scheduling offset is equal to or larger than timeDurationForQCL, if applicable, PDSCH QCL assumption is based on the CORESET with lower ID among the first and second CORESETs associated with the two PDCCH candidates.

For a periodic CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, the UE shall expect that a TCI-State indicates one of the following quasi co-location type(s):

-	'typeC' with an SS/PBCH block and, when applicable, 'typeD' with the same SS/PBCH block where SS/PBCH block may have a PCI different from the PCI of the serving cell. The UE can assume center frequency, SCS, SFN offset are the same for SS/PBCH block from the serving cell and SS/PBCH block having a PCI different from the serving cell, or

-	'typeC' with an SS/PBCH block and, when applicable,'typeD' with a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, where SS/PBCH block may have a PCI different from the PCI of the serving cell. The UE can assume center frequency, SCS, SFN offset are the same for SS/PBCH block from the serving cell and SS/PBCH block having a PCI different from the serving cell.

For periodic/semi-persistent CSI-RS, if the UE is configured with dl-OrJointTCI-StateList, the UE can assume that the indicated TCI-State is not applied.

For an aperiodic CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, the UE shall expect that a TCI-State indicates qcl-Type set to 'typeA' with a periodic CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, qcl-Type set to 'typeD' with the same periodic CSI-RS resource.

For a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without the higher layer parameter repetition, the UE shall expect that a TCI-State indicates one of the following quasi co-location type(s):

-	'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with the same CSI-RS resource, or

-	'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with an SS/PBCH block, where SS/PBCH block may have a PCI different from the PCI of the serving cell. The UE can assume center frequency, SCS, SFN offset are the same for SS/PBCH block from the serving cell and SS/PBCH block having a PCI different from the serving cell, or

-	'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or

-	'typeB' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info when 'typeD' is not applicable.

For a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, the UE shall expect that a TCI-State indicates one of the following quasi co-location type(s):

-	'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with the same CSI-RS resource, or

-	'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or

-	'typeC' with an SS/PBCH block and, when applicable, 'typeD' with the same SS/PBCH block, the reference RS may additionally be an SS/PBCH block having a PCI different from the PCI of the serving cell. The UE can assume center frequency, SCS, SFN offset are the same for SS/PBCH block from the serving cell and SS/PBCH block having a PCI different from the serving cell.

For the DM-RS of PDCCH, if the UE is not configured with dl-OrJointTCI-StateList, the UE shall expect that a TCI-State indicates one of the following quasi co-location type(s):

-	'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with the same CSI-RS resource, or

-	'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or

-	'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without higher layer parameter repetition and, when applicable, 'typeD' with the same CSI-RS resource.

When a UE is configured with sfnSchemePdcch set to 'sfnSchemeA', and CORESET is activated with two TCI states or is configured with applyIndicatedTCI-State set to 'both', the UE shall assume that the DM-RS port(s)of the PDCCH in the CORESET is quasi co-located with the DL-RSs of the two TCI states. When a UE is configured with sfnSchemePdcch set to 'sfnSchemeB', and a CORESET is activated with two TCI states or is configured with applyIndicatedTCI-State set to 'both', the UE shall assume that the DM-RS port(s)of the PDCCH is quasi co-located with the DL-RSs of the two TCI states except for quasi co-location parameters {Doppler shift, Doppler spread} of the second indicated TCI state.

When a UE is configured by higher layer parameter cjt-Scheme-PDSCH and dl-OrJointTCI-StateList and is indicated with two TCI-States applied for PDSCH reception and reports twoTCI-StatePDSCH-CJT-TxScheme:

-	if the UE is configured with cjtSchemeA, the UE assumes that PDSCH DM-RS port(s) are QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA.

-	if the UE is configured with cjtSchemeB, the UE assumes that PDSCH DM-RS port(s) are QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint TCI state.

For the DM-RS of PDSCH, if the UE is not configured with dl-OrJointTCI-StateList, the UE shall expect that a TCI-State indicates one of the following quasi co-location type(s):

-	'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with the same CSI-RS resource, or

-	'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition,or

-	typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without higher layer parameter repetition and, when applicable, 'typeD' with the same CSI-RS resource.

For the DM-RS of PDCCH, if the UE is configured with dl-OrJointTCI-StateList, the UE shall expect that an indicated TCI-State indicates one of the following quasi co-location type(s):

-	'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with the same CSI-RS resource, or

-	'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition.

For the DM-RS of PDSCH, if the UE is configured with dl-OrJointTCI-StateList, the UE shall expect that an indicated TCI-State indicates one of the following quasi co-location type(s):

-	'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with the same CSI-RS resource, or

-	'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition.

When a UE is configured with sfnSchemePDSCH set to 'sfnSchemeA', and the UE not configured with dl-OrJointTCI-StateList is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' in a DCI scheduling a PDSCH or the UE configured with dl-OrJointTCI-StateList is having two indicated TCI States to be applied to PDSCH, the UE shall assume that the DM-RS port(s)of the PDSCH is quasi co-located with the DL-RSs of the two TCI states. When a UE is configured with sfnSchemePDSCH set to 'sfnSchemeB', and the UE not configured with dl-OrJointTCI-StateList is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' in a DCI scheduling a PDSCH or the UE configured with dl-OrJointTCI-StateList is having two indicated TCI States to be applied to PDSCH, the UE shall assume that the DM-RS port(s)of the PDSCH is quasi co-located with the DL-RSs of the two TCI states except for quasi co-location parameters {Doppler shift, Doppler spread} of the second indicated TCI state.

When a UE is configured with dl-OrJointTCI-StateList or TCI-UL-State and is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet, an indicated TCI state is specific to a coresetPoolIndex value, when it is indicated by the DCI field 'Transmission Configuration Indication' in DCI format 1_1/1_2 associated with the coresetPoolIndex value.

When a UE is configured with dl-OrJointTCI-StateList and is having two indicated TCI-states, if the UE does not report its capability of defaultQCL-TwoTCI in frequency range 2 and when the offset between the reception of the scheduling/activation DCI format 1_0/1_1/1_2 and the scheduled or activated PDSCH reception is less than timeDurationForQCL in frequency range 2, the UE shall apply the first indicated TCI-State to the scheduled or activated PDSCH reception.

When a UE is configured with dl-OrJointTCI-StateList, is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet, if the UE does not report its capability of defaultQCL-PerCORESETPoolIndex in frequency range 2

-	when the offset between the reception of the scheduling/activation DCI format 1_0/1_1/1_2 in a CORESET associated with coresetPoolIndex value 0 and the scheduled or activated PDSCH reception is less than timeDurationForQCL in frequency range 2, the UE shall apply the indicated joint/DL TCI state specific to coresetPoolIndex value 0 to the scheduled or activated PDSCH reception.

-	the UE does not expect that the offset between reception of the scheduling/activation DCI format 1_0/1_1/1_2 in a CORESET associated with coresetPoolIndex value 1 and scheduled or activated PDSCH reception is less than timeDurationForQCL in frequency range 2.

When a UE is configured with dl-OrJointTCI-StateList and is having two indicated TCI-states:

-	Regardless of the offset between the reception of the scheduling DCI format 1_0/1_1/1_2 and the scheduled/activated PDSCH reception, if the UE is in frequency range 1, or the UE reports its capability of defaultQCL-TwoTCI in frequency range 2, or

-	If the UE does not report its capability of defaultQCL-TwoTCI in frequency range 2 and if the scheduling offset between the reception of the scheduling DCI format 1_0/1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than timeDurationForQCL

-	The UE can be configured by higher layer parameter applyIndicatedTCI-StateDCI-1-0 to indicate whether the first, the second, or both of the indicated TCI-state(s) is/are applied to PDSCH reception scheduled or activated by DCI format 1_0. The UE can be configured with applyIndicatedTCI-StateDCI-1-0 with value both only when the UE is configured with cjt-Scheme-PDSCH and the UE reports twoTCI-StatePDSCH-CJT-TxScheme or the UE is configured with sfnSchemePdsch. In that case, the UE shall apply both indicated TCI-states to PDSCH reception scheduled or activated by DCI format 1_0 on a search space other than Type0/0A/2 CSS on CORESET#0.

-	If the UE is not configured with applyIndicatedTCI-StateDCI-1-0, the first indicated TCI-state is applied to PDSCH reception scheduled or activated by DCI format 1_0.

-	When the UE is configured with tci-SelectionPresentInDCI jointly for both DCI formats 1_1 and 1_2 in the same DL BWP, and when the UE receives a DCI format 1_1/1_2 that schedules or activates PDSCH reception, the UE shall determine the indicated joint/DL TCI state(s) for the PDSCH reception according to the following:

-	If the DCI format 1_1/1_2 indicates codepoint "00" for the DCI field 'TCI selection', the UE shall apply the first one of two indicated joint/DL TCI states to all PDSCH DM-RS port(s) of corresponding PDSCH transmission occasion(s) scheduled or activated by the DCI format 1_1/1_2.

-	If the DCI format 1_1/1_2 indicates codepoint "01" for the DCI field 'TCI selection', the UE shall apply the second one of two indicated joint/DL TCI states to all PDSCH DM-RS port(s) of corresponding PDSCH transmission occasion(s) scheduled or activated by the DCI format 1_1/1_2.

-	If the DCI format 1_1/1_2 indicates codepoint "10" for the DCI field 'TCI selection', the UE shall apply both indicated joint/DL TCI states to the PDSCH reception scheduled or activated by the DCI format 1_1/1_2.

-	If the UE is not configured with tci-SelectionPresentInDCI and when the UE receives a DCI format 1_1/1_2 that schedules/activates PDSCH reception, the UE shall apply both indicated TCI-States to the scheduled or activated PDSCH reception.

When a UE is configured with dl-OrJointTCI-StateList, is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet and is having two indicated TCI-states, when the offset between the reception of the scheduling/activation DCI format 1_0/1_1/1_2 and the scheduled or activated PDSCH reception is less than timeDurationForQCL in frequency range 2, and if the PDSCH and a PDCCH overlaps in at least one symbol

-	If the UE does not report its capability of defaultQCL-PerCORESETPoolIndex, and if the 'QCL-TypeD' of the PDSCH DMRS is different from that of PDCCH DMRS, the UE is expected to prioritize the reception of PDCCH. This also applies to the intra-band CA case (when PDSCH and the PDCCH are in different component carriers).

When a UE is configured with dl-OrJointTCI-StateList and is having two indicated TCI-states, when the offset between the reception of the scheduling/activation DCI format 1_0/1_1/1_2 and the scheduled or activated PDSCH reception is less than timeDurationForQCL in frequency range 2, and if the PDSCH and a PDCCH overlaps in at least one symbol

-	If the UE does not report its capability of defaultQCL-TwoTCI, and if the 'QCL-TypeD' of the PDSCH DMRS is different from any one of those of PDCCH DMRS, the UE is expected to prioritize the reception of PDCCH. This also applies to the intra-band CA case (when PDSCH and the PDCCH are in different component carriers).

<omitted text>

### 5.2.3	CSI reporting using PUSCH

A UE shall perform aperiodic CSI reporting using PUSCH on serving cell c upon successful decoding of a DCI format 0_1 or DCI format 0_2 which triggers an aperiodic CSI trigger state. A UE shall perform aperiodic CSI reporting using PUSCH on the serving cell with the smallest serving cell index scheduled by DCI format 0_3 which triggers an aperiodic CSI trigger state.

When a DCI format 0_1 or DCI format 0_3 schedules two PUSCH allocations on the serving cell, the aperiodic CSI report is carried on the second scheduled PUSCH. When a DCI format 0_1 or DCI format 0_3 schedules more than two PUSCH allocations on the serving cell, the aperiodic CSI report is carried on the penultimate scheduled PUSCH.

<omitted text>

## 6.1	UE procedure for transmitting the physical uplink shared channel

PUSCH transmission(s) can be dynamically scheduled by an UL grant in a DCI, or the transmission can correspond to a configured grant Type 1 or Type 2. The configured grant Type 1 PUSCH transmission is semi-statically configured to operate upon the reception of higher layer parameter of configuredGrantConfig including rrc-ConfiguredUplinkGrant without the detection of an UL grant in a DCI. The configured grant Type 2 PUSCH transmission is semi-persistently scheduled by an UL grant in a valid activation DCI according to clause 10.2 of [6, TS 38.213] after the reception of higher layer parameter configuredGrantConfig not including rrc-ConfiguredUplinkGrant. If configuredGrantConfigToAddModList is configured, more than one configured grant configuration of configured grant Type 1 and/or configured grant Type 2 may be active at the same time on an active BWP of a serving cell.

The UE can be configured with a list of up to 64 TCI-UL-State configurations within the higher layer parameter BWP-UplinkDedicated. Each TCI-UL-State configuration contains a parameter for configuring one reference signal, if applicable, for determining UL TX spatial filter for dynamic-grant and configured-grant based PUSCH and PUCCH resource in a CC, and SRS.

If a UE is configured by higher layer parameter PDCCH-Config that contains ControlResourceSets with two different values of coresetPoolIndex for the active BWP of a serving cell, or if a UE is configured with SSB-MTC-AddtionalPCI and with PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet, and if the UE is configured with tag2-Id and is configured with dl-OrJointTCI-StateList or TCI-UL-State for a serving cell, each TCI-State or TCI-UL-State is associated with a tag-Id-ptr for determining timing adjustment for a corresponding UL transmission as described in Clause 4.2 of [6, TS 38.213]. The UE does not expect that TCI-states or TCI-UL-States associated with one coresetPoolIndex to correspond to two TAGs.

For the PUSCH transmission corresponding to a Type 1 configured grant or a Type 2 configured grant activated by DCI format 0_0 or 0_1, the parameters applied for the transmission are provided by configuredGrantConfig except for dataScramblingIdentityPUSCH, txConfig, codebookSubset, maxRank, scaling of UCI-OnPUSCH, which are provided by pusch-Config. A configured grant PUSCH can be transmitted with at most 4 layers. For the PUSCH transmission corresponding to a Type 2 configured grant activated by DCI format 0_2, the parameters applied for the transmission are provided by configuredGrantConfig except for dataScramblingIdentityPUSCH, txConfig, codebookSubsetDCI-0-2, maxRankDCI-0-2, scaling of UCI-OnPUSCH, resourceAllocationType1GranularityDCI-0-2 provided by pusch-Config. If the UE is provided with transformPrecoder in configuredGrantConfig, the UE applies the higher layer parameter tp-pi2BPSK, if provided in pusch-Config, according to the procedure described in clause 6.1.4 for the PUSCH transmission corresponding to a configured grant.

For PUSCH transmissions corresponding to a Type 1 configured grant based on eight antenna ports, when the higher layer parameter txConfig is set to ‘codebook’ and when maxRank is greater than 4, the determination of the precodingAndNumberOfLayers is based on the configuration of maxRank equal to 4; when the higher layer parameter txConfig is set to ‘nonCodebook’ and when Lmax [5, TS38.212] is greater than 4, the determination of the srs-ResourceIndicator is based on the configuration of Lmax equal to 4.

When the UE is configured dl-OrJointTCI-StateList or ul-TCI-StateList, the UE shall perform PUSCH transmission corresponding to a Type 1 configured grant or a Type 2 configured grant or a dynamic grant according to the spatial relation, if applicable, with a reference to the RS for determining UL Tx spatial filter. The RS is determined based on an RS configured with qcl-Type set to 'typeD' of the indicated TCI-State or an RS in the indicated TCI-UL-State. The reference RS in the indicated TCI-State can be a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info. The reference RS in the indicated TCI-UL-State can be a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, an SRS resource in an SRS resource set with the higher layer parameter usage set to 'beamManagement', or SS/PBCH block associated with the same or different PCI from the PCI of the serving cell. When nrofSlotsInCG-Period is configured for Type 1 configured grant or Type 2 configured grant, HARQ process ID for the first configured PUSCH grant and each subsequent valid configured PUSCH grant within a periodicity of the configuration is determined as in clause 5.4.1 of [10, TS 38.321], where a valid configured PUSCH grant is the one not colliding  with the DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, and not colliding with a symbol(s) of an SS/PBCH block with index provided by ssb-PositionsInBurst as described in clause 11.1 of [6, TS 38.213].

When a UE is configured with dl-OrJointTCI-StateList or TCI-UL-State and is having two indicated TCI-States or TCI-UL-States,

-	a UE having a PUSCH transmission scheduled or activated by DCI format 0_0 should apply the first indicated TCI state to the PUSCH transmission,

-	a UE configured with a PUSCH transmission corresponding to a Type 1 configured grant is expected to be configured with the higher layer parameter applyIndicatedTCI-State indicating the first, the second or both of the indicated TCI states to be applied for the PUSCH transmission. If 'both' TCI states are indicated, the UE should apply the first indicated TCI state to the PUSCH transmission occasion(s) or the PUSCH antenna port(s) associated with the first SRS resource set for CB/NCB transmission, and the second indicated TCI state to the PUSCH transmission occasion(s) or the PUSCH antenna port(s) associated with the second SRS resource set for CB/NCB transmission; otherwise the UE should apply either the 'first' or 'second' indicated TCI state to all PUSCH transmission occasions.

-	If the UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in different ControlResourceSets in the active DL BWP, the first and the second indicated TCI states correspond to the indicated TCI-States or TCI-UL-States specific to coresetPoolIndex value 0 and value 1, respectively, and applyIndicatedTCI-State does not indicate both of the indicated TCI states to be applied for the PUSCH transmission.

For the PUSCH retransmission scheduled by a PDCCH with CRC scrambled by CS-RNTI with NDI=1, the parameters in pusch-Config are applied for the PUSCH transmission except for p0-NominalWithoutGrant, p0-PUSCH-Alpha, powerControlLoopToUse, pathlossReferenceIndex described in clause 7.1 of [6, TS 38.213], mcs-Table, mcs-TableTransformPrecoder described in clause 6.1.4.1 and transformPrecoder described in clause 6.1.3.

For a UE configured with two uplinks in a serving cell, PUSCH retransmission for a TB on the serving cell is not expected to be on a different uplink than the uplink used for the PUSCH initial transmission of that TB.

A UE shall upon detection of a PDCCH with a configured DCI format 0_0, 0_1, 0_2 or 0_3 transmit the corresponding PUSCH as indicated by that DCI unless the UE does not generate a transport block as described in [10, TS 38.321]. Upon detection of a DCI format 0_1 or 0_2  with 'UL-SCH indicator' set to '0' and with a non-zero 'CSI request' where the associated reportQuantity in CSI-ReportConfig set to 'none' for all CSI report(s) triggered by 'CSI request' in this DCI format 0_1 or 0_2, the UE ignores all fields in this DCI except the 'CSI request' and the UE shall not transmit the corresponding PUSCH as indicated by this DCI format 0_1 or 0_2. Upon detection of a DCI format 0_3 with 'UL-SCH indicator' set to '0' and with a non-zero 'CSI request' where the associated reportQuantity in CSI-ReportConfig set to 'none' for all CSI report(s) triggered by 'CSI request' in this DCI format 0_3, the UE ignores all fields for the PUSCH that would carry an aperiodic CSI report according to clause 5.2.3 on the scheduled cell with the smallest serving cell index in this DCI except the 'CSI request' and the UE shall not transmit the corresponding PUSCH on the serving cell with the smallest serving cell index as indicated by this DCI format 0_3.

<omitted text>

#### 6.1.2.1	Resource allocation in time domain

When the UE is scheduled to transmit a transport block and no CSI report by a DCI or by a RAR UL grant or fallbackRAR UL grant, or the UE is scheduled to transmit a transport block and a CSI report(s) on PUSCH by a DCI, the 'Time domain resource assignment' field value m for the scheduled PUSCH on the serving cell of the DCI or the PUSCH time resource allocation field value m of the RAR UL grant or of the fallbackRAR UL grant provides a row index m + 1 to a resource allocation table. The determination of the used resource allocation table is defined in Clause 6.1.2.1.1. The indexed row defines the slot offset K2, the start and length indicator SLIV, or directly the start symbol S and the allocation length L, the PUSCH mapping type, the number of slots used for TBS determination (if numberOfSlotsTBoMS is present in the resource allocation table), and the number of repetitions (if numberOfRepetitions is present in the resource allocation table) to be applied in the PUSCH transmission.

When the UE is scheduled to transmit a PUSCH with no transport block and with a CSI report(s) by a 'CSI request' field on a DCI, the 'Time domain resource assignment' field value m of the DCI provides a row index m + 1 to the allocated table as defined in Clause 6.1.2.1.1. The indexed row defines the start and length indicator SLIV, or directly the start symbol S and the allocation length L, and the PUSCH mapping type to be applied in the PUSCH transmission and the K2 value is determined as , where  are the corresponding list entries of the higher layer parameter

-	reportSlotOffsetListDCI-0-2 or reportSlotOffsetListDCI-0-2-r17, if PUSCH is scheduled by DCI format 0_2 and reportSlotOffsetListDCI-0-2 or reportSlotOffsetListDCI-0-2-r17 is configured;

-	reportSlotOffsetListDCI-0-1 or reportSlotOffsetListDCI-0-1-r17, if PUSCH is scheduled by DCI format 0_1 or 0_3 and reportSlotOffsetListDCI-0-1 or reportSlotOffsetListDCI-0-1-r17 is configured;

-	reportSlotOffsetList or reportSlotOffsetList-r17, otherwise;

in CSI-ReportConfig for the  triggered CSI Reporting Settings and  is the (m+1)th entry of  including the omitted CSI Reporting Settings triggered for non-active DL BWPs, where the UE does not expect that (m+1) is larger than 16.

-	The slot Ks where the UE shall transmit the PUSCH is determined by K2 as Ks =, if UE is configured with ca-SlotOffset for at least one of the scheduled and scheduling cell, Ks=n⋅2μPUSCH2μPDCCH+K2+Koffset⋅2μPUSCH2μKoffset, otherwise, where μKoffsetis the subcarrier spacing configuration for Koffset with a value of 0 for frequency range 1 and for FR2-NTN, n is the slot with the scheduling DCI, K2 is based on the numerology of PUSCH,  and  are the subcarrier spacing configurations for PUSCH and PDCCH, respectively, and where

-	if the scheduling DCI is DCI format 0_0 with CRC scrambled by TC-RNTI, Koffset is provided by cellSpecificKoffset if configured and otherwise Koffset=0;

-	otherwise, Koffset is a parameter configured by higher layer as specified in clause 4.2 of [6 TS 38.213].

-	Nslot, offset, PDCCHCA and μoffset,PDCCH are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH, Nslot, offset, PUSCHCA and μoffset,PUSCH are the Nslot, offsetCA and the,respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the PUSCH, as defined in clause 4.5 of [4, TS 38.211], and

-	for PUSCH scheduled by DCI format 0_1, if pusch-RepTypeIndicatorDCI-0-1  is set to 'pusch-RepTypeB', the UE applies PUSCH repetition Type B procedure when determining the time domain resource allocation. For PUSCH scheduled by DCI format 0_2, if pusch-RepTypeIndicatorDCI-0-2 is set to 'pusch-RepTypeB', the UE applies PUSCH repetition Type B procedure when determining the time domain resource allocation. Otherwise, the UE applies PUSCH repetition Type A procedure when determining the time domain resource allocation for PUSCH scheduled by PDCCH, by RAR UL grant, or by fallbackRAR UL grant.

-	for PUSCH scheduled by DCI format 0_1, 0_2 or 0_3, if numberOfSlotsTBoMS is present and larger than 1, the UE applies TB processing over multiple slots procedure when determining the time domain resource allocation.

-	For PUSCH repetition Type A and TB processing over multiple slots, the starting symbol S relative to the start of the slot, and the number of consecutive symbols L counting from the symbol S allocated for the PUSCH are determined from the start and length indicator SLIV of the indexed row:

if  then

else

where, and

-	For PUSCH repetition Type B, the starting symbol S relative to the start of the slot, and the number of consecutive symbols L counting from the symbol S allocated for the PUSCH are provided by startSymbol and length of the indexed row of the resource allocation table, respectively.

-	For PUSCH repetition Type A and TB processing over multiple slots, the PUSCH mapping type is set to Type A or Type B as defined in Clause 6.4.1.1.3 of [4, TS 38.211] as given by the indexed row.

-	For PUSCH repetition Type B, the PUSCH mapping type is set to Type B.

The UE shall consider the S and L combinations defined in table 6.1.2.1-1 as valid PUSCH allocations

Table 6.1.2.1-1: Valid S and L combinations

| PUSCH mapping type | Normal cyclic prefix |  |  | Extended cyclic prefix |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | S | L | S+L | S | L | S+L |
| Type A (repetition Type A only) | 0 | {4,…,14} | {4,…,14} | 0 | {4,…,12} | {4,…,12} |
| Type B | {0,…,13} | {1,…,14} | {1,…,14} for repetition Type A, {1,…,27} for repetition Type B | {0,…, 11} | {1,…,12} | {1,…,12} for repetition Type A, {1,…,23} for repetition Type B |

For TB processing over multiple slots, when transmitting PUSCH scheduled by DCI format 0_1, 0_2 or 0_3 in PDCCH with CRC scrambled with C-RNTI, MCS-C-RNTI, or CS-RNTI with NDI=1,

-	the number of slots used for TBS determination N is indicated by numberOfSlotsTBoMS.

-	the number of repetitions K of the number of slots N used for TBS determination is determined as

-	if numberOfRepetitions is present in the resource allocation table, the number of repetitions K is equal to numberOfRepetitions;

-	otherwise, K=1.

-	when the UE supports repetition of TB processing over multiple slots, the UE does not expect that N∙K is larger than 32.

When configured with m= 5 or 6 the UE does not expect to be scheduled with more than one PUSCH in a slot, by a single DCI or multiple DCIs, where multiple DCIs are not associated with CORESETs having different coresetpoolIndex.

For PUSCH repetition Type A, when transmitting PUSCH scheduled by DCI format 0_1, 0_2 or 0_3 in PDCCH with CRC scrambled with C-RNTI, MCS-C-RNTI, or scheduled by DCI format 0_1 or 0_2 in PDCCH with CRC scrambled CS-RNTI with NDI=1, the number of repetitions K is determined as

-	if numberOfRepetitions is present in the resource allocation table, the number of repetitions K is equal to numberOfRepetitions;

-	elseif the UE is configured with pusch-AggregationFactor, the number of repetitions K is equal to pusch-AggregationFactor;

-	otherwise K=1.

-	the number of slots used for TBS determination N is equal to 1.

For PUSCH repetition type A, when transmitting PUSCH scheduled by RAR UL grant, the 2 MSBs of the MCS information field of the RAR UL grant provide a codepoint to determine the number of repetitions K according to Table 6.1.2.1-1A, based on whether or not the higher layer parameter numberOfMsg3-RepetitionsList is configured. The number of slots used for TBS determination N is equal to 1.

For PUSCH repetition type A, when transmitting PUSCH scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, the 2 MSBs of the MCS information field of the DCI format 0_0 with CRC scrambled by TC-RNTI provide a codepoint to determine the number of repetitions K according to Table 6.1.2.1-1A, based on whether or not the higher layer parameter numberOfMsg3-RepetitionsList is configured. The number of slots used for TBS determination N is equal to 1.

Table 6.1.2.1-1A: Number of repetition K as a function of 2 MSBs of MCS information field

| numberOfMsg3-RepetitionsList is configured |  |  | numberOfMsg3-RepetitionsList is not configured |  |
| --- | --- | --- | --- | --- |
| Codepoint | K |  | Codepoint | K |
| 00 | First value of numberOfMsg3-RepetitionsList |  | 00 | 1 |
| 01 | Second value of numberOfMsg3-RepetitionsList |  | 01 | 2 |
| 10 | Third value of numberOfMsg3-RepetitionsList |  | 10 | 3 |
| 11 | Fourth value of numberOfMsg3-RepetitionsList |  | 11 | 4 |

If a UE is configured with higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH but not configured with extendedK2 in pusch-TimeDomainAllocationListForMultiPUSCH or configured with higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH-DCI-0-3 but not configured with extendedK2 in pusch-TimeDomainAllocationListForMultiPUSCH-DCI-0-3, the UE does not expect to be configured with pusch-AggregationFactor.

If a UE is configured with extendedK2 in pusch-TimeDomainAllocationListForMultiPUSCH in which one or more rows contain multiple SLIVs for PUSCH on a UL BWP of a serving cell, the UE does not apply pusch-AggregationFactor, if configured, to DCI format 0_1 on the UL BWP of the serving cell and the UE does not expect to be configured with numberOfRepetitions in pusch-TimeDomainAllocationListForMultiPUSCH. If a UE is configured with extendedK2 in pusch-TimeDomainAllocationListForMultiPUSCH-DCI-0-3 in which one or more rows contain multiple SLIVs for PUSCH on a UL BWP of a serving cell, the UE does not apply pusch-AggregationFactor, if configured, to DCI format 0_3 on the UL BWP of the serving cell and the UE does not expect to be configured with numberOfRepetitions in pusch-TimeDomainAllocationListForMultiPUSCH-DCI-0-3.

If a UE is configured with extendedK2 in pusch-TimeDomainAllocationListForMultiPUSCH or pusch-TimeDomainAllocationListForMultiPUSCH-DCI-0-3 in which one or more rows contain multiple SLIVs for PUSCH on a UL BWP of a serving cell, when any two UL DCIs end in the same symbol and at least one of the DCIs scheduling multiple PUSCHs, the UE does not expect that the any scheduled multiple PUSCHs have overlapping spans, where the span associated with a DCI is defined from the beginning of the first scheduled PUSCH till the end of the last scheduled PUSCH.

For unpaired spectrum:

-	When AvailableSlotCounting is enabled, and in case K>1, the UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1, 0_2 or 0_3, based on tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst, and the TDRA information field value in the DCI format 0_1, 0_2 or 0_3.

-	A slot is not counted in the number of N∙K slots for PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_1, 0_2 or 0_3 if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

-	Otherwise, the UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1, 0_2 or 0_3, based on the TDRA information field value in the DCI format 0_1, 0_2 or 0_3.

-	The UE determines N∙K slots for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1, 0_2 or 0_3, based on tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst, and the TDRA information field value in the DCI format 0_1, 0_2 or 0_3.

-	A slot is not counted in the number of N∙K slots for a PUSCH transmission of TB processing over multiple slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

-	The UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by RAR UL grant, based on tdd-UL-DL-ConfigurationCommon and ssb-PositionsInBurst, and the TDRA information field value in the RAR UL grant.

-	A slot is not counted in the number of N∙K slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by RAR UL grant, if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

-	The UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, based on tdd-UL-DL-ConfigurationCommon and ssb-PositionsInBurst and the TDRA information field value in the DCI scheduling the PUSCH.

-	A slot is not counted in the number of N∙K slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_0 scrambled by TC-RNTI, if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

For paired spectrum and SUL band:

-	The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or 0_3 for paired spectrum only, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, or 0_3 for paired spectrum only, based on the TDRA information field value in the DCI format 0_1, 0_2 or 0_3.

-	For the case of a reduced capability half-duplex UE, the UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1, 0_2 or 0_3 when AvailableSlotCounting is enabled and K>1, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1,  0_2 or 0_3, based on the TDRA information field value in the DCI format 0_1, 0_2 or 0_3. A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot does not start or end at least NRx-Tx⋅Tc or NTx-Rx⋅Tc, respectively, from the last or first symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

-	The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by RAR UL grant, based on the TDRA information field value in the RAR UL grant.

-	The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, based on the TDRA information field value in the DCI scheduling the PUSCH.

If a UE would transmit a PUSCH of PUSCH repetition Type A when AvailableSlotCounting is enabled and K>1 or a TB processing over multiple slots over N∙K slots, and the UE does not transmit the PUSCH of a TB processing over multiple slots or the PUSCH repetition Type A in a slot from the N∙K slots, according to Clause 9, Clause 11.1, Clause 11.2A, Clause 15 and Clause 17.2 of [6, TS 38.213], and Clause 5.34.3 of [10, TS 38.321], the UE counts the slots in the number of N∙K slots.

For PUSCH repetition Type A, in case K>1,

-	If the PUSCH is scheduled by DCI format 0_1, 0_2 or 0_3

-	if AvailableSlotCounting is enabled, the same symbol allocation is applied across the N∙K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the N∙K slots determined for the PUSCH transmission, applying the same symbol allocation in each slot.

-	Otherwise, the same symbol allocation is applied across the N∙K consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the N∙K consecutive slots applying the same symbol allocation in each slot.

-	Else if the PUSCH is scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, the same symbol allocation is applied across the N∙K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the N∙K slots determined for the PUSCH transmission, applying the same symbol allocation in each slot.

For PUSCH repetition Type B:

-	If pusch-DMRS-Bundling is enabled, the PUSCH is limited to a single transmission layer.

For TB processing over multiple slots:

-	For unpaired spectrum, the same symbol allocation is applied across the N∙K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. The UE shall transmit the TB across the N∙K slots determined for the PUSCH transmission, applying the same symbol allocation in each slot.

-	For paired spectrum or supplementary uplink band, the same symbol allocation is applied across the  N∙K consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall transmit the TB across the N∙K consecutive slots applying the same symbol allocation in each slot.

-	For the case of reduced capability half-duplex UE, the same symbol allocation is applied across the N∙K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. The UE shall transmit the TB across the N∙K slots determined for the PUSCH transmission, applying the same symbol allocation in each slot.

For a PUSCH transmission scheduled by DCI format 0_1, 0_2 or 0_3, or 0_0 with CRC scrambled by TC-RNTI, the redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, … N∙K -1, is determined according to table 6.1.2.1-2.

For a PUSCH transmission of a PUSCH repetition Type A scheduled by RAR UL grant, the redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, … N∙K -1, is determined according to the first row of Table 6.1.2.1-2.

Table 6.1.2.1-2: Redundancy version for PUSCH transmission

| rvid indicated by the DCI scheduling the PUSCH | rvid to be applied to nth transmission occasion (repetition Type A) or TB processing over multiple slots) or nth actual repetition (repetition Type B) |  |  |  |
| --- | --- | --- | --- | --- |
|  | ((n-(n mod N))/N) mod 4 = 0 | ((n-(n mod N))/N) mod 4 = 1 | ((n-(n mod N))/N) mod 4 = 2 | ((n-(n mod N))/N) mod 4 = 3 |
| 0 | 0 | 2 | 3 | 1 |
| 2 | 2 | 3 | 1 | 0 |
| 3 | 3 | 1 | 0 | 2 |
| 1 | 1 | 0 | 2 | 3 |

When transmitting MsgA PUSCH on a non-initial UL BWP, if the UE is configured with startSymbolAndLengthMsgA-PO, the UE shall determine the S and L from startSymbolAndLengthMsgA-PO.

When transmitting MsgA PUSCH, if the UE is not configured with startSymbolAndLengthMsgA-PO, and if the TDRA list PUSCH-TimeDomainResourceAllocationList is provided in PUSCH-ConfigCommon, the UE shall use msgA-PUSCH-TimeDomainAllocation to indicate which values are used in the list. If PUSCH-TimeDomainResourceAllocationList is not provided in PUSCH-ConfigCommon, the UE shall use parameters S and L from table 6.1.2.1.1-2 or table 6.1.2.1.1-3 where msgA-PUSCH-TimeDomainAllocation indicates which values are used in the list. The time offset for PUSCH transmission is described in [6, TS 38.213].

For PUSCH repetition Type A and TB processing over multiple slots, a PUSCH transmission in a slot of a multi-slot PUSCH transmission is omitted according to the conditions in Clause 9, Clause 11.1, Clause 11.2A, Clause 15 and Clause 17.2 of [6, TS 38.213], and Clause 5.34.3 of [10, TS 38.321].

For PUSCH repetition Type B, except for PUSCH transmitting CSI report(s) with no transport block, the number of nominal repetitions is given by numberOfRepetitions. For the n-th nominal repetition, n = 0, …, numberOfRepetitions - 1,

-	The slot where the nominal repetition starts is given by , and the starting symbol relative to the start of the slot is given by .

-	The slot where the nominal repetition ends is given by , and the ending symbol relative to the start of the slot is given by .

Here is the slot where the PUSCH transmission starts, and is the number of symbols per slot as defined in Clause 4.3.2 of [4, TS 38.211].

For PUSCH repetition Type B, the UE determines invalid symbol(s) for PUSCH repetition Type B transmission as follows:

-	A symbol that is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated is considered as an invalid symbol for PUSCH repetition Type B transmission.

-	For operation in unpaired spectrum, symbols indicated by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon for reception of SS/PBCH blocks are considered as invalid symbols for PUSCH repetition Type B transmission.

-	For a reduced capability half-duplex UE in paired spectrum, symbols that do not start or end at least NRx-Tx⋅Tc or NTx-Rx⋅Tc, respectively, from the last or first symbol of an SS/PBCH block with index indicated by ssb-PositionsInBurst in SIB1 or by ssb-PositionsInBurst in ServingCellConfigCommon or by NonCellDefiningSSB, or by ssb-PositionsInBurst in SSB-MTC-AdditionalPCI associated to physical cell ID with active TCI states for PDCCH or PDSCH, or for a set of symbols of a slot corresponding to SS/PBCH blocks configured for L1 beam measurement/reporting for reception of SS/PBCH blocks are considered as invalid symbols for PUSCH repetition Type B transmission.

-	For operation in unpaired spectrum, symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set are considered as invalid symbol(s) for PUSCH repetition Type B transmission.

-	For operation in unpaired spectrum, if numberOfInvalidSymbolsForDL-UL-Switching is configured, numberOfInvalidSymbolsForDL-UL-Switching symbol(s) after the last symbol that is indicated as downlink in each consecutive set of all symbols that are indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated are considered as invalid symbol(s) for PUSCH repetition Type B transmission. The symbol(s) given by numberOfInvalidSymbolsForDL-UL-Switching are defined using the reference SCS configuration referenceSubcarrierSpacing provided in tdd-UL-DL-ConfigurationCommon.

-	For operation with shared spectrum channel access with semi-static channel occupancy, symbols in an idle duration associated with a periodic channel occupancy as described in Clause 4.3.1.1 of [16, TS 37.213], or in an idle duration in a period associated with an initiated channel occupancy as described in Clause 4.3.2. of [16, TS 37.213] are considered as invalid symbol(s) for PUSCH repetition Type B transmission.

-	The UE may be configured with the higher layer parameter invalidSymbolPattern, which provides a symbol level bitmap spanning one or two slots (higher layer parameter symbols given by invalidSymbolPattern). A bit value equal to 1 in the symbol level bitmap symbols indicates that the corresponding symbol is an invalid symbol for PUSCH repetition Type B transmission. The UE may be additionally configured with a time-domain pattern (higher layer parameter periodicityAndPattern given by invalidSymbolPattern), where each bit of periodicityAndPattern corresponds to a unit equal to a duration of the symbol level bitmap symbols, and a bit value equal to 1 indicates that the symbol level bitmap symbols is present in the unit. The periodicityAndPattern can be {1, 2, 4, 5, 8, 10, 20 or 40} units long, but maximum of 40 msec. The first symbol of periodicityAndPattern every 40 msec/P periods is a first symbol in frame 𝑛𝑓 mod 4 = 0, where P is the duration of periodicityAndPattern-r16 in units of msec. When periodicityAndPattern is not configured, for a symbol level bitmap spanning two slots, the bits of the first and second slots correspond respectively to even and odd slots of a radio frame, and for a symbol level bitmap spanning one slot, the bits of the slot correspond to every slot of a radio frame. If invalidSymbolPattern is configured, when the UE applies the invalid symbol pattern is determined as follows:

-	if the PUSCH is scheduled by DCI format 0_1, or corresponds to a Type 2 configured grant activated by DCI format 0_1, and if invalidSymbolPatternIndicatorDCI-0-1  is configured,

-	if invalid symbol pattern indicator field is set 1, the UE applies the invalid symbol pattern;

-	otherwise, the UE does not apply the invalid symbol pattern;

-	if the PUSCH is scheduled by DCI format 0_2, or corresponds to a Type 2 configured grant activated by DCI format 0_2, and if invalidSymbolPatternIndicatorDCI-0-2  is configured,

-	if invalid symbol pattern indicator field is set 1, the UE applies the invalid symbol pattern;

-	otherwise, the UE does not apply the invalid symbol pattern;

-	otherwise, the UE applies the invalid symbol pattern.

-	If the UE

-	is configured with multiple serving cells within a cell group and is provided with directionalCollisionHandling-r16 = 'enabled' for a set of serving cell(s) among the multiple serving cells, and

-	indicates support of half-DuplexTDD-CA-SameSCS-r16 capability, and

-	is not configured to monitor PDCCH for detection of DCI format 2-0 on any of the multiple serving cells,

-	a symbol indicated to the UE for reception of SS/PBCH blocks in a first cell of the multiple serving cells by ssb-PositionsInBurst in SIB1, or by ssb-PositionsInBurst in ServingCellConfigCommon, or by NonCellDefiningSSB, or by ssb-PositionsInBurst in SSB-MTC-AdditionalPCI associated to physical cell ID with active TCI states for PDCCH or PDSCH, or for a set of symbols of a slot corresponding to SS/PBCH blocks configured for L1 beam measurement/reporting is considered as an invalid symbol for PUSCH repetition Type B transmission in

-	any of the multiple serving cells if the UE is not capable of simultaneous transmission and reception as indicated by simultaneousRxTxInterBandCA among the multiple serving cells, and

-	any one of the cells corresponding to the same band as the first cell, irrespective of any capability indicated by simultaneousRxTxInterBandCA

and

-	a symbol is considered as an invalid symbol in another cell among the set of serving  cell(s) provided with directionalCollisionHandling-r16 for PUSCH repetition Type B transmission with Type 1 or Type 2 configured grant except for the first Type 2 PUSCH transmission (including all repetitions) after activation if the symbol is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated on the reference cell as defined in Clause 11.1 of [6, TS 38.213], or the UE is configured by higher layers to receive PDCCH, PDSCH, or CSI-RS on the reference cell in the symbol.

For PUSCH repetition Type B, after determining the invalid symbol(s) for PUSCH repetition type B transmission for each of the K nominal repetitions, the remaining symbols are considered as potentially valid symbols for PUSCH repetition Type B transmission. If the number of potentially valid symbols for PUSCH repetition type B transmission is greater than zero for a nominal repetition, the nominal repetition consists of one or more actual repetitions, where each actual repetition consists of a consecutive set of all potentially valid symbols that can be used for PUSCH repetition Type B transmission within a slot. An actual repetition with a single symbol is omitted except for the case of L=1. An actual repetition is omitted according to the conditions in Clause 9, Clause 11.1, Clause 11.2A, Clause 15 and Clause 17.2 of [6, TS 38.213], and Clause 5.34.3 of [10, TS 38.321]. The UE shall repeat the TB across actual repetitions. The redundancy version to be applied on the nth actual repetition (with the counting including the actual repetitions that are omitted) is determined according to table 6.1.2.1-2, where N=1.

For PUSCH repetition Type B, when a UE receives a DCI that schedules aperiodic CSI report(s) or activates semi-persistent CSI report(s) on PUSCH with no transport block by a 'CSI request' field on a DCI, the number of nominal repetitions is always assumed to be 1, regardless of the value of numberOfRepetitions. When the UE is scheduled to transmit a PUSCH repetition Type B with no transport block and with aperiodic or semi-persistent CSI report(s) by a 'CSI request' field on a DCI, the first nominal repetition is expected to be the same as the first actual repetition. For PUSCH repetition Type B carrying semi-persistent CSI report(s) without a corresponding PDCCH after being activated on PUSCH by a 'CSI request' field on a DCI, if the first nominal repetition is not the same as the first actual repetition, the first nominal repetition is omitted; otherwise, the first nominal repetition is omitted according to the conditions in Clause 9, Clause 11.1, Clause 11.2A, Clause 15 and Clause 17.2 of [6, TS 38.213], and Clause 5.34.3 of [10, TS 38.321].

For PUSCH repetition Type B, when a UE is scheduled to transmit a transport block and aperiodic CSI report(s) on PUSCH by a 'CSI request' field on a DCI, the CSI report(s) is multiplexed only on the first actual repetition. The UE does not expect that the first actual repetition has a single symbol duration.

For pusch-TimeDomainAllocationListForMultiPUSCH and pusch-TimeDomainAllocationListForMultiPUSCH-DCI-0-3 in pusch-Config, if a row indicates resource allocation for two to eight contiguous PUSCHs and extendedK2 is not configured, K2 given by k2-r16 indicates the slot where UE shall transmit the first PUSCH of the multiple PUSCHs. Each PUSCH has a separate SLIV and mapping type. The number of scheduled PUSCHs is signalled by the number of indicated valid SLIVs in the row of the pusch-TimeDomainAllocationListForMultiPUSCH signalled in DCI format 0_1 or in the row of the pusch-TimeDomainAllocationListForMultiPUSCH-DCI-0-3 signalled in DCI format 0_3.

For pusch-TimeDomainAllocationListForMultiPUSCH and pusch-TimeDomainAllocationListForMultiPUSCH-DCI-0-3 in pusch-Config, if a row indicates resource allocation of more than one PUSCH and extendedK2 is configured, each PUSCH has a separate SLIV, mapping type and K2 given by extendedK2. If a row indicates resource allocation of a single PUSCH, the PUSCH has a single SLIV, mapping type, and K2, where K2 is given by extendedK2, if configured, otherwise K2 is given by k2-r16. The number of scheduled PUSCHs is signalled by the number of indicated SLIVs in the row of the pusch-TimeDomainAllocationListForMultiPUSCH signalled in DCI format 0_1 or in the row of the pusch-TimeDomainAllocationListForMultiPUSCH-DCI-0-3 signalled in DCI format 0_3.

If a UE is configured with extendedK2 in pusch-TimeDomainAllocationListForMultiPUSCH in which one or more rows contain multiple SLIVs for PUSCH on a UL BWP of a serving cell, and the UE is indicated re-transmission of PUSCH by DCI format 0_1, where the PUSCH is correspond to a configured grant Type 1 or Type 2, the UE does not expect that the number of indicated SLIVs in the row of the pusch-TimeDomainAllocationListForMultiPUSCH by the DCI is more than one.

If a UE is configured with pusch-TimeDomainAllocationListForMultiPUSCH or pusch-TimeDomainAllocationListForMultiPUSCH-DCI-0-3 in which one or more rows contain multiple SLIVs for PUSCH on a UL BWP of a serving cell, the UE does not expect to be scheduled with one or multiple PUSCH transmissions by a single DCI format 0_1 or 0_3, where each PUSCH transmission overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

When the UE is configured with minimumSchedulingOffsetK2 in an active UL BWP it applies a minimum scheduling offset restriction indicated by the 'Minimum applicable scheduling offset indicator' field in DCI format 0_1, 0_3, 1_1 or 1_3 if the same field is available. When the UE is configured with minimumSchedulingOffsetK2 in an active UL BWP and it has not received 'Minimum applicable scheduling offset indicator' field in DCI format 0_1, 0_3, 1_1 or 1_3, the UE shall apply a minimum scheduling offset restriction indicated based on 'Minimum applicable scheduling offset indicator' value '0'. When the minimum scheduling offset restriction is applied the UE is not expected to be scheduled with a DCI in slot n to transmit a PUSCH scheduled with C-RNTI, CS-RNTI, MCS-C-RNTI or SP-CSI-RNTI with K2 smaller than K2min⋅2μ'2μ, where K2min and μ are the applied minimum scheduling offset restriction and the numerology of the active UL BWP of the scheduled cell when receiving the DCI in slot n, respectively, and μ' is the numerology of the new active UL BWP in case of active UL BWP change in the scheduled cell and is equal to μ, otherwise. The minimum scheduling offset restriction is not applied when PUSCH transmission is scheduled by RAR UL grant or fallbackRAR UL grant for RACH procedure, or when PUSCH is scheduled with TC-RNTI. The application delay of the change of the minimum scheduling offset restriction is determined in Clause 5.3.1.

When neither multipanelSchemeSDM nor multipanelSchemeSFN is configured and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook' or 'nonCodebook', for PUSCH repetition Type A, in case K>1, the same symbol allocation is applied across the K consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the K consecutive slots applying the same symbol allocation in each slot, and the association of the first and second SRS resource set in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 to each slot is determined as follows:

-	if a DCI format 0_1 or DCI format 0_2 indicates codepoint "00" for the SRS resource set indicator, the first SRS resource set is associated with all K consecutive slots,

-	if a DCI format 0_1 or DCI format 0_2 indicates codepoint "01" for the SRS resource set indicator, the second SRS resource set is associated with all K consecutive slots,

-	if a DCI format 0_1 or DCI format 0_2 indicates codepoint "10" for the SRS resource set indicator, the first and second SRS resource set association to K consecutive slots is determined as follows:

-	When K = 2, the first and second SRS resource sets are applied to the first and second slot of 2 consecutive slots, respectively.

-	When K > 2 and cyclicMapping in PUSCH-Config is enabled, the first and second SRS resource sets are applied to the first and second slot of K consecutive slots, respectively, and the same SRS resource set mapping pattern continues to the remaining slots of K consecutive slots.

-	When K > 2 and sequentialMapping in PUSCH-Config is enabled, first SRS resource set is applied to the first and second slots of K consecutive slots, and the second SRS resource set is applied to the third and fourth slot of K consecutive slots, and the same SRS resource set mapping pattern continues to the remaining slots of K consecutive slots.

-	Otherwise, a DCI format 0_1 or DCI format 0_2 indicates codepoint "11" for the SRS resource set indicator, and the first and second SRS resource set association to K consecutive slots is determined as follows,

-	When K = 2, the second and first SRS resource set are applied to the first and second slot of 2 consecutive slots, respectively.

-	When K > 2 and cyclicMapping in PUSCH-Config is enabled, the second and first SRS resource sets are applied to the first and second slot of K consecutive slots, respectively, and the same SRS resource set mapping pattern continues to the remaining slots of the K consecutive slots.

-	When K > 2 and sequentialMapping in PUSCH-Config is enabled, the second SRS resource set is applied to the first and second slot of K consecutive slots, and the first SRS resource set is applied to the third and fourth slot of K consecutive slots, and the same SRS resource set mapping pattern continues to the remaining slots of the K consecutive slots.

For PUSCH repetition Type B, when neither multipanelSchemeSDM nor multipanelSchemeSFN is configured and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook' or 'nonCodebook', the SRS resource set association to nominal PUSCH repetitions follows the same method as SRS resource set association to slots in PUSCH Type A repetition by considering nominal repetitions instead of slots.

When a UE is configured with dl-OrJointTCI-StateList or TCI-UL-State and is having two indicated TCI-States or TCI-UL-States, and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook' or 'nonCodebook', for PUSCH repetition Type A or Type B as described above, or for PUSCH transmission when multipanelSchemeSDM or multipanelSchemeSFN is configured, the association of the first and second indicated joint/UL TCI states to PUSCH transmission occasions or to corresponding PUSCH antenna ports is determined as follows:

-	if a DCI format 0_1 or DCI format 0_2 indicates codepoint “00” or “01” for the SRS resource set indicator, the first or second indicated joint/UL TCI state is applied to all PUSCH transmission occasions, respectively.

-	if a DCI format 0_1 or DCI format 0_2 indicates codepoint “10” or “11” for the SRS resource set indicator, and the multipanelSchemeSDM or multipanelSchemeSFN is not configured,

-	the first indicated joint/UL TCI state is applied to the PUSCH transmission occasion(s) associated with the first SRS resource set and the second indicated joint/UL TCI state is applied to the PUSCH transmission occasion(s) associated with the second SRS resource set, where the association of PUSCH transmission occasions to SRS resource sets is determined for K = 2 and K > 2, and depending on whether cyclicMapping or sequentialMapping in PUSCH-Config is enabled, based on the above description in this Clause.

-	if a DCI format 0_1 or DCI format 0_2 indicates codepoint “10” for the SRS resource set indicator and multipanelSchemeSDM or multipanelSchemeSFN is configured,

-	the first indicated TCI state is applied to the PUSCH antenna port(s), of corresponding PUSCH transmission occasion, associated with the first SRS resource set, and the second indicated TCI state is applied to the PUSCH antenna port(s), of corresponding PUSCH transmission occasion, associated with the second SRS resource set, where the association of PUSCH antenna ports to SRS resource sets is determined according to Clauses 6.1.1.1 and 6.1.1.2.

For both PUSCH repetition Type A and PUSCH repetition Type B, when a DCI format 0_1 or DCI format 0_2 indicates codepoint "10" or "11" for the SRS resource set indicator, the redundancy version to be applied on the nth transmission occasion (for PUSCH repetition Type A) of the TB, where n = 0, 1, … K-1, or nth actual repetition (for PUSCH repetition Type B, with the counting including the actual repetitions that are omitted) is determined according to Table 6.1.2.1-2 and Table 6.1.2.1-3. For all PUSCH repetitions associated with the SRS resource set of the first transmission occasion or actual repetition, the redundancy version to be applied is derived according to Table 6.1.2.1-2, where n is counted only considering PUSCH transmission occasions or actual repetitions associated with the same SRS resource set as the first transmission occasion or actual repetition. The redundancy version for PUSCH transmission occasions or actual repetitions that are associated with an SRS resource set other than the SRS resource set of the first transmission occasion or actual repetition is derived according to Table 6.1.2.1-3, where additional shifting operation for each redundancy version rvs is configured by higher layer parameter sequenceOffsetforRV in PUSCH-Config and n is counted only considering PUSCH transmission occasions or actual repetitions that are not associated with the SRS resource set of the first transmission occasion or actual repetition.

Table 6.1.2.1-3: Applied redundancy version for the other SRS resource set (SRS resource set not associated with the first transmission occasion or actual repetition) when sequenceOffsetforRV is present

| rvid indicated by the DCI scheduling the PUSCH | rvid to be applied to nth transmission occasion (repetition Type A) or nth actual repetition (repetition Type B) |  |  |  |
| --- | --- | --- | --- | --- |
|  | n mod 4 = 0 | n mod 4 = 1 | n mod 4 = 2 | n mod 4 = 3 |
| 0 | (0+ rvs) mod 4 | (2+ rvs) mod 4 | (3+ rvs) mod 4 | (1+ rvs) mod 4 |
| 2 | (2+ rvs) mod 4 | (3+ rvs) mod 4 | (1+ rvs) mod 4 | (0+ rvs) mod 4 |
| 3 | (3+ rvs) mod 4 | (1+ rvs) mod 4 | (0+ rvs) mod 4 | (2+ rvs) mod 4 |
| 1 | (1+ rvs) mod 4 | (0+ rvs) mod 4 | (2+ rvs) mod 4 | (3+ rvs) mod 4 |

For PUSCH repetition Type A, when higher layer parameter multipanelSchemeSDM or multipanelSchemeSFN is not provided and a DCI format 0_1 and DCI format 0_2 indicate codepoint "10" or "11" for the SRS resource set indicator and schedule aperiodic CSI report(s) on PUSCH with transport block by a 'CSI request' field on a DCI, the CSI report(s) multiplexing is determined as follows

-	if higher layer parameter ap-CSI-MultiplexingMode in CSI-AperiodicTriggerState is enabled and UCI other than CSI report(s) are not multiplexed on PUSCH, the CSI report(s) is transmitted separately only on the first transmission occasion associated with the first SRS resource set and the first transmission occasion associated with the second SRS resource set.

-	otherwise, the CSI report(s) is transmitted only on the first transmission occasion.

For PUSCH transmissions of TB processing over multiple slots, when a DCI format 0_1, 0_2 or 0_3 schedule aperiodic CSI report(s) on PUSCH with transport block by a 'CSI request' field on a DCI, the CSI report(s) is transmitted only on the first slot of the 𝑁 ∙ 𝐾 slots determined for the PUSCH transmission.

For PUSCH repetition Type B, when higher layer parameter multipanelSchemeSDM or multipanelSchemeSFN is not provided and a DCI format 0_1 and DCI format 0_2 indicate codepoint "10" or "11" for the SRS resource set indicator and schedule aperiodic CSI report(s) on PUSCH with transport block by a 'CSI request' field on a DCI, CSI report(s) multiplexing is determined as follows

-	if higher layer parameter ap-CSI-MultiplexingMode in CSI-AperiodicTriggerState is enabled and the first actual repetition associated with the first SRS resource set and the first actual repetition associated with the second SRS resource set have the same number of symbols and UCI other than CSI report(s) are not multiplexed on PUSCH, the CSI report(s) is multiplexed separately only on the first actual repetition associated with the first SRS resource set and first actual repetition associated with the second SRS resource set.

-	otherwise, the CSI report(s) is multiplexed only on the first actual repetition.

The UE does not expect a different number of actual PT-RS ports for the two actual repetitions when the CSI report(s) is transmitted separately on two actual repetitions.

For PUSCH repetition Type A, when higher layer parameter multipanelSchemeSDM or multipanelSchemeSFN is not provided and a DCI format 0_1 and DCI format 0_2 indicate codepoint "10" or "11" for the SRS resource set indicator and schedule aperiodic CSI report(s) on PUSCH with no transport block by a 'CSI request' field on a DCI, the number of repetitions is assumed to be 2 regardless of the value of numberOfRepetitions or pusch-AggregationFactor (if numberOfRepetitions is not present in the time domain resource allocation table), and transmission of CSI report(s) is determined as follows

-	if higher layer parameter ap-CSI-MultiplexingMode in CSI-AperiodicTriggerState is enabled and UCI other than CSI report(s) are not multiplexed on PUSCH, the CSI report(s) is transmitted separately on the first transmission occasion and the second transmission occasion

-	otherwise, the CSI report(s) is transmitted only on the first transmission occasion.

For PUSCH repetition Type B, when higher layer parameter multipanelSchemeSDM or multipanelSchemeSFN is not provided and a DCI format 0_1 and DCI format 0_2 indicate codepoint "10" or "11" for the SRS resource set indicator and schedule aperiodic CSI report(s) or activates semi-persistent CSI report(s) on PUSCH with no transport block by a 'CSI request' field on a DCI, the number of nominal repetitions is always assumed to be 2 regardless of the value of numberOfRepetitions, and the first and second nominal repetitions are expected to be the same as the first and second actual repetitions, and transmission of CSI report(s) is determined as follows:

-	if higher layer parameter ap-CSI-MultiplexingMode in CSI-AperiodicTriggerState is enabled for aperiodic CSI report(s) or higher layer paremeter SP-CSI-MultiplexingMode in CSI-SemiPersistentOnPUSCH-TriggerState is enabled for semi-persistent CSI report(s) and UCI other than CSI report(s) are not multiplexed on PUSCH, the CSI report(s) is transmitted separately on the first actual repetition and the second actual repetition

-	otherwise, the CSI report(s) is transmitted only on the first actual repetition.

The UE does not expect a different number of actual PT-RS ports for the two actual repetitions when the CSI report(s) is transmitted separately on two actual repetitions.

For PUSCH repetition Type A, when higher layer parameter multipanelSchemeSDM or multipanelSchemeSFN is not provided and a DCI format 0_1 and DCI format 0_2 indicate codepoint "10" or "11" for the SRS resource set indicator and activate semi-persistent CSI report(s) on PUSCH with no transport block by a 'CSI request' field on a DCI, or indicate the PUSCH repetition Type A carrying semi-persistent CSI report(s) without a corresponding PDCCH after being activated on PUSCH by a 'CSI request' field on a DCI, the number of repetitions is always assumed to be 2 regardless of the value of numberOfRepetitions or pusch-AggregationFactor (if numberOfRepetitions is not present in the time domain resource allocation table), and transmission of CSI report(s) is determined as follows

-	if higher layer parameter SP-CSI-MultiplexingMode in CSI-SemiPersistentOnPUSCH-TriggerState is enabled and UCI other than CSI report(s) are not multiplexed on PUSCH, the CSI report(s) is transmitted separately on the first transmission occasion and the second transmission occasion

-	otherwise, the CSI report(s) is transmitted only on the first transmission occasion.

For PUSCH repetition Type B, when higher layer parameter multipanelSchemeSDM or multipanelSchemeSFN is not provided and a DCI format 0_1 and DCI format 0_2 indicate codepoint "10" or "11" for the SRS resource set indicator and the PUSCH repetition Type B carrying semi-persistent CSI report(s) without a corresponding PDCCH after being activated on PUSCH by a 'CSI request' field on a DCI, the number of nominal repetitions is always assumed to be 2 regardless of the value of numberOfRepetitions, and transmission of CSI report(s) is determined as follows

-	if higher layer parameter SP-CSI-MultiplexingMode in CSI-SemiPersistentOnPUSCH-TriggerState is enabled and one of the first or second nominal repetition is the same as corresponding first or second actual repetition, the nominal repetition that is not having same actual repetition is omitted and the CSI report(s) is transmitted on the actual repetition that is not omitted.

-	if higher layer parameter SP-CSI-MultiplexingMode in CSI-SemiPersistentOnPUSCH-TriggerState is enabled and the first and second nominal repetitions are the same as the first and second actual repetitions and the UCI other than CSI report(s) are not multiplexed on PUSCH, the CSI report(s) is transmitted separately on the first actual repetition and the second actual repetition

-	otherwise, the CSI report(s) is transmitted only on the first actual repetition.

##### 6.1.2.1.1	Determination of the resource allocation table to be used for PUSCH

Table 6.1.2.1.1-1, Table 6.1.2.1.1-1A, Table 6.1.2.1.1-1B and Table 6.1.2.1.1-1C define which PUSCH time domain resource allocation configuration to apply.

Table 6.1.2.1.1-4 defines the subcarrier spacing specific values j. j is used in determination of K2 in conjunction to table 6.1.2.1.1-2, for normal CP or table 6.1.2.1.1.-3 for extended CP, where µPUSCH is the subcarrier spacing configurations for PUSCH.

Table 6.1.2.1.1-5 defines the additional subcarrier spacing specific slot delay value for the first transmission of PUSCH scheduled by the RAR or by the fallbackRAR. When the UE transmits a PUSCH scheduled by RAR or by the fallbackRAR, the Δ value specific to the PUSCH subcarrier spacing µPUSCH is applied in addition to the K2 value.

Table 6.1.2.1.1-1: Applicable PUSCH time domain resource allocation for common search space and DCI format 0_0 in UE specific search space

| RNTI | PDCCH search space | pusch-ConfigCommon includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationList | PUSCH time domain resource allocation to apply |
| --- | --- | --- | --- | --- |
| PUSCH scheduled by MAC RAR as described in clause 8.2 of [6, TS 38.213] or MAC fallbackRAR as described in clause 8.2A of [6, 38.213] or for MsgA PUSCH transmission |  | No | - | Default A |
|  |  | Yes |  | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
| C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI | Any common search space associated with CORESET 0 | No | - | Default A |
|  |  | Yes |  | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
| C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI | Any common search space not associated with CORESET 0, DCI format 0_0 in UE specific search space | No | No | Default A |
|  |  | Yes | No | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
|  |  | No/Yes | Yes | pusch-TimeDomainAllocationList provided in pusch-Config |

Table 6.1.2.1.1-1A: Applicable PUSCH time domain resource allocation for DCI format 0_1 in UE specific search space scrambled with C-RNTI, MCS-C-RNTI, CS-RNTI or SP-CSI-RNTI

| pusch-ConfigCommon includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationListDCI-0-1 | pusch-Config includes pusch-TimeDomainAllocationListForMultiPUSCH | PUSCH time domain resource allocation to apply |
| --- | --- | --- | --- | --- |
| No | No | No | No | Default A |
| Yes | No | No | No | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
| No/Yes | Yes | No | No | pusch-TimeDomainAllocationList provided in pusch-Config |
| No/Yes | No | Yes | - | pusch-TimeDomainAllocationListDCI-0-1 provided in pusch-Config |
| No/Yes | No | - | Yes | pusch-TimeDomainAllocationListForMultiPUSCH provided in pusch-Config |

Table 6.1.2.1.1-1B: Applicable PUSCH time domain resource allocation for DCI format 0_2 in UE specific search space scrambled with C-RNTI, MCS-C-RNTI, CS-RNTI or SP-CSI-RNTI

| pusch-ConfigCommon includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationListDCI-0-2 | PUSCH time domain resource allocation to apply |
| --- | --- | --- | --- |
| No | No | No | Default A |
| Yes | No | No | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
| No/Yes | Yes | No | pusch-TimeDomainAllocationList provided in pusch-Config |
| No/Yes | No | Yes | pusch-TimeDomainAllocationListDCI-0-2 provided in pusch-Config |

Table 6.1.2.1.1-1C: Applicable PUSCH time domain resource allocation for DCI format 0_3 in UE specific search space scrambled with C-RNTI or MCS-C-RNTI

| pusch-ConfigCommon includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationListDCI-0-1 | pusch-Config includes pusch-TimeDomainAllocationListForMultiPUSCH-DCI-0-3 | PUSCH time domain resource allocation to apply |
| --- | --- | --- | --- | --- |
| No | No | No | No | Default A |
| Yes | No | No | No | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
| No/Yes | Yes | No | No | pusch-TimeDomainAllocationList provided in pusch-Config |
| No/Yes | No | Yes | No | pusch-TimeDomainAllocationListDCI-0-1 provided in pusch-Config |
| No/Yes | No/Yes | No/Yes | Yes | pusch-TimeDomainAllocationListForMultiPUSCH-DCI-0-3 provided in pusch-Config |

<omitted text>

### 6.1.3	UE procedure for applying transform precoding on PUSCH

For a PUSCH scheduled by RAR UL grant, or for a PUSCH scheduled by fallbackRAR UL grant, or for a PUSCH scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, the UE shall consider the transform precoding either 'enabled' or 'disabled' according to the higher layer configured parameter msg3-transformPrecoder.

For a MsgA PUSCH, the UE shall consider the transform precoding either 'enabled' or 'disabled' according to the higher layer configured parameter msgA-TransformPrecoder. If higher layer parameter msgA-TransformPrecoder is not configured, the UE shall consider the transform precoding either 'enabled' or 'disabled' according to the higher layer configured parameter msg3-transformPrecoder.

For PUSCH transmission scheduled by a PDCCH with CRC scrambled by CS-RNTI with NDI=1, C-RNTI, or MCS-C-RNTI or SP-CSI-RNTI:

-	If the DCI with the scheduling grant was received with DCI format 0_0, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder from rach-ConfigCommon included directly within BWP configuration.

-	If the DCI with the scheduling grant was not received with DCI format 0_0

-	If the DCI with the scheduling grant was received with DCI format 0_1 or 0_2 with CRC scrambled by C-RNTI, MCS-RNTI, or CS-RNTI with NDI=1 and if the UE is configured with a higher layer parameter dynamicTransformPrecoderFieldPresenceDCI-0-1 in pusch-Config for DCI format 0_1 or dynamicTransformPrecoderFieldPresenceDCI-0-2 in pusch-Config for DCI format 0_2 and the higher layer parameter is set to 'enabled',

-	the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the Transform precoder indicator field in the DCI with the scheduling grant.

-	For pusch-TimeDomainAllocationListForMultiPUSCH in pusch-Config, the UE shall, for all PUSCH transmissions, consider the transform precoding either enabled or disabled according to Transform precoder indicator field in the DCI format 0_1 with the scheduling grant.

-	If resourceAllocation in pusch-Config for DCI format 0_1 or resourceAllocationDCI-0-2 in pusch-Config for DCI format 0_2 is set to resourceAllocationType0, or if the resource allocation is set to resource allocation type 0 according to the DCI configuration as described in clauses 7.3.1.1.2 and 7.3.1.1.3 of [6, TS 38.212], or if dmrs-Type in DMRS-UplinkConfig is set to ‘type 2’ for this PUSCH transmission, the UE does not expect that the Transform precoder indicator field in the DCI with the scheduling grant indicates that transform precoding is enabled.

-	If the UE is configured with the higher layer parameter dmrs-TypeEnh in DMRS-UplinkConfig, and if the scheduling grant indicates that transform precoding is enabled for the scheduled PUSCH transmission, the UE ignores the higher layer parameters dmrs-TypeEnh in DMRS-UplinkConfig, if configured, for the DM-RS transmission of the scheduled PUSCH transmission.

-	Otherwise,

-	If the UE is configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to this parameter.

-	If the UE is not configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder.

For PUSCH transmission with a configured grant

-	If the UE is configured with the higher layer parameter transformPrecoder in configuredGrantConfig, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to this parameter.

-	If the UE is not configured with the higher layer parameter transformPrecoder in configuredGrantConfig, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder.

<omitted text>

### 6.3.1	Frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots

For PUSCH repetition Type A other than the PUSCH scheduled by RAR UL grant or fallbackRAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH), a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingDCI-0-2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in configuredGrantConfig for configured PUSCH transmission. For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, a UE is configured for frequency hopping by the frequency hopping flag information field of the RAR UL grant, and by the frequency hopping flag information field of DCI format 0_0 with CRC scrambled by TC-RNTI, respectively. One of two frequency hopping modes can be configured:

-	Intra-slot frequency hopping, applicable to single slot and multi-slot configured PUSCH transmission, multi-slot PUSCH transmission scheduled by DCI format 0_1, 0_2 or 0_3, each of multiple PUSCH transmissions on a serving cell scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH or pusch-TimeDomainAllocationListForMultiPUSCH-DCI-0-3  is configured and each of multiple configured grant PUSCH transmissions in a configuration where the higher layer parameters cg-nrofSlots and cg-nrofPUSCH-InSlot are provided.

-	Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission.

For operation with shared spectrum channel access in FR1, the UE does not expect that two hops of a PUSCH transmission are in different RB sets.

In case of resource allocation type 2, the UE transmits PUSCH without frequency hopping.

In case of resource allocation type 1, whether or not transform precoding is enabled for PUSCH transmission, the UE may perform PUSCH frequency hopping, if the frequency hopping field in a corresponding detected DCI format or in a random access response UL grant is set to 1, or if for a Type 1 PUSCH transmission with a configured grant the higher layer parameter frequencyHoppingOffset is provided, otherwise no PUSCH frequency hopping is performed. When frequency hopping is enabled for PUSCH, the RE mapping is defined in clause 6.3.1.6 of [4, TS 38.211].

For a PUSCH scheduled by RAR UL grant, fallbackRAR UL grant, or by DCI format 0_0 with CRC scrambled by TC-RNTI, frequency offsets are obtained as described in clause 8.3 of [6, TS 38.213]. Otherwise, for a PUSCH scheduled by DCI format 0_0/0_1/0_3 or a PUSCH based on a Type2 configured UL grant activated by DCI format 0_0/0_1 and for resource allocation type 1, frequency offsets are configured by higher layer parameter frequencyHoppingOffsetLists in pusch-Config. For a PUSCH scheduled by DCI format 0_2 or a PUSCH based on a Type2 configured UL grant activated by DCI format 0_2 and for resource allocation type 1, frequency offsets are configured by higher layer parameter frequencyHoppingOffsetListsDCI-0-2 in pusch-Config.

-	When the size of the active BWP is less than 50 PRBs, one of two higher layer configured offsets is indicated in the UL grant.

-	When the size of the active BWP is equal to or greater than 50 PRBs, one of four higher layer configured offsets is indicated in the UL grant.

For PUSCH based on a Type1 configured UL grant the frequency offset is provided by the higher layer parameter frequencyHoppingOffset in rrc-ConfiguredUplinkGrant.

For a MsgA PUSCH the frequency offset is provided by the higher layer parameter as described in [6, TS 38.213].

In case of intra-slot frequency hopping, the starting RB in each hop is given by:

,

where i=0 and i=1 are the first hop and the second hop respectively, and  is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) or as calculated from the resource assignment for MsgA PUSCH (described in [6, TS 38.213]) and is the frequency offset in RBs between the two frequency hops. The number of symbols in the first hop is given by , the number of symbols in the second hop is given by , where NsymbPUSCH,s is the length of the PUSCH transmission in OFDM symbols in one slot.

In case of inter-slot frequency hopping and when pusch-DMRS-Bundling is not enabled, or for inter-slot frequency hopping for a PUSCH scheduled by RAR UL grant or DCI format 0_0 with CRC scrambled by TC-RNTI, the starting RB during slot  is given by:

,

where  is the current slot number within a system radio frame, where a multi-slot PUSCH transmission can take place,  is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) and is the frequency offset in RBs between the two frequency hops.

In case of inter-slot frequency hopping and when pusch-DMRS-Bundling is enabled, and when a PUSCH is not scheduled by RAR UL grant or DCI format 0_0 with CRC scrambled by TC-RNTI, the starting RB during slot  is given by:

RBstartnsμ=RBstartRBstart+RBoffsetmodNBWPsizensμNFH mod 2=0nsμNFH mod 2=1

where nsμ is the current slot number within a system radio frame, NFH is the value of the higher layer parameter pusch-FrequencyHoppingInterval,  is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) andis the frequency offset in RBs between the two frequency hops.

<omitted text>